3GPP TSG RAN Meeting #106	RP-242493

Madrid, Spain, December 9th – 12th, 2024

Agenda Item:	15.2

Source:	InterDigital, Inc.

Title:	Scope for Rel-20 NTN for 5G-A

Document for:	Discussion, Decision

# Introduction

Non-terrestrial networks (NTN) facilitate deployment of wireless networks in areas where land-based antennas are impractical, for example due to geography or cost. It is envisioned that, coupled with terrestrial networks, NTN will enable truly ubiquitous coverage of 5G networks.

3GPP work on NR NTN began in Rel-15 with an initial feasibility study, expanding into a general study item in Rel-16 and a work item in Rel-17. IoT NTN started in Rel-17 with a brief study item phase leveraging existing work from NR and focusing on identifying LTE-specific issues. It was subsequently converted to a work item later in the release.

Initial Rel-17 work items introduced basic NTN functionality to NR and LTE, focusing on the transparent satellite payload scenario and the necessary adaptations to work in a non-terrestrial environment. Key enhancements include introduction of time/frequency pre-compensation procedures, modifications to timing relationships, mobility enhancements (e.g., time/location-based CHO), miscellaneous enhancements to HARQ, DRX, LCP, Tracking Area management, and IoT-specific support for discontinuous coverage.

Subsequent work in Rel-18 refined the initial NTN release by improving service continuity between terrestrial and non-terrestrial networks, enhancing UL coverage to better support commercial smartphones, support for >10GHz deployments, and finalizing the leftover aspects of Rel-17 IoT NTN.

Finally, while still ongoing, work in Rel-19 includes DL coverage enhancements (e.g., to address the satellite beam power sharing scenario), improved UL capacity and throughput, enhanced support for MBS, regenerative payload, and Redcap devices.

Release 20 will improve reachability, facilitate deployment and integration of NTN into existing networks, and reduce reliance on GNSS.

# Proposed scope for Rel-20

Many candidate topics have been proposed for non-terrestrial networks (NTN) in previous discussion. It is suggested that a focused scope be agreed as a baseline for specification based on requirements of initial deployments of NTN. This including the following topics: Robust notification/alert; Enhanced mobility and service continuity; and GNSS-less NTN access.

- Proposal 1:	Rel-20 NR NTN objectives include at least: 1) Robust notification/alert; 2) Enhanced mobility and service continuity; and 3) GNSS-less NTN access.

## Notification/Alert channel for NLOS UEs

DL coverage enhancements work from Rel-19 primarily targets LOS UEs experiencing poor DL coverage due to EIRP density reduction. However, according to TR 38.811 other issues affecting NLOS UEs (approximately 10% of rural UEs) such as clutter loss results in coverage drops of 18dB. This reduction severely impacts the ability for such UEs to receive DL signalling such as paging.

Observation 1:	According to TR 38.811, clutter loss for NLOS UEs can be over 18dB, which impacts approximately 10% of rural UEs and 50% in urban scenarios.

In NTN, user actions (e.g., taking a phone outside of a briefcase or moving outdoors) can greatly improve the coverage conditions of a UE, which is known as “user cooperation”. To prompt such cooperation, a simple notification/alert can be sent to the UE through a heavily coverage enhanced channel, hopefully resulting in channel conditions being improved sufficiently through user actions to receive paging via traditional paging channels. To support the notification/alert channel, it is preferred that the existing paging channel be used as a baseline and enhanced, considering definition of a new channel may require large specification effort.

- Proposal 2:	NTN notification/alert channel prioritizes enhancing the existing paging channel rather than defining a new channel [RAN1].

The notification/alert channel should be considered as a fallback when existing paging is insufficient. Impacts to the existing paging procedure should be limited and focus on when the UE should fall back to the notification channel, or when to renew monitoring the legacy paging channel once radio conditions have sufficiently improved.

- Proposal 3:	NTN notification/alert channel is considered as a fallback when existing paging procedure fails.

Previous discussion has indicated that the notification/alert message should be dedicated. However, a dedicated message may have challenges due to the size of NTN cells and number of UE’s which it will support. In case a group-based notification/alert message is supported, false alarms should be minimized.

- Proposal 4:	If a group-based notification/alert message is supported, address the impact of false alarm notifications to unintended UEs.

## Enhanced mobility and service continuity

Release 18 has specified enhanced integration for TN-NTN, specifically for cell reselection. However, considering the different deployments available under specification (e.g., LEO up to GEO), there is a broad difference in terms of coverage and latency characteristics. So far prioritization of one network type over another has not been addressed, however a UE under multi-network coverage would be able to leverage the characteristics based on the given scenario.

Observation 2:	UEs under overlapping coverage from different network types (TN/NTN or NTN/NTN with different orbits) can leverage latency and coverage characteristics to support different use cases.

One example of this would be paging interaction between TN and NTN. NTN offers many advantages from a coverage perspective since a single cell can cover a very wide area. This would be very useful for paging, where things like tracking area update would be much less frequent. However, once reached the NTN cell may not be suitable for the QoS of the connection, so the paging message can direct the UE to access one network type over another.

- Proposal 5:	Improve paging integration between NTN-TN (e.g., a UE paged on one network type can be directed to another network type) [RAN2].

Similarly, if the UE is initiating the connection (e.g., based on data arrival from the application layer), a UE under overlapping coverage of multiple network types can select the most suitable network type to initiate the connection even though it may be originally camped on a different network type.

- Proposal 6:	Support QoS-based network selection (e.g., between TN/NTN, or NTN/NTN with different orbits) based on access cause [RAN2].

- An alternative to QoS-based NW selection (while still leveraging the benefits of different characteristics of TN/NTN and NTN with different orbits) includes multi-connectivity between TN and NTN, as well as between NTN of different orbits. An added advantage would be enhanced reliability due to redundant NTN connections.

- Proposal 7:	Support NTN-based dual connectivity (e.g., between TN/NTN, or NTN/NTN with different orbits) [RAN2].

## GNSS-less NTN access

Accounting for increased propagation delay in cell access and various timing relationships was a key issue in the initial releases of NTN. Methods to resolve this include increased value ranges, offsets, and timing pre-compensation. Performing timing pre-compensation has so far relied on GNSS based location information, which has several drawbacks including lack of availability (e.g., due to poor atmospheric conditions or visibility), constant updating, additional power consumption, and lower throughput due to additional measurement gaps for lower capability devices.

Rel-20 may consider reduced reliance on GNSS, for example, during the Random Access procedure which relies on the GNSS-based timing pre-compensation. Examples of possible solutions include distance-based RACH partitioning and/or timer/window extension.

- Proposal 8:	Enhance the NTN Random Access procedure to support access without GNSS [RAN2].

# Conclusion

In this contribution, R20 scope for non-terrestrial networks (NTN) is discussed. The following observations and proposals are provided:

Observation 1:	According to TR 38.811, clutter loss for NLOS UEs can be over 18dB, which impacts approximately 10% of rural UEs and 50% in urban scenarios.

Observation 2:	UEs under overlapping coverage from different network types (TN/NTN or NTN/NTN with different orbits) can leverage latency and coverage characteristics to support different use cases.

- Proposal 1:	Rel-20 NR NTN objectives include at least: 1) Robust notification/alert; 2) Enhanced mobility and service continuity; and 3) GNSS-less NTN access.

- Proposal 2:	NTN notification/alert channel prioritizes enhancing the existing paging channel rather than defining a new channel [RAN1].

- Proposal 3:	NTN notification/alert channel is considered as a fallback when existing paging procedure fails.

- Proposal 4:	If a group-based notification/alert message is supported, address the impact of false alarm notifications to unintended UEs.

- Proposal 5:	Improve paging integration between NTN-TN (e.g., a UE paged on one network type can be directed to another network type) [RAN2].

- Proposal 6:	Support QoS-based network selection (e.g., between TN/NTN, or NTN/NTN with different orbits) based on access cause [RAN2].

- Proposal 7:	Support NTN-based dual connectivity (e.g., between TN/NTN, or NTN/NTN with different orbits) [RAN2].

- Proposal 8:	Enhance the NTN Random Access procedure to support access without GNSS [RAN2].

# References

RP-241789 – NR NTN (Non-Terrestrial Networks) enhancements – Thales (WI Rapporteur)