3GPP TSG RAN Meeting #106										RP-242522

Madrid, Spain, December 9-13, 2024

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242394 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 70% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#118bis, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2,

- The differential L1-RSRP is quantized to a 4-bit value with 2 dB step size

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A

- Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2

  - FFS: DCI format 0_3

- FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, resource mapping/configuration between first and second UL channel in Mode-B, at least Option-1 is supported

- Option-1 (one-to-one): Only one periodic PUCCH resource for the first channel and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Down-select one of the following in RAN1#118bis

    - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

    - Option-1B: No restriction in terms of periodicity.

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for both Mode-A and Mode-B, the first PUCCH and the second PUSCH can be from the same or different CC(s)

- FFS: whether the first PUCCH and the second PUSCH should be from the same PUCCH group

- The first PUCCH and the second PUSCH should be in the same CG

  - FFS: Different CGs

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

- Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI.

FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

Working Assumption

The following working assumption in RAN1#118 is revised in red.

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th Q-th best quality.

  - M Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B,

- The UE expects that there is the same periodicity (in ms) between first PUCCH resource and pre-configured resource for second UL channel.

  - FFS: Whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms)

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, there is no RAN1 consensus on the following enhancement for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW.

When the Scheme-1 is used, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, confirm the following working assumption

- Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, the following working assumption in RAN1#117 is confirmed with modification:

- (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

  - Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

Agreement

Regarding RS measurement for the current beam for Event 2, for enabling one of either Scheme-1 or Scheme-2 by NW in Option-2a, the following implicit manner is supported:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

Agreement

Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

Agreement

Regarding the triggering event determination for Event 2, down-select among the following alternatives for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured]

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s)

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS;

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS;

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s):

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}:

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s):

Note: There is the same periodicity for the new beam RS(s).

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- For new beam measurement, in a CSI report configuration, configure legacy RRC parameter carrier that indicates the CC that the RS resource set associated with the CSI reporting configuration can be found

- FFS: Whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement.

- FFS: Whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement.

Conclusion

There is no RAN1 consensus on the following proposal:

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ can further comprise {5, 6, 7, 8}, besides for previously agreed candidate value of {1, 2, 3, 4}.

CSI

Agreement

For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource):

- SRS port group 0, corresponding to CW0, comprises the even PSRS/2 out of PSRS ports; and

- SRS port group 1, corresponding to CW1, comprises the odd PSRS/2 out of PSRS ports

The above feature is applicable only for reportQuantity = ‘cri-RI-CQI’

No other spec enhancement is introduced on new CW-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

Note: The above grouping assumption is to align NW and UE on the association between SRS ports and reported CQIs for the two CWs when reportQuantity = ‘cri-RI-CQI’.

Note: different SRS ports are associated with different UE antenna ports.

Note: if one single CW is scheduled, both SRS port groups can correspond to the same CW, i.e. no enhancement is needed for the single-CW case

Note: This feature is a separate UE capability and, for UEs supporting this capability, configured via RRC (FFS details on the extend of RRC configuration)

Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

FFS (by RAN1#118bis): Whether there is impact on mapping between CWs to CSI-RS ports

For SRS antenna switching with multiple aperiodic SRS resource sets, PSRS ports indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, active resource counting is:

- For Capability 1 timeline: 1

- For Capability 2 timeline: 1

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the support for the 3-bit scaling factor(s) for RI=v >1, support only for RI=v=2 without per-SD-basis-vector/layer power adjustment/boosting

- FFS: Details on per-layer scaling factor applied to each of the selected SD basis vectors, extending the agreed scaling factor for RI= v =1 si∈1, 12, 13 ,14,16,18,112,116 (in RAN1#117)

This feature is a separate UE capability from soft scaling for RI=v=1. Introduce new RRC parameter to enable the feature.

Note: This doesn’t preclude the use of power boosting at the NW side by implementation

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the inter-pol co-phase selection indicator row from the UCI parameter table for Scheme-B for RI=5-8 is amended as follows:

Scheme-B

| Parameter | UCI | Details/description |
| --- | --- | --- |
| … | … | … |
| Inter-pol co-phase selection indicator for each layer | Part 2 Wideband or Subband (**) | v=5-8: QPSK: 2-bit indicator per layer group l=1, …, LG v=5, 7: QPSK: - For a layer group with 2 layers: 1-bit indicator {(1, -1), (j, -j)} - For a layer group with 1 orphan layer: 2-bit indicator {1, -1, j, -j} v=6, 8: QPSK: - 1-bit indicator for each layer group {(1, -1), (j, -j)} |

Agreement

For the Rel-19 Type-I SP codebook refinement for P (the total number of aggregated ports)=48, 64, 128 CSI-RS ports, regarding timeline for the port subset indication for the SD NES Type-1,

- Capability 1 timeline: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2)

- Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources

Agreement

For the Rel-19 Type-I SP codebook refinement for P (the total number of aggregated ports)=48, 64, 128 CSI-RS ports, regarding active resource/port counting for the port subset indication for the SD NES Type-1,

- active resource counting is M, where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources

- active port counting is maxi=1MPi, P, where Pi is the number of CSI-RS ports in i-th sub-configuration derived from the corresponding antenna port subset indicator portSubsetIndicator

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, when a CSI-RS resource set for aggregating K NZP CSI-RS resources (to attain a total of 48, 64, and 128 ports) is configured as the associated NZP CSI-RS for each of the SRS resource set(s) with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’, support to change the starting position of the time gap between CSI-RS and SRS from the last symbol of the reception of the aperiodic NZP-CSI-RS resource to the last symbol of the reception of the aperiodic NZP-CSI-RS resource set.

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, except for that based on the Rel-18 Type-II Doppler, the following rule is supported:

- After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise.

Agreement

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler, the following rule is supported:

- After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or KP consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding priority 0 (G0)/wideband in CSI part 2, the UCI packing order is as follows:

- The G0/wideband for the 1st configured CMR among the non-reported MR CRIs;

- …

- The G0/wideband for the last configured CMR among the non-reported MR CRIs;

- The G0/wideband for the 1st reported CRI;

- …

- The G0/wideband for the (M- MR)-th reported CRI;

The entire G0/wideband is either reported or dropped entirely, following the legacy principle.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding one-part CSI wideband CQI/PMI reporting, the UCI packing order is as follows:

- The wideband CSI for the 1st reported CRI;

- …

- The wideband CSI for the M-th reported CRI;

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports,

the following rule is supported:

- After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port (corresponding to the ‘reference UE antenna port’) out of the y available SRS ports (from an xTyR SRS resource for antenna switching) can be configured per CSI reporting setting.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource,

- Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic”

- When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report

  - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option

  - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, confirm the following working assumptions as agreement with the following refinement:

- When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting

- When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance

FFS: Whether an additional UE procedure is needed when the reported DO value is ‘out of range’

FFS: Whether the Dd report codepoints need to be reinterpreted from intervals/ranges to values when the linkage mechanism is configured, or in general, a rule is needed to determine the value of DO for the compensation for each CMR for CJT CSI report (including if multiple Dos are reported)

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support at least AP-CSI-RS as the CMR for the Rel-18 eType-II CJT reporting

- FFS: The support for P/SP CSI-RS

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, support to include an indicator in the trigger for a Rel-18 eType-II CJT CSI, which indicates whether the UE should perform delay offset (DO) compensation based on the linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI or not.

- This feature is a separate UE capability

- No new DCI field is introduced. This does not preclude increasing the bit width for CSI request.

- FFS: Details on signalling design for the indicator including whether it is per CSI-RS resource/Dd value and the associated UE behaviour(s)

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (PO) and >1:

- The UCI parameters are captured in the tables below:

| Parameter | Details/description |
| --- | --- |
| nref | Reference CSI-RS resource index, based on the ordering from RRC configuration: logNTRP bits |
| {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} | DL/UL phase offsets for CSI-RS resource n, (n=0, 1, …, NTRP – 1, n≠nref): NTRP-1NSB-P logMΦbits |

- The UCI mapping order is as follows:

  - nref,

  - {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource ID

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding active resource counting and OCPU, when ReportQuantity is ‘cjtc-P’ (phase offset), for both =1 and >1, fully reuse those from Rel-18 TDCP reporting

- OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (delay offset), ‘cjtc-F’ (frequency offset), or, ‘cjtc-Dd-F’ (joint delay + frequency offset),

- after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each CSI-RS resource in the KTRS CSI-RS Resource Sets of the CSI-RS Resource Setting for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- after the CSI report (re)configuration, serving cell activation, BWP change, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement no later than the CSI reference resource within the same DRX active time, when DRX is configured, and drop the report otherwise.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1), the maximum value of configured NSB-P is 16 per CSI reporting setting

Conclusion

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, there is no consensus in supporting P/SP CSI-RS as the CMR for the Rel-18 eType-II CJT reporting (in addition to the agreed AP CSI-RS).

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v ∈{2} for the 3-bit scaling factor(s), decide, by RAN1#119, from the following alternatives:

- Alt1: min1υ,si2ri

- Alt2: min1υ,si2k∈Girk

- Alt3: si2

- Alt4: υsi2ri

- Alt5: minriυ,si2

- Alt6: si2υ

- Alt7:

  - if two different vectors for υ=2, min1υ,si2 for the vector with smaller scaling factor si≤sj, and min⁡(1-min⁡(1υ,si2),sj2) for the other vector configured with larger scaling factor sj;

  - otherwise (υ=1 or one same vector for υ=2),  min1,si2 for the vector

Where ri denotes the number of layers associated with the i-th SD basis vector.

The same scheme applies to both Mode-A and Mode-B.

Note: si∈1, 12, 13 ,14,16,18,112,116 as agreed in RAN1#117.

Normalization of precoder should be taken into account in the final specification design.

Conclusion

For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, when SRS port grouping is configured, regarding the mapping between CSI-RS ports and SRS port groups, no additional specification support is introduced

- It is up to gNB implementation to ensure adequate operation of this feature

Agreement

For the Rel-19 Type-I SP codebook refinement for P (the total number of aggregated ports)=48, 64, 128 CSI-RS ports, regarding CPU occupation for the port subset indication for the SD NES Type-1,

|  | Timeline capability 1 | Timeline capability 2 |
| --- | --- | --- |
| P-report (L subConfigs) | OCPU=L | OCPU=L |
| AP/SP-report (N triggered) | OCPU=N | OCPU=N |

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, introduce a 1-bit indicator per CSI trigger state:

- The 1-bit indicator is an RRC parameter

- This is done without increasing the bit-width of the CSI request field of the DCI triggering a Rel-18 CJT eType-II CJT CSI report

- FFS (RAN1#119): Whether the 1-bit indicator applies to all the NTRP CSI-RS resources, or to 1-bit indicator per CSI-RS resource

- FFS (RAN1#119): How this applies to a single CSI trigger state associated with >1 CSI reports

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger:

- Reuse the CPU occupation and active resource counting for the Rel-18 eType-II CJT

- FFS (RAN1#119): Re timeline, decide whether to reuse or further relax the timeline for the Rel-18 eType-II CJT

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support linking the CMRs in the two CSI Report Settings so that UE knows which CMRs in the two report settings correspond to the same TRP.

- Based on a fixed correspondence between the set of TRS resource set IDs in sequential order and the set of CSI-RS resource IDs in sequential order of configuration in their respective Resource Setting

FFS: linking, when the number of resources configured for CJT CSI is < number of resource sets configured for CJT Dd, in case of separate triggers

Agreement

For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, change the maxNumberTxPortsPerResource to maxNumberTxPortsPerReport for Rel-19 codebook triplet capability

- Note: Since ARC=1 was agreed, the K aggregated resources are perceived as 1 resource for ARC, and maxNumberTxPortsPerResource cannot be larger than 32.

3Tx uplink

Agreement

To support 3T6R antenna switching for a 3TX UE,

- The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’,

- Reuse the principles used for 4T8R, i.e.,

  - For 4T8R 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources, transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or

  - For 4T8R 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports.

Note: This is not meant to be a text proposal to be captured for the specifications.

Agreement

To support non-codebook-based PUSCH transmission by a 3TX UE,

- UE reports a capability of up to a maximum of 3 layers.

Conclusion

To support non-codebook-based precoding by a 3TX UE, for SRI indication, re-use the legacy-based solution according to NSRS ≤ maxNumberSRS-ResourcePerSet.

Conclusion

To support non-codebook-based PUSCH transmission by a 3TX UE,

- For sTRP, a single SRS resource set, with up to NSRS ≤ maxNumberSRS-ResourcePerSet single-port SRS resources, can be configured,

- For Rel-17 M-TRP PUSCH repetition, two SRS resource sets, each with up to NSRS ≤ maxNumberSRS-ResourcePerSet single-port SRS resources, can be configured.

Asymmetric DL-sTRP/UL-mTRP

Agreement

The lower limit of the value range of PL offset is extended to -12 dB

Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Agreement

For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0:

- This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset.

- When one joint/UL TCI state is indicated in Rel-17 unified TCI,

  - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power.

- FFS: Whether the bit field can be used to indicate other information

- FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

Agreement

For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission:

- Alt2:

  - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation.

  - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed.

FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

Agreement

About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1:

- Alt1: X = 45 (to be captured in RAN2 spec)

  - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not.

  - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

Agreement

Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH:

- (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

  - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

- (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

  - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

In RAN1#119, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported:

- Option-2: Type-1 CG PUSCH can NOT be transmitted

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group.

- (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups

  - Subject to separate UE capability

Conclusion

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the case that the first PUCCH and the second PUSCH are from the different CG is NOT supported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, down-select one of Alt-1 and Alt-2 for both mode-A and mode-B.

- Alt-1 (dedicated SR): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR/reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

  - FFS: whether/how to enhance legacy SR to support notification of beam report transmission in Mode-B

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission for mode A and mode B will be separately discussed.

Agreement

On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation:

- Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality.

- Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality.

- Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs

Note: For Event-2, we have the following definition for scheme-1 and scheme-2

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

- Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Agreement

On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement

- Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

Agreement

Confirm the following working assumption in RAN1#117:

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured

- Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured.

- Option-3: No further enhancement.

Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2

FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120

- Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

- Option-2: A CSI trigger state can be associated with

  - only UE-initiated/event-driven beam reporting

  - or only legacy AP-CSI configuration

  - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases:

- Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR

- Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH

- Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

  - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

    - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

    - update TCI state(s) with the reported new beam(s)

    - activate new beam(s) without additional SSB reception

  - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

    - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

    - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

  - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

  - Other alternatives are not precluded

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC).

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s).

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS.

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS.

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s).

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}.

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s).

Note: There is the same periodicity for the new beam RS(s).

FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical.

Send an LS to RAN4. Final LS in R1-2410914.

CSI

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, the 1-bit indicator per CSI trigger state applies to all the NTRP configured CSI-RS resources.

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax

- The value of Drelax is a UE capability, taken from {0, drelax}

  - FFS: The value of drelax (>0), including whether it depends on SCS

- For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if the UCI associated with each of the M CRIs comprises two parts and is multiplexed on PUCCH, the UE determines the PUCCH resource, the number of PRBs for the PUCCH resource, assuming that the CSI associated with each of the M reported CRIs corresponds to rank 1

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, regarding linking the CMRs in the two CSI Report Settings, for separate triggering, the UE expects that the number of configured CSI-RS resources associated with the Rel-18 eType-II CJT CSI is always the same as the number of TRS resource sets associated with the Rel-19 CJTC Dd report

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI reported in CSI part-1, if resource-specific RI restriction is configured, zero padding bits are introduced in CSI part 1 for each of the (M-MR) reported CRIs:

- For a k-th CRI from the (M-MR) reported CRIs (where k=1, 2, …, (M-MR)), OP,k zero padding bits are inserted where:

  - OP,k=Nmax-Nreported,k

    - Nmax=maxi∈S(Ni), where Ni is the size of RI field corresponding to the i-th CRI, and S is the set of CRIs corresponding to the (Ks-MR) resources

    - Nreported,k is the size of RI field corresponding to the k-th CRI

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, when a single CSI trigger state associated with >1 Rel-18 Type-II CJT CSI reports, the 1-bit indicator per CSI trigger state applies to all the Rel-18 Type-II CJT CSI reports linked with the CJTC Dd report(s).

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s):

- The scaling formula is ρ.si where ρ is a multiplicative factor independent of i

  - Reuse legacy precoder normalization (per discretion of the spec editor)

  - FFS (RAN1#119): Whether min(ρ.si , 1) operation is needed

  - FFS (RAN1#119): Whether ρ other than 1 (baseline) is needed (e.g. v or 1v)

- Regarding the configuration of the si value (3-bit indicator per SD basis vector group), decide, by RAN1#119, between the following:

  - Alt1. RI=1 and RI=2 are separately configured (RI-specific)

  - Alt2. A same configuration is used for RI=1 and RI=2 (RI-common)

Conclusion

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order and priority levels of NRep CSI reports with M CRIs, the UCI packing order and the consecutive priority levels (higher to lower) follows the legacy specifications.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding wideband P/SP-CSI reported using one-part CSI, if resource-specific RI restriction is configured, the zero padding bits for each of the M reported CRI are determined as follows:

- For a k-th CRI from the M reported CRIs, OP,k=Nmax-Nreported,k, where:

  - Nmax= maxj∈Q  (Nmax,j) , where Q is the set of CRIs corresponding to Ks resources and Nmax,j is the maximum payload size of associated CSI fields for a j-th CRI, and Nmax,j=NRI(j)+maxrj∈Srank,jB(rj), where Srank,j is the set of rank values rj that are allowed to be reported for the j-th CRI;

  - Nreported,k= NRI(k)+B(Rk) , where Rk is the reported rank for k-th CRI;

  - Note: NRI(k) is the size of RI field corresponding to k-th CRI.

  - Note: The definition of the operator B(∙) is as legacy (as defined in 38.212).

Note: Here k, j=1, 2, …, KS

Note: How this is captured in the spec (including exact formulation) is up to the editor(s).

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, an RRC-configured resource-level slot offset (relative to the resource-set-level slot offset, using the same design as Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports) is supported for aperiodic CSI-RS resource set

- FFS: The number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources, including the value(s) of the slot offset

- FFS: Whether, in addition, configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set

  - Note: “Available slot offset” is analogous to the Rel-17 SRS triggering offset enhancement

Agreement

For the Rel-19 Type-I SP codebook refinement for P=48, 64, 128 CSI-RS ports, when the UE reports or multiplexes the CSI that include Part 2 CSI reports on PUCCH, the PUCCH resource, the number of PRBs for the PUCCH resource, and/or the number of Part 2 CSI reports are determined based on the RI value that results in the largest UCI payload.

- For Scheme-A, the RI value is:

  - 8 when the RI value(s) allowed by the configured RI restriction per CSI reporting configuration include 5, 6, 7, and/or 8,

  - 1 otherwise

- For Scheme-B, the RI value is 4

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint triggering carried on a same PUSCH (hence on a same slot), the UCI associated with the CJTC Dd report is multiplexed in CSI Part 1

- The previously agreed UCI design and mapping order for CJTC Dd report are reused

- The legacy UCI design, UCI mapping order, and UCI omission for the Rel-18 eType-II CJT CSI are reused

Note: The above proposal reuses the legacy UCI design principles, where

the UCI associated with the CJTC Dd is placed in the part of UCI as TS 38212 Table 6.3.1.1.2-13; the CSI part 1 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-13 and the CSI part 2 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-14

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the UE assumes that the NTRP CSI-RS resources are transmitted without DL/UL switching in between the NTRP resources

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s), ρ=1

- Note: In this case, the min(si,1) operation is not needed

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), the configuration of the si value (3-bit indicator per SD basis vector group) is RI-common (a same configuration is used for RI=1 and RI=2)

Conclusion

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports with the Rel-15 Type-I SP codebook, joint operation with the Rel-18 NES framework is not supported.

3Tx uplink

Conclusion

To support 3T6R antenna switching for a 3TX UE,

- Reuse the power control parameters configuration principles of 4T8R for 3T6R, i.e., when two aperiodic SRS resource sets are configured,

  - if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE shall expect that the two SRS resource sets are associated with the same values of the higher layer parameters p0AlphaSetforSRS and pathlossReferenceRS-Id

  - Otherwise, the UE shall expect that the two SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

Agreement

The following capabilities are introduced for Rel-19

- 't3r3' for 3T3R.

- 't3r6' for 3T6R.

Agreement

Support the following replies to Questions 2 and 3 in RAN4 LS,

Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

- Reply 1: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders.

- Reply 2: No, a 3Tx UE supporting UL full power transmission mode 0, shall support full power with both 1- and 2-layer 3-port codebook.

R1-2410762	Draft Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement for 8TX UE	InterDigital, OPPO

Agreement

Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement in endorsed in R1-2410870

Agreement

For a UE operating with 3TX,

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook.

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching.

- Note: The RRC parameter mentioned in the above two bullets is configured per set

A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.

Asymmetric DL-sTRP/UL-mTRP

Agreement

Support to apply PL offset on PDCCH-order PRACH in FR2

- The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here.

Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19

Agreement

The answer to the Question 1 in LS R1-2409353 is:

- From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Agreement

The working assumption part of the following previous agreement is confirmed.

Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH:

- (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

  - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

- (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

  - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

Agreement

For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA:

- The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs.

Agreement

For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts:

- Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement.

- Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional.

- Alt3:

  - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17)

  - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources.

- Alt4:

  - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17)

  - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources.

    - FFS how to define the PL offset and CLPC adjustment state

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Remaining details on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Remaining details on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Remaining details on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Remaining details on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Remaining details on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Remaining details on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Remaining details on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - Remaining details on SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Remaining details on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Remaining details on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Remaining details on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Asymmetric DL-sTRP/UL-mTRP:

- Remaining details on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Remaining details on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Remaining details on two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

## 2.2	RAN2

#### 2.2.1	Agreements

- RAN2#128

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

#### 2.2.2	Remaining Open issues

UE-initiated/event-driven beam management:

- FFS MAC impacts based on RAN1 agreements

- RRC signaling design based on RAN1 RRC parameter list

CSI:

- RRC signaling design based on RAN1 RRC parameter list

3Tx uplink:

- RRC signaling design based on RAN1 RRC parameter list

Asymmetric DL-sTRP/UL-mTRP:

- Detailed format of PL offset update MAC CE.

- FFS PL offset impact to PHR trigger.

- FFS PL offset impact (if any) to RACH procedure.

- Removing the restriction of coresetPoolIndex in Rel-18 2TA operation to support 2TA operation in Asymmetric DL-sTRP/UL-mTRP.

- RRC signaling design based on RAN1 RRC parameter list.

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#112bis, the following agreements were made.

RF

3Tx uplink

<Agreement>: PC for 3Tx requirement

- PC for 3Tx requirement

  - PC3

  - PC2

  - FFS on PC1.5

    - FFS on UE type, i.e., FWA and/or handheld UE

    - FFS on support of full power mode 0

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

<Agreement>: Assumption for power splitting between antenna ports

- Equal power split among non-zero ports according to RAN1 spec

<Agreement>: PA configurations

- For MPR evaluation, the assumptions of PA architecture are:

  - For PC3, 3x23dBm

  - For PC2

    - 3x23dBm

    - 3x26dBm is not precluded and interested company can provide the evaluation

<Agreement>: Different MPR for different UE type

- Define separate MPR requirements for handheld and FWA UE

Asymmetric DL-sTRP/UL-mTRP

<Agreement>: Whether to assume different TCI state between DL and UL

- Assume different TCI state between DL and UL

RRM

UE-initiated/event-driven beam management

Issue 1-1: How to define Event triggered measurement reporting delay?

< Agreement>

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a [first PUCCH] triggered by the event.

- “[first PUCCH]” can be updated based on further RAN1/2 agreement.

Issue 1-3: Applicability of Event triggered measurement reporting delay requirements?

< Agreement>

- Applicable for all events agreed in RAN1/2

- Applicable to both Mode A and B

- Note: applicable to the event/mode supported by UE

Issue 1-7: Whether to support CSI-RS based L1-RSRP for aperiodic?

<Agreement >

- RAN4 to define CSI-RS based UEIBM L1-RSRP requirements for periodic CSI-RS. CSI-RS based UEIBM L1-RSRP requirements for semi-persistent CSI-RS are FFS pending on RAN1 further agreement to support semi-persistent CSI-RS.

CJT calibration (CJTC) reporting

Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration?

Agreement:

- RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later.

- Further discuss the simulation assumptions offline during this meeting.

3Tx uplink

Issue 3-1: RRM core/performance impacts by introducing 3TX support?

Agreement:

- No new RRM core requirements by introducing 3TX support.

Asymmetric DL-sTRP/UL-mTRP

Issue 4-1: Whether RRM impacts exist by 2TA enhancement?

Agreement:

- RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP]

- As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID.

- Further discuss the details in the next meeting.

In RAN4#113, the following agreements were made.

RF

3Tx uplink

<Agreement>: PC for 3Tx requirement

- 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19

<Agreement>: PC3 for 3Tx

- Use the same MPR as MPR for PC3 single carrier.

<Agreement>: PC2 for 3Tx

- Use the same MPR as MPR for PC2 single carrier.

  - FFS on whether PC2 MPR with 1Tx or 2Tx will be applied

<Agreement>: Supported layer for mode 0

- For the performance verification of 3Tx, use 1-layer only for mode 0.

RRM

UE-initiated/event-driven beam management

Issue 1-1: How to define Event triggered measurement reporting delay?

< Agreement>

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay?

< Agreement>

Agreement: For specifying the RAN4 delay requirement for event 2

- Measure both the current beam and new beam after the starting point, FFS the details.

- Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion.

- Start the RAN4 RRM discussion for FR2.

- Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2?

Case 1: no time window based

Case 2: with time window based, M is configurable

< Agreement>

Agreement:

It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

CSI

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement?

Agreement:

- No RRM impact by added SRS port grouping to support more than 4 layers.

- Note: the agreement can be revisited if any impact observed based on further RAN1 progress.

CJT calibration (CJTC) reporting

Issue 2-1: On the simulation assumption for CJT calibration

Agreement is captured in R4-2420209.

3Tx uplink

Issue 3-1: RRM core/performance impacts by introducing 3TX support?

Agreement:

- No new RRM performance requirements by introducing 3TX support.

Asymmetric DL-sTRP/UL-mTRP

No further agreement based on last meeting.

#### 2.4.2	Remaining Open issues

RF

3Tx uplink

FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type.

FFS on whether PC2 MPR with 1Tx or 2Tx will be applied

FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx. Subject to Tx Diversity support

Asymmetric DL-sTRP/UL-mTRP

FFS on study of UE RF requirement for different DL/UL TCI states in the WI or TEI

RRM

UE-initiated/event-driven beam management

FFS on the detailed RRM requirements for event-triggered L1 reporting without/with time-window-based configured.

FFS on whether to define new TCI state switching delay or existing requirements are sufficient

FFS on impact of beam on known/unknown conditions for TCI states

FFS on whether to enhance beam sweeping in event triggered L1 beam reporting

FFS on performance part of UE-initiated/event-driven beam management including accuracy and so on

CJT calibration (CJTC) reporting

FFS on whether to have RRM impacts based on simulation results.

Asymmetric DL-sTRP/UL-mTRP

FFS on detailed RRM requirements by 2TA enhancement

FFS on whether to have RRM core impacts of active uplink TCI state switching delay for unified TCI. If yes, how to define detailed RRM requirements for asymmetric DL-sTRP/UL-mTRP

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template