3GPP TSG RAN Meeting #106	RP-242529

Madrid, Spain, 09 – 12 December, 2024

Status Report to TSG

Agenda item:			9.3.2.4

| WI / SI Name | NR sidelink multi-hop relay |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No | Testing part: No |
| Acronym | NR_SL_relay_multihop |  |  |  |
| Unique ID | 1040114 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242349 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 09/2025 | Performance part: mm/yyyy | Testing part: mm/yyyy |
| Overall Completion level | Study Item: xx % | Core part: 40% | Performance Part: x% | Testing part: xx% |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Youngdae Lee |
|  | Company | LG Electronics Inc. |
|  | Email | youngdae.lee@lge.com |
| Rapporteur | Name | Martino Freda |
|  | Company | InterDigital Communications |
|  | Email | martino.freda@interdigital.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#127bis (October 2024)

No TU

RAN2#128 (November 2024)

63 contributions ([1] ~ [63]) were submitted in this meeting. The following agreements were made:

Scenarios

- The following cases are supported for L2 multihop relay:

  - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

  - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

  - FFS if the last relay UE can use the same L2ID for both of the connections in either case.

- Cases with two indirect paths to the gNB for the same remote UE are not supported.

- An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs.

- If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

Relay discovery and (re)selection for multi-hop U2N relay

- A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

- If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

- The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

- RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

- The LS to SA2 on relay discovery announcement was approved in [63].

Control plane procedures for multi-hop U2N relay

- In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.  FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE.

- The figure and description under P1 of [16] serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

- In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.

- In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection.

- The figure and description under P4 of [16] serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

- For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

- If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

- In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

- Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

  - Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

  - Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

  - All agreements in this WI apply to both these cases unless otherwise specified.

- Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.  This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs.

- Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

Service continuity for multi-hop U2N relay

- The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

- Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

- The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

- The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

  - Event Y2

  - Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

2.2.2	Remaining Open issues

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay.

  - Relay discovery and (re)selection

  - Impact on SRAP and QoS handling for multi-hop

  - Control plane procedures

- Specify intra-gNB service continuity for multi-hop U2N relay

  - Intra-gNB direct to multi-hop indirect path switching

  - Intra-gNB single-hop indirect to multi-hop indirect path switching

2.3	RAN3

2.3.1	Agreements

The work starts from RAN3#127.

2.3.2	Remaining Open issues

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay.

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed

  - Control plane procedures

- Specify intra-gNB service continuity for multi-hop U2N relay

  - Intra-gNB multi-hop indirect to direct path switching using existing framework

  - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

  - Intra-gNB direct to multi-hop indirect path switching

  - Intra-gNB single-hop indirect to multi-hop indirect path switching

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#128

- R2-2409632	Discussion on topology and intermediate relay UE (re)selection	vivo

- R2-2409633	Discussion on CP and SRAP impact for Approach 1	vivo

- R2-2409634	Discussion on Service continuity for multi-hop relay	vivo

- R2-2409728	Discussion on multi-hop U2N relay discovery and relay selection	NEC Corporation

- R2-2409730	Discovery and relay (re)selection for multi-hop U2N relay	OPPO

- R2-2409731	Service continuity of multi-hop U2N relay	OPPO

- R2-2409732	Control plane procedures of multi-hop U2N relay	OPPO

- R2-2409796	CP and SRAP for Multi-hop Relay	NEC

- R2-2409859	Discussion on Multi-hop Discovery and (Re)selection	CATT

- R2-2409860	Discussion on the Control Plane Procedures	CATT

- R2-2409861	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT

- R2-2409906	Discussion on relay discovery and (re)selection for NR sidelink multi-hop relay	TOYOTA InfoTechnology Center

- R2-2409967	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple

- R2-2409968	Discussion on End-to-End Connection Setup Approaches for Multi-hop UE-to-NW Relay	Apple

- R2-2409969	Discussion on SRAP for Multi-hop Layer-2 U2N Relay	Apple

- R2-2410006	Report of [Post127][402][Relay] Multi-hop relay control plane	InterDigital

- R2-2410007	Discovery and Relay (Re)Selection for Multi-hop U2N Relays	InterDigital

- R2-2410008	Control Plane Aspects for Multi-hop U2N Relays	InterDigital

- R2-2410032	Discussion on multi-hop Relay discovery and (re)selection	ZTE Corporation, Sanechips

- R2-2410033	Discussion on control plane procedures for multi-hop SL relay	ZTE Corporation, Sanechips

- R2-2410034	Discussion on service continuity for multi-hop SL relay	ZTE Corporation, Sanechips

- R2-2410104	Multi-hop relay discovery and reselection	China Telecom

- R2-2410105	Discussion on control plane aspects for NR sidelink multi-hop relay	China Telecom

- R2-2410106	Discussion on service continuity for multi-hop relay	China Telecom

- R2-2410139	Discussion on control plane and QoS handling for NR sidelink multi-hop relay	Spreadtrum, UNISOC

- R2-2410149	discussion on control plane procedure	Ericsson

- R2-2410150	discussion on discovery and relay (re)selection	Ericsson

- R2-2410183	Remaining issues on multi-hop U2N Relay Discovery message forwarding	ASUSTeK

- R2-2410184	Remaining issues on SRB0 message forwarding in multi-hop U2N Relay	ASUSTeK

- R2-2410185	Discussion on measurement report for multi-hop U2N Relay	ASUSTeK

- R2-2410201	Service Continuity for Multi-Hop Relays	Ericsson

- R2-2410281	Relay (re)selection in Multi-hop relay	Lenovo

- R2-2410282	Control plane in Multi-hop relay	Lenovo

- R2-2410283	Service continuity for Multi-hop system	Lenovo

- R2-2410288	Relay discovery aspects for multi-hop relay	Nokia

- R2-2410290	SRAP impacts on MH relay	Nokia

- R2-2410297	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.

- R2-2410298	Discussion on the discovery and relay (re)selection for multi-hop U2N relay	LG Electronics Inc.

- R2-2410299	Discussion on service continuity for multi-hop U2N relay	LG Electronics Inc.

- R2-2410305	Report of [POST127][401][Relay] MH relay discovery and (re)selection	LG Electronics Inc.

- R2-2410354	Considerations on Service Continuity of Multi-hop Relay	NEC

- R2-2410392	Multi-hop relay selection/re-selection	Sony

- R2-2410569	Control Plane under multihop L2 U2N relaying	Kyocera

- R2-2410570	Discovery and (re)selection under multihop relay	Kyocera

- R2-2410587	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon

- R2-2410588	Control plane procedures for multi-hop relay	Huawei, HiSilicon

- R2-2410589	Discussion on service continuity for Multi-hop Relay	Huawei, HiSilicon

- R2-2410619	Relay discovery and (re)selection	TCL

- R2-2410631	On approach 1	Nokia

- R2-2410704	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp

- R2-2410705	discussion on C-plane procedure for multi-hop relay	Sharp

- R2-2410706	discussion on service continuity for multi-hop relay	Sharp

- R2-2410734	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated

- R2-2410735	Control procedure for multi-hop L2 based U2N relay	Qualcomm Incorporated

- R2-2410736	Consideration on multi-hop U2N relay service continuity	Qualcomm Incorporated

- R2-2410756	Consideration on CP and UP issues for multi-hop SL relay	Samsung

- R2-2410827	Considerations on relay discovery and (re)selection	Samsung

- R2-2410828	Initial considerations on service continuity	Samsung

- R2-2410840	Relay discovery and (re)selection	TCL

- R2-2410981	Report of [POST127][401][Relay] MH relay discovery and (re)selection	LG Electronics Inc.

- R2-2411125	LS to SA2 on relay discovery announcement	ZTE Corporation, Sanechips

- R2-2411128	Way forward on SL multi-hop relay	LG Electronics Inc.

- R2-2411184	LS to SA2 on relay discovery announcement	RAN2

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template