3GPP TSG RAN Meeting #106                                           RP-242555

Madrid, Spain, December 9-12, 2024

Agenda item:	10.4.5

Source: 	Qualcomm Incorporated

Title: 	Scope for IOT NTN TDD

Document for:	Discussion and Decision

# Background

In RAN1#119, the following agreements were reached on the feasibility of the TDD frame structure from the downlink synchronization point of view and compatibility with the legacy system in the 1616-1626.5 MHz MSS band:

Observation

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

Observation

In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view:

- DL synchronization, based on the link level evaluations submitted at RAN#119

- Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band

Observation

RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Conclusion

For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits.

[text omitted for brevity]

Observation

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

Observation

In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view:

- DL synchronization, based on the link level evaluations submitted at RAN#119

- Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band

Observation

RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Conclusion

For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits.

[text omitted for brevity]

Additionally, a way forward [1] was agreed in RAN4 during RAN4#113. In this document, RAN4 agreed to initial aspects on the impact on the TDD pattern in RAN4.

The work item [2] includes a checkpoint for RAN#106, as follows:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

In this contribution, we present out views on the outcome of the study phase of RAN1 and RAN4, and how to update the work item to capture the conclusion.

# Discussion

RAN1 discussed several TDD patterns, with the following conclusions:

- The value of N=9 was the one preferred by most companies due to the WID recommendation [2] and the compatibility with the legacy system.

- On the number of downlink subframes (denoted as D in the RAN1 agreements), it was recognized that the studied values of D (D=8, 20, 30) can meet the link budget requirements, but only D=8 would be compatible with the legacy system deployed in this band. Synchronization and capacity performance would improve for larger values of D.

- Similarly, for uplink, RAN1 concluded that a number of uplink subframes equal to 8 (U=8) would be compatible with the legacy system.

Based on RAN1 discussions, there is a desire to specify a system that can coexist with the legacy deployments in the 1616-1626.5 MHz MSS band. At the same time, if there is limited specification effort to generalize the system to operate in other cases (mainly larger values of D and U), it would benefit potential future deployments that are not constrained by the legacy system. In our view, this is achievable with limited specification effort with the following limitations:

- For downlink:

  - Initial search is performed using D=8 only: generalizing to different values of D would lead to blind decoding at the UE side when detecting NPSS/NSSS/NPBCH/SIB1-NB, which is not desirable.

  - After initial search / access, more downlink subframes can be made available.

  - RRM requirements are defined only for D=8, U=8.

- For uplink:

  - RAN1 specifications should be able to accommodate larger values of U as long as the rules (e.g. dropping, postponement) can be defined in a U-agnostic manner.

Therefore, we propose to update the WID objective as follows:

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern based on N=9 including confirming the value of N, and associated UE procedures [RAN1, RAN2]

    - D=8, U=8 are prioritized.

    - D=8 is used for initial access / search. Larger values of D can be used after initial access / search.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

    - Requirements are defined only for D=8

Proposal: For the normative work on Rel-19 NTN IOT TDD:

- Specify only N=9.

- Prioritize D=8, U=8.

- During initial access, D=8 is assumed. Larger values of D can be used after initial access.

- RAN4 requirements are based on D=8.

# Summary

In this contribution we presented our views on the scope for the normative work of NB-IoT NTN TDD. We made the following proposal:

Proposal: For the normative work on Rel-19 NTN IOT TDD:

- Specify only N=9.

- Prioritize D=8, U=8.

- During initial access, D=8 is assumed. Larger values of D can be used after initial access.

- RAN4 requirements are based on D=8.

And proposed to update the work item as follows:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern based on N=9 including confirming the value of N, and associated UE procedures [RAN1, RAN2]

    - D=8, U=8 are prioritized.

    - D=8 is used for initial access / search. Larger values of D can be used after initial access / search.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

    - Requirements are defined only for D=8

# References

[1] R4-2419795 - Way Forward for [113][317] IoT_NTN_TDD, Moderator (Qualcomm Incorporated)

[2] RP-242415 - New WID on introduction of IoT-NTN TDD mode, Moderator (RAN1 Vice-Chair)