3GPP TSG RAN Meeting #106								RP-242597

Madrid, Spain, December 9-12th, 2024

Agenda Item:	9.3.1.3

Source:	Ericsson

Title:	Completion of AI/ML for positioning

Document for:	Discussion, Decision

# 1 Summary

Based on observing recent RAN1 and RAN4 discussion and agreements in Q4, the AI/ML based positioning had slow progress for the three 1st priority use cases. The two 2nd priority use cases are UE-centric and requires substantially more standardization effort than the corresponding 1st priority cases. It is unrealistic to expect that all 5 use cases can be completed in the remaining time (e.g., 3 RAN1 meetings) before the work item closes.

To increase the likelihood that we can complete the positioning feature in Rel.19, we propose to remove the 2nd priority cases from the WID. Further motivation and discussion are given in this document.

On the overall AI/ML PHY WID completion topic, it can also be discussed whether to remove the RAN1 agenda item 9.1.4.2 “Other aspects of AI/ML” in RAN1 to allocate more time to the WI agenda points. The work item objectives for RAN1 that are discussed in 9.1.4.2 is already addressed in the use case agenda items. Removing the agenda item would be more efficient use of the RAN1 AI/ML PHY time units. We notice that there is a continued low interest among companies in agreeing on these (non-critical) general AI/ML aspects. This can be further discussed in the plenary, no direction action to the plenary needed as this is a chairman decision.

# 2 Regarding 2nd priority cases of positioning accuracy enhancements

The WID have the following objectives for the use case of positioning accuracy enhancements.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

RAN2 Chair guidance for each RAN2 meeting thus far:

#### 8.1.2.3 LCM for Positioning use case Contributions should focus on LCM for UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases. Aspects related to data collection should be covered in 8.1.3

During Q4, RAN1 focused on the 1st priority cases. Case 3a made reasonable progress, however, case 1 and case 3b still have a long way to go, with many details to be discussed on monitoring, data collection and inference phases.

- For case 3b, we have only now settled the debate on “path vs sample” for LMF model input with an agreement to support both at a high level. However, many details for measurement report remain to be settled.

  - Also, the same issue may need to be discussed and resolved for case 1 and case 3a, if training data collection of case 1 is specified (pending RAN2 progress), or if split architecture is supported for case 3a (pending RAN3 discussion).

- Model monitoring support still has multiple options to be discussed. There are 5 remaining options for case 1.

- For case 3a model output, the use of LOS indication is not settled yet.

- Training data collection for all cases has multiple open issues, e.g. pairing of label and measurements, detailed content on training dataset, etc.

With only three meetings left, we do not see a realistic path where both 1st priority and 2nd priority cases can be completed in RAN1. Furthermore, the 2nd priority cases have a greater impact on specifications in both RAN2 and RAN4 compared to the 1st priority cases. However, the work in these two groups is lagging considerably behind.

The issue raised on completion of case 2a/2b within the WI time frame mentioned in our previous contribution at RAN#106 remain [RP-242030]:

From RP-242030 Beyond the fact that Case 2a and 2b are assigned 2nd priority, they are more difficult to make progress. The reasons include: - Case 2a/2b require tight coordination between UE and LMF, e.g., PRS configuration, measurement type and format, whether/how/what NW-side additional condition(s) the UE should be made aware. Additionally, it’s unclear where the boundaries are to declare consistency vs inconsistency between training and inference. - For Case 2a/2b, signaling support needs to be explicitly defined for all LCM stages, including training data collection, inference, monitoring, functionality/model management. However, this part is lacking since RAN2 only focus on 1st priority cases for LCM. - Many issues of Case 2a/2b involve UE capability, e.g., measurement capability, model LCM related capabilities. Different UE vendors have different preferences on UE implementation. On the other hand, there is also strong concern that too much flexibility in UE implementation will lead to market fragmentation. - The SI phase did not decide on the mapping between the AI ML positioning cases and existing positioning methods, or whether new methods should be introduced. RAN2 will need to discuss the integration of AIML in the positioning methods and inform RAN1 on how much legacy can be re-used. - Many aspects of case 2a/2b requires defining RAN4 requirements to ensure consistent UE behaviour. However, RAN4 work is dependent on RAN1 progress that is lagging behind. Five cases together with many variations on UE capability will lead to a large number of possible combinations to support in Rel-19. This discourages companies to invest the effort to bring AI/ML based positioning to real life. Also, from workload perspective, it is expected to take much effort for RAN1/RAN2/RAN3 to coordinate and finish the work for the three 1st priority cases on time.

Thus, we suggest that Rel-19 work continues only on the three 1st priority cases listed in the current WID.

For the use case of AI/ML based positioning accuracy enhancements, update the WID to remove the 2nd priority cases.

If the above proposal is agreed, RAN to send an LS to SA and SA2 to inform them about the decision.

# 4	Conclusion

We propose the following:

Proposal 1	For the use case of AI/ML based positioning accuracy enhancements, update the WID to remove the 2nd priority cases.

Proposal 2	If the above proposal is agreed, RAN to send an LS to SA and SA2 to inform them about the decision.