3GPP TSG RAN meeting #106										RP-242601

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.4.1.5

| WI Name | Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_combinedLband |  |  |  |  |
| Unique ID | 1040134 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241856 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2025 | Performance part: 06/2025 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 35% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Luca Lodigiani |
|  | Company | Inmarsat |
|  | Email | l.lodigiani@outlook.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

### 2.4.1	Agreements after RAN4#112-bis

#### Sub-topic 3-1: General Aspects and Regulatory Input

Sub-topic description: Regulatory aspects

#### Issue 3-1-1: ETSI additional blocking Requirements

- Agreement

  - Further discuss whether and how the additional ETSI requirements for in-band/out-of-band blocking should be captured.

#### Sub-topic 3-2: UE RF

Sub-topic description: System parameters and UE RF aspects

#### Issue 3-2-1: UE Duplexer

- Agreement:

  - Further discuss whether implementation with one duplexer is feasible, considering the requirements to protect Radio Astronomy in the 1660-1668MHz range between the two UL.

#### Sub-topic 3-3: SAN RF

Sub-topic description: SAN RF requirements

#### Issue 3-3-1: SAN Operating Bands

- Agreement

  - Specify the following SAN operating bands for bands n253, n251 and n250:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n253 | 1668 MHz - 1675 MHz | 1518 MHz - 1525 MHz | FDD |
| n251 | 1626.5 MHz - 1660.5 MHz | 1518 MHz - 1559 MHz | FDD |
| n250 | 1668 MHz - 1675 MHz | 1518 MHz - 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

#### Issue 3-3-2: SAN Channel Bandwidths

- Agreement:

  - Specify the following initial SAN DL channel bandwidths for bands n253, n251 and n250:

  - FFS which channel bandwidth applies for UL

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 |  |  |  |  |
| n253 | 30 |  |  |  |  |  |
|  | 60 |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n251 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n250 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

#### Issue 3-3-3: SAN NR-ARFCN

- Agreement

  - Specify the SAN NR-ARFCN for 100kHz channel raster for bands n253, n251 and n250 as follows:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n253 | 100 | 333600– <20> – 335000 | 303600 – <20> – 305000 |
| n251 | 100 | 325300– <20> – 332100 | 303600 – <20> – 311800 |
| n250 | 100 | 333600– <20> – 335000 | 303600 – <20> – 311800 |

#### Issue 3-3-4: SAN NR-ARFCN for enhanced channel raster

- Agreement

    - Specify the SAN NR-ARFCN for 10kHz enhanced channel raster for bands n253, n251 and n250 as follows:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n253 | 10 | 333600– <2> – 335000 | 303600 – <2> – 305000 |
| n251 | 10 | 325300– <2> – 332100 | 303600 – <2> – 311800 |
| n250 | 10 | 333600– <2> – 335000 | 303600 – <2> – 311800 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

#### Issue 3-3-5: SAN SS Raster

- Agreement

  - FFS

#### Sub-topic 3-4: RRM Requirements

Sub-topic description: RRM aspects

#### Issue 3-4-1: Band grouping

- Agreement:

  - Agree Option 1 as follows

- Table 3.5.2A-1: NR frequency band groups for satellite access in FR1

| Group | NR FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NR_FDD_SAB_FR1_A | n254, n255, n256, n253, n251, n250, |
| B | NR_FDD_SAB_FR1_B |  |
| C | NR_FDD_SAB_FR1_C |  |
| D | NR_FDD_SAB_FR1_D |  |
| E | NR_FDD_SAB_FR1_E |  |
| F | NR_FDD_SAB_FR1_F |  |
| G | NR_FDD_SAB_FR1_G |  |
| H | NR_FDD_SAB_FR1_H |  |
| I | NR_FDD_SAB_FR1_I |  |
| J | NR_FDD_SAB_FR1_J |  |

### 2.4.2	Agreements after RAN4#113

The discussion in RAN4#113 has mostly focused on how to capture radiated regulatory requirements, ETSI requirements and on the requirements and mechanisms for protection of Radio Astronomy service.

#### Issue 3-1-1: General Approach for Capturing Additional Regulatory Radiated Emission Requirements (e.g. ITU, ETSI, FCC)

- Agreement:

  - Further study how to capture radiated regulatory requirements in the specification(s), in such a way that ensures compliance to regulations in all operational scenarios, but does not create unnecessary implementation burden.

    - (E.g. basic FR1 OTA emission requirements, additional conversion to conducted requirements with reference antenna gain or scaling factor, etc)

#### Issue 3-1-3: ETSI additional blocking Requirements

Agreement:

  - For two new NR NTN L bands, referring to the conclusions in Rel-18 for band 253, do not define the additional in-band / out-of-band blocking requirements in Rel-19 until RAN4 gets clear information from ETSI.

#### Issue 3-1-4: Protection of Radio Astronomy stations – Duplexer dependency

Agreement:

  - The requirements to protect Radio Astronomy service operation are independent with the duplexer implementation.

#### Issue 3-1-5: Requirements and Mechanisms on Protection of Radio Astronomy stations

- Agreement:

  - Further study mechanisms to implement protection of Radio Astronomy stations, at least the following options could be considered:

    - TX Disable, considering possible impact on backwards compatibility

    - RB restrictions

    - NS

#### Issue 3-1-6: Signalling of proximity to Radio Astronomy stations

- Agreement:

  - Further study how to determine proximity of NTN UE to Radio Astronomy stations

### 2.4.3	Remaining Open issues after RAN4#113

UL Channel Bandwidths and final channel bandwidth table are still missing, alongside a number of other details on system parameters (e.g. actual TX-RX frequency separation range, SS Raster)

Companies are expected to further study how to capture radiated regulatory requirements and further consider how to implement requirements for protection of Radio Astronomy services.

The majority of UE RF requirements and SAN RF requirements are still pending

CR work split agreement amongst companies for the Core part should be targeted for the next meeting.

Performance has not started yet.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- RP-241856 - Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges – Inmarsat, Viasat

- R4-2415215	Topic summary for [112bis][111] NR_IoT_NTN_Bands	Moderator(Inmarsat)

- R4-2415814	draftCR to TS38.108 Introduction of NR-NTN combined L-band	ZTE Corporation, Sanechips

- R4-2416050	Discussion on the full NTN L band definition	Huawei, HiSilicon

- R4-2416374	Introduction of NR-NTN L-band	ZTECorporation,Sanechips

- R4-2416473	Input to UE RF requirements for NTN L-bands n253, n521, n250	Inmarsat, Viasat

- R4-2417206	WF on NTN and IoT-NTN bands	Inmarsat

- R4-2418135	Topic summary for [113][113] NR_IoT_NTN_Bands	Moderator (Inmarsat)

- R4-2420465	WF on requirements for NR IoT NTN bands	Inmarsat

- R4-2418357	draftCR to TS38.108 Introduction of NR-NTN combined L-band	ZTE Corporation, Sanechips

- R4-2418486	Introduction of NR-NTN L-band	ZTECorporation,Sanechips

- R4-2419067	Discussion on the full NTN L band definition	Huawei, HiSilicon

- R4-2419523	Discussion on UE RF requirements for NR-NTN bands to support the Extended L-band and the combined MSS L-band and Extended L-band ranges	Mediatek India Technology Pvt.

- R4-2419555	Input to UE RF requirements for NTN L-bands n253, n521, n250	Inmarsat, Viasat