3GPP TSG RAN Meeting #106																RP-242618

Madrid, Spain, December 9-12, 2024

Source:	vivo

Title:	Discussion on 3T3R SRS antenna switching capability in Rel-19 MIMO

Agenda Item:	9.3.1.1

Document for:	Discussion and decision

Introduction

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

The revised “NR MIMO Phase 5” WID [1] includes the following objective with the revised objective to include support of 3T6R and 3T3R antenna switching capabilities, and the second Note was added in RAN#105.

In RAN1#119, following agreement was reached [2].

Agreement

The following capabilities are introduced for Rel-19

- 't3r3' for 3T3R.

- 't3r6' for 3T6R.

There was different interpretation of the second note in the objective in RAN1#119, hence we clarify our understanding in this contribution.

Discussion

In RAN1#119, some companies argued that the UE capability of “'t3r3' for 3T3R” is for reporting incapability to the network, that means the UE reports to the network that it is not capable of supporting “'t3r3' for 3T3R”! Since the UE capability of “'t3r3' for 3T3R” has been agreed, the UE not supporting this capability will not report it to the network, hence the argument of reporting incapability is not valid. When the UE reports the capability of “'t3r3' for 3T3R”, gNB can configure corresponding SRS resource(s) in the set with usage as antennaSwitching. To support “'t3r3' for 3T3R”, SRS resource(s) can be configured as below, which was agreed in RAN1#119 for a UE operating with 3TX:

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching.

- A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.

Conclusions

In this contribution we discuss SRS resource(s) configuration for a UE reporting “'t3r3' for 3T3R” antenna switching capability, corresponding RRC parameter can be configured as below.

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antennaswitching.

Reference

[1] RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105

[2] RAN1#119 Chair note