3GPP TSG RAN Meeting #106					RP-242638

Madrid, Spain

Agenda Item:		9.1.4 Proposals led by RAN4 (not spectrum related)

Source:		Airbus, Thales, Novamint, FirstNet, Bosch, SyncTechno Inc., Fraunhofer IIS, Fraunhofer HHI, Siemens, BMW, Gilat, Softil, SKY Perfect JSAT, Erillisverkot, ESA, Sharp, EDF, Hispasat, MITRE, Eutelsat Group, SES, Intelsat, UIC

Title:			Market considerations on Mobile VSAT operating with NGSO/GSO from verticals point view

Document for:	Discussion

# Introduction

This paper proposes 3GPP to define a limited number of Mobile Very Small Aperture Terminals (Mobile VSATs), ‘VSAT-like’ terminals mounted on moving platforms such as cars, aircraft, trains, ships, etc.) operating in above 10 GHz frequency bands for the benefits of most satellite network user groups.

Observation 1: Mobile VSAT refers to UE with directive antenna mounted on moving platforms and operating in above 10 GHz frequency band

In this document, the following points are discussed:

- interest and motivation for introducing Mobile VSAT terminals connected to NGSO/GSO satellite, including an explanation of the rationale behind this approach and potential use cases of interest.

- the technology trends on ‘VSAT-like’ antennas.

- the necessary terminal types to be defined and their key characteristics.

Note that this contribution doesn’t address:

- terminals with omnidirectional antenna such as handheld or wearable devices.

- terminals with directional antenna and operating in FR1 bands.

# Discussion

## Context

The global demand for seamless connectivity is surging as mobility becomes a defining characteristic of modern life. From in-flight internet services to connected vehicles and real-time maritime communications, the ability to ensure reliable connectivity across various transport modalities is becoming increasingly essential. Mobile VSAT technology, also recognized as Earth Stations in Motion (ESIM) as defined by the International Telecommunication Union (ITU), connected to Non-Geostationary Orbit (NGSO) and/or Geostationary Orbit (GSO) is uniquely positioned to meet these evolving connectivity demands.

ITU has defined Earth Station In Motion (ESIM) in various documents. ITU refers to Earth Station in Motion (ESIM) as “an Earth station that is placed on a moving platform, such as a ship at sea, a moving train, or an aircraft in flight” [1]. 3GPP added support for ESIMs under the term ‘Mobile VSAT’ in Rel18 as part of the broader support for NTN in bands above 10GHz. In Rel18, Mobile VSAT support was limited to communication with a GSO satellite due to a historical Ka-band constraint which was removed at WRC23. Mobile VSAT for the communication with non-GSO was not included in Rel-18 [2], as discussed in [3].

Mobile VSATs can be installed on different moving platforms, including aircraft, ships, and different types of land vehicles, which can provide data-rich and reliable connections to moving or stationary satellite access networks. These terminals have evolved from the traditional large parabolic dish antennas to electronically steered small advance devices capable of tracking multiple satellites.

There are a wide range of Mobile VSATs capable of providing connectivity to both GEO and NGSO based satellite access networks in Ku or Ka band.

ITU defines three ESIM categories:

- Land Mobile

- Maritime Mobile

- Aeronautical Mobile

Observation 2: Mobile VSATs are needed for both GSO and NGSO based satellite access networks operating in Ku or Ka band

## Mobile VSAT targeted use cases

Mobile VSATs support a wide array of applications across different transport modalities and present unique opportunities and challenges as follows.

Automotive Connectivity

The automotive sector is witnessing a rapid shift toward connected vehicles, where mobile VSAT can play a pivotal role. By supporting navigation, infotainment, vehicle-to-everything (V2X) communications, and remote controlling, the mobile VSAT enhances resilience of connectivity and guarantees efficiency in transportation systems. As the industry moves toward autonomous driving, reliable connectivity will be essential for enabling real-time data exchange between vehicles (cars, trucks, buses, etc..) and infrastructure, ensuring resilient and efficient operations. From its side, the 3GPP MRP 5GAA has published a detailed analysis about possible scenarios, antenna options and expected performance in its technical report about satellite communications for automotive [4].

Aviation

In the aviation sector, the demand for in-flight connectivity is rapidly increasing. Passengers now expect high-speed internet access during flights, and airlines are looking for reliable and standard solutions to meet these expectations. Mobile VSAT systems provide the necessary infrastructure for delivering consistent connectivity, enabling airlines to enhance the passenger experience while supporting operational communications. Furthermore, the ability to maintain connectivity during turbulence or adverse weather conditions is critical for safety and efficiency.

Maritime Connectivity

For the maritime industry, reliable and mobile broadband communication is critical for ensuring maritime safety, including support for Marine Aids to Navigations (Marine AtoNs), connected ocean initiatives particularly in light of upcoming digital transformations such as autonomous ships, and crew welfare. Mobile VSATs can play a key role in enabling this connectivity, providing real-time information on weather, navigation, shipment status, operational management, and internet access for onboard passengers and crew.

Rail Transport

In rail transport, mobile VSAT can significantly improve operational efficiency railway staff and passenger connectivity, especially in rural areas where terrestrial connectivity is poor, not available or not economically feasible. Trains equipped with mobile VSAT will have to support real-time critical voice, data and video communications for train operations, allowing for better coordination and management of services. Furthermore, mobile VSAT can offer also non-critical communication services such as internet services to passengers, enhancing the travel experience. Full details of rail use cases are described in [5], and associated performance requirements are described in [6].

Public safety

There are multiple potential use cases for the use of Mobile VSAT for public safety. These mobile VSAT can be mounted on vehicles, drones of large variety of sizes, vessels, motorbikes, jet ski,.. addressing all types of moving platforms that public safety users can have. For instance, after natural disasters, a Mobile VSAT can provide emergency responders with internet access, allowing them to coordinate relief efforts, share real-time data, and communicate with command centres. In remote areas, rugged mobile VSAT enables search and rescue teams to share updates and receive instructions efficiently. Another example is, enabling remote law enforcement to monitor activities in real time, thus enhancing situational awareness, and facilitating rapid responses. In situations where medical personnel are deployed in remote areas, VSAT can support telemedicine applications, allowing healthcare providers to consult with specialists or access critical medical information on-site. One specific use case for public safety are nomadic terminals. The goal for the public safety personnel is to arrive at the emergency areas and set up the terminal and be ready to establish a communication in less than 30 seconds.

Off road commercial vehicles

Off-road commercial vehicles, such as those used in mining, agriculture, and construction, can significantly benefit from Mobile VSAT connectivity.  Mobile VSAT enables off-road vehicles to transmit operational data in real time, including sensor information, location tracking, fuel consumption, and equipment status. This helps fleet managers optimize routes, improve efficiency, get the sensors data on real time to improve services, and control operation remotely if needed. In this case, operators can remotely monitor the health of machinery and equipment, and also adapt the service based on real time retrieved data by using Mobile VSAT. As an example to elaborate, predictive maintenance reduces downtime and operational costs by addressing issues before they become critical. It also improves the service efficiency reducing the associated cost by providing a tailored service. Mobile VSAT connectivity supports better coordination among various vehicles and personnel working in the field. This is particularly important in large-scale operations like mining, construction or large agricultural sites.

Utilities Connectivity

On the utilities needs mobile satellite communication for different type of needs. In most of the cases the goal is to bring connectivity to a set of users, which can be under a moving cell. This moving cell needs to have a direct link to the core network, thus the best way to do it in remote areas where no terrestrial coverage is available, is by bringing connectivity through a Mobile VSAT, that could forward the connection to the tactical cell. It is important to remark, that the use cases could be quite different depending on the speed of the moving cell. For instance, for utilities nomadic use cases are quite important. In such cases, the main requirement is to set up the terminal in less than 30 minutes. Other high mobility use cases, required a MVSAT integrate to different type of vehicles being able to communicate while moving at speeds up to 100 km/h.

Media and entertainment

5G NTN will significantly enhance the media and entertainment industry by supporting a range of innovative use cases that improve content delivery, offer new interactive experiences, and broaden access to global audiences. With 5G NTN, individuals or content creators can stream live events, personal broadcasts, or videos in real-time from remote or rural areas enhancing social media platforms, allowing for richer, live user-generated content from any part of the world. In addition, 5G NTN can enable live broadcasting from any location, even in remote or underserved areas where traditional terrestrial infrastructure is unavailable. This is ideal for broadcasting sports, concerts, news gathering, and emergency events, ensuring high-quality transmission without delay. From live event broadcasting from any location to, real-time audience analytics and disaster and recovery and resilient broadcasting, NTN technology has the potential to reshape how entertainment is produced, consumed, and monetized worldwide.

Observation 3: Mobile VSATs to address a wide range of use cases across vertical market segments

## Mobile VSAT in 3GPP

Despite its advantages, Mobile VSAT technology faces several challenges that standardization within the 3GPP framework can help address:

1. Interoperability Issues

A major challenge in the deployment of Satellite Communication systems (Satcoms) addressing mobile VSAT is the lack of standardized protocols, which can lead to no compatibility or compatibility issues at best between different systems and networks. The absence of common standards hinders effective communication and service delivery, making it difficult for operators to integrate various technologies. Establishing standardized protocols within the 3GPP framework will facilitate seamless interoperability among diverse mobile VSAT systems, promoting a more cohesive and stronger ecosystem.

2. Integration with Next-Generation Networks maximizing market potential

As the telecommunications industry transitions to 5G and beyond, ensuring that mobile VSAT can effectively integrate with these advanced networks already in 5G-Advance is essential. We hope that 3GPP standardization will support the development of capabilities necessary for future technologies, including autonomous vehicles, aerial connectivity and smart transportation systems, maximizing the market potential. By aligning mobile VSAT with next-generation network standards, we can ensure its relevance and effectiveness in an increasingly connected world.

3. Performance Variability

Mobile VSAT legacy systems often exhibit variability in performance, including latency, bandwidth, and reliability. These discrepancies can negatively impact user experience and hinder the adoption of mobile VSAT solutions in critical applications. By defining clear performance metrics tailored to the different scenarios, needs and requirements, 3GPP standardization can help ensure that mobile VSAT systems consistently meet user expectations for quality and reliability, and user experience.

Observation 4: Standardisation of a limited number of Mobile VSAT types will help to organise the market

## ‘VSAT-like’ Antenna technology trends

The evolution of electronically steered antennas in satellite communications has been marked by significant technological advancements and shifts in industry needs over the last years. As satellite technology advanced, there was growing interest in applying electronically steered antennas for commercial satellite communications. The ability to steer beams electronically without moving parts presented a significant advantage compared to bulky dish antennas. Advances in digital signal processing and computing allowed for more sophisticated beamforming techniques, improving the performance and efficiency of phased array antennas.

Recently, innovations in materials, including lighter and more efficient designs with flat panel antennas, led to significant reductions in cost and size, making electronically steered antennas more viable for a broader range of applications with an easier integration (internet to the home, connected moving platforms: cars, trucks aircrafts…), providing connectivity in remote and underserved areas. With the current convergence of satellite communications with terrestrial networks, with 5G NTN, flat panel electronic steered antennas began to gain traction.

Ongoing research focuses on further miniaturizing and enhancing the performance of these electronically steered antennas, making them suitable for consumer devices and small satellites, particularly with small form factor thanks, among others, on the flat panel designs. The integration of AI for real-time optimization of beam patterns is becoming more common, improving network performance and user experience. As satellite communications evolve, electronically steered antennas are finding applications beyond traditional sectors, including IoT, disaster recovery, and global broadband initiatives.

The evolution of electronically steered antennas in satellite communications has been marked by continuous innovation, driven by technological advancements and changing market needs. As satellite networks become more complex and demand for connectivity increases, particularly with the introduction of 5G NTN, these antennas will play a pivotal role in shaping the future of global communications.

Observation 5: Most Mobile VSATs will feature an electronic beam steering antenna packaged as Flat Panel Antenna.

## Most representatives Mobile VSAT key characteristics

Since mobile VSAT need to address different markets and use cases, we will list the key technical characteristics that they will to be compliant with in order to ensure a full utilisation of these devices for all the different verticals that are requiring it.

For instance, and depending on the use cases, the SWAP (Size, Weight, Aperture and Power) could be very different not only depending on the frequency band, but more importantly on the use cases. For example, for heat dissipation constraints, a mobile VSAT and/or the antenna associated to it installed in a car or aircraft, could not consume the same amount of power.

In addition to the SWAP constraints, there are also other operational constraints, like the maximum speed that the mobile platform needs to support, that need to be taken into account. For instance, aircrafts can move up to 1500km/h compared to the ground.

In order to gather all different uses cases presented by the different verticals, and knowing the huge disparity on the different use cases needs and requirements, we intend to define limited number of Mobile VSATs, hence maximizing the market potential. The intention is to group the different needs of the verticals in small number of Mobile VSAT devices, in order to focus 3GPP efforts on providing good and representative definition and characteristics of the most demanding MVSATs setting the scene for the introduction of Mobile VSAT connecting to NGSO. Table 1 illustrates the desired form factors and performance goals from verticals.

Table 1 Mobile VSAT characteristics from verticals

|  | MVSAT A | MVSAT B | MVSAT C |
| --- | --- | --- | --- |
| Min Indicative DL Target throughput | 50 Mbps | 200 Mbps | [1] Gbps |
| Min Indicative UL Target throughput | 15 Mbps | 60 Mbps | [300] Mbps |
| Max antenna aperture | 20x20 cm | 40x40 cm | 60x60 cm |
| Max thickness | 2-4 cm | 7 cm | 10-15 cm |
| Indicative Max Power consumption (No TX power - Note 1) | <35 Watts | 150 Watts | 500 Watts |
| Max VSAT speed with respect to ground | Up to 1500 km/h. Note 2 |  |  |
| Specific requirements and regulations | Note 3 |  |  |
| Note 1: Total power consumption can affect the feasibility of the antenna and considerations (such as cooling) in some vehicles, e.g. passenger vehicles. |  |  |  |
| Note 2: Up to 250 km/h for land vehicle/vessels/drones. Up to 500 km/h for high-speed trains. Up to 1500 km/h for aircraft |  |  |  |
| Note 3: for each vertical sectors, specific requirements for the certification of terminals will have to be fulfilled (e.g. EMC, interference, health protection, …) |  |  |  |

Table 2: Mobile VSAT vertical use cases examples

| Verticals use cases | MVSAT A | MVSAT B | MVSAT C |
| --- | --- | --- | --- |
| Passenger vehicle | X |  |  |
| Trucks | X |  |  |
| Commercial Buses | X | X |  |
| Off-road vehicles | X |  |  |
| Railway trains | X | X | X |
| Aircrafts | X | X | X |
| Professional vehicles (Public safety, Utilities) | X | X | X |
| Railway staff | X |  |  |
| Media & entertainment / News gathering production | X |  | X |
| Media & entertainment audience | X |  |  |
| Drones (various sizes) | X | X |  |
| Vessels / Marine AtoNs | X | X | X |

Observation 6:  A limited number of Mobile VSAT types served by NGSO are sufficient to meet the need of multiple verticals use cases

Observation 7: To minimize workload in 3GPP, Mobile VSAT B and C can be mapped on existing VSAT type 5, defined in TS 38.101-5 Table 9.2.1.0-1, communicating to both GSO and NGSO

Observation 8: To minimize workload in 3GPP, Mobile VSAT A could be defined as a new VSAT type 6, in TS 38.101-5 Table 9.2.1.0-1, based on fixed VSAT type 3 RF characteristics, connecting to NGSO only.

The RF parameters defined by 5GAA [4] (5GAA, Technical Report, Maximising the benefit of future satellite communications for automotive, September 2024) for Mobile VSAT A operating Ka-band can be considered as starting point for 3GPP normative work

## Way forward in 3GPP

Proposal 1: 3GPP to upgrade VSAT type 5 for Mobile VSAT communicating with NGSO, in addition to GSO, in both Ku and Ka bands

Proposal 2: 3GPP to define a Mobile VSAT type 6 communicating to NGSO based on Fixed VSAT type 3 RF characteristics

Proposal 3: 3GPP to reuse the minimum performance values from existing VSAT type 3 and 5 to the maximum extent possible, including but not limited to:

- Minimum peak EIRP

- Minimum OTA Rx sensitivity

- Minimum Pointing/Tracking accuracy

# Conclusion

As conclusion, in this document we would like to take in consideration the following observations and proposals:

Observation 1: Mobile VSAT refers to UE with directive antenna mounted on moving platforms and operating in above 10 GHz frequency band

Observation 2: Mobile VSATs are needed for both GSO and NGSO based satellite access networks operating in Ku or Ka band

Observation 3: Mobile VSATs to address a wide range of use cases across vertical market segments

Observation 4: Standardisation of a limited number of Mobile VSAT types will help to organise the market

Observation 5: Most Mobile VSATs will feature an electronic beam steering antenna packaged as Flat Panel Antenna.

Observation 6: A limited number of Mobile VSAT types served by NGSO are sufficient to meet the need of multiple verticals use cases

Observation 7: To minimize workload in 3GPP, Mobile VSAT B and C can be mapped on existing VSAT type 5, defined in TS 38.101-5 Table 9.2.1.0-1, communicating to both GSO and NGSO

Observation 8: To minimize workload in 3GPP, Mobile VSAT A could be defined as a new VSAT type 6, in TS 38.101-5 Table 9.2.1.0-1, based on fixed VSAT type 3 RF characteristics, connecting to NGSO only.

Proposal 1: 3GPP to upgrade VSAT type 5 for Mobile VSAT communicating with NGSO, in addition to GSO, in both Ku and Ka bands

Proposal 2: 3GPP to define a Mobile VSAT type 6 communicating to NGSO based on Fixed VSAT type 3 RF characteristics

Proposal 3: 3GPP to reuse the minimum performance values from existing VSAT type 3 and 5 to the maximum extent possible, including but not limited to:

- Minimum peak EIRP

- Minimum OTA Rx sensitivity

- Minimum Pointing/Tracking accuracy

# References

- Satellite issues: Earth Station in Motion (ESIM) https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

- 3GPP TS 38.101-5 v18.7.0 NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements.

- RP-240079 Motivation to support mobile VSAT in NGSO deployment scenarios. Source: Eutelsat Group et al.

- 5GAA, Technical Report, Maximising the benefit of future satellite communications for automotive, September 2024. https://5gaa.org/maximising-the-benefit-of-future-satellite-communications-for-automotive/

- 3GPP TR 22.989 v19.4.0 Technical Specification Group Services and System Aspects; Study on Future Railway Mobile Communication System; Stage 1.

- 3GPP TS 22.289 v18.0.1 Technical Specification Group Services and System Aspects; Mobile Communication System for Railways; Stage 1.