# 7	Key performance indicators

This section describes the definitions of all KPIs.

The KPI targets defined in this section as well as in Report ITU-R M.[IMT-2030. TECH PERF REQ] shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ].

## 7.1	Peak data rate

Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

Wi × SEpi

where Wi and SEpi (i = 1,…Q) are the component bandwidths and Spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The targets for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.

## 7.2	Peak Spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

The targets for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is TBD bit/s/Hz.

–	Uplink peak spectral efficiency is TBD bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink.

## 7.3	User experienced data rate

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency. Let W denote the channel bandwidth and SEuser denote the 5th percentile user [184] spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the related TBD test environment(s).

The target values for the user experienced data rate are as follows in the TBD usage scenarios and test environment(s):

–	Downlink user experienced data rate is TBD Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

## 7.4	5th  percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

The targets for 5th percentile user spectral efficiency for various test environments are summarized in Table 7.4-1.

Table 7.4-1: 5th percentile spectral efficiency

| Test Environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| A |  |  |
| B |  |  |
| … |  |  |

## 7.5	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

The minimum requirements for average spectral efficiency for various test environments are summarized in Table 7.5-1.

Table 7.5-1: Average spectral efficiency

| Test Environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| A |  |  |
| B |  |  |
| … |  |  |

## 7.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and  the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6).

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands. (Editor’s note: This requires further discussion, and contributions are encouraged)

This requirement is defined for the purpose of evaluation in the related Immersive Communication test environment.

The target value for Area traffic capacity in downlink is TBD in the – TBD test environment.

## 7.7	Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2) ), where the target QoS is to ensure a system packet drop rate less than 1% under given packet arrival rate l and packet size S. Packet drop rate = (Number of packet in outage) / (number of generated packets), where a packet is in outage if this packet failed to be successfully received by destination receiver beyond packet dropping timer.

Connection density should be achieved for a limited bandwidth and number of TRxPs. [The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability]

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

The minimum requirement for connection density is TBD devices per km2.

3GPP should develop standards with means of high connection efficiency (measured as supported number of devices per TRxP per unit frequency resource) to achieve the desired connection density.

## 7.8	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

[–	Stationary: 0 km/h

–	Pedestrian: 0 km/h to 10 km/h

–	Vehicular: 10 km/h to 120 km/h

–	High speed vehicular: 120 km/h to TBD km/h.]

Table 7.8-1 defines the mobility classes that shall be supported in the respective test environments.

Table 7.8-1: Mobility classes

|  | Test environments for TBD |  |  |
| --- | --- | --- | --- |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 7.8-2. This assumes the user is moving at the maximum speed in that mobility class in each of the test environments.

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

Table 7.8-2: Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |

These values were defined assuming an antenna configuration as described in TBD.

## 7.9	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

The target for mobility interruption time should be 0ms.

This KPI is for both intra-frequency and inter-frequency mobility for intra-NR mobility.

Mobility support can be relaxed for extreme rural scenarios for the Provision of minimal services for very low-ARPU areas: Inter RAT mobility functions can be removed. Intra-RAT mobility functions can be simplified if it helps decreasing the cost of infrastructure and devices. Basic idle mode mobility shall be supported as a minimum.

## 7.10	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	TBD ms for TBD

## 7.11	Control plane latency

Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE).

This requirement is defined for the purpose of evaluation in the TBD usage scenarios.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is TBD ms.

## [7.12	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a TBD data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the TBD usage scenarios.

The minimum requirement for the reliability is TBD success probability of transmitting a layer 2 PDU (protocol data unit) of TBD bytes within TBD ms in channel quality of coverage edge for the TBD test environment, assuming TBD application data (e.g. TBD bytes application data + protocol overhead). ]

## 7.13	Joint requirement on data rate, latency and reliability

The joint performance requirement relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

NOTE:	The exact definition will be similar to Reliability and can be seen as a generalization of the reliability requirement with a similar evaluation methodology.

[Reliability is the success probability of transmitting application layer packets within a required maximum time, which is the time it takes to deliver a TBD packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. The size of the application layer packets is determined by the data rate and traffic arrival rate, e.g. frame rate.]

This requirement is defined for the purpose of evaluation in the [Immersive Communication and Hyper-Reliable Low Latency Communication] usage scenario.

The joint minimum requirement for various test environments is summarized in Table 7.13-1.

Table 7.13-1: Joint requirement on data rate, latency, and reliability

| Test environment | Data rate |  | Latency |  | Reliability |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Dense Urban - Immersive Communication |  |  |  |  |  |  |
| Rural – [Immersive Communication] |  |  |  |  |  |  |
| Indoor – HRLLC |  |  |  |  |  |  |

## 7.14	Positioning

Positioning is the ability to estimate the position of connected devices. The device positioning accuracy is the [90%] point of the CDF of the device positioning error. The device positioning error is defined as the difference between the estimated horizontal/vertical position and the actual horizontal/vertical position of a device.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for the device positioning accuracy for various test environments are summarized in Table 7.14-1.

Table 7.14-2: Device positioning accuracy

| Test Environment | Bandwidth (MHz) | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- | --- |
| Indoor Hotspot –Immersive communication | [20] |  |  |
|  | [100] |  |  |
| Dense Urban – Immersive communication | [20] |  |  |
|  | [100] |  |  |
| … |  |  |  |

## 7.15	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is TBD.

The RIT/SRIT shall support scalable bandwidth. Scalable bandwidth is the ability of the candidate RIT/SRIT to operate with different bandwidths.

## 7.16	Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, [object classification, movement characterization, passive object localization, mapping, and composite sensing tasks such as tracking. SLAM, etc.

A sensing task is a processing function in an ISAC-capable system, which is performed to achieve a certain sensing goal with respect to one or more sensing target(s), based on radio frequency measurements. Performance of a sensing task is characterized by at least one minimum requirement.

Technical Performance Requirements can be set for the sensing tasks, where the sensing tasks could be Detection, Characterization and Localization. A Confidence level of TBD should be specified.

Table 7.16-1: Sensing requirement (example format)

|  | SENSING TASKS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test environment | Presence/absence detection |  | [Sensing Task 2] |  |  |
|  | PD | PFA | Req 1 | Req 2 | Req 3 |
| [Urban macro – ISAC @ Ground level] | TBD1 | TBD1 | [TBD] | [TBD] | [TBD] |
| [Urban macro – ISAC @ Aerial vehicles] | TBD2 | TBD2 | [TBD] | [TBD] | [TBD] |
| NOTE 1: Applicable for ground-level objects. NOTE 2: Applicable for low-altitude aerial vehicles. |  |  |  |  |  |

## 7.17	Energy efficiency

Network energy efficiency is the capability to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability to minimize the power consumed by the device modem in relation to the traffic characteristics.

Energy efficiency of the network and the device can relate to the support for the following two aspects:

a)	Efficient data transmission in a loaded case;

b)	Low energy consumption when there is no data.

Efficient data transmission in a loaded case is demonstrated by the Average spectral efficiency (see section 7.5).

Low energy consumption when there is no data can be estimated by the sleep ratio. The sleep ratio is the fraction of unoccupied time resources (for the network) or sleeping time (for the device) in a period of time corresponding to the cycle of the control signalling (for the network) or the cycle of discontinuous reception (for the device) when no user data transfer takes place. Furthermore, the sleep duration, i.e. the continuous period of time with no transmission (for network and device) and reception (for the device), should be sufficiently long.

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

NOTE: The following example requirement is based on Ambition level 3 for energy efficiency.

The RIT/SRIT shall demonstrate high power savings through the capability to support a high sleep ratio and long sleep duration. The minimum requirement for the power savings is summarized in Table 7.17-1, where the power consumption reference is [a “typical” Rel-15 NR reference case].

Table 7.17-1: TRxP power savings through sleep modes

| Sleep duration | Relative power consumption (%) |
| --- | --- |
| [> X ms] | [Y%] |
| NOTE: The value of X should be selected such that deep sleep operation is possible in a scenario with zero user data traffic (e.g. >50 ms). |  |

Average spectral efficiency corresponds to “spectrum efficiency” in Recommendation ITU-R M.2160.