3GPP TSG RAN Meeting #106						RP-242639

Madrid, Spain, December 9-12, 2024

Agenda item:	16

Source:	Ericsson

Title:	Proposed Technical Performance Requirements (TPR) for IMT-2030

Document for:	Discussion

# Background

In ITU-R WP5D, a new report on Technical Performance Requirements (TPR) for IMT-2030 is being developed [1] that corresponds to ITU-R report M.2410 [5] for IMT-2020. The report will define a set of minimum technical performance requirements for IMT-2030 candidate radio interface technologies. In an LS from WP5D [2], WP5D invites External Organizations to provide input on the TPR as follows:

- Proposed candidate items for minimum technical performance requirements based on Usage Scenarios and Capabilities from ITU-R Recommendation M.2160 [6], including necessary background information and justification, which are requested to be provided preferably by WP5D #48 (February 2025 TBC).

- Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP5D #49 (June/July 2025 TBC)

This contribution proposes candidate TPRs, based on the ongoing discussions in WP5D on the TPR Working document [1] and WP5D input documents [3,4]. Some of the new proposed TPR are presented in more detail. In an Annex to this contribution, a text proposal for a future 3GPP TR is made, with proposed definitions for the TPR.

# Candidate IMT-2030 TPR

The Working document for the draft new Report on TPR [1] lists technical performance requirements, based on the capabilities in ITU-R Recommendation M.2160 [6]. Definitions of the TPR in [1] are derived from that recommendation, the legacy definitions for IMT-2020 in Report ITU-R report M.2410 [5], plus additional input made to WP5D. Being a “working document” means that the text in [1] is under discussion has varying levels of stability, ranging from text with wide consensus to some proposed TPR with weak support within WP5D. No text is formally agreed.

The following are “legacy” TPR from IMT-2020 that are proposed to be applied also for IMT-2030.

- Peak data rate

- Peak Spectral Efficiency

- User experienced data rate

- Average Spectral efficiency

- 5th percentile user Spectral efficiency

- Area traffic capacity

- Connection Density

- Mobility

- Mobility interruption time

- User plane latency

- Control plane latency

- [Reliability]

- Bandwidth

- Energy Efficiency / Sustainability

Definitions can largely remain with some updates, noting that the definitions in 3GPP did not fully align with ITU-R definitions from IMT-2020. For Energy efficiency, a revised quantitive TPR is proposed. This is further described below in section 3. In WP5D Energy efficiency is treated as part Sustainability, but the broader aspects of that capability will most likely not remain as TPR, but could potentially be part of the submission template.

The following are “new” TPR which were proposed for IMT-2030, based on the extended and new Usage scenarios described in ITU-R Recommendation M.2160 [6].

- Joint requirement on data rate, latency and reliability (to replace Reliability)

- Positioning

- Sensing-related capabilities

Further details on the three new TPR are given below in section 3.

Several additional TPR are presently in the WP5D working document but are not proposed here for inclusion. These include AI-related capabilities, that has potential to be describes as a TPR, at least for inspection. Sustainable data rate is proposed as a potential TPR for UE energy efficiency and there are several TPR specifically proposed for the XR use case. Security, Resilience and Interoperability are proposed, but may instead be covered by the submission template for IMT-2030. The same is true for Coverage, which will most likely be covered by link budgets to be provided in the submission template.

# Revised and new TPR for IMT-2030

Further details on the following TPR parts are presented here:

- Joint requirement on Data rate, Latency, and Reliability

- Positioning

- Sensing

- Energy efficiency

Detailed text proposals for the Working Document related to the TPR are given below.

## Joint requirement on Data rate, Latency, and Reliability

IMT-2020 has separate capabilities for latency, data rate (user experienced), and reliability. The corresponding TPR for latency was evaluated through analysis and the TPRs for data rate and reliability were evaluated by simulation.

Although being fundamental performance metrics for a Radio Interface Technology (RIT), improved peak bitrate or reduced minimum latency would enable few new use cases seen as individual metrics. IMT-2030 is however not only an extension of the legacy IMT-2020 capabilities, but also an expansion into new use cases within the six usage scenarios of the “wheel diagram” in ITU-R Recommendation M.2160 [6]. Many new use cases rely on combinations of data rate, latency, and reliability to function as shown in Figure 1. Examples include extended reality (XR), remote driving and video conferencing. To reflect these new use cases, a Technical Performance Requirement (TPR) is proposed that jointly evaluates the capabilities Data rate, Latency and Reliability.

The joint requirement can be seen as a generalization of the reliability requirement and the evaluation methodology would be similar. By setting and evaluating such a requirement with higher data rates than for IMT-2020 it would replace Reliability and could cover multiple combinations of Data rate, Latency, and Reliability for various use cases.

Figure 1: Example use cases requiring different combinations of data rate, latency, and reliability.

## Energy efficiency

For IMT-2020, Energy efficiency of the network and the device related to the support for two aspects:

- Efficient data transmission in a loaded case (demonstrated by average spectral efficiency);

- Low energy consumption when there is no data (demonstrated by the capability to support a high sleep ratio and long sleep duration).

While the requirement for IMT-2020 in ITU-R was to demonstrate the presence of the capability, evaluations were in fact quantitative, though no limits were put on the resulting sleep ratio and duration. Quantitative evaluation for IMT-2030 is possible, based on the sleep ratio/duration parameters or on related metrics also involving power models. One possible choice of power models are the ones from the 3GPP study in TR 38.864.

It is proposed to introduce a quantitative TPR on Energy Efficiency for IMT-2030. There are several possibilities for the ambition level of the requirement:

- An evaluation to demonstrate the capability to support a high sleep ratio and long sleep duration (Inspection; Similar to IMT-2020 evaluation)

- A quantitative evaluation to demonstrate the level of sleep ratio and long sleep duration supported by the RIT (Analysis; With TPR limits on sleep ratio and sleep duration)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Analysis without user data traffic; With TPR limits on relative energy savings)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Simulation with user data traffic; With TPR limits on relative energy savings)

Level 1) above is the same TPR as for IMT-2020, while level 2) is based on the same type of evaluation, but with limits for the TPR (sleep ratio and duration).

Level 3) and 4) are further advancement that also involve power models. In 3) the power model is applied to the different sleep modes of the RIT and the related sleep durations for a case without data traffic, to estimate how much power saving they would achieve. Level 4) is a further step where the power model is applied in a system level simulation, with simulated user data traffic.

An important qualitative difference between the ambition levels 1 to 4 above, is the use of a power model in the latter two. By using a power model with level 3, a tangible measure of power savings is provided, and the way power is saved is concretized. A power model used for evaluating the energy saving potential of a future 6G RIT should preferably be future-looking and demonstrate that a RIT has potential for exploiting future advances on efficient power savings.

The difference between ambition level 3 and 4 above is the use of user data traffic in level 4, while in level 3 no user data traffic is included in the evaluations. While user data traffic can have a very large impact on the energy consumption of an individual radio base station in a lab environment, it has low impact on the total energy consumption of a live nationwide network. The reason is that averaging over time and space, the average physical resource utilization of a typical 5G network is in the single-digit percentage range. The energy consumption in a nationwide network is therefore today very much dominated by the energy cost of providing coverage rather than by the actual user data traffic. Hence an evaluation that ignores user data traffic, as in ambition level 3 above, still captures most of the total network energy consumption.

It is for the reasons above proposed to use ambition level 3 for further work on Energy efficiency.

## Positioning

For positioning of active/connected objects, minimum requirements should be defined for a limited number of use cases. It is noted that the positioning performance depends on the geometry and density of transmitters and receivers. Furthermore, the positioning signals typically need to be heard from/at multiple sites, which will impact the minimum required signal strength and quality at the receiver. Thus, deployment with advantageous (serving cell) connectivity does not in itself guarantee good positioning performance. Additionally, both horizontal and vertical positioning accuracy are important, at least for some use cases.

It is further noted that positioning accuracy on cm-level may only be reached in some specific environments and deployments and therefore cannot be a generic requirement.

### Usage scenario for positioning

The Immersive Communication usage scenario extends the enhanced Mobile Broadband (eMBB) usage scenario and covers use cases that provide an immersive experience to users, including interactions with machine interfaces. Such immersive applications will merge digital objects, information, and overlays to provide virtual, mixed, and augmented reality (VR, MR, and AR respectively) experiences, which are commonly referred to as extended reality (XR) applications.

To maintain proper orientation in time and 3D space in XR, there is a need for keeping track of the position and location of various connected objects, including handheld devices (smart phones) as well as wearables such as AR glasses, connected vehicles, and collaborative robots (cobots). It is therefore expected that Immersive Communication use cases will require support of high reliability and low latency communication as well as precise positioning of connected objects and devices for responsive and accurate interaction. Therefore, TPRs for the Positioning capability should be defined and evaluated in the Immersive Communication usage scenario.

### Proposals on Positioning for a TPR in a future TR

The following aspects need to be highlighted in a TPR for positioning:

- Horizontal and vertical position estimation performance can be different depending on the system configuration and deployment. Additionally, the requirements on the horizontal error and vertical error put by an application can be different. For this reason, there needs to be separate requirement entries for horizontal and vertical accuracy.

- The position estimation error should be seen as having a spatial distribution. The accuracy should thus be defined as a percentile point of the cumulative distribution (CDF) of the positioning errors.

- Position estimation accuracy is dependent on many parameters, including signal bandwidth, measurement capabilities, deployment, radio conditions, etc. Those will be defined for the test environment in the respective evaluation configuration. Since bandwidth has an impact on the possible positioning accuracy but is also dependent on the operating band frequency, it is proposed that minimum requirements are defined for more than one bandwidth configuration.

## Sensing-related capabilities

In a first step, the objective of sensing and the related sensing tasks need to be defined, applicable for a limited number of use cases (e.g., Transportation, Industry, UAV, Smart home). Some examples of sensing tasks are presence/absence detection, localisation of passive/non-connected objects, velocity/direction estimation, distance estimation, condition detection (e.g., weather), etc.

### Sensing tasks and use cases

There are many possible use cases for sensing. As an example, the 3GPP SA1 study in 3GPP TR 22.837 suggest 62 use cases. Looking forward the number of use cases may increase further, making an adequate and flexible standardization framework necessary to accommodate the growing number, without unnecessary complications.

It is further noted that even though there can potentially be a large number of sensing use cases, there is a limited set of actual sensing task categories that need to be supported in a system capable of sensing. With a set of standardized common sensing tasks, enabling a new use case would entail selecting sensing tasks needed from the standardized set and ensuring that the relevant Key Performance Indicator (KPI) target values for the new use case exist. Thus, supporting a new sensing use case would not affect the standardized elements and will make the sensing TPR scalable.

Example sensing tasks:

- Object detection (e.g., presence/absence detection)

- Object characterization (e.g., classification, size determination, feature extraction),

- Movement detection (e.g., detection of a movement occurrence),

- Movement characterization (e.g., velocity estimation),

- Localization (at least of passive/non-connected objects),

- Mapping (building a map of unknown environment).

Each basic sensing task is characterized with at least one KPI that defines the minimum requirement. This is illustrated in Figure 4.

A sensing task can be defined as follows:

A sensing task is a processing function in an ISAC-capable system, which is performed to achieve a certain sensing goal with respect to one or more sensing target(s), based on radio frequency measurements.

Figure 4

An example on how a small set of sensing tasks provide a general interface to accommodate a large number of use cases.

### Relevant test environments for Sensing

Prioritization among possible test environments will also be needed. Even for an outdoor deployment with macro BSs, different sensing test environments may need to be defined for outdoor sensing of target objects at a ground level and outdoor sensing for low-altitude aerial vehicles, where at least the channel characteristics are likely to be different, and a new parameters for aerial vehicles may need to be developed. Two outdoor test environments are proposed, one for low-altitude aerial vehicles and one for ground-level objects.

# Evaluation parameters

For the TPR listed above, several parameters need to be set to define the requirements while others will be defined for the evaluation. In ITU-R WP5D, a new report on Guidelines for the evaluation of IMT-2030 is being developed [7] that corresponds to ITU-R report M.2412 [8] for IMT-2020. The document is presently at an early stage, with much of the evaluation methodology inherited from IMT-2020.

As a first input to the update of the evaluation parameters, it is proposed to:

- Add a frequency band in the range 7-8 GHz to the existing set of IMT-2020 frequency bands.

- For the frequency band in the range 7-8GHz, use up to four times more antennas elements than used for 4GHz for IMT-2020.

# Proposals

This contribution presents a list of candidate TPR, proposed to be used in the LS response to ITU-R WP5D:

- Peak data rate

- Peak Spectral Efficiency

- User experienced data rate

- Average Spectral efficiency

- 5th percentile user Spectral efficiency

- Area traffic capacity

- Connection Density

- Mobility

- Mobility interruption time

- User plane latency

- Control plane latency

- [Reliability]

- Bandwidth

- Energy Efficiency / Sustainability

- Joint requirement on data rate, latency and reliability (to replace Reliability)

- Positioning

- Sensing-related capabilities

For some revised and new TPR, further details and reasoning is given above in Section 3.

In the attachment, a text proposal is given as input to a future TR on the Study on Scenarios and Requirements for IMT-2030, specifically for a chapter on Key Performance Indicators.

Based on the above we propose:

- That the attached text proposal is agreed to be included in the TR for the Study on Scenarios and Requirements for IMT-2030

- The attached draft LS is agreed to be sent to ITU-R WP5D as a response

# References

- “Working Document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to Document 5D/413 (Working Party 5D Chairman’s Report).

- “Minimum requirements related to technical performance for IMT-2030 radio interface(s); Liaison Statement to External Organizations”, Source: ITU-R WP5D, Tdoc RT-240015.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/218, 13 June 2024.

- “Proposals for the Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]”, Source: Telefon AB – LM Ericsson, Document 5D/387, 23 September 2024.

- ITU-R Report M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020radio interface(s)”.

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- “Working Document towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL]: Guidelines for evaluation of radio interface technologies for IMT-2030”, Annex 5.2 Document 5D/413 (Working Party 5D Chair’s Report).

- M.2412