3GPP TSG RAN meeting #106									           		RP-242652

Madrid, Spain, December 9-12th, 2024

## Status Report to TSG

Agenda item:			10.4.5

| WI / SI Name | New WID on introduction of IoT-NTN TDD mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_TDD |  |  |  |  |
| Unique ID | 1050123 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242415 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: Overall: 20% RAN1: 40% RAN2: 0% RAN4: 10% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Andjela Ilic-Savoia |
|  | Company | Iridium Satellite LLC |
|  | Email | andjela.savoia@iridium.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1 Agreements during RAN1#118-bis

Agreements to serve as baseline for evaluations and simulations, during study phase, to evaluate feasibility of DL sync

Agreement

The target operating DL CNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz:

- For LEO-600, the operating DL CNR is 4.91dB (i.e. with 0dBi UE antenna gain)

- For LEO-1200, the operating DL CNR is 5.51dB (i.e. with 0dBi UE antenna gain)

- When reporting link level simulation results, companies are encouraged to report the CNR margin with respect to the above operating DL CNRs.

- Companies may report values for other scenarios (e.g. UE antenna gain of -5.5 dBi, LEO-800) with the corresponding assumptions.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations (which includes DL synchronization as per the WID) that downlink PHY channels and signals are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the DL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and signals and eNB will not transmit the corresponding instances.

  - FFS: What is the TDD pattern

Agreement

RAN1 evaluates via link level simulations the following:

- NPSS/NSSS detection

- NPBCH decoding

Agreement

The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.):

- SIB1-NB

- Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31)

- Paging

- NPRACH / RAR

- NPSS/NSSS/NPBCH

- NPDCCH, NPUSCH, NPDSCH

- Note: Study for the above channels / signals may continue during the normative phase.

- It is up to companies to perform link level evaluations for some of the aspects above

Agreement

For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions:

| Parameter | Value |
| --- | --- |
| Carrier frequency | 1.6GHz |
| Channel model | NTN TDL-C rural, 30 degrees |
| Frequency error / timing drift (including XO error) | 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH |
| Variation of frequency error / variation of timing drift | 0.27ppm/s for NPSS / NSSS 0 for NPBCH |
| UE velocity | 3km/h |
| Target performance | For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability |
| Other assumptions (e.g. combining length, frequency error hypothesis, etc.) | To be reported and justified by companies |

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes):

- N = 9:

  - D = 8

  - D = 20

  - D = 30

- N = 8:

  - D = 20

- Companies to report which subframe indices are included in the set of consecutive downlink subframes.

- Other combinations are not precluded to be reported by companies

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 evaluates the following combination of N (in radio frames) and U (U = number of consecutive uplink subframes):

- N = 9:

  - U = 8

  - U = 20

  - U = 30

- N = 8:

  - U = 20

- Companies to report which subframe indices are included in the set of consecutive uplink subframes.

- Other combinations are not precluded to be reported by companies

- For evaluations including both uplink and downlink (if any), companies are encouraged to prioritize the case of D=U

NOTE: This does not imply that the above combinations are the only ones to be considered during the normative phase.

Agreement

For the study phase, RAN1 assumes as a baseline for evaluations that uplink PHY channels are transmitted on the anchor carrier following subframe index/location and SFN mapping of NB-IoT NTN FDD.

- For the purpose of study, the UL subframes of the Rel-19 TDD pattern may not fully cover all the instances of the PHY channels and eNB will not receive the corresponding instances.

  - FFS: What is the TDD pattern

#### 2.1.2 Agreements during RAN1#119

Agreements and observations stemming from evaluations of feasibility of DL sync and other aspects

Observation

RAN1 makes the following observations on downlink synchronization:

Case 1: For N=9, D=8:

- For NPSS:

  - For one-shot detection (90ms delay), 9 sources provided simulation results, with a median required SNR of 1.30dB. The link budget margin is (median margin):

    - 4.21 dB for LEO-1200 (0dBi antenna gain)

    - -1.89 dB for LEO-600 (-5.5dBi antenna gain).

  - For two-shot detection (180ms delay), 3 sources provided simulation results, with a median required SNR of -1.20dB. The link budget margin is (median margin):

    - 6.71dB for LEO 1200 (0dBi antenna gain)

    - 0.61dB for LEO-600 (-5.5dBi antenna gain).

  - For four-shot detection (360ms delay), 3 sources provided simulation results, with a median required SNR of -3.80dB. The link budget margin is (median margin):

    - 9.31dB for LEO-1200 (0dBi antenna gain)

    - 3.21dB for LEO-600 (-5.5dBi antenna gain).

  - NOTE: The assumptions for combining for 2-shot and 3-shot detection are different across companies.

- For NSSS:

  - For one-shot detection (180ms delay), 5 sources provided simulation results, with a median required SNR of -2.50dB. The link budget margin is (median margin):

    - 8.01dB for LEO-1200 (0dBi antenna gain)

    - 1.91dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin):

    - 2.03dB for LEO-1200 (0dBi antenna gain)

    - -4.07dB for LEO-600 (-5.5dBi antenna gain)

  - For 640ms combining, 6 sources provided results, with a median required SNR of  -4.25dB. The link budget margin is (median margin):

    - 9.76dB for LEO-1200 (0dBi antenna gain)

    - 3.66dB for LEO-600 (-5.5dBi antenna gain)

Case 2: For N=9, D=20:

- For NPSS:

  - For a combining of 2 consecutive NPSS (80ms delay), 1 source provided simulation results, with a median required SNR of -0.8dB. The link budget margin is (median margin):

    - 6.31dB for LEO-1200 (0dBi antenna gain)

    - 0.21dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 640ms combining, 1 source provided simulation results, with a median required SNR of -5.10dB. The link budget margin is (median margin):

    - 10.61dB for LEO-1200 (0dBi antenna gain)

    - 4.51dB for LEO-600 (-5.5dBi antenna gain)

Case 3: For N=9, D=30:

- For NPSS:

  - For combining of 3 consecutive NPSS (90ms delay), 1 source provided simulation results, with a median required SNR of -2.4 dB. The link budget margin is (median margin):

    - 7.91dB for LEO-1200 (0dBi antenna gain)

    - 1.81dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 640ms combining, 1 source provided simulation results, with a median required SNR of
 -7.10dB the link budget margin is (median margin):

    - 12.61dB for LEO-1200 (0dBi antenna gain)

    - 6.51dB for LEO-600 (-5.5dBi antenna gain)

NOTE: Other evaluated scenarios (LEO-600 with 0dBi antenna gain, LEO-1200 with -5.5dBi antenna gain, LEO-800 with 0dBi antenna gain) have margins in between LEO-1200 (0dBi antenna gain) and LEO-600 (-5.5dBi antenna gain)

NOTE: Different companies provided input for different cases. For the companies that evaluated multiple cases, the performance (in terms of acquisition delay) of Case 3 and Case 2 is better than the performance of Case 1.

Observation

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

Observation

In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view:

- DL synchronization, based on the link level evaluations submitted at RAN#119

- Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band

Agreement

Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints:

- At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods.

  - The same downlink slot is used in all the 90ms periods.

- At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods.

  - The same uplink slot is used in all the 90ms periods.

- The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4).

- NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

Observation

RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Conclusion

For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits.

- When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16:

  - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier.

  - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier.

  - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included.

- The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB.

Agreement

The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

  - Option 1-2: H-SFN duration is unchanged

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

  - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Agreement

RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-U NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of U NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes.

- Impact on segmented pre-compensation.

Agreement

RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-D NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of D NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes.

- The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

Agreement

The downlink subframes within D=8 are to be down-selected among the following:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

- Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames)

- Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames)

- Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

2.1.2	Remaining open issues

Study phase successfully completed and the normative phase can start. Topics to be considered are identified:

Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures.

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1 Agreements during RAN4#113 meeting:

General Aspect

Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

SNR, the above analysis focuses on the requirements for normal coverage.

RF Aspect

Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

RRM Aspect

Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demod Aspect

No observation in this meeting.

2.4.2 Remaining open issues

Study phase successfully completed and the normative phase can start.Topics to be considered are identified and listed below.

At least the following are identified as open issues:

Impact on SAN RF requirements

  - For the requirements to be impacted w.r.t  [249] band, NTN IoT Tx/Rx requirements in TS 36.108 will serve as a baseline (minimum specification impact has been identified). New requirements such as transient period, can be further discussed for SAN.

Impact on UE RF requirements:

  - The assessment of the impact on UE RF requirements (TS 36.102) was carried out and minimum impact has been identified. Requirements such as REFSENS can be further discussed for TDD NB-IoT UE.

Impact on performance requirements:

  - New demodulation and performance requirements shall be defined with respect to the new periodic pattern(s).

Impact on RRM requirements:

  - Some RRM requirement impact has been already identified, such as RLM, MSG3-based, cell re-selection, RRC re-establishment,  RRC connection redirection to non-anchor carrier, cell measurements) with respect to the new periodic pattern(s).

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1		SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN1

RAN1#118bis meeting, Heifei, China, October 14-18th, 2024:

- R1-2408400	Analysis - loT-NTN TDD mode DL sync	Iridium Satellite LLC

- R1-2408871	IOT-NTN TDD mode	Qualcomm Incorporated

- R1-2407772	Discussion on IoT-NTN TDD mode	THALES

- R1-2408736	On TDD mode for IoT-NTN	Ericsson

- R1-2407882	Discussion on IoT-NTN TDD mode	vivo

- R1-2407689	Discussion on IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2407725	Discussion on IoT-NTN TDD mode	Spreadtrum Communications

- R1-2407924	Discussion on the new NB-IoT NTN TDD mode	CMCC

- R1-2407937	Discussion on the IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R1-2407960	Discussion on IoT-NTN TDD mode	Xiaomi

- R1-2408037	Considerations on IoT-NTN TDD mode	CATT

- R1-2408078	Discussion on introduction of IoT-NTN TDD mode	SageRAN

- R1-2408139	Discussion on IoT-NTN TDD mode	OPPO

- R1-2408273	WP for Introduction of IoT-NTN TDD mode WI	Iridium Satellite LLC

- R1-2408302	Discussion on IoT-NTN TDD mode	LG Electronics

- R1-2408347	Discussion on IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

- R1-2408491	Discussion on IoT-NTN TDD mode	Apple

- R1-2408667	Discussion on IoT-NTN TDD mode	Samsung

- R1-2408874	Discussion on introduction of IoT-NTN TDD mode	SageRAN

- R1-2408919	On R19 TDD IoT-NTN	Nordic Semiconductor ASA

- R1-2409038	Feature lead summary#1 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2409039	Feature lead summary#2 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2408273 	Work plan for New WID on introduction of IoT-NTN TDD mode	Iridium Satellite LLC

RAN1#119 meeting, Orlando, USA, November 18-22th, 2024:

- R1-2409409	Discussion on IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2409440	Discussion on IoT-NTN TDD mode	THALES

- R1-2409531	Discussion on the new NB-IoT NTN TDD mode	CMCC

- R1-2409565	Analysis - loT-NTN TDD mode DL synchronization and SIB scheduling	Iridium

- R1-2409618	Discussion on IoT-NTN TDD mode	Samsung

- R1-2409654	Discussion on IoT-NTN TDD mode	Spreadtrum, UNISOC

- R1-2409702	Discussion on IoT-NTN TDD mode	vivo

- R1-2409730	Discussion on the IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R1-2409827	Discussion on IoT-NTN TDD mode	Apple

- R1-2409835	Discussion on IoT-NTN TDD mode	LG Electronics

- R1-2409849	Discussion on IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

- R1-2409887	Discussion on IoT-NTN TDD mode	Xiaomi

- R1-2409982	Evaluation on IoT-NTN TDD mode	CATT

- R1-2410083	Discussion on IoT-NTN TDD mode	OPPO

- R1-2410307	On TDD mode for IoT-NTN	Ericsson

- R1-2410366	Discussion on IoT-NTN TDD mode	Lenovo

- R1-2410499	IOT-NTN TDD mode	Qualcomm Incorporated

- R1-2410570	On R19 TDD IoT-NTN	Nordic Semiconductor ASA

- R1-2410687	Feature lead summary #1 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2410874	Feature lead summary #2 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

- R1-2409564	Work plan for WID: introduction of IoT-NTN TDD mode

## 4.2	RAN2

RAN2#128 meeting, Orlando, USA, November 18-22th, 2024:

- R2-2409694 Work plan for WID: introduction of IoT-NTN TDD mode, Iridium

## 4.3	RAN3

## 4.4	RAN4

RAN4#113 meeting, Orlando, USA, November 18-22th, 2024:

- R4-2417569	Discussion on IOT-NTN TDD mode in L-band and UE RF Requirements, Iridium

- R4-2419720	Discussion on the First Introduction of an NTN NB-IoT TDD Frequency Band, THALES

- R4-2417760	On IoT NTN TDD mode frequency error, Apple

- R4-2417772	RRM and demodulation considerations and simulation assumptions for IoT_NTN_TDD, Iridium

- R4-2418247	Initial assessment of the impact on RRM, Qualcomm Incorporated

- R4-2418484	Discussion on IoT NTN TDD mode, ZTECorporation,Sanechips

- R4-2418658	Discussion on RRN impact due to periodic pattern for IoT NTN, Huawei, HiSilicon

- R4-2419108	Initial discussions on IoT NTN TDD mode: Study of impact due to periodic pattern, Ericsson

- R4-2419154	Discussion on UE RF impact due to the periodic pattern, China Telecom

- R4-2419634	Discussion on the impact of periodic patterns of new TDD configuration in NTN, Nokia

- R4-2419604	Topic summary for [113][317] IoT_NTN_TDD Source: Moderator (Qualcomm)

- R4-2419795	Way Forward for [113][317] IoT_NTN_TDD Source: Moderator (Qualcomm)

- R4-2419663 	Work plan for WID: introduction of IoT-NTN TDD mode, Iridium

- R4-2419795	Way Forward for [113][317] IoT_NTN_TDD, Moderator (Qualcomm Incorporated)