3GPP TSG RAN Meeting #106	RP-242654

Madrid, Spain, December 9 - 12, 2024

Agenda Item:	10.4.5

Source: 	Iridium

Title:	Motivation for WID revision: Introduction of IoT_NTN TDD mode

Document for:	Information

# Introduction

In RAN#105 new WID on Introduction of IoT NTN TDD mode operating in L-band (MSS band 1616 -1626.5 MHz) was approved in [1]. This contribution explains motivation for the WID revision [2] and lists proposed changes to the WID text. The changes suggested here stem from work progress during two working groups meetings, the desire to allow for configuration flexibility and to improve the WID text clarity.

# List of proposed changes and justification

## Acronym and Unique Identifier

## The Acronym and Unique Identifier were assigned to the WID only after RAN105 Plenary in September. The first change is to add them into WID.

## Proposal 1: Add acronym and unique identifier
Acronym: IoT_NTN_TDD
Unique identifier: 1050123

## Changes in Table in the section 2.3	“Other related Work Items and dependencies”

## We propose change of text explaining “Nature of relationship” for the three of the listed related WIs, as below. We think that this rewording provides clarity and removes ambiguity on the relationship to these WI:

## LTE_NBIOT_eMTC_NTN (NB-IoT/eMTC support for Non-Terrestrial Networks)

Proposal 2: use three new “Nature of Relationship” descriptions for three WI referred to below:

Original “Nature of Relationship”: “The proposed WID will use the Release 17 specification resulting from this work as a baseline for the evolution”
Proposed new “Nature of Relationship”: “This WI enhanced the IoT usage via NTN in Rel-17 LTE”

## IoT_NTN_enh (IoT NTN enhancements)

Original “Nature of Relationship”: “The proposed WID will use the Release 18 specification resulting from this work as a baseline for the evolution”
Proposed new “Nature of Relationship”: “This WI enhanced the IoT usage via NTN in Rel-18 LTE”

## IoT_NTN_Ph3 (NTN for IoT Phase 3)

Original “Nature of Relationship”: “The proposed WID will use the Release 19 specification resulting from this work as a baseline for the evolution”
Proposed new “Nature of Relationship”: “This REL-19 WI IoT_NTN_Ph3 is enhancing IoT usage via NTN based on FDD, so it is a parallel enhancement to the present WI but no dependencies between WIs are expected”

## Justification section 3 – add a sentence

A sentence needs to be added to the start of the Justification section, as an introduction, to emphasize that prior to this work item, other IoT NTN efforts were FDD focused, and this WI is the first one with the TDD focus.

Proposal 3: add the following sentence at the beginning of section 3:
“The NTN version of NB-IoT (REL-17 WI LTE_NBIOT_eMTC_NTN, REL-18 WI IoT_NTN_enh, REL-19 WI IoT_NTN_Ph3) have all been defined assuming FDD mode”.

## Clarifying text in “4.1 Objective of SI or Core part WI or Testing part WI”

During RAN1 meetings, some companies suggested that the approach in the WID [1] could be useful for other values of N, to other companies or other bands in the future. Therefore suggesting (additions in blue):

Proposal 4: adding “at least” N=9 as a baseline under the 5th bullet in 4.1.

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline.

Further, we propose to change the following paragraph (see below) to support the configuration of the periodic pattern per radio cell, thus allowing for flexible deployment, rather than having the periodic pattern fixed per band. The pattern is illustrated in the Figure 1 below:

Figure 1 Definition of the periodic TDD-DL-UL-Pattern

Proposal 5: Change of text of the fifth bullet in section 4.1, to how it reads below, to allow flexibility and configurability; Additions are indicated in blue, and deletions are crossed off.

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. The value of N is fixed per band. Other parameters of the configuration of the periodic pattern (e.g. downlink to uplink guard period for TDD operation and the number of consecutive/contiguous DL and UL subframes) can be defined per cell.

Proposal 6: adding the bullet below, as it describes the minimum configuration to be supported for the deployment in the legacy system:

- For this target band N=9 and minimum of D=8 and U=8 to be supported.

Proposal 7: Remove the paragraph referring to Study phase under section 4.1, as that has been completed during October and November WGs meetings.

- Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4]

  - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24

Proposal 8: Rewording the paragraph that starts with ”This work item includes the following objectives:” under section 4.1 to how it reads below; Additions are indicated in blue, and deletions are crossed off.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, necessary adaptation and associated UE procedures [RAN1, RAN2]

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]
Note: RAN4 work to be aligned with RAN1 assumptions.

# References

[1]	RP-242415, “New WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

[2]   RP-242650, “Revised WID for introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024