3GPP TSG-RAN Meeting #106	RP-242762

Madrid, Spain, 9th – 12th December, 2024

Source:	Nokia

Title:	Confirmation of Feasibility Definition and progress of 6Rx

Agenda item:	9.3.4.1

Document for:	Discussion

Introduction

The ongoing work item on UE RF enhancements in Rel-19 [2] includes the following objective (highlighting added for emphasis):

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

RAN4#112 discussed the number of MIMO layers to consider for 6Rx, but was unable to conclude on the feasibility of supporting 6 layers. Some guidance is needed from RAN Plenary on the meaning of feasibility in order for RAN4 to be able to move forward.

Discussion

General considerations for MIMO layer evaluation for 6Rx UE

Tx EVM assumptions for 6-Layer Performance Evaluation

In RAN4#112-bis the topic of TxEVM was discussed, with the following described in the Way Forward [1] :

Issue 3-1-1: Tx EVM assumptions for 6-layer performance evaluation Way Forward: RAN4 to further discuss the assumption of Tx EVM for 6-layer performance evaluation and decide during performance part of work.

Nokia would like to add some clarification to this discussion and in particular would like to focus this with the ‘lens’ of ‘feasibility’.

Firstly, the definition of ‘feasibility’ from the “Shorter Oxford English Dictionary”[14] as defined in TR 21.801 [12] (3GPP Specification drafting rules) is as follows:

Capability of being done; practicability.

Therefore, for 6-layer MIMO to be feasible then it must have the ‘capability of being done’ in at least one scenario.

From a network perspective, a UE must be capable to work with a wide variety of RAN vendors in various configurations. If there is any configuration of the RAN whereby a 6-Layer UE is capable of supporting 6 Layer MIMO, then by definition 6-Layers is feasible.

- One factor that has been discussed in RAN4 is the TxEVM to be assumed, ranging among 6%, 3.5%, 2.5% or none, as has been captured in the WF documentation [1]. In practice, a wide range of values can be used, for example in small cell deployments, with multiple beams being produced collectively adding to TxEVM, the value can reach close to that seen in TS 38.104 [13] of 3.5% or in some cases higher (noting that 256 QAM would not be supported in those specific deployments). However, in scenarios (such as those employed by 6Rx devices) with low mobility, or even for Fixed Wireless Access, the RAN can be designed to support 6 layers with 8 Tx ports and minimize both TxEVM and BS antenna correlation, ensuring that a high SNR can be achieved in practice with minimal correlation across BS antenna elements. Therefore a TxEVM of either 1% or 0% can realistically be assumed for the assessment of feasibility, with no correlation between BS Tx ports.

Unlike 4-layers, a 6-layer MIMO configuration would have the benefit of utilizing two TBs and as such two codewords, such that higher maximum throughput at higher SNRs can be achieved which is beneficial for deployments such as FWA.

Simulation results can be observed in [15], where we can see that 6 Layer MIMO is indeed feasible and tracks well the performance of 4 Layers, offering higher performance at higher SNRs in a spatial channel such as CDL.

Since feasibility is defined as the capability of being done; these simulations show that unequivocally 6 Layer MIMO is indeed capable of being done, thus 6 Layer MIMO should be deemed feasible.

- RAN4 for several meetings now has attempted alignment under realistic deployment scenarios (noting that ‘realistic’ is a variable term amongst companies, with much disagreement); however progress has stalled. Full details of the simulation campaign aligned with the way forward can be found in [1].

We observed that several companies, including Qualcomm [9], Nokia [11], Oppo [8] had identified that indeed 6-layer MIMO was feasible on devices that were able to accommodate 6Rx, and some companies such as Huawei [10], showed that it was feasible, even if in their simulations they showed it had no gain over 4-layers in SNR performance.

However, a minority of companies were not content with the demonstrated feasibility of 6-layers in appropriate scenarios and seemed to consider that feasibility must be shown in all scenarios, thus preventing RAN4 from reaching consensus that 6 layers are feasible for 6 Rx.

Thus, Nokia would like to request the following:

- RAN Plenary is requested to provide a view on the definition of feasible or this work item (and more importantly the ethos of RAN4) will come to a halt.

Specifically, Nokia would be looking for clarification on the following aspects:

- How RAN and RAN4 collectively agrees feasibility (i.e. do all companies require to agree a feature is feasible)

- Does feasibility include gain over a legacy feature (in this case 6-layer vs 4-layer)

- Does the definition of feasibility correspond to feasibility in (1) one case, (2) a majority of cases or (3) all cases.

Conclusion

The paper discusses the issue of the feasibility of 6 Layer MIMO in the 6Rx work in RAN4.

In order to enable progress to be made, it would be helpful for RAN Plenary to give guidance to RAN4 that demonstration of feasibility by multiple companies in appropriate scenarios can be considered to confirm feasibility, so that RAN4 can conclude that 6 layers are feasible and move forward with the objective of the WI.

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024

- TS 38.822 v17.1.0 “NR; User Equipment (UE) feature list”, June 2023

- R4-072223, “TP for 36.803 for MIMO correlation matrices for LTE”, Motorola, Ericsson, Nokia, NEC, NXP, InterDigitial, Agilent, RAN4#45, Jeju, Korea, November 2007

- R4-080620, “High Dimension E-UTRA MIMO Correlation matrixes”, Ericsson, RAN4#46-bis, Shenzhen, China, April 2008

- R4-1708533, “Discussion on SI for 8Rx”, Huawei, RAN4#84, Berlin, Germany, Aug 2017

- R4-2311072, “Discussion on PDSCH Demodulation Requirements for 8Rx”, Nokia, Nokia Shanghai Bell, RAN4#108, Aug 2023

- R4-2412352, “R19 Simulation results of 6Layer performance”, OPPO, RAN4#112, Aug 2024

- R4-2411680, “Discussion on the demodulation performance requirements for 6Rx devices”, Qualcomm, RAN4#112, Aug 2024

- R4-2412572, “On MIMO layer evaluation for 6Rx UE”, Huawei, HiSilicon, RAN4#112, Aug 2024

- R4-2411525, “Discussion on MIMO layer evaluation for 6Rx UE, Nokia, RAN4#112, Aug 2024

- TR 21.801 v18.0.0 “Specification drafting rules”, March 2024

- TS 38.104 v18.7.0 “NR; Base Station (BS) radio transmission and reception”, Sept 2024

- Oxford English Dictionary, 2024 (https://www.oed.com/dictionary/feasibility_n)

- R4-2418011, “Simulation Results for on MIMO layer evaluation for 6Rx UE”, Nokia, RAN4#113, Orlando, US, Nov 24