3GPP TSG RAN Meeting #106																    	 	RP-242777

Madrid, Spain, December 9-12, 2024

Agenda item:	10.1.4

Source: 	Qualcomm Incorporated

Title: 	Low band low band switching

Document for:	Approval

# Introduction

A new WI to introduce a CA based DL switching scheme will be proposed in RAN#113. The motivation for this work was presented in [1]. In this paper we discuss some high level technical aspects related to this proposal.

# Discussion

The proposal for introducing a DL switching scheme with CA for low FDD bands is motivated by the need to pair low band DL spectrum with an UL band with similar coverage (also a low band). The scenarios of interest are presented in [1] and listed here below for convenience.

Spectrum holding scenarios to be considered for low band – low band switching in [1]

The scenario above can be categorized into two large groups as shown below.

- Band combinations for which DL and UL cannot be paired due to proximity of DL and UL spectrum. For such a case, a filtering based solutions is not practical/feasible. From the scenarios shown above, these would be n12-n29, n28-n67 and n26-n106.

- Band combinations for which the bands to be aggregated are relatively far in frequency and antenna implementation is very challenging. For such bands, RF front end parts(e.g. filters) are available, however, the antenna efficiency in all the aggregated spectrum parts would be too low. From the scenarios shown above, these would be n5-n29, n14-n29 and n29-n71.

For the two groups above, the switching schemes can be different and implementation could also be different.

For both groups, the basic switching scheme is between UL+DL(e.g. n12 UL+DL) <-> “unpaired” DL (e.g. n29DL).

For group 1 listed above, in the cases in which the DL bands are very close in frequency, as is the case with n29DL and n12DL, it could be possible to design an Rx filter covering both DLs. In this case, both DLs can be aggregated when the UE switches to DL only. Such a scheme has several advantages or the basic scheme described above because PCell DL is available all the time and available DL bandwidth is wider. Hence, this scheme should also be included if a work item for low band – low band DL switching is to be approved. For the case of n12-n29, this scheme would mean switching between n12 UL+DL and n12 DL + n29 DL.

Proposal 1. Switching between UL+DL and DL+DL should also be covered in the low band – low band DL switching work item.

For the combinations of group 1, switching will require the physical switching of front end filters. For the combinations of group 2, switching will also require antenna retuning besides the switching of front end filters. The longer switching delay will have to be taken into account when the switching schemes are designed.

Observation 1. Band combinations in group 2 will also require some antenna retuning when switching DLs, this could impact the switching delay.

The switching schemes will have some RAN1 impact also because the UE will not be active on all aggregated DLs in all the slots. It was proposed to reuse as much as possible the UL switching framework, however, there is a fundamental difference between the already defined UL switching and the proposed DL switching schemes. For UL switching, the DLs on all aggregated CCs are always available for transmissions to the UE and the impact to physical layer operations is minimal. For the DL switching schemes, the DL is not always available (even PCell DL is not always available) and the UE will not be able to monitor all DL slots on all CCs. This will create new issues in RAN1 and RAN4.

Observation 2. DL switching is different compared to UL switching as DL is not always available to schedule the UE. RAN1/4 impact compared to UL switching is expected to be different.

# Conclusion

In this paper we discussed the proposal to introduce a low band low band DL switching scheme to enhance the use of low band spectrum. We made the following proposal and observations:

Proposal 1. Switching between UL+DL and DL+DL should also be covered in the low band – low band DL switching work item.

Observation 1. Band combinations in group 2 will also require some antenna retuning when switching DLs, this could impact the switching delay.

Observation 2. DL switching is different compared to UL switching as DL is not always available to schedule the UE. RAN1/4 impact compared to UL switching is expected to be different.

# References

- R4-2417690, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, RAN4#113, AT&T et al.