3GPP TSG-RAN RP #106	RP-242780

Madrid, Spain, December 9-12, 2024

Agenda item:	9.3.4.1

Source: 	Qualcomm Incorporated

Title: 	Support of 6 MIMO Layers for 6Rx

Document for:	Discussion

# Introduction

The Rel-19 work item on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 addresses, among other topics, 6Rx for handheld and FW devices [1]. One goal is to specify the core requirements for higher frequency bands above 2.5 GHz. This includes to study the gain and feasibility and, if feasible, to support 6 MIMO layers. According to the work item description, support of 6 MIMO layers shall be introduced in Rel-19 for 6Rx for handheld and FWA devices, if the feasibility has been shown.

6Rx for handheld and FWA UE [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this paper, we provide a summary of the work that has been done so far in RAN4 regarding the feasibility analysis of 6 MIMO layers and share our interpretation of the status of the implementation feasibility.

# Discussion

RAN4 discussed in the past four meetings #111, #112, 112bis and #113 simulation assumptions for feasibility analysis and identified the impact of several key parameters on performance of 6 MIMO layers. The identified key parameters include

- UE Rx antenna correlation including antenna efficiency

- Tx EVM and antenna correlation in the base station

- DMRS configuration for 6 MIMO layers

The simulation assumptions for the feasibility study are listed in [2]. For the key parameters, there are still many different options being proposed, and no progress could be made in RAN4 #112bis and RAN4 #113 to align them further. It was shown in various simulations by all companies that the key parameters have a very strong impact on performance and all effects are well understood. Since there is a fundamental disagreement about the values for the key parameters, it cannot be expected that any further progress on the alignment can be made in the upcoming meetings. Hence, in our opinion it is not an efficient use of time to continue discussing simulation assumptions further in the next RAN4 meetings. Enough information is already available to decide whether 6 MIMO layers is feasible for handheld and FWA devices.

The first key parameter for the performance of 6 MIMO layers is the UE Rx antenna correlation. This correlation matrix depends on the antenna design, the distance and isolation between the antennas and therefore also on the formfactor of the device. Also, the carrier frequency matters and in general for higher carrier frequencies the correlation values will become smaller. Since this parameter depends significantly on the actual antenna design, the correlation matrix cannot be realistically determined by a parametric model but needs to be measured by UE vendors. Several companies provided measurements for the envelope correlation coefficients (ECC) [3], [4], [5]. It has been shown by several companies that the ECC matrices provided in [3], [4] yield the same performance and have a negligible performance loss to zero correlation. The matrices in [5] yield to a small degradation by 1 – 1.5 dB. Qualcomm has also done ECC measurements of the antenna design [6] but decided not to publish the measurements. The resulting performance is identical to the performance with the ECC matrices in [3] and [4]. Although ECC matrices with close-to-zero correlation impact have been measured by several UE vendors, feasibility is still questioned by some companies.

The second key parameter for the performance of 6 MIMO layers is the Tx EVM and the antenna correlation in the base station. Again, these values cannot be determined by any parametric model or by the minimum requirements in RAN4 but must be measured and confirmed for the feasibility analysis by base station vendors. Two base station vendors confirmed that a Tx EVM of 1% or 1.25% and zero antenna correlation can be assumed for the feasibility analysis [7], [8]. The values for Tx EVM are significantly lower than the minimum requirements for 256QAM and allow to achieve a SNR of 40 dB and 38 dB, respectively. Although low Tx EVM and close-to-zero correlation impact have been confirmed by main base station vendors, feasibility is still questioned by some companies.

The third key parameter is the DMRS configuration. Rel-15 Type 1 DMRS supports only four orthogonal ports for a single-symbol DMRS. Hence, to support six layers with Rel-15 Type 1 DMRS double-symbol DMRS needs to be configured which increases the DMRS overhead of six layers with respect to four layers. However, eType 1 DMRS has been introduced in the Rel-18 MIMO evolution work item for the purpose of reducing the DMRS overhead for more than four MIMO layers.  Although Rel-18 eType 1 DMRS has been introduced to reduce DMRS overhead for more than four MIMO layers, this assumption is questioned by some companies for feasibility analysis.

Based on the confirmation of feasibility of these key parameters, it has been shown by several companies that 6 MIMO layers provides a throughput gain at achievable SNR values and it is proposed by many operators to introduce 6 MIMO layers in Rel-19 as an optional feature for handheld and FWA devices [10]. However, some companies still question the feasibility and block the introduction of an optional feature, but it remains unclear what additional feasibility analysis these companies expect to see. Feasibility cannot mean that all companies regard the implementation of a feature as feasible since otherwise all progress can be blocked. Feasibility can also not mean that only deployed features must be considered for the analysis as the opposing companies do by saying that Rel-15 Type 1 DMRS must be the baseline for six layers. By that all progress of 5G specification in Rel-17/Rel-18 in 3GPP and potential upgrades in field deployments in the upcoming years is ignored and 5G will not make any progress till 6G availability.

Since several companies regard 6 MIMO layers as feasible and no further progress is expected in RAN4, we propose that RAN #106 decides to introduce 6 MIMO layers in Rel-19 as an optional feature. Since six MIMO layers is just an optional feature, we do not see any harm to include it in the Rel-19 specification in addition to many other optional features in 5G. It can be left to interested companies whether this feature is implemented and deployed or not.

Proposal 1: Introduce six layers for FWA devices as an optional feature in Rel-19.

Proposal 2: Introduce six layers for handset devices as an optional feature in Rel-19.

# Conclusions

Since several companies regard 6 MIMO layers as feasible and no further progress is expected in RAN4, we propose that RAN #106 decides to introduce 6 MIMO layers in Rel-19 as an optional feature.

Proposal 1: Introduce six layers for FWA devices as an optional feature in Rel-19.

Proposal 2: Introduce six layers for handset devices as an optional feature in Rel-19.

# References

- RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN #104, June 2024

- R4-2420376, WF on 6Rx requirements, RAN4 #113, November 2024

- R4-2412352, R19 Simulation results of 6Layer performance, OPPO, RAN4 #112, August 2024

- R4-2412572, On MIMO layer evaluation for 6Rx UE, Huawei, RAN4 #112, August 2024

- R4-2415681, Discussion and simulation results on MIMO layer evaluation for 6Rx UE, Samsung RAN4 #112bis, October 2024

- R4-2411680, Discussion on the demodulation performance requirements for 6Rx devices, Qualcomm, RAN4 #112, August 2024

- R4-2417559, Discussion on MIMO layer evaluation for 6Rx UE, Nokia, RAN4 #113, November 2024

- R4-2419550, Discussion on the MIMO layers for 6Rx UE, Ericsson, RAN4 #113, November 2024

- RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, RAN #98-e, December 2022

- R4-2418056, Six Layer Support for Devices with 6Rx in Rel-19, T-Mobile USA, Verizon, CMCC, BT, China Telecom, SK Telecom, Deutsche Telekom, Vodafone, Boost Mobile Network, NTT Docomo, Telecom Italia, AT&T, RAN4 #113, November 2024