3GPP TSG RAN Meeting #106			RP-242798

Madrid, Spain, December 9-12, 2024

Agenda item:	9.3.1.3

Title:	Discussion on Data Collection for UE Sided Model Training

Source:	OPPO

Document for:	Discussion and Decision

- Introduction

In RAN#105, a revised WI on AI/ML for NR air interface [1] was approved on beam management, positioning and CSI prediction. Data collection for UE-side model training are still pending for further study.

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

It is time to check the progress of the study in RAN#106 and make decision on whether to have normative work on this topic. In this contribution, we analyze the current status on this topic across all working groups and then give our suggestions on the way forward.

- Discussion on data collection for UE-sided model training

In RAN2#126 and RAN2#127, data collections for UE-side model training has been updated in TR 38.843 [2] as follow.

7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

RAN2 have conducted comprehensive analysis on the meaning of each option, especially on controllability and visibility aspects. Due to some dependency with SA2/SA5 on how each option works from SA point of view, RAN sent a LS to SA for clarification [3]:

To TSG SA, WG SA2, WG SA3, WG SA5

ACTION: 	TSG RAN kindly asks TSG SA to initiate the work and at least for SA WG2 to provide inputs on solutions 1b, 2, and 3 by RAN#106.

Observation 1: During RAN#105, RAN sent a LS to SA for clarification on options of data collection for UE-sided model training, RAN would like to revisit this topic in RAN#106 after receiving more inputs from SA.

During RAN2#128 meeting in Orlando, RAN2 has received more questions from SA2 and SA5 for further clarification on data collection for UE-sided model training [4][5]. It seems the options are still controversial from SA point of view. RAN2 replied the LSs from SA2 and SA5 during RAN2#128 meeting [6][7]. SA2 discussed the RAN2 reply LS, after further discussion, SA2 replied the RAN LS [8]:

1. Overall Description:

SA2 thanks TSG RAN for the LS on AIML data collection, RP-242389 (S2-2411455).

Regarding the action on SA2 from TSG RAN: “The standardized solutions for data collection (if standardized) need to meet the requirements above. TSG RAN kindly asks TSG SA to initiate the work and at least for SA2 to provide inputs on solutions 1b, 2, and 3 by RAN#106”, SA2 did not reach consensus on the feasibility, for any of the options outlined above, to meet RAN requirements specified in RP-242389 (S2-2411455).

A full evaluation of potential solutions for all UE side data collection options would require further study in SA2, and coordination with SA5, SA3, and RAN2.

2. Actions:

To TSG RAN and TSG SA

ACTION:  SA2 kindly asks TSG RAN and TSG SA to take the above into account.

Based on above SA2 reply LS, it’s quite obvious that SA2 is not ready to complete the study for each candidate option from RAN.

Observation 2: Based on the latest reply LS from SA2, it’s quite obvious that SA2 is not ready to complete the study for each candidate option from RAN.

From our understanding, it is very difficult for RAN to further down select those options for UE-sided model training without detailed analysis from SA, so the safer way in RAN is to suspend the study of all options until the end of Rel-19 before getting more inputs from SA.

Proposal: RAN will suspend the study of all options on data collection for UE-sided model training by the end of Rel-19 before getting more inputs from SA.

3. Conclusion

To draw attention from readers, allow us to repeat observations and proposals in what follows.

Observation 1: During RAN#105, RAN sent a LS to SA for clarification on options of data collection for UE-sided model training, RAN would like to revisit this topic in RAN#106 after receiving more inputs from SA.

Observation 2: Based on the latest reply LS from SA2, it’s quite obvious that SA2 is not ready to complete the study for each candidate option from RAN.

Proposal: RAN will suspend the study of all options on data collection for UE-sided model training by the end of Rel-19 before getting more inputs from SA.

4. References

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] R2-2407807, “CR on RAN2 inputs to TR 38.843”

[3] RP-242389, LS on AIML data collection

[4] S2-2411191 Reply LS on AIML data collection

[5] S5-246299 Reply LS on AIML Data Collection

[6] R2-2411152 Reply LS to SA2 on AIML data collection

[7] R2-2411114 Reply LS to SA5 on AIML data collection

[8] S2-2412726 Reply LS on AIML data collection