3GPP TSG RAN Meeting #106 	          	      		            	 RP-242807

Madrid, Spain, December 9-12, 2024

Agenda Item:   	9.3.2.2

Source:   	LG Electronics

Title: 	Discussion on NTN for NR Phase 3 in Rel-19

Document for:   	Discussion and decision

- Introduction

For the objective of DL coverage enhancements on NR-NTN WID [1], there are checkpoints that RAN1 should provide RAN plenary with the following yellow marked reports to be used in determining its normative work scope.

The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study […]

In this contribution, we will provide our view on the scope of normative work related to NR-NTN DL coverage enhancements.

- Discussion

The following list of system/link level candidate options was agreed upon by RAN1 to be considered for NR-NTN DL coverage enhancements.

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

Regarding system level enhancements, after further discussion in RAN1, it was agreed that RAN1 would support SSB periodicity extension with the maximum of the additional default value of at least 160ms during the initial access (see details below). Note that there was a lot of discussion about whether to support 320ms or lager as the maximum for additional default values, but RAN1 failed to reach a consensus on it. Given this situation, we believe that it is inefficient to continue debating whether to adopt the maximum of the additional default value greater than 160ms, and prefer not to consider it any further. Also RAN1 made the observation that the backward compatibility of legacy UEs assuming the default SSB periodicity of 20ms is not guaranteed within the cell where SSB periodicity greater than 20ms is used for the initial frequency scan, and that legacy UEs cannot camp on the cell with SSB periodicity greater than 160ms. This kind of impact on the backward compatibility resulting from introducing 160ms as the maximum of the additional default value will be taken into account by RAN1 in the design of the remaining details of this feature.

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

Additionally, as part of the system level enhancements, the proposal below to support the operation where wider beams are used for SSB/DL common channels, while narrower beams are used for DL/UL dedicated channels (including PRACH), was discussed, but its support was not agreed upon. We think that this feature could be deprioritized, considering that it will take a considerable amount of time to finalize even the remaining issues for other system/link level features that RAN1 has agreed to support.

Proposal 5-1-v1 Regarding wider beam for SSB/ transmitting the DL common channels, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint, RAN1 further studies the following candidate options: - Option 1: Network broadcasts a set of ROs associated with each narrow beams, and the UE chooses the RO based on the indicated information (e.g. reference Location, or elevation angle of reference Location) of the RO and its own location - Option 2: Network indicates in SIB narrow beam signal (CSI-based or NCD-SSB) and associated ROs, and the UE selects the RO based on measurements on indicated narrow beam RS - Option 3: Network implementation based on existing specification e.g. network detection based on PRACH repetitions. No spec change.

Proposal 1: For DL coverage system level enhancements, RAN1 starts the normative work of specifying SSB periodicity extension with the maximum of the additional default value of 160ms.

As link level enhancements, RAN1 has so far agreed to support repetitions on PDSCH with Msg4, PDSCH with SIB1, and PDCCH for CSS (except Type-3). Details can be found below.

Agreement Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3. - The following techniques are for further study: - PDCCH repetition, including: - Option 1: Intra-slot PDCCH repetition - Option 2: Inter-slot PDCCH repetition - CORESET length (i.e. number of OFDM symbols) extension - DCI format optimization (e.g. size reduction, etc) - Note: the same technique is intended to apply to all search space types targeted for link level enhancements - For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques: - Configuration - Backward compatibility and UE behaviour of legacy UE - Linked Search Space - Blocking probability - DCI format size budget - Resource overhead - Impact on CORESET0 - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS Agreement For PDSCH with Msg4 Link level enhancement: - Continue studying PDSCH repetition - Further discuss the specification impact for at least the following: - Procedure and signaling - Repetition factor - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

However, for PDSCH with SIB19, RAN1 failed to agree to support link level enhancements based on the proposal below due to different views among companies on its necessity. From our perspective, it would be okay with considering not having this feature, given the limited number of remaining RAN1 meetings.

Proposal 4-1b-v2 For link level enhancement of PDSCH with SIB19: - Support PDSCH repetition within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: the target coverage enhancement to bridge the gap is 1.1 dB for SIB19 - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

Proposal 2: For DL coverage link level enhancements, RAN1 starts the normative work of specifying repetitions of the PHY channels/signals listed below:

- PDSCH with Msg4

- PDSCH with SIB1

- PDCCH for CSS (except Type-3)

- Conclusion

In this contribution, we discussed the scope of normative work on NR-NTN DL coverage enhancements to be decided in the upcoming plenary meeting. The following proposals are given:

Proposal 1: For DL coverage system level enhancements, RAN1 starts the normative work of specifying SSB periodicity extension with the maximum of the additional default value of 160ms.

Proposal 2: For DL coverage link level enhancements, RAN1 starts the normative work of specifying repetitions of the PHY channels/signals listed below:

- PDSCH with Msg4

- PDSCH with SIB1

- PDCCH for CSS (except Type-3)

- Reference

[1] RP-241789, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Thales, CATT.