3GPP TSG RAN Meeting #106								                    RP-242832

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:		9.2.6

| WI / SI Name | Study on NR frequency range 2 (FR2) OTA (Over the Air) testing Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_FR2_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030074 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242400 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: Sep/2025 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 90% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Han, Bin |
|  | Company | Qualcomm Incorporated |
|  | Email | binhan@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#112bis

Default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing

- Agreements:

  - RAN4 confirms the baseline multi-RX/STxMP system is suitable to guarantee the same relative angular separations between specific UE test orientations and probes.

  - The following test procedure can be considered as the solution to select default AoA separation and UE orientation EIRP PUMAX,f,c,k testing:

    - AoA1 direction for STxMP testing should re-use the legacy beam peak direction (from single carrier test) for the declared UE orientation (selected from any of the approved STxMP device orientations

      - For AoA1, no new BP search is necessary for device orientations used for STxMP test that have not been used for single carrier test

    - AoA2 direction is offset by AoA1 by the declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA

- Agreements:

  - The test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA if the test results based on the default AoA separation fail the limits in the spec. The following options can be considered:

    - Option 1: A local scan in xz plane is conducted. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the legacy beam peak EIRP limit in both directions. FFS on the measurement grid of local scan.

    - Option 2: Alternative test directions on top of the default AoA separation can be declared by UE. FFS on the allowed offset of alternative test directions to the default test directions.

  - Option 1 and Option 2 only apply for EIRP PUMAX,f,c,k testing and will not have the impact on the EIRPmax and TRPmax test directions selection.

  - RAN4 to make down selection from Option 1 and Option 2 in next meeting

AoA separation and UE orientation EIRPmax testing

- Agreements:

  - The following two sets of AoA separation and UE orientation needs to be tested for EIRPmax compliance

    - The default AoA separation and UE orientation declared by UE for EIRP PUMAX,f,c,k testing.

    - For EIRPmax test, one of AoA is at the beam peak direction of single CC operation without STxMP enabled, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and 30deg AoA separation.

Additional margin for skipping rule for EIRPmax testing

- Agreements:

  - 0.5dB as an additional margin for EIRPmax test skipping rule.

AoA separation and UE orientation TRPmax testing

- Agreements:

  - Default AoA separation and UE orientation for EIRP PUMAX,f,c,k testing should be reused for TRPmax testing.

Additional margin for skipping rule for TRPmax testing

- Agreements:

  - 0.5dB as an additional margin for TRPmax test skipping rule.

Measurement grid for TRPmax testing

- Agreements:

  - Reuse the legacy TRP grid for TRPmax testing of STxMP

Link polarization combinations

- Agreements:

  - Further discuss the link polarization combinations for EIRP PUMAX,f,c,k, EIRPmax and TRPmax testing.

  - The following options can be considered:

    - Option 1: Limit the link polarization combinations for the 2AoA, and crossed link polarization (‘’ and ‘’) can be considered for the sake of lower interference

    - Other options are not precluded.

Work split for TR 38.771

| Section | Title | Company |
| --- | --- | --- |
| 5.1 | General | Qualcomm |
| 5.2 | Measurement setup | Keysight |
| 5.3 | Test system aspects | Keysight |
| 5.4 | Test procedure | vivo, Samsung, Qualcomm |
| Annex A | Measurement uncertainty budget for STxMP UE RF testing methodology | Keysight |

RAN4#113

Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA

- Agreements:

  - Alternative test directions rather than the default AoA1 to be declared by UE.

    - The declaration of AoA1 direction can be any direction. If UE doesn’t declare AoA1, the default AoA1, i.e., the beam peak direction of single CC operation without STxMP, will be applied.

    - AoA2 direction is to use AoA1 + or - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

    - AoA1+ UE declared offset or AoA1 - UE declared offset needs to be declared by UE

Ambiguity in AoA2 selection between AoA+offset and AoA-offset

- Agreements

  - If UE doesn’t declare AoA1, the default AoA1, i.e., the beam peak direction of single CC operation without STxMP is applied, the following AoA2 directions should be tested. If any of the two directions pass EIRP PUMAX,f,c,k  requirements, the DUT pass the test.

    - AoA2 direction is AoA1 + the declared offset

    - AoA2 direction is AoA1 - the declared offset

  - If UE declare the alternative AoA1, only one AoA2 direction should be tested.

    - AoA2 direction is either AoA1+ the declared offset or AoA1- the declared offset according to UE declaration

Skipping rule for EIRPmax and TRPmax testing

- Agreements:

  - The test methodology defined in the TR 38.771 only applies for STxMP testing with two-layer transmission (i.e., one layer per TCI).

  - The test methodology and skipping rule for single layer transmission might be different from two-layer transmission case.

  - Test methodology for single layer transmission needs to be further studied in further release.

Link polarization combinations for STxMP testing

- Agreements

  - Limit the link polarization combinations, i.e., transmission polarizations from TE, to ‘’ and ‘’.

#### 2.4.2	Remaining Open issues

The remaining issues need to be solved:

- Preliminary uncertainty assessments

- Complete TPs to TR38.771

## 4.	References

RAN4 #112bis

- R4-2415308	Discussion on FR2 OTA test method on STxMP		Qualcomm

- R4-2415564	On FR2 OTA testing of STxMP					Samsung

- R4-2415760	Discussion on the FR2 sTxMP OTA testing			vivo

- R4-2416476	On absolute probe locations					Keysight

- R4-2416546	Topic summary for [112bis][325] NR_FR2_OTA		Qualcomm

- R4-2416580	Way Forward for [112bis][325] NR_FR2_OTA		Qualcomm

RAN4#113

- R4-2418164	TR 38.771 v0.1.0 for study on FR2 OTA testing Phase 3	Qualcomm

- R4-2419809	TP to TR 38.771 on the test procedure of sTxMP		vivo

- R4-2418179	Discussion on the FR2 OTA remaining issue			vivo

- R4-2418343	On remaining issues of FR2 STxMP testing			Samsung

- R4-2418846	Views on FR2 OTA test method on STxMP			Qualcomm

- R4-2418847	Reply LS on FR2 UE Beamlock test function			Qualcomm

- R4-2419398	On STxMP Test Directions					Keysight

- R4-2419614	Topic summary for [113][327] NR_FR2_OTA			Qualcomm

- R4-2419810	Way Forward for [113][327] NR_FR2_OTA			Qualcomm

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template