3GPP TSG RAN Meeting #106　   　　                                                                  RP-242842

Madrid, Spain, December 9-12, 2024

Status Report to TSG

Agenda item:			9.3.4.5

| WI / SI Name | # Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NonCol_intraB_ENDC_NR_CA_Ph2 |  |  |  |  |
| Unique ID | 1030082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242083 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2025 | Performance part: 03/2026 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 45% (The WI has been started from Q2 2024) | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colours below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Yasuki Suzuki |
|  | Company | KDDI |
|  | Email | ui-suzuki@kddi.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#112-bis

UE RF requirement

< Issue 2-1: How to differentiate UE Capability Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC >

- Legacy capabilities, such as MIMO-CapabilityDL-r10 on E-UTRA carriers.

< Issue 2-2:  UE Capability Type 4a(6Rx/UE) for NR-CA >

- Do not consider Type 4a for NR-CA.

< Issue 2-5:  Switch between Type 4a/4b and single CC operation >

- No new NW signaling needed from Type 4 to single carrier fallback.

< Issue 2-7:  When to inform RAN2 the demand on new UE capability(s) and new BS signaling >

- After RAN4 finish the work according the WID including all UE capability and NW signalling, the RAN4 will send the decisions related into RAN2 on one go.

< Issue 3-3:  Harmonized approach on UE Type definition with RRM part >

- It would be better to have the RRM CRs after the RF CRs in order to ensure a harmonized approach.

< WF >

- WF on Rel-19 non-collocated scenario is approved. [1]

- WF on rest issues for intra-band non-collocated EN-DC/NR-CA is approved. [2]

RRM requirement

< Issue 1-1: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19 >

- Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

< Issue 1-2: Whether to introduce MRTD/MTTD requirements for FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in Rel-19 >

- On whether or not to introduce MRTD/MTTD requirements for FR1 inter-band asynchronous EN-DC with overlapping DL bands for TDD-TDD band combinations in Rel-19, apply the same agreement as for Rel-18.

< Issue 1-4: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition >

- When Type 4 capable UE operates in non-collocated condition, interruption requirements (below listed) defined for type 2 UE in non-collocated condition in R18 can be applied

  - Interruption at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

< Issue 1-5: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition >

- When Type 4 capable UE operates in collocated condition, SCell activation delay defined for type 2 UE in collocated scenario in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

< Issue 1-6: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition >

- When type 4 capable UE operates in collocated scenario, scheduling availability defined for type 2 UE in collocated scenario in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability and network signalling. The specific UE capability and network signalling can wait for the RF session conclusion.

< Issue 1-7: SCell BFD/CBD requirements for Type 4 capable UE >

- Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band.

< Issue 1-3: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE >

- The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE

  - Interruption

  - SCell activation delay

  - Scheduling availability

  - SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

< WF >

- WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2 is approved. [3]

RAN4#113

UE RF requirement

< Issue 2-3-2: Reference UE architecture >

- No need to discuss the table in the future RAN4 meetings.

  - The table in WID is not reference point for RAN4.

< LS >

- LS to RAN5 on frequency separation for type 4 UE is endorsed. [4]

< WF >

- WF on non-collocated intra-band EN-DC and NR-CA [5]

RRM requirement

< Issue 1-1: When to update RRM requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19 >

- Discuss the CR work split in this meeting.

- Companies can submit draft CRs from the next meeting.

< WF >

- WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2 [6]

2.4.2	Remaining Open issues

UE RF requirement

- UE Capability

  - UE Capability Type definition for EN-DC and NR-CA

- UE behavior and BS signalling for Type 4 EN-DC/NR-CA

  - New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA)

  - New BS Signalling to switch between Type 4a/4b and Type 1(collocated)

  - UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signaling

RRM requirement

- None. Discuss the CR work split in next meeting.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R4-2417114	WF on Rel-19 non-collocated scenario, KDDI

- R4-2417115	WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI

- R4-2416850	WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2, Huawei

- R4-2418579	LS on frequency separation for type 4 UE, Apple

- R4-2420399	WF on non-collocated intra-band EN-DC and NR-CA, KDDI

- R4-2420090	WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2, Huawei