3GPP TSG RAN Meeting #106	RP-242858

Madrid, Spain, December 9-12, 2024

Source:	China Telecom

Title:	Motivation for introducing power class 2 RedCap for FDD bands

Agenda Item:	9.4.3.3

Document for:	Discussion

- Introduction

In RAN#104, the new WID on NR power class 2 RedCap (Reduced Capability) UE in FR1 was approved [1] with the following objectives captured:

- Apply the PC2 RF requirements for non-RedCap UE to RedCap UE, based on 1Tx architecture with 26 dBm PA.

  - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed.

  - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way.

    - Note: Not differentiate the applicable form factors for TDD bands in the specification.

  - Check at RAN #106 on whether to define PC2 RF requirements for FDD bands and the applicable form factors for FDD bands (if defined).

Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI.

This contribution discusses enabling PC2 RedCap for FDD bands as highlighted in the objectives.

- Discussion

Whether to enable PC2 RedCap for FDD bands

The work plan [2] suggests to consider pre-discussion for the feasibility study to enable PC2 RedCap for FDD bands in RAN4#113 meeting before the checking point in coming RAN meeting. Also the draft CRs [3][4] for TDD bands has been endorsed in last RAN4 meeting, thus this contribution proposes the request for FDD bands.

- 3GPP RAN4#113 (Nov. 2024)

  - Further discussion and endorse the draft CR to cover any open issues for PC2 RedCap for TDD bands

  - Initial discussion for the feasibility study to enable PC2 RedCap for FDD bands

Typically, the RedCap UE and non-RedCap normal UE are operated in the same network and on the same carrier. It is not constraint that RedCap UE works on the frequency bands. On the contrary, RedCap UE and non-RedCap UE share the same frequency bands is a potential commercial deployment solution. For non-RedCap normal UE, the RF requirements for power class 2 (PC2) with 3dB higher conducted output power have been specified for a few TDD and FDD bands. In addition, the antenna performance for RedCap UE may not enable good radiated power due to the size and cost constraints.

Observation 1: The RedCap UE and non-RedCap normal UE are operated in the same network with same frequency band.

Observation 2: For non-RedCap normal UE, both FDD band and TDD band support high power transmission, The 1Tx or 2Tx architectures depend on UE implementation.

Therefore, to extend cell coverage for NR deployment as well as improve the data rate of cell edge RedCap UE, not only TDD bands but also FDD bands need to increase the maximum transmit power from PC3 to PC2.

As seen, from the RAN4 specification perspective, only PC2 is targeted in the new WID, thus it is suitable to make PC2 RedCap also apply for FDD bands.

Proposal 1: From network deploy point, it is proposed to enable PC2 RedCap UE for FDD bands in Dec. RAN meeting.

- Conclusions

In this contribution, we discuss enabling PC2 RedCap for FDD bands and have the following proposals:

Proposal 1: From network deploy point, it is proposed to enable PC2 RedCap UE for FDD bands in Dec. RAN meeting.

- References

RP-241665, New WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek, RAN#104, June, 2024

R4-2412465, Work plan on power class 2 RedCap in FR1, China Telecom

R4-2417086, Draft CR to 38.101-1 power class 2 RedCap for TDD bands, China Telecom, ZTE, CATT, China Unicom, Huawei, HiSilicon, Ericsson

R4-2420357, (NR_PC2_RedCap_UE) CR to 38.101-1: PC2 Redcap support for TDD bands, Ericsson