3GPP TSG RAN Meeting #106						RP-242873

Madrid, Spain, December 9-12, 2024

Agenda item:	16

Source:	Huawei, HiSilicon, CAICT

Title:	Scope of RAN-level SID focusing on ITU IMT-2030 and initial considerations on IMT-2030 TPRs

Document for:	Discussion

Background

According to the endorsed 3GPP 6G timeline in RP-240823 [1], RAN Plenary in December 2024 will start the Study Item Rel-20 focusing on ITU-R IMT-2030. At last RAN#105 meeting, input contribution RP-242022 [2] introduced the status of work on IMT-2030 in ITU-R WP5D. An LS has been sent from ITU-R WP 5D to 3GPP in [7], indicating that WP 5D plans to preliminarily agree on the list of minimum Technical Performance Requirements (TPRs) for IMT-2030 radio interface(s) at its #48 meeting in February 2025, and start discussion of target values from WP 5D#49 meeting in June 2025. The ITU-R report of IMT-2030 TPR is expected to be completed in WP 5D#51 in February 2026. To facilitate such discussion, this document provides our proposal on the scope of the 3GPP relevant study and the initial consideration on candidate items and target values.

Consideration on the scope of 3GPP study item for IMT-2030

For the first RAN-P SI to be approved in December 2024, the focus shall only be on ITU-R related matters, in particular with respect to IMT-2030 TPRs. The output of this RAN-P SI can be documented in a 3GPP external TR (i.e. a 9xx TR), similar as TR38.913 for IMT-2020. The specific discussion can be handled by the “ITU-R ad hoc” agenda and in the ITU-R ad hoc session. The scope of such discussion in Q4 2024 is limited to IMT-2030 TPR.

The SI aims at least developing a high-level 3GPP view on IMT-2030 TPRs, which can be used to convey 3GPP’s view to the ITU before the finalization of the TPRs at WP5D. Given that this RAN-P SI is only for the duration from December 2024 to June 2025, while the WP5D discussion will continue till Feb 2026, it is expected that the work on ITU-R IMT-2030 TPRs can be continued (if necessary) within the second RAN-P SI to be approved in March 2025.

Proposal 1: Regarding the first RAN-P SI on IMT-2030 to be approved in December 2024:

- Its scope shall only focus on ITU-R IMT-2030 related work, i.e. IMT-2030 TPRs and corresponding deployments;

- Any remaining work from the first RAN-P SI on IMT-2030 after Jun. 2025 can be continued in the second RAN-P SI;

- A 3GPP external TR is used to document the output of this study;

- Handled by the ITU-R ad hoc agenda and in the ITU-R ad hoc session.

Proposed candidate items

Table 1: Proposal on candidate items for minimum technical performance requirements for IMT-2030 radio interface(s) in ITU-R WP 5D

|  | Candidate IMT-2030 TPRs under discussion in WP 5D [3] |  |  | TPRs in ITU IMT-2020 [4] | Proposed to be defined as ITU IMT-2030 TPR | Proposed not to be defined as ITU IMT-2030 TPR |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate |  |  | Yes | X |  |
| 2 | Peak spectral efficiency |  |  |  | X |  |
| 3 | User experienced data rate |  |  |  | X |  |
| 4 | 5th percentile user spectral efficiency |  |  |  | X |  |
| 5 | Average spectral efficiency |  |  |  | X |  |
| 6 | Area traffic capacity |  |  |  | X |  |
| 7.1 | Latency | User plane latency |  |  | X |  |
| 7.2 |  | Control plane latency |  |  | X |  |
| 8 | Connection density |  |  |  | X |  |
| 9 | Sustainability/Energy efficiency |  |  |  | X (1) |  |
| 10 | Reliability |  |  |  | X |  |
| 11 | Mobility |  |  |  | X |  |
| 12 | Mobility interruption time |  |  |  | X |  |
| 13 | Bandwidth |  |  |  | X |  |
| 14 | Sustainable data rate |  |  | No | Combined with energy efficiency |  |
| 15 | XR connection density/connection capacity |  |  |  | To be down selected/ combined as one TPR item, named e.g. “Immersive system/communication capacity” (2) |  |
| 16 | XR area capacity |  |  |  |  |  |
| 17 | XR area efficiency |  |  |  |  |  |
| 18 | Joint requirement on data rate, latency, and reliability |  |  |  |  |  |
| 19 | Joint requirement of data rate, latency, reliability and capacity |  |  |  |  |  |
| 20 | Positioning |  |  |  | X (3) |  |
| 21 | Sensing-related capabilities |  | Detection/False alarm probability |  | X |  |
|  |  |  | Localization accuracy |  | X |  |
|  |  |  | Velocity accuracy |  | X |  |
|  |  |  | Sensing resolution |  | X |  |
|  |  |  | Reconstruction accuracy |  | X (4) |  |
|  |  |  | … |  |  |  |
| 22 | AI-related capabilities |  | Qualitative metric: AI service functionality requirements |  | X (4) |  |
|  |  |  | Quantitative metric: AI service accuracy |  | X (4) |  |
|  |  |  | Quantitative metric: AI service latency |  | X (4) |  |
|  |  |  | Quantitative metric: AI service density |  | X (4) |  |
|  |  |  | … |  |  |  |
| 23 | Security and Resilience |  |  |  |  | X (5) |
| 24 | Interoperability |  |  |  |  | X (5) |
| 25 | Coverage |  |  |  | FFS (6) |  |

(1) Focus on Energy efficiency as in ITU IMT-2020. Propose to define separate items for Network energy efficiency and Device energy efficiency as definitions in annex 1.

(2) Candidate items #15 to #19 are similar, because they are all expanding legacy TPRs (connection density, area capacity, or reliability) to additionally evaluate new use case/traffic model, e.g. XR (in all proposals), digital twin and other relevant use case (in some proposals).

(3) Note: It is not intended to restrict the implementation details of IMT-2030 technology on how to meet these requirements. For example, setting “positioning accuracy” as performance requirements for IMT-2030 radio interface technology doesn’t imply that LMF (Location management function) is required in IMT-2030 RAN. LMF is located in core network of NR technology. This is up to implementation details of IMT-2030 technology.

(4) See definitions in annex 1.

(5) Security and Interoperability are important design principle for communication system. However, they are not appropriate to be the technical performance requirement of radio interface technology.

(6) Depends on its definition, coverage may relate to network deployment which is beyond the scope of technical performance requirement of radio interface technology

Proposal 2: To adopt the candidate items in above table 1 for minimum technical performance requirements for IMT-2030 radio interface(s), and preferably provide feedback to ITU-R WP 5D#48 meeting by February 2025.

Preliminary views on target values

Table 2: Preliminary views on target values for minimum technical performance requirements for IMT-2030 radio interface(s) in ITU-R WP 5D

| Proposed candidate items for ITU IMT-2030 TPR |  |  | Preliminary views on target value | Comments |
| --- | --- | --- | --- | --- |
| Peak data rate |  |  | DL: 200, UL: 100Gbps | 10 x IMT-2020 |
| Peak spectral efficiency |  |  | DL: 60~90, UL: 30~45bps/Hz | 2~3 x IMT-2020 |
| User experienced data rate |  |  | DL: 1000, UL: 500Mbps | 10 x IMT-2020 |
| 5th percentile user spectral efficiency |  |  | Indoor Hotspot 0.9-1.5 (DL), 0.63-1.05 (UL) Dense Urban 0.675-1.125, 0.45-0.75 Rural 0.36-0.6, 0.135-0.225 | 3~5 x IMT-2020 |
| Average spectral efficiency |  |  | Indoor Hotspot 27-45 (DL), 20.25-33.75 (UL) Dense Urban 23.4-39, 16.2-27 Rural 9.9-16.5, 4.8-8 bps/Hz | 3~5 x IMT-2020 |
| Area traffic capacity |  |  | 100 Mbit/s/m2 @ Indoor Hotspot | 10 x IMT-2020 |
| latency | User plane latency |  | 0.5-1ms for HRLLC 1ms for Immersive communication | 1/2-1 x IMT-2020 |
|  | Control plane latency |  | 10ms | 1/2 x IMT-2020 |
| Connection density |  |  | 107 - 108 devices/km2 for IoT traffic | 10-100 x IMT-2020 |
| Energy efficiency |  |  | Target value TBD |  |
| Reliability |  |  | 1-10-6 - 1-10-7 for HRLLC | 10-100 x IMT-2020 |
| Mobility |  |  | Target value TBD |  |
| Mobility interruption time |  |  | 0ms |  |
| Bandwidth |  |  | At least 200MHz | 2 x IMT-2020 |
| “Immersive system/communication capacity” |  |  | 500-1000 users/km2 for XR use case | Corresponding to 6-12 users per cell of ISD=200m |
| Positioning |  |  | Horizontal accuracy: [20cm] @90% for indoor, [3-5m] @90% for outdoor Vertical accuracy: [1-3m] @90% for indoor |  |
| Sensing-related capabilities |  | Detection/False alarm probability | Target value TBD | Awaiting the outcome of channel model study |
|  |  | Localization accuracy |  |  |
|  |  | Velocity accuracy |  |  |
|  |  | Sensing resolution |  |  |
|  |  | Reconstruction accuracy |  |  |
| AI-related capabilities |  | Qualitative metric: AI service functionality requirements | Target value TBD |  |
|  |  | Quantitative metric: AI service accuracy |  |  |
|  |  | Quantitative metric: AI service latency |  |  |
|  |  | Quantitative metric: AI service density |  |  |

In previous IMT-2020 discussion the values adopted are several times higher than IMT-Advanced, e.g. requirements on cell average/cell edge spectral efficiency are 3 x IMT-A, which is to show the advanced capabilities for a new generation compared with the old ones. The values in table 2 are suggested by the same principle. The relation between these target values and the real network performance was mentioned in previous discussion, e.g. due to the limitation of operating conditions (availability of cell sites, spectrum etc.), it may be difficult for real network to achieve downlink user experience data rate > 100Mbps as required by ITU IMT-2020 TPRs. This is true and has been clarified in ITU document with following text in section 4 of IMT-2020 TPR [4].

“These requirements are not intended to restrict the full range of capabilities or performance that candidate RITs/SRITs for IMT-2020 might achieve, nor are they intended to describe how the RITs/SRITs might perform in actual deployments under operating conditions that could be different from those presented in other ITU-R Recommendations and Reports on IMT-2020.”

According to the working method in ITU-R, TPRs are intended for the evaluation of candidate technology proposals (RITs/SRITs: Radio interface technologies/Set of radio interface technologies) on the fulfilment of ITU IMT-2030 requirements. It is conducted with associated evaluation methodology of simulation which may not be reflecting the real network conditions.

Observation: TPRs are intended for the simulation evaluation of candidate technology proposals on the fulfilment of ITU requirements. They are not intended to describe how the RITs/SRITs might perform in actual deployments.

Proposal 3: To further discuss the target values in above table 2 targeting a generational performance enhancement of IMT-2030 over incumbent IMT systems.

Conclusion

Proposal 1: Regarding the first RAN-P SI on IMT-2030 to be approved in December 2024:

- Its scope shall only focus on ITU-R IMT-2030 related work, i.e. IMT-2030 TPRs and corresponding deployments;

- Any remaining work from the first RAN-P SI on IMT-2030 after Jun. 2025 can be continued in the second RAN-P SI;

- A 3GPP external TR is used to document the output of this study;

- Handled by the ITU-R ad hoc agenda and in the ITU-R ad hoc session.

Proposal 2: To adopt the candidate items in above table 1 for minimum technical performance requirements for IMT-2030 radio interface(s), and preferably provide feedback to ITU-R WP 5D#48 meeting by February 2025.

Observation: TPRs are intended for the simulation evaluation of candidate technology proposals on the fulfilment of ITU requirements. They are not intended to describe how the RITs/SRITs might perform in actual deployments.

Proposal 3: To further discuss the target values in above table 2 targeting a generational performance enhancement of IMT-2030 over incumbent IMT systems.

References

- RP-240823, Additional Considerations for 6G Timeline, TSG Chairs, 3GPP TSG#103 Joint Session, March 2024

- RP-242022, Status of work on IMT-2030 in ITU-R WP5D, Ericsson, Nokia, Interdigital, Huawei, Samsung, ZTE, Intel Corporation, TSG RAN#105, September 2024

- “Working document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to Working Party 5D Chairman’s Report.

- Report ITU-R M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”.

- R1-2009433, Evaluation results for Rel-16 positioning and Rel-17 enhancement, Huawei, HiSilicon, 3GPP TSG RAN1#103-e, October, 2020

- Report ITU-R M.2412-0 (11/2017), “Guidelines for evaluation of radio interface technologies for IMT-2020”.

- RP-243202 Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG#106, December 2024

Annex 1: proposed new TPR definitions for IMT-2030

XR connection density/connection capacity

XR connection density/connection capacity is the total number of devices fulfilling a specific quality of service (QoS) of XR (Extended Reality) use cases per unit area (per km2).

Connection density should be achieved for a given bandwidth and number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability, as specified in Report ITU-R M.[IMT-2030.EVAL].

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirement for XR connection density/connection capacity is TBD (e.g. [500-1000]) devices per km2.]

Sensing-related capabilities

- Detection/False alarm probability: Detection probability refers to the probability of correctly detecting an event when the event representing the characteristics of a target object or environment occurs actually. False alarm probability refers to the probability of an event is detected but detected event representing the characteristics of a target object or environment does not occurs actually.

- Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value.

- Velocity accuracy refers to the closeness of the measured sensing result of velocity of the target object’s velocity to its true velocity.

- Sensing resolution refers to the minimum difference in the measured magnitude (e.g. range, angle, Doppler, and location) of any two distinguishable target objects.

- Reconstruction accuracy refers to the closeness of the reconstructed scatter points set to the real scatter points set of sensing target, measured by e.g. Root Mean Square Error (RMSE)

AI-related capabilities

- AI service functionality requirements [Qualitative metrics]

This requirement is defined for the purpose of evaluation in the usage scenario of artificial intelligence and communication.

The minimum requirements for AI service functionality requirements are that the candidate RITs/SRITs shall have mechanisms and/or signalling related to at least [N] of functionalities including distributed data processing, distributed learning, AI computing, AI model execution, AI model inference as exposed capabilities to external applications, [AI to improve internal-IMT communication performance], or any other functionalities which the proponent(s) of candidate RITs/SRITs consider relevant to better supporting AI enabled applications*.

*Note: In this case, the fulfilment of requirement is subject to the discussion in WP 5D.

- AI Service Accuracy [Quantitative metrics]

AI service accuracy relates to the capability of processing a given AI inference/learning task with high accuracy within a predetermined time duration.

AI service accuracy is the relative accuracy of AI inference/learning operation compared to the reference case where no data transmission or latency constraint is involved in the AI inference/learning operation, for the given AI inference/learning task, as specified in report [ITU-R.Eval].

This requirement is defined for the purpose of evaluation in the usage scenario of artificial intelligence and communication.

The minimum requirement for AI service accuracy is higher than TBD (e.g. 95%~99%) of relative accuracy within TBD (e.g. 5~15ms) for the [Dense Urban] test environment, assuming given AI inference/learning task.

- AI Service latency [Quantitative metrics]

AI service latency is the time from beginning to the end of a given AI inference/learning task with a target service accuracy.

The target service accuracy is to process an AI inference/learning operation with a certain relative accuracy, for the given AI inference/learning task, as specified in report [ITU-R.Eval].

This requirement is defined for the purpose of evaluation in the usage scenario of artificial intelligence and communication.

The minimum requirement for AI service latency is TBD (e.g. 5~15ms) for the [Dense Urban] test environment, assuming given AI inference/learning task.

- AI Service density [Quantitative metrics]

AI service density is the total number of AI services satisfying given AI service accuracy and AI service latency requirements that can be supported in the network of unit coverage area, for the given AI inference/learning task, as specified in report [ITU-R.Eval].

This requirement is defined for the purpose of evaluation in the usage scenario of artificial intelligence and communication.

The minimum requirement for AI service density is TBD AI service connections per km2 per TBD MHz bandwidth for the [Dense Urban] test environment, assuming given AI inference/learning task.

Energy efficiency

Network energy efficiency

Network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. It can be estimated by relative power saving for both loaded case and unloaded case.

Unloaded case:

In this case, the power consumption of full duty setup (i.e. network transmit system information or detects user request information with full duty setup) is the baseline. The evaluation is conducted towards relative power saving between full duty setup and DTX/DRX with certain duty cycle configurations for both transmission and reception operations.

Let Pon denotes the power consumed during Ton which is the time for the discontinuous transmission/reception. Examples of discontinuous transmission/reception include synchronization, measurement, and the detection of control signaling. Let Poff denotes the power consumed during Toff which is the time for sleeping. The energy consumption Eno_data is calculated as follows:

Eno_data = Pon ×Ton + Poff ×Toff

The proponent should report the energy saving gain of the Eno_data achieved based on DTX/DRX with certain duty cycle configurations compared to the Eno_data calculated with full duty setup (i.e. network transmit system information or detects user request information with full duty setup). The proponent should also report the sleep ratio and sleep duration of above DTX/DRX and full duty setup used in the evaluation, separately.

Associated impacts on communication performance and overheads should be accounted for. Examples include latency, coverage and additional resource utilization.

The minimum requirement for network energy efficiency is [TBD]% relative power consumption between full duty setup and DTX/DRX with certain duty cycle configurations.

Loaded case

In this case, power consumption of fully loaded case (i.e. transmitting at full power and receiving at full BW all the time) of the same IMT-2030 candidate technology is the baseline. The evaluation is conducted towards the relative power saving between fully loaded case and un-fully loaded case for both transmission and reception operations.

Let Pactive denotes the power consumed during Tacitve which is the time for BS transmission/reception. Examples of transmission/reception include DL control and data transmission, DL common signaling transmission, UL reception. Let Psleep denotes the power consumed during Tsleep which is the time for sleeping. The energy consumption Edata is calculated as follows:

Edata = Pactive ×Tactive + Psleep ×Tsleep

The proponent should report the energy saving gain of the Edata in a loaded case which is the relative energy saving comparing the consumed energy to the fully loaded case. The proponent should also report the achieved sleep ratio and sleep duration.

Associated impacts on communication performance and overheads should be accounted for. Examples include QoS satisfaction ratio, coverage and additional resource utilization. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability, as specified in Report ITU-R M.[IMT-2030.EVAL]. QoS satisfaction ratio is the ratio of devices fulfilling a specific quality of service (QoS) target.

QoS Satisfaction Ratio = Number of UEs fulfilling QoS targetTotal Number of UEs

The minimum requirement for network energy efficiency is [TBD]% relative power consumption between fully loaded case and loaded case with a given load condition (e.g. low load, light load) under the constraint certain UE QoS satisfaction ratio of [TBD] is achieved (e.g. 90% UEs fulfilling its QoS target).

Device energy efficiency

Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics. It can be estimated by relative power saving for both loaded case and unloaded case.

Unloaded case:

In this case, the power consumption of full duty setup (i.e. UE detects system information or paging information with full duty setup) is the baseline. The evaluation is conducted towards relative power saving between full duty setup and DTX/DRX with certain duty cycle configurations.

Let Pon denotes the power consumed during Ton which is the time for the discontinuous transmission/reception. Examples of discontinuous transmission/reception include synchronization, measurement, and the detection of control signaling. Let Poff denotes the power consumed during Toff which is the time for sleeping. The energy consumption Eno_data is calculated as follows:

Eno_data = Pon ×Ton + Poff ×Toff

The proponent should report the energy saving gain of the Eno_data achieved based on DTX/DRX with certain duty cycle configurations compared to the Eno_data calculated with full duty setup. The proponent should also report the sleep ratio and sleep duration of above DTX/DRX and full duty setup used in the evaluation, separately.

Associated impacts on communication performance and overheads should be accounted for. Examples include latency, coverage and additional resource utilization.

The minimum requirement for network energy efficiency is [TBD]% relative power consumption between full duty setup and DTX/DRX with certain duty cycle configurations.

Loaded case

In this case, power consumption of fully loaded case (i.e. peak data rate) of the same IMT-2030 candidate technology is the baseline. The evaluation is conducted towards the relative power saving between fully loaded case (e.g. peak data rate.) and un-fully loaded case with certain data rate for both transmission and reception operations.

let R(T) denotes the number of correctly transmitted or received bits during the period time of T. Further, let P denote the power used to transmit or receive the data bits and control information. The energy efficiency is given by:

EEv=R(T)T*P EEv=R(T)T∙P

Where v is the data rate, v=R(T)T .

The proponent should report the energy efficiency gain in a loaded case which is the relative energy saving comparing the consumed energy in certain data rate to the fully loaded case of peak data rate.

Associated impacts on communication performance and overheads should be accounted for. Examples include latency (as long as QoS is met), coverage and additional resource utilization.

The minimum requirement for device energy efficiency is [TBD]% relative energy saving between fully loaded case (i.e. peak data rate) and loaded case with certain data rate of [TBD] [e.g. 500 Mbps or “User experienced data rate” defined above].

Annex 2: preliminary analysis and simulation results

Spectral efficiency

Based on extension of ITU IMT-2020 evaluation methodologies [6], spectral efficiency is simulated for three eMBB test environments of Dense urban, Rural and Indoor. The evaluated carrier frequency is replaced from 4GHz to 7 GHz. The extended MIMO configuration is BS up to 1536 Tx/Rx antenna elements, UE up to 16 Tx/Rx antenna elements. ITU IMT-2020’s configuration is BS up to 256 Tx/Rx antenna elements, UE up to 8 Tx/Rx antenna elements (AE) for 4GHz carrier frequency [6].

The simulation results are in figures and summarized in table below. The total overhead (control channel, reference signal etc.) is counted as 20%. The effect of real channel estimation is considered. Compared with ITU IMT-2020 requirements, 4 to 35 times improvement of spectral efficiency are observed with different extended MIMO configurations at 7 GHz.

Table: Spectral efficiency of different MIMO configurations@7GHz

| eMBB test environments |  | ITU IMT-2020 requirements of Spectral efficiency (Average/Cell edge, bps/Hz) | Spectral efficiency of different MIMO configurations@7GHz (Average/Cell edge, N x ITU IMT-2020 requirements) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 256x8* | 256x16* | 512x8* | 512x16* |
| DL | Dense urban | 7.8/0.225 | 4.8x/5.6x | 7.2x/9.4x | 6.4x/7.8x | 9x/13.4x |
|  | Rural | 3.3/0.12 | 11.7x/8.4x | 17.7x/14.8x | 16x/12.3x | 22.5x/22.9x |
|  | Indoor | 9/0.3 | 4.1x/6.3x | 6.2x/8.7x | 6.1x/8.9x | 8x/13x |
| UL | Dense urban | 5.4/0.15 | 10.9x/4.2x | 12.4x/5.2x | 13.2x/5.7x | 14.6x/7.4x |
|  | Rural | 1.6/0.05 | 26.7x/8.9x | 29.8x/11.6x | 32.5x/13.6x | 35.4x/17.8x |
|  | Indoor | 6.75/0.21 | 8.3x/16.8x | 10x/21.2x | 11.2x/27x | 12.9x/33.4x |

* BS 256: (M, N, P, Mg, Ng; Mp, Np) = (24,16,2,1,1,8,16), (8,16,2,1,1,8,16) for indoor, BS 512: (24,32,2,1,1,8,32), (8,32,2,1,1,8,32) for indoor. UE 8: (2,2,2,1,1,2,2), UE 16: (2,4,2,1,1,2,4)

XR connection density/connection capacity

Using the evaluation methodology of 3GPP study on XR (Extended Reality) evaluations for NR in TR38.838, XR capacity is evaluated for eMBB dense urban test environment with similar extended MIMO configurations as above section 7.1 at 7 GHz.

The simulation results are in figures below for DL and UL respectively. It can be seen that, under the simulated cell configurations,

For downlink, 256x8 MIMO configuration supports about 54 user per cell of ISD=200m (corresponding to 4676 users/km2), with 90% of UEs being satisfied.

For uplink, 256x8 MIMO configuration supports about 12.5 user per cell of ISD=200m (corresponding to 1082 users/km2), with 90% of UEs being satisfied.

Table: Traffic model: GOP (Group-Of-Picture) based I/P-Stream traffic model from 3GPP TR38.838

| Application |  | DL I/P-stream | UL pose/control | UL I/P-stream |
| --- | --- | --- | --- | --- |
| Frame per second/Sampling Frequency |  | 60 FPS | 250 Hz | 60 FPS |
| Average data rate |  | R = 30 Mbps α = 2 | 0.2 Mbps | R = 10 Mbps α = 2 |
| Packet delay budget |  | 10ms | 10ms | 30ms |
| Packet success rate |  | 99% | 99% | 99% |
| GOP size |  | K = 8 |  | K = 8 |
| Packet size | Distribution | Truncated Gaussian Distribution | Fixed Size | Truncated Gaussian Distribution |
|  | I-stream Mean | 111112 Bytes | - | 37037 Bytes |
|  | P-stream Mean | 55556 Bytes | - | 18519 Bytes |
|  | STD | 0.105*mean | - | 0.105*mean |
|  | Max | 1.5*mean | - | 1.5*mean |
|  | Min | 0.5*mean | - | 0.5*mean |
| Jitter | Distribution | Truncated Gaussian Distribution |  | Truncated Gaussian Distribution |
|  | Mean | 0 ms |  | 0 ms |
|  | STD | 2 ms |  | 2 ms |
|  | Range | [-4, 4] ms |  | [-4, 4] ms |

AI-related capabilities

- Examples of AI indicators evaluation

- Use cases

AI-enabled robots need to perceive the environment through cameras, such as real-time detection of object categories. Utilize the AI ​​capabilities of robots and networks to process the collected images to achieve real-time high-precision AI inference.

AI models may be deployed in a variety of ways, including : 1. The AI ​​model collects data from the central node for training and inferencing, as shown on the left side of the figure below; the trained AI model can be deployed on the network for inferencing; 2. The AI ​​model is jointly trained and inferenced by distributed nodes, as shown on the right side of the figure below; the network can train the AI ​​model and perform inference; 3. The central node pre-trains the AI ​​model, the distributed nodes obtain the pre-trained model, and obtain the model through fine-tuning for inference, that is, combining the left and right of the figure below; the network can fine-tune the pre-trained model to obtain the AI ​​model used for inference.

Figure: examples of model deployment

The AI ​​inference service process includes: 1) The user uses the user-side AI model to process the input data to obtain intermediate data; 2) The user transmits the intermediate data to the network; 3) The network uses the network-side AI model to process the received intermediate data, get the inference results.

Figure: example of AI inference service process

- Evaluation configurations

Test environment: [Dense Urban]

Communication setup: [Same as immersive communication]

AI use case: [image recognition]

AI data set: [ImageNet-1k test dataset]

AI model: [Taking AlexNet as an example, as shown in the figure below, the neural network layer on the left is processed by the user, and the neural network layer on the right is processed by the network.]

Model processing time: [UE: 0.75ms, Network: 0.45ms]

- Evaluation method

AI service accuracy: evaluated through simulation. 1) The user processes each sample Si, i=1, …, n in the test data set based on the user-side AI model to obtain the intermediate data Zi. 2) Based on the evaluation environment and transmission configurations, the user sends the intermediate data and the network receives the intermediate data. 3) The network processes the received intermediate data Zi based on the AI ​​model on the network side, and obtains the inference result Y ̃_i corresponding to each sample. 4) Compare and calculate the inference results of each sample with the label Yi of each sample in the data set, and obtain the degree to which the output is the same as the true value acc=1ni =1n1Yi==Yi, i.e. AI service accuracy.

AI service latency: The time used for intermediate data transmission (tcomm) is added to the user and network AI model processing time (tproc,UE, tproc,NW) to obtain the AI ​​service latency tservice=tcomm+tproc,UE+tproc,NW.

- Evaluation results

AI service accuracy and AI service delay under different transmission settings, e.g. different quantization bit length of intermediate data transmitted from UE to Network

| Number of quantization bits | 2 | 4 | 6 | 8 | 10 | 12 | 16 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Absolute AI inference accuracy (%) | 0.14 | 10.35 | 52.94 | 56.47 | 56.53 | 56.55 | 56.55 | 56.56 |
| AI service accuracy for evaluation (%) (relative accuracy, compared with theoretical accuracy of the reference AI model without distributed mechanism) | 0.24 | 18.30 | 93.61 | 99.84 | 99.95 | 99.99 | 99.99 | 100 |
| AI service latency (ms) | 1.4 | 1.6 | 1.8 | 1.9 | 2.1 | 2.3 | 2.7 | 4.2 |

Energy efficiency

Network energy efficiency

An example evaluation of the energy consumption for the loaded case is given based on BS power consumption model described in 3GPP TR 38.864. Various energy saving schemes can be considered depending on the details of UE traffic model and UE QoS target, which will result in various BS sleep ratio and sleep duration. One energy saving example is given, where there are 10 XR UEs in each cell and the latency budget is 30ms. According to the remaining latency budget, BS can keep sleeping to reduce power consumption, as shown in below figure.

Figure: example network energy saving scheme and evaluation for the loaded case based on 3GPP TR 38.864

The simulation results show the scheme can achieve 76% BS power reduction as compared to the reference fully loaded case, while all UE can still fulfil its QoS target. This is because BS could sleep longer and sleep continuously.

Device energy efficiency

Figure below gives an example calculation of device energy efficiency of loaded case based on UE power consumption model described in 3GPP TR 38.840. In this case, various power saving schemes can be considered depending on the detail level of power model e.g. bandwidth, modulation, MIMO, duty cycle, etc. The last two are considered in this example cases.  It can be done by calculation (“analytical” in ITU’s term). Simulation approach is also possible, which will consider the effect of wireless channel model, interferences, etc., if desired.

Figure: example evaluation of Device energy efficiency of loaded case based on NR