3GPP TSG-RAN WG1 Meeting #119 	R1-2410931

Orlando, US, 18-22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0209 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 editorial corrections for TS 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, Netw_Energy_NR-Core, NR_pos_enh2-Core, NR_SL_enh2-Core, NR_cov_enh2-Core, NR_Mob_enh2-Core,NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The name of some higher layer parameters in TS 38.212 is not aligned with the latest ones defined in RAN2 per the RAN2 LS R1-2407592(R2-2407827). - Capture the correction on the field of Open-loop power control parameter set indication in DCI format 0_3 per the agreement based on R1-2408629 from RAN1#118bis. - Capture the correction on the field of Downlink assignment index in DCI format 1_0 per the agreement on draft CR R1-2409277 from RAN1#118bis. - Correction some typos for the Tables in DCI format 1_3. - Capture the endorsed TP on DCI format 2_9 per the agreement from RAN1#119 meeting. - Capture the endorsed draft CR R1-2410598 on bit-width determination of Type-1B DCI field in DCI format 0_3 and DCI format 1_3 from RAN1#119 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the name of higher layer parameters in TS 38.212 per the RAN2 LS R1-2407592(R2-2407827). - Update the description of the field Open-loop power control parameter set indication in DCI format 0_3 to reflect the agreement from RAN1#118bis. - Update the correction on the field of Downlink assignment index in DCI format 1_0 to reflect the agreement on draft CR R1-2409277 from RAN1#118bis. - Correction some typos for the Tables in DCI format 1_3. - Reflect the endorsed TP on DCI format 2_9 per the agreement from RAN1#119 meeting. - Reflect the endorsed draft CR R1-2410598 on bit-width determination of Type-1B DCI field in DCI format 0_3 and DCI format 1_3 from RAN1#119 meeting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 6.3.1.1.2, 6.3.2.1.2, 7.3.1, 7.3.1.1.2, 7.3.1.1.3, 7.3.1.1.4, 7.3.1.2.1, 7.3.1.2.2, 7.3.1.2.3, 7.3.1.2.4, 7.3.1.3.10, 8.3.1.1, 8.4.1.3, 8.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

For one TB for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2:

-	if the PDSCH is scheduled by DCI format 4_1/4_2:

-	maximum number of layers is given by X, where:

-	if the higher layer parameter maxMIMO-Layers of pdsch-ConfigMulticast is configured, X is given by that parameter;

-	otherwise, X equals to 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMulticast or by pdsch-ConfigMTCH for at least one common frequency resource (CFR) is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the PDSCH is scheduled by DCI format 4_0:

-	maximum number of layers is 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMCCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMTCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	nPRB=nPRB,LBRM is given by Table 5.4.2.1-1, where the value of nPRB,LBRM for DL-SCH is determined according to the size of the associated CFR if configured to the UE;

-	maximum coding rate of 948/1024;

-	NRE=156∙nPRB;

-	C is the number of code blocks of the transport block determined according to Clause 5.2.2.

For one TB for UL-SCH, or for one TB for DL-SCH/PCH except for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2:

-	maximum number of layers for one TB for UL-SCH is given by the minimum of X and 4, where:

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured and if neither the higher layer parameter multipanelSchemeSFN nor the higher layer parameter multipanelSchemeSDM is configured, X is given by that parameter;

-	elseif the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured and if the higher layer parameter multipanelSchemeSFN is configured, X is given by max{maxMIMO-Layers, maxMIMO-LayersforSFN};

-	elseif the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured and if the higher layer parameter multipanelSchemeSDM is configured, X is given by max{maxMIMO-Layers, 2*maxMIMO-LayersforSDM};

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured and if neither the higher layer parameter multipanelSchemeSFN nor the higher layer parameter multipanelSchemeSDM is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell;

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured and if the higher layer parameter multipanelSchemeSFN is configured, X is given by max{maxRank, maxRankSFN} across all BWPs of the serving cell;

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured and if the higher layer parameter multipanelSchemeSDM is configured, X is given by max{maxRank, 2*maxRankSDM} across all BWPs of the serving cell;

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell;

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where:

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter;

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell;

-	if the higher layer parameter mcs-Table-r17 or mcs-TableDCI-1-2-r17 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam1024', maximum modulation order Qm=10 is assumed for DL-SCH, else if the higher layer parameter mcs-Table or mcs-TableDCI-1-2 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for DL-SCH; otherwise a maximum modulation order  is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder or mcs-TableDCI-0-2 or mcs-TableTransformPrecoderDCI-0-2 given by a pusch-Config or the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for UL-SCH; otherwise a maximum modulation order  is assumed for UL-SCH;

-	maximum coding rate of 948/1024;

-	 is given by Table 5.4.2.1-1, where the value of  for DL-SCH is determined according to the initial downlink bandwidth part if there is no other downlink bandwidth part configured to the UE;

-	;

-	 is the number of code blocks of the transport block determined according to Clause 5.2.2.

Table 5.4.2.1-1: Value of

| Maximum number of PRBs across all configured DL BWPs and UL BWPs of a carrier for DL-SCH and UL-SCH, respectively, or Maximum number of PRBs across all CFRs of a carrier for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2 |  |
| --- | --- |
| Less than 33 | 32 |
| 33 to 66 | 66 |
| 67 to 107 | 107 |
| 108 to 135 | 135 |
| 136 to 162 | 162 |
| 163 to 217 | 217 |
| Larger than 217 | 273 |

Denoting by  the rate matching output sequence length for the -th coded block, where the value of  is determined as follows:

Set

for  to

if the -th coded block is not scheduled for transmission as indicated by CBGTI according to Clause 5.1.7.2 for DL-SCH and 6.1.5.2 for UL-SCH in [6, TS 38.214]:

;

else

if

;

else

;

end if

;

end if

end for

where:

-	 is the number of transmission layers that the transport block is mapped onto;

-	 is the modulation order;

-	 is the total number of coded bits available for transmission of the transport block;

-	 if CBGTI is not present in the DCI scheduling the transport block and  is the number of scheduled code blocks of the transport block if CBGTI is present in the DCI scheduling the transport block.

Denote by  the redundancy version number for this transmission ( = 0, 1, 2 or 3), the rate matching output bit sequence , , is generated as follows, where  is given by Table 5.4.2.1-2 according to the value of  and LDPC base graph:

;

;

while

if

;

;

end if

;

end while

Table 5.4.2.1-2: Starting position of different redundancy versions,

|  | LDPC base graph 1 | LDPC base graph 2 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

< Unchanged parts are omitted >

#### 6.3.1.1	UCI bit sequence generation

##### 6.3.1.1.2	CSI only

If cqi-BitsPerSubband is configured, this Clause 6.3.1.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the bitwidth of a CSI field of the CSI sub-report is determined following the procedure in this clause 6.3.1.1.2 by taking configurations in CSI-ReportSubConfig when applicable. If CSI-ReportSubConfig configures a list of CSI-RS resource IDs, for the determination of the bitwidth of a CRI field, the value of KsCSI-RS is the number of CSI-RS resources configured in the corresponding CSI-ReportSubConfig.

The bitwidth for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports is 2 for Rank=1 and 1 for Rank=2, according to Clause 5.2.2.2.1 in [6, TS 38.214].

The bitwidth for PMI of codebookType=typeI-SinglePanel with more than 2 CSI-RS ports is provided in Table 6.3.1.1.2-1, where the values of and  are given by Clause 5.2.2.2.1 in [6, TS 38.214].

Table 6.3.1.1.2-1: PMI of codebookType=typeI-SinglePanel

|  | Information field for wideband PMI |  |  | Information field for wideband PMI or per subband PMI |  |
| --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |
|  | codebookMode=1 | codebookMode=2 |  | codebookMode=1 | codebookMode=2 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (,) | N/A | 2 | 4 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (, 0) | N/A | 2 | 4 |
| Rank=2 with 4 CSI-RS ports, | (,) | (, 0) | 1 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (,) | 2 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (, 0) | 2 | 1 | 3 |
| Rank=3 or 4, with 4 CSI-RS ports | (,) |  | 0 | 1 |  |
| Rank=3 or 4, with 8 or 12 CSI-RS ports | (,) |  | 2 | 1 |  |
| Rank=3 or 4 , with >=16 CSI-RS ports | (, ) |  | 2 | 1 |  |
| Rank=5 or 6 | (,) |  | N/A | 1 |  |
| Rank=7 or 8, | (, ) |  | N/A | 1 |  |
| Rank=7 or 8, | (,) |  | N/A | 1 |  |
| Rank=7 or 8, with or or | (,) |  | N/A | 1 |  |

The bitwidth for PMI of codebookType= typeI-MultiPanel is provided in Table 6.3.1.1.2-2, where the values of and  are given by Clause 5.2.2.2.2 in [6, TS 38.214].

Table 6.3.1.1.2-2: PMI of codebookType= typeI-MultiPanel

|  | Information fields for wideband |  |  |  |  | Information fields for wideband or per subband |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |  |  |  |  |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | N/A | N/A | 2 | N/A | N/A | N/A |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | 2 | 2 | 2 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=1 with codebookMode=2 | (,) | N/A | 2 | 2 | N/A | N/A | 2 | 1 | 1 |
| Rank=2 with , codebookMode=2 | (,) | 1 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=3 or 4 with , codebookMode=2 | (,) | 0 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=2 or 3 or 4 with , codebookMode=2 | (,) | 2 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |

The bitwidth for PMI with 1 CSI-RS port is 0.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel or reportQuantity set to 'cri-RI-CQI' or 1 CSI-RS port is provided in Table 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel, or
reportQuantity set to 'cri-RI-CQI', or 1 CSI-RS port

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator when codebookType=typeI-SinglePanel | 0 |  |  |  |  |
| Rank Indicator when reportQuantity set to 'cri-RI-CQI' | 0 | 1 | 2 | 3 | 3 |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214].  is the value of the rank. The value of  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. For higher layer parameter reportQuantity set to 'cri-RI-CQI', the values of the rank indicator field are mapped to rank indicator values with increasing order, where '0' is mapped to rank-1.

Table 6.3.1.1.2-3A: RI, LI, CQI, and CRI associated with one CSI-RS resource pair and
csi-ReportMode= Mode 1 or Mode 2

| Field | Bitwidth |  |
| --- | --- | --- |
|  | 1 antenna port per Resource | > 1 antenna ports per Resource |
| Rank Combination Indicator | 0 | min(2,log2 nRI,NCJT) |
| The first Layer Indicator | 0 | log2v1 |
| The second Layer Indicator | 0 | log2v2 |
| Wide-band CQI for the first TB | 4 | 4 |
| Subband differential CQI for the first TB | 2 | 2 |
| CRI if csi-ReportMode= Mode 1 | log2N | log2N |
| CRI if csi-ReportMode= Mode 2 | log2M1+M2+N | log2M1+M2+N |

Table 6.3.1.1.2-3B: RI, LI, CQI, and CRI associated with one CSI-RS resource and
csi-ReportMode= Mode 1 or Mode 2

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | 0 | min(1,log2 nRI,sTRP) | min(2,log2 nRI,sTRP) | log2 nRI,sTRP | log2 nRI,sTRP |
| Layer Indicator | 0 | log2v | min⁡(2, log2v) | min⁡(2, log2v) | min⁡(2, log2v) |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI if csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1 = 1 | log2M1+M2 | log2M1+M2 | log2M1+M2 | log2M1+M2 | log2M1+M2 |
| CRI if csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1 = 2 | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI |
| CRI if csi-ReportMode= Mode 2 | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N |

nRI,NCJT in Table 6.3.1.1.2-3A is the number of allowed rank combination indicator values associated with one CSI-RS resource pair according to Clause 5.2.1.4.2 [6, TS 38.214]. The values of the rank combination indicator field are mapped to allowed rank combinations in the following order: {1,1}, {1,2}, {2,1},{2,2}, where '0' is mapped to the first allowed rank combination. v1 and v2 are the values of the first and the second rank associated with two CSI-RS resources of the CSI-RS resource pair respectively.

nRI, sTRP in Table 6.3.1.1.2-3B is the number of allowed rank indicator values associated with one CSI-RS resource according to Clause 5.2.1.4.2 [6, TS 38.214]. v is the value of the rank associated with the CSI-RS resource. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The value of N in Table 6.3.1.1.2-3A and Table 6.3.1.1.2-3B is the number of CSI-RS resource pairs configured within a CSI-RS resource set. The values of M1 and M2 in Table 6.3.1.1.2-3A and Table 6.3.1.1.2-3B are given by

-	If sharedCMR = "Enabled", M1 = K1 and M2 = K2

-	If sharedCMR is absent and N = 1, M1 = K1 - 1 and M2 = K2 - 1

-	If sharedCMR is absent and N = 2,

-	M1 = K1 - 2 and M2 = K2 - 2, if the two resource pairs do not share any CSI-RS resource

-	M1 = K1 - 1 and M2 = K2 - 2, if the two resource pairs share the same CSI-RS resource from the first CSI-RS resource group

-	M1 = K1 - 2 and M2 = K2 - 1, if the two resource pairs share the same CSI-RS resource from the second CSI-RS resource group

where the values of K1 and K2 are the numbers of CSI-RS resources in the first and second CSI-RS resource groups within the CSI-RS resource set respectively.

The bitwidth for RI/LI/CQI/CRI of codebookType= typeI-MultiPanel is provided in Table 6.3.1.1.2-4.

Table 6.3.1.1.2-4: RI, LI, CQI, and CRI of codebookType=typeI-MultiPanel

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| CRI |  |

where  is the number of allowed rank indicator values according to Clause 5.2.2.2.2 [6, TS 38.214],  is the value of the rank, and  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for RI/LI/CQI of codebookType= typeII or codebookType=typeII-PortSelection is provided in Table 6.3.1.1.2-5.

Table 6.3.1.1.2-5: RI, LI, and CQI of codebookType=typeII or typeII-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the number of non-zero wideband amplitude coefficients for layer |  |

where  is the number of allowed rank indicator values according to Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and  is the value of the rank. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for CRI, SSBRI, RSRP, differential RSRP, and CapabilityIndex are provided in Table 6.3.1.1.2-6.

Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |

where  is the number of CSI-RS resources in the corresponding resource set, and  is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index'.

The bitwidth for CRI, SSBRI, SINR, differential SINR, and CapabilityIndex are provided in Table 6.3.1.1.2-6A.

Table 6.3.1.1.2-6A: CRI, SSBRI, SINR, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI | log2KsCSI-RS |
| SSBRI | log2KsSSB |
| SINR | 7 |
| Differential SINR | 4 |
| CapabilityIndex | 2 |

where KsCSI-RS  is the number of CSI-RS resources in the corresponding resource set, and KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-SINR' or 'ssb-Index-SINR-Index'.

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the mapping order of CSI fields of one CSI sub-report is determined following the procedure in this clause 6.3.1.1.2, by replacing CSI report #n in the following Tables 6.3.1.1.2-7, 6.3.1.1.2-9 and 6.3.1.1.2-10 with CSI sub-report #n, and taking only Tables 6.3.1.1.2-1/2/3/4 for the determination of the bitwidth of a CSI field.

Table 6.3.1.1.2-7: Mapping order of CSI fields of one CSI report,
pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI or reportQuantity
set to 'cri-RI-CQI' and cqi-FormatIndicator=widebandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Zero padding bits , if needed |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4, if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4, if reported |

The number of zero padding bits  in Table 6.3.1.1.2-7 is 0 for 1 CSI-RS port and  for more than 1 CSI-RS port, where:

-	 and  is the set of rank values  that are allowed to be reported;

-	, where  is the reported rank;

-	For 2 CSI-RS ports, ;

-	For more than 2 CSI-RS ports, ;

-	if PMI is reported,  and ; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/2; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/2; otherwise, ;

-	if CQI is reported,  is obtained according to Tables 6.3.1.1.2-3/4; otherwise, ;

-	if LI is reported,  is obtained according to Tables 6.3.1.1.2-3/4; otherwise, .

Table 6.3.1.1.2-7A: Mapping order of CSI fields of one CSI report, pmi-FormatIndicator=widebandPMI, cqi-FormatIndicator=widebandCQI, csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1=0

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3A, if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | Zero padding bits OP, if needed |
|  | PMI wideband information fields X1, from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X2, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X1, from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X2, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |

The number of zero padding bits OP in Table 6.3.1.1.2-7A is 0 for 1 CSI-RS port and OP=Nmax-Nreported for more than 1 CSI-RS port, where:

-	Nmax=maxr∈SRankB(r)  and SRank is the set of rank combination values of r={r1,r2} that are allowed to be reported;

-	Nreported=B(R) where R is the reported rank combination;

-	For 2 CSI-RS ports, Br=NPMI(r1)+NPMI(r2)+NCQIr+NLI(r1)+NLI(r2);

-	For more than 2 CSI-RS ports, Br=NPMI,i1(r1)+NPMI,i1(r2)+NPMI,i2(r1)+NPMI,i2(r2)+NCQIr+NLI(r1)+NLI(r2) ;

-	if PMI is reported, NPMI1=2 and  NPMI2=1; otherwise, NPMI=0;

-	if PMI i1 is reported, NPMI,i1(r1) and NPMI,i1(r2) are obtained according to Tables 6.3.1.1.2-1; otherwise, NPMI,i1=0;

-	if PMI  i2 is reported, NPMI,i2(r1) and NPMI,i2(r2) are obtained according to Tables 6.3.1.1.2-1; otherwise,  NPMI,i2=0;

-	if CQI is reported, NCQIr is obtained according to Tables 6.3.1.1.2-3A; otherwise, NCQIr=0;

-	if LI is reported, NLI(r1) and NLI(r2) are obtained according to Tables 6.3.1.1.2-3A; otherwise , NLI=0.

Table 6.3.1.1.2-8: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting, or mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #3 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #4 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #1 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #2 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #3 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-8A: Mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6A, if reported |
|  | SINR #1 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #2 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #3 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #4 as in Table 6.3.1.1.2-6A, if reported |
|  | CapabilityIndex #1 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #2 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #3 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-8B: Mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Resource set indicator |
|  | CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | RSRP of CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6 |
|  | Differential RSRP of CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6 |
|  | Differential RSRP of CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported |

where the 1-bit resource set indicator, with value of 0 or 1, indicates the 1st or the 2nd channel measurement resource set respectively, from which CRI or SSBRI #1 of 1st resource group is reported from; and all remaining resource groups, if reported, follow the same mapping order as the 1st resource group where CRI or SSBRI #1 of all remaining resource groups is reported from the indicated channel measurement resource set. For all reported resource groups, CRI or SSBRI #1 and CRI or SSBRI #2 are reported from different channel measurement resource sets.

Table 6.3.1.1.2-8C: Mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | SSBRI # L×M as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | Differential RSRP # L×M as in Table 6.3.1.1.2-6, if reported |
| NOTE: L is the number of reported cells provided by higher layer parameter nrOfReportedCells and M is the number of reported SSBRI/RSRP pairs per cell and equal to the value provided by higher layer parameter nrofReportedRS-PerCell. |  |

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

Table 6.3.1.1.2-9A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

Table 6.3.1.1.2-9B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

The number of zero padding bits OP in Table 6.3.1.1.2-9B is 0 for 1 CSI-RS port and OP=Nmax-Nreported(R) for more than 1 CSI-RS port, where:

-	Nmax=maxr∈SRankN(r) . SRank is the set of rank and rank combination values r that are allowed to be reported. Nr is obtained according to Tables 6.3.1.1.2-3A/3B for rank combination indicator and rank indicator respectively.

-	Nreported R is obtained according to Tables 6.3.1.1.2-3A for rank combination indicator and R is the reported rank combination.

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3B for rank indicator and R is the reported rank.

Table 6.3.1.1.2-10: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-10A: Mapping order of CSI fields of one CSI report,
CSI part 2 wideband, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the first CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the second CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.1.1.2-10B: Mapping order of CSI fields of one CSI report,
CSI part 2 wideband, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, if reported part 1 is associated with one CSI-RS resource pair, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second CSI-RS resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

Table 6.3.1.1.2-11A: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
csi-ReportMode= Mode 1

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.1.1.2-11B: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
csi-ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.1.1.2-11C: Mapping order of CSI fields of one CSI report containing Nnsub CSI sub-report(s), CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI of CSI sub-report #1 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields X2 of CSI sub-report #1 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #1 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #1 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index of for 2 antenna ports CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nnsub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence ,
without two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 as in Table 6.3.1.1.2-7/7A/8/8B |
|  | CSI report #2 as in Table 6.3.1.1.2-7/7A/8/8B |
|  | … |
|  | CSI report #n as in Table 6.3.1.1.2-7/7A/8/8B |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, all CSI sub-reports within the CSI report #i are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence  is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

Table 6.3.1.1.2-13: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 if CSI report #1 is not of two parts, or CSI report #1, CSI part 1, if CSI report #1 is of two parts, as in Table 6.3.1.1.2-7/7A/8/8B/9/9A/9B |
|  | CSI report #2 if CSI report #2 is not of two parts, or CSI report #2, CSI part 1, if CSI report #2 is of two parts, as in Table 6.3.1.1.2-7/7A/8/8B/9/9A/9B |
|  | … |
|  | CSI report #n if CSI report #n is not of two parts, or CSI report #n, CSI part 1, if CSI report #n is of two parts, as in Table 6.3.1.1.2-7/7A/8/8B/9/9A/9B |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, either all CSI sub-reports not of two parts or CSI part 1 of all CSI sub-reports of two parts, are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-13 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.1.1.2-14: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B/11C if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B/11C if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B/11C if CSI part 2 exists for CSI report #n |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}: - CSI part 2 widebands of all CSI sub-reports are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number; - CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-14 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

< Unchanged parts are omitted >

#### 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.2.1.2	CSI

If cqi-BitsPerSubband is configured, this Clause 6.3.2.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the bitwidth of a CSI field of the CSI sub-report is determined following the procedure in this clause 6.3.2.1.2 by taking configurations in CSI-ReportSubConfig when applicable. If CSI-ReportSubConfig configures a list of CSI-RS resource IDs, for the determination of the bitwidth of a CRI field, the value of KsCSI-RS is the number of CSI-RS resources configured in the corresponding CSI-ReportSubConfig.

The bitwidth for PMI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Clause 6.3.1.1.2.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Clause 6.3.1.1.2.

The bitwidth for PMI/RI/LI/CQI/CRI with 1 CSI-RS port is specified in Clause 6.3.1.1.2.

The bitwidth for PMI of codebookType=typeII is provided in Tables 6.3.2.1.2-1, where the values of , , , , , , and  are given by Clause 5.2.2.2.3 in [6, TS 38.214].

Table 6.3.2.1.2-1: PMI of codebookType= typeII

|  | Information fields for wideband PMI |  |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-r16 is provided in Tables 6.3.2.1.2-1A, where the values of (N1, N2), (O1, O2), L, KNZ, N3, and Mll=1,…, υ are given by Clause 5.2.2.2.5 in [6, TS 38.214].

Table 6.3.2.1.2-1A: PMI of codebookType= typeII-r16

|  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | i1,1 |  |  |  | i1,2 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
| Rank=1 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 | i1,5 |  | i1,6,1 |  | i1,6,2 | i1,6,3 |  | i1,6,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A | N/A |  | log2N3-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A | N/A |  | log2N3-1M2-1 |  | log2N3-1M2-1 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A | N/A |  | log2N3-1M3-1 |  | log2N3-1M3-1 | log2N3-1M3-1 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 | N/A |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | log2N3-1M4-1 |  | 3(KNZ-4) | 4(KNZ-4) |  | 8LM4 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A | log2(2M1) |  | log22M1-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | 2LM1 |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A | log2(2M2) |  | log22M2-1M2-1 |  | log22M2-1M2-1 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | 4LM2 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A | log2(2M3) |  | log22M3-1M3-1 |  | log22M3-1M3-1 | log22M3-1M3-1 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 6LM3 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 | log2(2M4) |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | log22M4-1M4-1 |  | 3(KNZ-4) | 4(KNZ-4) |  | 8LM4 |
| Note: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-CJT is provided in Tables 6.3.2.1.2-1B, where the values of (N1, N2), (O1, O2), O3, KNZ, N3, N0, Ln, σ(n) and Mll=1,…, υ are given by Clause 5.2.2.2.8 in [6, TS 38.214].

Table 6.3.2.1.2-1B: PMI of codebookType= typeII-CJT

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,1 |  | i1,2 |  | i1,8,1 | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2KNZ | N/A |  | N/A |  | N/A |
| Rank=2 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | N/A |  | N/A |
| Rank=3 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | N/A |
| Rank=4 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |
| Rank=1 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2KNZ | N/A |  | N/A |  | N/A |
| Rank=2 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | N/A |  | N/A |
| Rank=3 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | N/A |
| Rank=4 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 |  | i1,5 |  | i1,6,1 | i1,6,2 |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A |  | N/A |  | log2N3-1M1-1 | N/A |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A |  | N/A |  | log2N3-1M2-1 | log2N3-1M2-1 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A |  | N/A |  | log2N3-1M3-1 | log2N3-1M3-1 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 |  | N/A |  | log2N3-1M4-1 | log2N3-1M4-1 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A |  | log2(2M1) |  | log22M1-1M1-1 | N/A |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A |  | log2(2M2) |  | log22M2-1M2-1 | log22M2-1M2-1 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A |  | log2(2M3) |  | log22M3-1M3-1 | log22M3-1M3-1 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 |  | log2(2M4) |  | log22M4-1M4-1 | log22M4-1M4-1 |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |
|  | i1,6,3 |  |  | i1,6,4 | i1,9 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=1N0Lσ(n)M1 |
| Rank=2 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=1N0Lσ(n)M2 |
| Rank=3 N3≤19 | log2N3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=1N0Lσ(n)M3 |
| Rank=4 N3≤19 | log2N3-1M4-1 |  |  | log2N3-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=1N0Lσ(n)M4 |
| Rank=1 N3>19 | N/A |  |  | N/A | N0-1log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=1N0Lσ(n)M1 |
| Rank=2 N3>19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=1N0Lσ(n)M2 |
| Rank=3 N3>19 | log22M3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=1N0Lσ(n)M3 |
| Rank=4 N3>19 | log22M4-1M4-1 |  |  | log22M4-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=1N0Lσ(n)M4 |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2n=1N0Lσ(n)Mυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.8 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler is provided in Tables 6.3.2.1.2-1C, where the values of (N1, N2), (O1, O2), L, KNZ, N3, N4, Q and Mll=1,…, υ are given by Clause 5.2.2.2.10 in [6, TS 38.214].

Table 6.3.2.1.2-1C: PMI of codebookType=typeII-Doppler

|  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i1,1 |  |  |  | i1,2 |  |  |  | i1,8,1 |  |  | i1,8,2 |  |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
| Rank=1 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | i1,5 |  | i1,6,1 |  |  | i1,6,2 |  |  | i1,6,3 |  |  | i1,6,4 |  |  |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A |  | N/A |  | log2N3-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A |  | N/A |  | log2N3-1M2-1 |  |  | log2N3-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A |  | N/A |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 |  | N/A |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A |  | log2(2M1) |  | log22M1-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A |  | log2(2M2) |  | log22M2-1M2-1 |  |  | log22M2-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A |  | log2(2M3) |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 |  | log2(2M4) |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i1,10,1 |  |  | i1,10,2 |  |  |  | i1,10,3 |  |  | i1,10,4 |  |  | {i2,4,l}l=1,…,υ |  |  | {i2,5,l}l=1,…,υ |  |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| Rank=1 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυQ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,τ(3), kl,i,f,τ(2), and cl,i,f,τ, respectively), where KlNZ as defined in Clause 5.2.2.2.10 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType= typeII-PortSelection is provided in Tables 6.3.2.1.2-2, where the values of , , , , , , and  are given by Clause 5.2.2.2.4 in [6, TS 38.214].

Table 6.3.2.1.2-2: PMI of codebookType= typeII-PortSelection

|  | Information fields for wideband PMI |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r16 is provided in Tables 6.3.2.1.2-2A, where the values of PCSI-RS, d, L, KNZ, N3, and  Mll=1,…, υ are given by Clause 5.2.2.2.6 in [6, TS 38.214].

Table 6.3.2.1.2-2A: PMI of codebookType= typeII-PortSelection-r16

|  |  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | i1,1 |  |  |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |  |  |
| Rank=1 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
| Rank=1 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 | i1,5 | i1,6,1 |  | i1,6,2 |  | i1,6,3 | i1,6,4 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A | N/A | log2N3-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A | N/A | log2N3-1M2-1 |  | log2N3-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A | N/A | log2N3-1M3-1 |  | log2N3-1M3-1 |  | log2N3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 | N/A | log2N3-1M4-1 |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A | log2(2M1) | log22M1-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A | log2(2M2) | log22M2-1M2-1 |  | log22M2-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A | log2(2M3) | log22M3-1M3-1 |  | log22M3-1M3-1 |  | log22M3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 | log2(2M4) | log22M4-1M4-1 |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r17 is provided in Tables 6.3.2.1.2-2B, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.7 in [6, TS 38.214].

Table 6.3.2.1.2-2B: PMI of codebookType= typeII-PortSelection-r17

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.7 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the mapping order of CSI fields of one CSI sub-report is determined following the procedure in this clause 6.3.2.1.2, by replacing CSI report #n in the following Tables 6.3.2.1.2-3 and 6.3.2.1.2-4 with CSI sub-report #n, and taking only Tables 6.3.1.1.2-1/2/3/4 for the determination of the bitwidth of a CSI field.

The bitwidth for PMI of codebookType=typeII-CJT-PortSelection is provided in Tables 6.3.2.1.2-2C, where the values of PCSI-RS, K1,n, KNZ, N3, N, M and σn are given by Clause 5.2.2.2.9 in [6, TS 38.214].

Table 6.3.2.1.2-2C: PMI of codebookType= typeII-CJT-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  |  | i1,6 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | N/A |  | N/A |  | N/A |
| Rank=2 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | N/A |  | N/A |
| Rank=3 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |  | N/A |
| Rank=4 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | i1,9 |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=n=1N0K1,σ(n)M; n=1N0K1,σ(n)M otherwise |  |
| Rank=2 | 4 | 4 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=2n=1N0K1,σ(n)M; 2n=1N0K1,σ(n)M otherwise |  |
| Rank=3 | 4 | 4 | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3∙n=1N0K1,σ(n)M |  |
| Rank=4 | 4 | 4 | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 4∙n=1N0K1,σ(n)M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are n=1N0K1,σ(n)M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.9 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler-PortSelection is provided in Tables 6.3.2.1.2-2D, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.11 in [6, TS 38.214].

Table 6.3.2.1.2-2D: PMI of codebookType= typeII-Doppler-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2D is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.11 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

For CSI on PUSCH, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence  starting with . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence  starting with .

The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8A. The mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8B. The mapping order of CSI fields of one report for TDCP reporting is provided in Table 6.3.2.1.2-3C. The mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility is provided in Table 6.3.1.1.2-8C. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, CRI/SINR, SSBRI/SINR reporting, or TDCP reporting.

Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4/6, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
|  | Indicator of the N0 selected CSI-RS resources by a bitmap with NTRP bits, this field is present only if NTRP>1 and restrictedCMR-Selection is configured to OFF |
|  | Indicator of selected Ln value combination or αn value combination with bitwidth of log2(NL), this field is present only if NL>1 |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ as in Tables 6.3.2.1.2-8/8A/8B/9/9A, if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Table 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Table 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3C: Mapping order of CSI fields of one CSI report for reportQuantity=tdcp

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Amplitude value for the configured delay values as in Table 6.3.2.1.2-10 based on the order from the first configured delay to the last configured delay |
|  | Phase value for the configured delay values as in Table 6.3.2.1.2-10 based on the order from the first configured delay to the last configured delay, if reported |

The number of zero padding bits OP in Table 6.3.1.1.2-9B is 0 for 1 CSI-RS port and OP=Nmax-Nreported(R) for more than 1 CSI-RS port, where:

-	Nmax=maxr∈SRankN(r) . SRank is the set of rank and rank combination values r that are allowed to be reported. Nr is obtained according to Tables 6.3.1.1.2-3A/3B for rank combination indicator and rank indicator respectively.

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3A for rank combination indicator and R is the reported rank combination

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3B for rank indicator and R is the reported rank

Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.2.1.2-4A: Mapping order of CSI fields of one CSI report, CSI part 2 wideband,
csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the first CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the second CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.2.1.2-4B: Mapping order of CSI fields of one CSI report, CSI part 2 wideband,
csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, if reported part 1 is associated with one CSI-RS resource pair, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second CSI-RS resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.2.1.2-5: Mapping order of CSI fields of one CSI report, CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-r16 or typeII-PortSelection-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |

Table 6.3.2.1.2-5B: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2B/2D, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2B/2D: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*K1M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2B/2D: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |

Table 6.3.2.1.2-5C: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
ReportMode= Mode 1

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.2.1.2-5D: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.2.1.2-5E: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-CJT

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1B, if reported; |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,9, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2∙n=1N0Lσ(n)Mυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if present and if reported; |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if reported; |

Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType= typeII-Doppler

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.2.1.2-8B, if present and reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f for N4=1 or Pril,i,f,j for N4>1 defined in clause 5.2.3 of TS 38.214 [6], if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.2.1.2-8B, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f for N4=1 or Pril,i,f,j for N4>1 defined in clause 5.2.3 of TS 38.214 [6], if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.2.1.2-8B, if present and if reported |

Table 6.3.2.1.2-5G: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-CJT-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2C, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ}, i1,9 and ν*n=1N0K1,σ(n)M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if reported |

Table 6.3.2.1.2-5H: Mapping order of CSI fields of one CSI report containing Nnsub CSI sub-report(s), CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI of CSI sub-report #1 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields X2 of CSI sub-report #1 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #1 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #1 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nnsub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

Table 6.3.2.1.2-6: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI part 1 of CSI report #1 as in Table 6.3.2.1.2-3/3A/3B or Table 6.3.1.1.2-8/8A/8B/8C or Table 6.3.2.1.2-3C |
|  | CSI part 1 of CSI report #2 as in Table 6.3.2.1.2-3/3A/3B or Table 6.3.1.1.2-8/8A/8B/8C or Table 6.3.2.1.2-3C |
|  | … |
|  | CSI part 1 of CSI report #n as in Table 6.3.2.1.2-3/3A/3B or Table 6.3.1.1.2-8/8A/8B/8C or Table 6.3.2.1.2-3C |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, CSI part 1 of all CSI sub-reports are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-6 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.2.1.2-7: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D/5H, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D/5H, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D/5H, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #n |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, - CSI part 2 wideband of all CSI sub-reports are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number; - CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-7 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

The bitwidth for RI/CQI of codebookType= typeII-r16 or codebookType=typeII-PortSelection-r16 is provided in Table 6.3.2.1.2-8.

Table 6.3.2.1.2-8: RI and CQI of codebookType=typeII-r16 or typeII-PortSelection-r16

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5 and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType= typeII-CJT is provided in Table 6.3.2.1.2-8A.

Table 6.3.2.1.2-8A: RI and CQI of codebookType= typeII-CJT

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers, and all CSI-RS resources if configured, KNZ | log22βM1Lmax if max allowed rank is 1; log222βM1Lmaxotherwise |

Where nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.8 TS 38.214 [6], Lmax is the maximum of n=1NTRPLn for all NL configured Ln combinations, where Lnn=1,…, NTRP, M1, and β are given by Clause 5.2.2.2.8 in TS 38.214 [6]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clause 5.2.2.2.8 TS 38.214 [6], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType= typeII-Doppler is provided in Table 6.3.2.1.2-8B.

Table 6.3.2.1.2-8B: RI and CQI of codebookType= typeII-Doppler

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| The second time-domain wide-band CQI | 4 |
| The second time-domain subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

Where nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.10 TS 38.214 [6], K0=2Lp1×N3RβQ, where p1, N3, R, β and Q are given by Clause 5.2.2.2.10 in TS 38.214 [6]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clause 5.2.2.2.10 TS 38.214 [6], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection is provided in Table 6.3.2.1.2-9.

Table 6.3.2.1.2-9: RI and CQI of codebookType=typeII-PortSelection-r17 or
typeII-Doppler-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.7 and Clauses 5.2.2.2.11 [6, TS 38.214], K0=K1Mβ, where K1, M, and β are given by Clause 5.2.2.2.7 and Clauses 5.2.2.2.11 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.7 and Clauses 5.2.2.2.11 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType=typeII-CJT-PortSelection is provided in Table 6.3.2.1.2-9A.

Table 6.3.2.1.2-9A: RI and CQI of codebookType= typeII-CJT-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers, and all CSI-RS resources if configured, KNZ | log2βMK1,max if max allowed rank is 1; log22βMK1,max otherwise |

Where nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.9 TS 38.214 [6], K1,max is the maximum of n=1NTRPK1, n for all NL configured αn combinations, where K1.nn=1,…, NTRP, M, and β are given by Clause 5.2.2.2.9 in TS 38.214 [6], and. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clause 5.2.2.2.9 TS 38.214 [6], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for reportQuantity=tdcp is provided in Table 6.3.2.1.2-10.

Table 6.3.2.1.2-10: Amplitude and phase values for reportQuantity=tdcp

| Field | Bitwidth |
| --- | --- |
| Amplitude value | 4 |
| Phase value | 4 |

< Unchanged parts are omitted >

### 7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI) |
| 0_2 | Scheduling of PUSCH in one cell |
| 0_3 | Scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of one or multiple PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback |
| 1_2 | Scheduling of PDSCH in one cell |
| 1_3 | Scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell |
| 2_0 | Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |
| 2_4 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE |
| 2_5 | Notifying the availability of soft resources as defined in Clause 9.3.1 of [10, TS 38.473] |
| 2_6 | Notifying the power saving information outside DRX Active Time for one or more UEs |
| 2_7 | Notifying paging early indication and TRS availability indication for one or more UEs |
| 2_8 | Notifying the aperiodic beam indication and associated time resources |
| 2_9 | Activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs |
| 3_0 | Scheduling of NR sidelink in one cell |
| 3_1 | Scheduling of LTE sidelink in one cell |
| 3_2 | Scheduling of NR SL PRS in one cell |
| 4_0 | Scheduling of PDSCH with CRC scrambled by MCCH-RNTI/G-RNTI for broadcast or by Multicast MCCH-RNTI for multicast in RRC_INACITIVE state |
| 4_1 | Scheduling of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast in RRC_CONNECTED state or by G-RNTI for multicast in RRC_INACITIVE state |
| 4_2 | Scheduling of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast in RRC_CONNECTED state |

The fields defined in the DCI formats below are mapped to the information bits  to  as follows.

Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit  and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to .

If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12.

The size of each DCI format except for DCI format 0_3/1_3 is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary.

For a cell set configured by higher layer parameter mc-DCI-SetofCellsToAddModList, the size of DCI format 0_3/1_3 is determined as follows and shall be adjusted as described in Clause 7.3.1.0 if necessary:

-	If scheduledCellComboListDCI-0-3 for the cell set is configured, the size of DCI format 0_3 is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-0-3; Otherwise, the size of DCI format 0_3 is determined by the configuration of the corresponding active bandwidth part(s) of the cells configured by higher layer parameter scheduledCellListDCI-0-3 for the cell set.

-	If scheduledCellComboListDCI-1-3 for the cell set is configured, the size of DCI format 1_3 is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-1-3; Otherwise, the size of DCI format 1_3 is determined by the configuration of the corresponding active bandwidth part(s) of the cells configured by higher layer parameter scheduledCellListDCI-1-3 for the cell set.

If a UE is configured with pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookList-r16 in this clause.

If a UE is configured with pdsch-HARQ-ACK-CodebookListMulticast-r17, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookListMulticast-r17 in this clause.

For a cell detected in cell search procedure with synchronization raster defined in Table 5.4.3.1-2 or Table 5.4.3.1-3 of [13, TS 38.101-1], the size of CORESET 0 for the cell in this clause refers to the size of punctured CORESET 0 as defined in clause 7.3.2.2 of [4, TS 38.211] if any.

< Unchanged parts are omitted >

#### 7.3.1.1	DCI formats for scheduling of PUSCH

< Unchanged parts are omitted >

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator - 0 or 3 bits, as defined in Clause 10.1 of [5, TS38.213]. This field is reserved when this format is carried by PDCCH on the primary cell and the UE is configured for scheduling on the primary cell from an SCell, with the same number of bits as that in this format carried by PDCCH on the SCell for scheduling on the primary cell.

-	DFI flag - 0 or 1 bit

-	1 bit if the UE is configured to monitor DCI format 0_1 with CRC scrambled by CS-RNTI and for operation in a cell with shared spectrum channel access when the higher layer parameter cg-RetransmissionTimer is configured. For a DCI format 0_1 with CRC scrambled by CS-RNTI, the bit value of 0 indicates activating or releasing type 2 CG transmission and the bit value of 1 indicates CG-DFI. For a DCI format 0_1 with CRC scrambled by C-RNTI/SP-CSI-RNTI/MCS-C-RNTI and for operation in a cell with shared spectrum channel access, the bit is reserved.

-	0 bit otherwise;

If DCI format 0_1 is used for indicating CG-DFI, all the remaining fields are set as follows:

-	HARQ-ACK bitmap - 16 bits if nrofHARQ-Processes-v1700 in ConfiguredGrantConfig is not configured or 32 bits if nrofHARQ-Processes-v1700 in ConfiguredGrantConfig is configured, where the order of the bitmap to HARQ process index mapping is such that HARQ process indices are mapped in ascending order from MSB to LSB of the bitmap. For each bit of the bitmap, value 1 indicates ACK, and value 0 indicates NACK.

-	TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	All the remaining bits in format 0_1 are set to zero.

Otherwise, all the remaining fields are set as follows:

-	UL/SUL indicator - 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator - 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment - number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured

-	 bits if only resource allocation type 0 is configured, where  is defined in Clause 6.1.2.2.1 of [6, TS 38.214],

-	bits if only resource allocation type 1 is configured, or  bits if resourceAllocation is configured as 'dynamicSwitch'.

-	If resourceAllocation is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the  LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocation is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured

-	5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

-	6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12  where NRB-set,ULBWP  is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214].

-	Time domain resource assignment - 0, 1, 2, 3, 4, 5, or 6 bits

-	If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is not configured and if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is not configured and if the higher layer parameter pusch-TimeDomainAllocationList is configured, 0, 1, 2, 3, or 4 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList;

-	If the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 is configured or if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-1 or pusch-TimeDomainAllocationListForMultiPUSCH;

-	otherwise the bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the default table.

-	Frequency hopping flag - 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured, or if the higher layer parameter frequencyHopping is not configured and the higher layer parameter pusch-RepTypeIndicatorDCI-0-1 is not configured to pusch-RepTypeB, or if the higher layer parameter frequencyHoppingDCI-0-1 is not configured and pusch-RepTypeIndicatorDCI-0-1 is configured to pusch-RepTypeB, or if only resource allocation type 2 is configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

For transport block 1:

-	Modulation and coding scheme - 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator - 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version - - number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2 (only present if maxRank or maxMIMO-Layers is larger than 4):

-	Modulation and coding scheme - 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part, maxRank is larger than 4 or the value of maxMIMO-Layers for the indicated bandwidth part is larger than 4 and the value of maxRank or maxMIMO-Layers for the active bandwidth part is no more than 4, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields for transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	Transform precoder indicator - 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderFieldPresenceDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_1 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the bit is reserved.

-	0 bit otherwise.

-	HARQ process number - 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-0-1 is configured; otherwise 4 bits

-	1st downlink assignment index - 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook for unicast and multicast if pdsch-HARQ-ACK-Codebook = semiStatic is configured for both unicast and multicast and the higher layer parameter fdmed-ReceptionMulticast is not configured; otherwise for semi-static HARQ-ACK codebook for unicast;

-	2 bits for dynamic HARQ-ACK codebook for unicast, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included = true.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st  downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	2nd downlink assignment index - 0, 2 or 4 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks for unicast, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included = true;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	3rd downlink assignment index - 0, 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook for multicast if the higher layer parameter fdmed-ReceptionMulticast is configured;

-	2 bits for the dynamic HARQ-ACK codebook for multicast;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 3rd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 3rd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 3rd downlink assignment index until the bit width of the 3rd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	Second TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213] if higher layer parameter SecondTPCFieldDCI-0-1 is configured; 0 bit otherwise.

-	SRS resource set indicator - 0 or 2 bits

-	2 bits according to Table 7.3.1.1.2-36 if

-	txConfig = nonCodeBook, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with the usage of value 'nonCodeBook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, or

-	txConfig=codebook, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided;

-	0 bit otherwise.

-	SRS resource indicator -number of bits determined by the following:

-	 bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B/31C/31D/31E/31F if the higher layer parameter txConfig = nonCodebook, where

-	 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel,

-	otherwise NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList and associated with the higher layer parameter usage of value 'nonCodeBook',

and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSDM} if maxMIMO-LayersforSDM is configured

-	Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSFN} if maxMIMO-LayersforSFN is configured

-	Lmax is given by maxMIMO-Layers otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook, where

-	 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel,

-	otherwise NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList and associated with the higher layer parameter usage of value 'codeBook'.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel,  and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook' or 'nonCodeBook', the first SRS resource set is associated with coresetPoolIndex value 0 and the second SRS resource set is associated with coresetPoolIndex value 1, where the first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets.

-	Second SRS resource indicator - number of bits determined by the following:

-	log2(maxk∈{1,2,…,min⁡{Lmax,NSRS}}NSRSk) bits according to Tables 7.3.1.1.2-28/29A/30A/31A with the same number of layers indicated by SRS resource indicator field if the higher layer parameter txConfig = nonCodebook, the higher layer paramtere maxMIMO-LayersforSDM is not configured, and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set, and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by maxMIMO-LayersforSFN if maxMIMO-LayersforSFN is configured

-	Lmax is given by maxMIMO-Layers otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-28/29 if the higher layer parameter txConfig = nonCodebook, the higher layer paramtere maxMIMO-LayersforSDM is configured and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set, and Lmax is given by maxMIMO-LayersforSDM.

-	log2(NSRS) bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set.

-	0 bit otherwise.

-	Precoding information and number of layers - number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} if multipanelSchemeSDM is configured, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 2 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 2 if multipanelSchemeSDM is configured, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 1 if multipanelSchemeSDM is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 1 if multipanelSchemeSDM is configured, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 2 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 2 if multipanelSchemeSDM is configured, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN}=1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM}=1 if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 1 if multipanelSchemeSDM is configured, and the values of higher layer parameter codebookSubset;

-	7 bits according to Table 7.3.1.1.2-5B for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is disabled, maxRank = 8, and according to codebook1;

-	7 bits according to Table 7.3.1.1.2-5C for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is disabled, maxRank = 7, and according to codebook1;

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is disabled, maxRank = 4, 5 or 6, and according to maxRank;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is enabled or maxRank = 1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRank;

-	8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookTypeUL= codebook4, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookTypeUL= codebook4, transform precoder is disabled, maxRank = 2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookTypeUL= codebook4, transform precoder is enabled or maxRank = 1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRank = 2, 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is enabled or maxRank = 1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

-	5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRank = 1, 2, 3, or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

-	8 or 9 bits according to Table 7.3.1.1.2-5Q for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	10 bits according to Table 7.3.1.1.2-5R for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	10 bits according to Table 7.3.1.1.2-5S for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is disabled, maxRank = 5, 6, 7, or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports or 8 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 4, and at least one SRS resource with 8 antenna ports is configured in the SRS resource set with usage set to 'codebook', and an SRS resource with 4 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-2 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same.

-	Second Precoding information - number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=2 if multipanelSchemeSFN is configured, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=1 if multipanelSchemeSFN is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=2 if multipanelSchemeSFN is configured, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=1 if multipanelSchemeSFN is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSDM if multipanelSchemeSDM is configured, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSDM = 2 if multipanelSchemeSDM is configured, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankSDM if multipanelSchemeSDM is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSDM = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSDM if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRankSDM = 2 if multipanelSchemeSDM is configured, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankSDM= 1 if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSDM = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Second Precoding information field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Second Precoding information field for the case with smaller bit width until the bit width of the Second Precoding information field for the two cases are the same.

-	Antenna ports - number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by Tables 7.3.1.1.2-6A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, TS 38.211 [4];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, TS 38.211 [4];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/10A/11 according to the value of rank, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is not configured, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/14A/15/15A/15B/15C/15D according to the value of rank, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is not configured, and maxLength=2;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/18A/19/19A/19B according to the value of rank, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/22A/23/23A/23B/23C/23D according to the value of rank, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, and maxLength=2.

-	4 bits as defined by Tables 7.3.1.1.2-38/39/40/40A/41/42/43/44/45, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-46/47/48/48A/49/50/51/52/53, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is configured, and maxLength=2;

-	5 bits as defined by Tables 7.3.1.1.2-54/55/56/56A/57/58/59/60/61, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, and maxLength=1;

-	6 bits as defined by Tables 7.3.1.1.2-62/63/64/64A/65/66/67/68/69, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, and maxLength=2.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively, and the value of rank is:

-	the sum of the value determined according to the SRS resource indicator field and the value determined according to the second SRS resource indicator field, if txConfig = nonCodebook, multipanelSchemeSDM is configured and SRS resource set indicator field equals "10"

-	the sum of the value determined according to the Precoding information and number of layers field and the value determined according to the Second Precoding information, if txConfig = codebook, multipanelSchemeSDM is configured and SRS resource set indicator field equals "10"

-	determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and multipanelSchemeSDM is not configured, or if the higher layer parameter txConfig = nonCodebook, multipanelSchemeSDM is configured and SRS resource set indicator field equals "00" or “01”,

-	determined according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook and multipanelSchemeSDM is not configured, or if the higher layer parameter txConfig = codebook, multipanelSchemeSDM is configured and SRS resource set indicator field equals "00" or "01".

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

When the Transform precoder indicator field is present, if the bit width of the Antenna ports field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Antenna ports field for the case with smaller bit width until the bit width of the Antenna ports field for the two cases are the same.

-	SRS request - 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CSI request - 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) - 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits as defined in Clause 6.1.5 of [6, TS38.214], determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock and maxRank or maxMIMO-Layers for PUSCH.

-	PTRS-DMRS association - number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRank=1 and maxRankSFN=1, or if maxRank=1 and maxRankSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM;

-	2 or 4 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26A are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", and maxRank<=4 or maxMIMO-Layers<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or 4 or maxMIMO-Layers=3 or 4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when two PTRS ports are configured by maxNrofPorts-SDM in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port(s) corresponding to the selected codeword according to Table 7.3.1.1.2-25B, where the selected codeword is the codeword with higher MCS for the initial PUSCH if the MCS indices of the two codewords are different for the initial PUSCH, or codeword 0 otherwise.

-	4 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-26A.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same.

-	Second PTRS-DMRS association - 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; 0 bit otherwise. Tables 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator - 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList or by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled by higher layers and the Transform precoder indicator field is not present; 1 bit if transform precoder is disabled by higher layers or if the Transform precoder indicator field is present. If the Transform precoder indicator field is present and set to '0', the bit is reserved.

-	UL-SCH indicator - 0 or 1 bit as follows

-	0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1;

-	1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s). The UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", when the indicated number of layers is larger than 4.

-	ChannelAccess-CPext-CAPC - 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

-	Open-loop power control parameter set indication - 0 or 1 or 2 bits.

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator is present in the DCI format 0_1;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator is not present in the DCI format 0_1.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-0-1 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Invalid symbol pattern indicator - 0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-1 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214].

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK2 is not configured;

-	1 bit if higher layer parameter minimumSchedulingOffsetK2 is configured. The 1 bit indication is used to determine the minimum applicable K2 for the active UL BWP and the minimum applicable K0 value for the active DL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP shall be the same as the minimum applicable K0 value.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

-	Sidelink assignment index - 0, 1 or 2 bits:

-	1 bit if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static and, in addition,  the UE is configured with a SL configured grant type 1 or to monitor DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI;

-	2 bits if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic and, in addition, the UE is configured with a SL configured grant type 1 or to monitor DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI;

-	0 bit otherwise.

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.

If the number of information bits in DCI format 0_1 scheduling a single PUSCH prior to padding is not equal to the number of information bits in DCI format 0_1 scheduling multiple PUSCHs for the same serving cell, zeros shall be appended to the DCI format 0_1 with smaller size until the payload size is the same for scheduling a single PUSCH and multiple PUSCHs.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 0_1 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 0_1 with smaller size until the payload size is the same.

-	If application of step 4C in clause 7.3.1.0 results in additional zero padding for DCI format 0_1 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 0_1 monitored on the primary cell and DCI format 0_1 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | Configured BWP with BWP-Id = 1 |
| 01 | Configured BWP with BWP-Id = 2 |
| 10 | Configured BWP with BWP-Id = 3 |
| 11 | Configured BWP with BWP-Id = 4 |

Table 7.3.1.1.2-2: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4 or max{maxRank, maxRankSFN} = 2 or 3 or 4 or max{maxRank, maxRankSDM} = 2 or 3 or 4 or maxRankSDM= 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-2A: Precoding information and number of layers for 4 antenna ports or Second Precoding information,, if transform precoder is disabled, maxRank = 2 or max{maxRank, maxRankSFN} = 2 or max{maxRank, maxRankSDM} = 2 or maxRankSDM= 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 1 layer: TPMI=13 | 10 | 1 layer: TPMI=13 |
| 11 | 2 layer: TPMI=6 | 11 | 2 layer: TPMI=6 |
| 12 | 1 layer: TPMI=4 | 12-15 | Reserved |
| … | … |  |  |
| 20 | 1 layer: TPMI=12 |  |  |
| 21 | 1 layer: TPMI=14 |  |  |
| 22 | 1 layer: TPMI=15 |  |  |
| 23 | 2 layers: TPMI=7 |  |  |
| … | … |  |  |
| 29 | 2 layers: TPMI=13 |  |  |
| 30-31 | Reserved |  |  |

Table 7.3.1.1.2-2B: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=13 | 12 | 1 layer: TPMI=13 |
| 13 | 2 layer: TPMI=6 | 13 | 2 layer: TPMI=6 |
| 14 | 3 layer: TPMI=1 | 14 | 3 layer: TPMI=1 |
| 15 | 1 layer: TPMI=4 | 15 | Reserved |
| … | … |  |  |
| 23 | 1 layer: TPMI=12 |  |  |
| 24 | 1 layer: TPMI=14 |  |  |
| 25 | 1 layer: TPMI=15 |  |  |
| 26 | 2 layers: TPMI=7 |  |  |
| … | … |  |  |
| 32 | 2 layers: TPMI=13 |  |  |
| 33 | 3 layers: TPMI=2 |  |  |
| 34 | 4 layers: TPMI=1 |  |  |
| 35 | 4 layers: TPMI=2 |  |  |
| 36-63 | Reserved |  |  |

Table 7.3.1.1.2-2C: Second precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4 or maxRankSFN = 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 27 | 1 layer: TPMI=27 | 11 | 1 layer: TPMI=11 | 3 | 1 layer: TPMI=3 |
| 28-31 | 1 layer: reserved | 12-15 | 1 layer: reserved | 4-7 | 1 layer: reserved |
| 0 | 2 layers: TPMI=0 | 0 | 2 layers: TPMI=0 | 0 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 21 | 2 layers: TPMI=21 | 13 | 2 layers: TPMI=13 | 5 | 2 layers: TPMI=5 |
| 22-31 | 2 layers: reserved | 14-15 | 2 layers: reserved | 6-7 | 2 layers: reserved |
| 0 | 3 layers: TPMI=0 | 0 | 3 layers: TPMI=0 | 0 | 3 layers: TPMI=0 |
| … | … | … | … | 1-7 | 3 layers: reserved |
| 6 | 3 layers: TPMI=6 | 2 | 3 layers: TPMI=2 | 0 | 4 layers: TPMI=0 |
| 7-31 | 3 layers: reserved | 3-15 | 3 layers: reserved | 1-7 | 4 layers: reserved |
| 0 | 4 layers: TPMI=0 | 0 | 4 layers: TPMI=0 |  |  |
| … | … | … | … |  |  |
| 4 | 4 layers: TPMI=4 | 2 | 4 layers: TPMI=2 |  |  |
| 5-31 | 4 layers: reserved | 3-15 | 4 layers: reserved |  |  |

Table 7.3.1.1.2-2D: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or maxRankSFN = 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … |
| 14 | 1 layer: TPMI=14 | 3 | 1 layer: TPMI=3 |
| 15 | 1 layer: TPMI=15 | 4 | 1 layer: TPMI=13 |
| 0 | 2 layers: TPMI=0 | 5-7 | 1 layer: reserved |
| … | … | 0 | 2 layers: TPMI=0 |
| 13 | 2 layers: TPMI=13 | … | … |
| 14-15 | 2 layers: reserved | 6 | 2 layers: TPMI=6 |
|  |  | 7 | 2 layers: reserved |

Table 7.3.1.1.2-2E: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … |
| 14 | 1 layer: TPMI=14 | 3 | 1 layer: TPMI=3 |
| 15 | 1 layer: TPMI=15 | 4 | 1 layer: TPMI=13 |
| 0 | 2 layers: TPMI=0 | 5-7 | 1 layer: reserved |
| … | … | 0 | 2 layers: TPMI=0 |
| 13 | 2 layers: TPMI=13 | … | … |
| 14-15 | 2 layers: reserved | 6 | 2 layers: TPMI=6 |
| 0 | 3 layers: TPMI=0 | 7 | 2 layers: reserved |
| … | … | 0 | 3 layers: TPMI=0 |
| 2 | 3 layers: TPMI=2 | 1 | 3 layer: TPMI=1 |
| 3-15 | 3 layers: reserved | 2-7 | 3 layers: reserved |
| 0 | 4 layers: TPMI=0 | 0 | 4 layers: TPMI=0 |
| … | … | 1-7 | 4 layers: reserved |
| 2 | 4 layers: TPMI=2 |  |  |
| 3-15 | 4 layers: reserved |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission is either not configured or configured to fullpowerMode2 or configured to fullpower, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSFN} = 1 or max{maxRank, maxRankSDM} = 1 or maxRankSDM= 1 or maxRankSFN= 1, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-3A: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission = fullpowerMode1, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSFN} = 1 or max{maxRank, maxRankSDM} = 1 or maxRankSDM= 1 or maxRankSFN= 1, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=13 | 4 | 1 layer: TPMI=13 |
| 5 | 1 layer: TPMI=4 | 5-7 | Reserved |
| … | … |  |  |
| 13 | 1 layer: TPMI=12 |  |  |
| 14 | 1 layer: TPMI=14 |  |  |
| 15 | 1 layer: TPMI=15 |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers or Second Precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or max{maxRank, maxRankSFN} = 2 or max{maxRank, maxRankSDM} = 2 or maxRankSDM= 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-4A: Precoding information and number of layers or Second Precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or max{maxRank, maxRankSFN} = 2 or max{maxRank, maxRankSDM} = 2 or maxRankSDM= 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 |

Table 7.3.1.1.2-4B: Second precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or maxRankSFN = 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | 0 | 2 layers: TPMI=0 |
| 5 | 1 layer: TPMI=5 | 1 | 2 layers: reserved |
| 6-7 | 1 layer: reserved |  |  |
| 0 | 2 layers: TPMI=0 |  |  |
| … | … |  |  |
| 2 | 2 layers: TPMI=2 |  |  |
| 3-7 | 2 layers: reserved |  |  |

Table 7.3.1.1.2-4C: Second precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or maxRankSFN = 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: reserved |
| 0 | 2 layers: TPMI=0 |
| 1-3 | 2 layers: reserved |

Table 7.3.1.1.2-5: Precoding information and number of layers or Second Precoding information, for 2 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSFN} = 1 or max{maxRank, maxRankSDM} = 1 or maxRankSDM= 1 or maxRankSFN= 1, and and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

Table 7.3.1.1.2-5A: Precoding information and number of layers or Second Precoding information, for 2 antenna ports or Second Precoding information, if transform precoder is enabled and ul-FullPowerTransmission = fullpowerMode1, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSFN} = 1 or max{maxRank, maxRankSDM} = 1 or maxRankSDM= 1 or maxRankSFN= 1, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | Reserved |

Table 7.3.1.1.2-5B: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 8, and CodebookTypeUL=codebook1

| Bit field mapped to index | codebook1 = ng1n4n1 | Bit field mapped to index | codebook1 = ng1n2n2 |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layer2: TPMI=0 |
| 17 | 2 layers: TPMI=1 | 17 | 2 layer2: TPMI=1 |
| … | … | … | … |
| 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
| 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 |
| 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 |
| … | … | … | … |
| 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 |
| 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 |
| 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 |
| … | … | … | … |
| 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 |
| 96 | 5 layers: TPMI=0 | 96 | 5 layers: TPMI=0 |
| 97 | 5 layers: TPMI=1 | 97 | 5 layers: TPMI=1 |
| … | … | … | … |
| 103 | 5 layers: TPMI=7 | 103 | 5 layers: TPMI=7 |
| 104 | 6 layers: TPMI=0 | 104 | 6 layers: TPMI=0 |
| 105 | 6 layers: TPMI=1 | 105 | 6 layers: TPMI=1 |
| … | … | … | … |
| 111 | 6 layers: TPMI=7 | 111 | 6 layers: TPMI=7 |
| 112 | 7 layers: TPMI=0 | 112 | 7 layers: TPMI=0 |
| 113 | 7 layers: TPMI=1 | 113 | 7 layers: TPMI=1 |
| … | … | … | … |
| 115 | 7 layers: TPMI=3 | 119 | 7 layers: TPMI=7 |
| 116 | 8 layers: TPMI=0 | 120 | 8 layers: TPMI=0 |
| 117 | 8 layers: TPMI=1 | 121 | 8 layers: TPMI=1 |
| … | … | … | … |
| 119 | 8 layers: TPMI=3 | 127 | 8 layers: TPMI=7 |
| 120-127 | reserved |  |  |

Table 7.3.1.1.2-5C: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 7, and CodebookTypeUL=codebook1

| Bit field mapped to index | codebook1= ng1n4n1 | Bit field mapped to index | codebook1= ng1n2n2 |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layer2: TPMI=0 |
| 17 | 2 layers: TPMI=1 | 17 | 2 layer2: TPMI=1 |
| … | … | … | … |
| 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
| 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 |
| 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 |
| … | … | … | … |
| 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 |
| 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 |
| 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 |
| … | … | … | … |
| 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 |
| 96 | 5 layers: TPMI=0 | 96 | 5 layers: TPMI=0 |
| 97 | 5 layers: TPMI=1 | 97 | 5 layers: TPMI=1 |
| … | … | … | … |
| 103 | 5 layers: TPMI=7 | 103 | 5 layers: TPMI=7 |
| 104 | 6 layers: TPMI=0 | 104 | 6 layers: TPMI=0 |
| 105 | 6 layers: TPMI=1 | 105 | 6 layers: TPMI=1 |
| … | … | … | … |
| 111 | 6 layers: TPMI=7 | 111 | 6 layers: TPMI=7 |
| 112 | 7 layers: TPMI=0 | 112 | 7 layers: TPMI=0 |
| 113 | 7 layers: TPMI=1 | 113 | 7 layers: TPMI=1 |
| … | … | … | … |
| 115 | 7 layers: TPMI=3 | 119 | 7 layers: TPMI=7 |
| 116-127 | reserved | 120-127 | reserved |

Table 7.3.1.1.2-5D: Precoding information and number of layers, for 8 antenna ports,
if transform precoder is disabled, maxRank = 4, 5 or 6, CodebookTypeUL=codebook1

| Bit field mapped to index | maxRank = 4 | Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layer2: TPMI=0 |
| 17 | 2 layers: TPMI=1 | 17 | 2 layers: TPMI=1 | 17 | 2 layer2: TPMI=1 |
| … | … | … | … | … | … |
| 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
| 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 |
| 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 |
| … | … | … | … | … | … |
| 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 |
| 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 |
| 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 |
| … | … | … | … | … | … |
| 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 |
| 96-127 | reserved | 96 | 5 layers: TPMI=0 | 96 | 5 layers: TPMI=0 |
|  |  | 97 | 5 layers: TPMI=1 | 97 | 5 layers: TPMI=1 |
|  |  | … | … | … | … |
|  |  | 103 | 5 layers: TPMI=7 | 103 | 5 layers: TPMI=7 |
|  |  | 104-127 | reserved | 104 | 6 layers: TPMI=0 |
|  |  |  |  | 105 | 6 layers: TPMI=1 |
|  |  |  |  | … | … |
|  |  |  |  | 111 | 6 layers: TPMI=7 |
|  |  |  |  | 112-127 | reserved |

Table 7.3.1.1.2-5E: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 or 2 or 3 if transform precoder is disabled, CodebookTypeUL=codebook1

| Bit field mapped to index | transform precoder is enabled, or maxRank=1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled, and maxRank=2 | Bit field mapped to index | transform precoder is disabled, and maxRank=3 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
|  |  | 16 | 2 layer2: TPMI=0 | 16 | 2 layer2: TPMI=0 |
|  |  | 17 | 2 layer2: TPMI=1 | 17 | 2 layer2: TPMI=1 |
|  |  | … | … | … | … |
|  |  | 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
|  |  | 48-63 | reserved | 48 | 3 layers: TPMI=0 |
|  |  |  |  | 49 | 3 layers: TPMI=1 |
|  |  |  |  | … | … |
|  |  |  |  | 71 | 3 layers: TPMI=23 |
|  |  |  |  | 72-127 | reserved |

Table 7.3.1.1.2-5F: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookTypeUL=codebook4

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
| … | … | … | … | … | … | … | … |
| 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
| 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 |
| … | … | … | … | … | … | … | … |
| 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 |
| 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 |
| … | … | … | … | … | … | … | … |
| 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 |
| 218-255 | reserved | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 |
|  |  | … | … | … | … | … | … |
|  |  | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 |
|  |  | 246-255 | reserved | 246 | 7 layers: TPMI=246 | 246 | 7 layers: TPMI=246 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 253 | 7 layers: TPMI=253 | 253 | 7 layers: TPMI=253 |
|  |  |  |  | 254-255 | reserved | 254 | 8 layers: TPMI=254 |
|  |  |  |  |  |  | 255 | reserved |

Table 7.3.1.1.2-5G: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookTypeUL=codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | maxRank = 2 | Bit field mapped to index | maxRank = 3 | Bit field mapped to index | maxRank = 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36-63 | reserved | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
|  |  | … | … | … | … |
|  |  | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
|  |  | 92-127 | reserved | 92 | 4 layers: TPMI=92 |
|  |  |  |  | … | … |
|  |  |  |  | 161 | 4 layers: TPMI=161 |
|  |  |  |  | 162-255 | reserved |

Table 7.3.1.1.2-5H: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookTypeUL=codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | Precoding information and number of layers |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 7 | 1 layer: TPMI=7 |

Table 7.3.1.1.2-5I: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookTypeUL=codebook2

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
| 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
| 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
| 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 |
| 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 |
| 698-1023 | reserved | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 |
|  |  | 684-1023 | reserved | 684 | 7 layers: TPMI=0 | 684 | 7 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 691 | 7 layers: TPMI=7 | 691 | 7 layers: TPMI=7 |
|  |  |  |  | 692-1023 | reserved | 692 | 8 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 695 | 8 layers: TPMI=3 |
|  |  |  |  |  |  | 696-1023 | reserved |

Table 7.3.1.1.2-5J: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookTypeUL=codebook2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 | Bit field mapped to index | transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
|  |  | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
|  |  | 303-511 | reserved | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
|  |  |  |  | 568-1023 | reserved | 568 | 4 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 635 | 4 layers: TPMI=67 |
|  |  |  |  |  |  | 636-1023 | reserved |

Table 7.3.1.1.2-5K: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookTypeUL=codebook3

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
| 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
| 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 |
| 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 |
| 864-1023 | reserved | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 |
|  |  | 944-1023 | reserved | 944 | 7 layers: TPMI=0 | 944 | 7 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 975 | 7 layers: TPMI=31 | 975 | 7 layers: TPMI=31 |
|  |  |  |  | 976-1023 | reserved | 976 | 8 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 991 | 8 layers: TPMI=15 |
|  |  |  |  |  |  | 992-1023 | reserved |

Table 7.3.1.1.2-5L: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookTypeUL=codebook3, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 | Bit field mapped to index | transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
|  |  | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
|  |  | 119-127 | reserved | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
|  |  |  |  | 424-511 | reserved | 424 | 4 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 703 | 4 layers: TPMI=279 |
|  |  |  |  |  |  | 704-1024 | reserved |

Table 7.3.1.1.2-5M: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookTypeUL=codebook4, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | maxRank = 2 | Bit field mapped to index | maxRank = 3 | Bit field mapped to index | maxRank = 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 1 layer: TPMI=255 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
| 37 | 2 layers: TPMI=256 | … | … | … | … |
| 38-63 | reserved | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
|  |  | 92 | 1 layer: TPMI=255 | 92 | 4 layers: TPMI=92 |
|  |  | 93 | 2 layers: TPMI=256 | … | … |
|  |  | 94 | 3 layers: TPMI=257 | 161 | 4 layers: TPMI=161 |
|  |  | 95-127 | reserved | 162 | 1 layer: TPMI=255 |
|  |  |  |  | 163 | 2 layers: TPMI=256 |
|  |  |  |  | 164 | 3 layers: TPMI=257 |
|  |  |  |  | 165 | 4 layers: 258 |
|  |  |  |  | 166-255 | reserved |

Table 7.3.1.1.2-5N: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookTypeUL=codebook4, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | Precoding information and number of layers |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 7 | 1 layer: TPMI=7 |
| 8 | 1 layer: TPMI=255 |
| 9-15 | reserved |

Table 7.3.1.1.2-5O: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookTypeUL=codebook2, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | Transform precoder is disabled and maxRank = 2 | Bit field mapped to index | Transform precoder is disabled and maxRank = 3 | Bit field mapped to index | Transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
| 32 | 1 layer: TPMI=32 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
| 33-63 | reserved | … | … | … | … | … | … |
|  |  | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
|  |  | 304 | 1 layer: TPMI=32 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
|  |  | 305-511 | reserved | … | … | … | … |
|  |  |  |  | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
|  |  |  |  | 568 | 1 layer: TPMI=32 | 568 | 4 layers: TPMI=0 |
|  |  |  |  | 569-1023 | reserved | … | … |
|  |  |  |  |  |  | 635 | 4 layers: TPMI=67 |
|  |  |  |  |  |  | 636 | 1 layer: TPMI=32 |
|  |  |  |  |  |  | 637-1023 | reserved |

Table 7.3.1.1.2-5P: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookTypeUL=codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 | Bit field mapped to index | transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 1 layer: TPMI=16 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| 17-31 | reserved | … | … | … | … | … | … |
|  |  | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
|  |  | 120 | 1 layer: TPMI=16 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
|  |  | 121 | 2 layers: TPMI=104 | … | … | … | … |
|  |  | 122-127 | reserved | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
|  |  |  |  | 424 | 1 layer: TPMI=16 | 424 | 4 layers: TPMI=0 |
|  |  |  |  | 425 | 2 layers: TPMI=104 | … | … |
|  |  |  |  | 426 | 3 layers: 304 | 703 | 4 layers: TPMI=279 |
|  |  |  |  | 427-511 | reserved | 704 | 1 layer: TPMI=16 |
|  |  |  |  |  |  | 705 | 2 layers: TPMI=104 |
|  |  |  |  |  |  | 706 | 3 layers: TPMI=304 |
|  |  |  |  |  |  | 707-1023 | reserved |

Table 7.3.1.1.2-5Q: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookTypeUL=codebook4, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
| … | … | … | … | … | … | … | … |
| 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
| 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 |
| … | … | … | … | … | … | … | … |
| 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 |
| 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 |
| … | … | … | … | … | … | … | … |
| 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 |
| 218 | 1 layer: TPMI=255 | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 |
| 219 | 2 layers: TPMI=256 | … | … | … | … | … | … |
| 220 | 3 layers: TPMI=257 | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 |
| 221 | 4 layers: TPMI=258 | 246 | 1 layer: TPMI=255 | 246 | 7 layers: TPMI=246 | 246 | 7 layers: TPMI=246 |
| 222-255 | reserved | 247 | 2 layers: TPMI=256 | … | … | … | … |
|  |  | 248 | 3 layers: TPMI=257 | 253 | 7 layers: TPMI=253 | 253 | 7 layers: TPMI=253 |
|  |  | 249 | 4 layers: TPMI=258 | 254 | 1 layer: TPMI=255 | 254 | 8 layers: TPMI=254 |
|  |  | 250-255 | reserved | 255 | 2 layers: TPMI=256 | 255 | 1 layer: TPMI=255 |
|  |  |  |  | 256 | 3 layers: TPMI=257 | 256 | 2 layers: TPMI=256 |
|  |  |  |  | 257 | 4 layers: TPMI=258 | 257 | 3 layers: TPMI=257 |
|  |  |  |  | 258-511 | reserved | 258 | 4 layers: TPMI=258 |
|  |  |  |  |  |  | 259-511 | reserved |

Table 7.3.1.1.2-5R: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookTypeUL=codebook2, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
| 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
| 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
| 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 |
| 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 |
| 668 | 1 layer: TPMI=32 | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 |
| 669-1023 | reserved | … | … | … | … | … | … |
|  |  | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 |
|  |  | 684 | 1 layer: TPMI=32 | 684 | 7 layers: TPMI=0 | 684 | 7 layers: TPMI=0 |
|  |  | 685-1023 | reserved | … | … | … | … |
|  |  |  |  | 691 | 7 layers: TPMI=7 | 691 | 7 layers: TPMI=7 |
|  |  |  |  | 692 | 1 layer: TPMI=32 | 692 | 8 layers: TPMI=0 |
|  |  |  |  | 693-1023 | reserved | … | … |
|  |  |  |  |  |  | 695 | 8 layers: TPMI=3 |
|  |  |  |  |  |  | 696 | 1 layer: TPMI=32 |
|  |  |  |  |  |  | 697-1023 | reserved |

Table 7.3.1.1.2-5S: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookTypeUL=codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
| 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
| 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 |
| 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 |
| 864 | 1 layer: TPMI=16 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 |
| 865 | 2 layers: TPMI=104 | … | … | … | … | … | … |
| 866 | 3 layers: TPMI=304 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 |
| 867-1023 | reserved | 944 | 1 layer: TPMI=16 | 944 | 7 layers: TPMI=0 | 944 | 7 layers: TPMI=0 |
|  |  | 945 | 2 layers: TPMI=104 | … | … | … | … |
|  |  | 946 | 3 layers: TPMI=304 | 975 | 7 layers: TPMI=31 | 975 | 7 layers: TPMI=31 |
|  |  | 944-1023 | reserved | 976 | 1 layer: TPMI=16 | 976 | 8 layers: TPMI=0 |
|  |  |  |  | 977 | 2 layers: TPMI=104 | … | … |
|  |  |  |  | 978 | 3 layers: TPMI=304 | 991 | 8 layers: TPMI=15 |
|  |  |  |  | 979-1023 | reserved | 992 | 1 layer: TPMI=16 |
|  |  |  |  |  |  | 993 | 2 layers: TPMI=104 |
|  |  |  |  |  |  | 994 | 3 layers: TPMI=304 |
|  |  |  |  |  |  | 995-1023 | reserved |

Table 7.3.1.1.2-6: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=1,
except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and
π/2-BPSK modulation is used

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |

Table 7.3.1.1.2-6A: Antenna port(s), transform precoder is enabled, dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2-BPSK modulation is used, dmrs-Type=1, maxLength=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0, nSCID= 0 |
| 1 | 2 | 0, nSCID= 1 |
| 2 | 2 | 2, nSCID= 0 |
| 3 | 2 | 2, nSCID= 1 |

Table 7.3.1.1.2-7: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=2,
except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and
π/2-BPSK modulation is used

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 1 |
| 1 | 2 | 1 | 1 |
| 2 | 2 | 2 | 1 |
| 3 | 2 | 3 | 1 |
| 4 | 2 | 0 | 2 |
| 5 | 2 | 1 | 2 |
| 6 | 2 | 2 | 2 |
| 7 | 2 | 3 | 2 |
| 8 | 2 | 4 | 2 |
| 9 | 2 | 5 | 2 |
| 10 | 2 | 6 | 2 |
| 11 | 2 | 7 | 2 |
| 12-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-7A: Antenna port(s), transform precoder is enabled, dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2-BPSK modulation is used, dmrs-Type=1, maxLength=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0, nSCID= 0 | 1 |
| 1 | 2 | 0, nSCID= 1 | 1 |
| 2 | 2 | 2, nSCID= 0 | 1 |
| 3 | 2 | 2, nSCID= 1 | 1 |
| 4 | 2 | 0, nSCID= 0 | 2 |
| 5 | 2 | 0, nSCID= 1 | 2 |
| 6 | 2 | 2, nSCID= 0 | 2 |
| 7 | 2 | 2, nSCID= 1 | 2 |
| 8 | 2 | 4, nSCID= 0 | 2 |
| 9 | 2 | 4, nSCID= 1 | 2 |
| 10 | 2 | 6, nSCID= 0 | 2 |
| 11 | 2 | 6, nSCID= 1 | 2 |
| 12-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-8: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6-7 | Reserved | Reserved |

Table 7.3.1.1.2-9: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4-7 | Reserved | Reserved |

Table 7.3.1.1.2-10: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not
configured, dmrs-Type=1, dmrs-TypeEnh is not configured,
maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1-7 | Reserved | Reserved |

Table 7.3.1.1.2-10A: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is configured, dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | 0,2,3 |
| 2-7 | Reserved | Reserved |

Table 7.3.1.1.2-11: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1-7 | Reserved | Reserved |

Table 7.3.1.1.2-12: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14A: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is configured, dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 0,2,3 | 1 |
| 4-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15A: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15C: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15D: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is not configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=1, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-17: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=1, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7-15 | Reserved | Reserved |

Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not
configured, dmrs-Type=2, dmrs-TypeEnh is not configured,
maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-18A: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is configured, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 2 | 0,2,3 |
| 4 | 3 | 0,2,3 |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-19: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=1, rank =4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2-15 | Reserved | Reserved |

Table 7.3.1.1.2-19A: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-19B: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
enhanced-dmrs-Typedmrs-TypeEnh is not configured, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not
configured, dmrs-Type=2, dmrs-TypeEnh is not configured,
maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22A: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is configured, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 2 | 0,2,3 | 1 |
| 7 | 3 | 0,2,3 | 1 |
| 8-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank=4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23A: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23C: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23D: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is not configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field, or value of ‘SRS request’ index for each cell in the scheduled cell set indicated by SRS request field in DCI 0_3 or 1_3 | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 0_3, 1_1, 1_2, 1_3, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 1 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 2 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 3 when triggered by DCI formats 0_1, 0_2, 0_3, 1_1, 1_2 and 1_3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

Table 7.3.1.1.2-25: PTRS-DMRS association or Second PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |

Table 7.3.1.1.2-25A: PTRS-DMRS association for UL PTRS port 0 or for the actual UL PT-RS port if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or PTRS-DMRS association for UL PTRS port 0 and 1 if multipanelSchemeSDM is configured and maxNrofPorts-SDM is set to 2

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st scheduled DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field |  | 0 | 1st scheduled DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field |
| 1 | 2nd scheduled DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field |  | 1 | 2nd scheduled DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field |

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0, maxRank>4 or maxMIMO-Layers>4

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port corresponding to the selected Codeword |
| 1 | 2nd scheduled DMRS port corresponding to the selected Codeword |
| 2 | 3rd scheduled DMRS port corresponding to the selected Codeword |
| 3 | 4th scheduled DMRS port corresponding to the selected Codeword |

Table 7.3.1.1.2-26: PTRS-DMRS association or Second PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-26A: PTRS-DMRS association for UL PTRS ports 0 and 1, maxRank>4 or maxMIMO-Layers>4

| Value of 2 MSBs | DMRS port | Value of 2 LSBs | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-27: void

Table 7.3.1.1.2-28: SRI indication or Second SRI indication, for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 2 | 2 | 2 | 2 |
|  |  | 3 | reserved | 3 | 3 |

Table 7.3.1.1.2-28A: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=1

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5-7 | reserved | 5 | 5 | 5 | 5 | 5 | 5 |
|  |  | 6-7 | reserved | 6 | 6 | 6 | 6 |
|  |  |  |  | 7 | reserved | 7 | 7 |

Table 7.3.1.1.2-29: SRI indication or Second SRI indication, for
non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6-7 | reserved | 6 | 0,3 |
|  |  |  |  | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10-15 | reserved |

Table 7.3.1.1.2-29A: Second SRI indication for non-codebook based PUSCH transmission, Lmax=2

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0,1 | 2 | 2 | 2 | 2 |
| 1 | 2 layers: reserved | 3 | 1 layer: reserved | 3 | 3 |
|  |  | 0 | 0,1 | 4-7 | 1 layer: reserved |
|  |  | 1 | 0,2 | 0 | 0,1 |
|  |  | 2 | 1,2 | 1 | 0,2 |
|  |  | 3 | 2 layers: reserved | 2 | 0,3 |
|  |  |  |  | 3 | 1,2 |
|  |  |  |  | 4 | 1,3 |
|  |  |  |  | 5 | 2,3 |
|  |  |  |  | 6-7 | 2 layers: reserved |

Table 7.3.1.1.2-29B: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=2

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | … | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | … | 7 | s00,s10 | 7 | 7 |
| 15 | reserved | 20 | s014,s114 | … | … | 8 | s00,s10 |
|  |  | 21-31 | reserved | 27 | s020,s120 | … | … |
|  |  |  |  | 28-31 | reserved | 35 | s027,s127 |
|  |  |  |  |  |  | 36-63 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-29B-1: SRI(s) for 2 layers, NSRS>4

| SRI(s) | SRI(s) for 2 layers, NSRS=5 | SRI(s) for 2 layers, NSRS=6 | SRI(s) for 2 layers, NSRS=7 | SRI(s) for 2 layers, NSRS=8 |
| --- | --- | --- | --- | --- |
| s00,s10 | 0,1 | 0,1 | 0,1 | 0,1 |
| s01,s11 | 0,2 | 0,2 | 0,2 | 0,2 |
| s02,s12 | 0,3 | 0,3 | 0,3 | 0,3 |
| s03,s13 | 0,4 | 0,4 | 0,4 | 0,4 |
| s04,s14 | 1,2 | 0,5 | 0,5 | 0,5 |
| s05,s15 | 1,3 | 1,2 | 0,6 | 0,6 |
| s06,s16 | 1,4 | 1,3 | 1,2 | 0,7 |
| s07,s17 | 2,3 | 1,4 | 1,3 | 1,2 |
| s08,s18 | 2,4 | 1,5 | 1,4 | 1,3 |
| s09,s19 | 3,4 | 2,3 | 1,5 | 1,4 |
| s010,s110 |  | 2,4 | 1,6 | 1,5 |
| s011,s111 |  | 2,5 | 2,3 | 1,6 |
| s012,s112 |  | 3,4 | 2,4 | 1,7 |
| s013,s113 |  | 3,5 | 2,5 | 2,3 |
| s014,s114 |  | 4,5 | 2,6 | 2,4 |
| s015,s115 |  |  | 3,4 | 2,5 |
| s016,s116 |  |  | 3,5 | 2,6 |
| s017,s117 |  |  | 3,6 | 2,7 |
| s018,s118 |  |  | 4,5 | 3,4 |
| s019,s119 |  |  | 4,6 | 3,5 |
| s020,s120 |  |  | 5,6 | 3,6 |
| s021,s121 |  |  |  | 3,7 |
| s022,s122 |  |  |  | 4,5 |
| s023,s123 |  |  |  | 4,6 |
| s024,s124 |  |  |  | 4,7 |
| s025,s125 |  |  |  | 5,6 |
| s026,s126 |  |  |  | 5,7 |
| s027,s127 |  |  |  | 6,7 |

Table 7.3.1.1.2-30: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14-15 | reserved |

Table 7.3.1.1.2-30A: Second SRI indication for non-codebook based PUSCH transmission, if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, Lmax=3

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0,1 | 2 | 2 | 2 | 2 |
| 1 | 2 layers: reserved | 3 | 1 layer: reserved | 3 | 3 |
|  |  | 0 | 0,1 | 4-7 | 1 layer: reserved |
|  |  | 1 | 0,2 | 0 | 0,1 |
|  |  | 2 | 1,2 | 1 | 0,2 |
|  |  | 3 | 2 layers: reserved | 2 | 0,3 |
|  |  | 0 | 0,1,2 | 3 | 1,2 |
|  |  | 1-3 | 3 layers: reserved | 4 | 1,3 |
|  |  |  |  | 5 | 2,3 |
|  |  |  |  | 6-7 | 2 layers: reserved |
|  |  |  |  | 0 | 0,1,2 |
|  |  |  |  | 1 | 0,1,3 |
|  |  |  |  | 2 | 0,2,3 |
|  |  |  |  | 3 | 1,2,3 |
|  |  |  |  | 4-7 | 3 layers: reserved |

Table 7.3.1.1.2-30B: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=3

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25-31 | reserved | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
|  |  | 41-63 | reserved | 62 | s034,s134,s234 | … | ... |
|  |  |  |  | 63-127 | reserved | 91 | s055,s155,s255 |
|  |  |  |  |  |  | 92-127 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1 and Table 7.3.1.1.2-30B-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-30B-1: SRI combinations for 3 layers, NSRS>4

|  | SRI(s) for 3 layers, NSRS=5 | SRI(s) for 3 layers, NSRS=6 | SRI(s) for 3 layers, NSRS=7 | SRI(s) for 3 layers, NSRS=8 |
| --- | --- | --- | --- | --- |
| s00,s10,s20 | 0,1,2 | 0,1,2 | 0,1,2 | 0,1,2 |
| s01,s11,s21 | 0,1,3 | 0,1,3 | 0,1,3 | 0,1,3 |
| s02,s12,s22 | 0,1,4 | 0,1,4 | 0,1,4 | 0,1,4 |
| s03,s13,s23 | 0,2,3 | 0,1,5 | 0,1,5 | 0,1,5 |
| s04,s14,s24 | 0,2,4 | 0,2,3 | 0,1,6 | 0,1,6 |
| s05,s15,s25 | 0,3,4 | 0,2,4 | 0,2,3 | 0,1,7 |
| s06,s16,s26 | 1,2,3 | 0,2,5 | 0,2,4 | 0,2,3 |
| s07,s17,s27 | 1,2,4 | 0,3,4 | 0,2,5 | 0,2,4 |
| s08,s18,s28 | 1,3,4 | 0,3,5 | 0,2,6 | 0,2,5 |
| s09,s19,s29 | 2,3,4 | 0,4,5 | 0,3,4 | 0,2,6 |
| s010,s110,s210 |  | 1,2,3 | 0,3,5 | 0,2,7 |
| s011,s111,s211 |  | 1,2,4 | 0,3,6 | 0,3,4 |
| s012,s112,s212 |  | 1,2,5 | 0,4,5 | 0,3,5 |
| s013,s113,s213 |  | 1,3,4 | 0,4,6 | 0,3,6 |
| s014,s114,s214 |  | 1,3,5 | 0,5,6 | 0,3,7 |
| s015,s115,s215 |  | 1,4,5 | 1,2,3 | 0,4,5 |
| s016,s116,s216 |  | 2,3,4 | 1,2,4 | 0,4,6 |
| s017,s117,s217 |  | 2,3,5 | 1,2,5 | 0,4,7 |
| s018,s118,s218 |  | 2,4,5 | 1,2,6 | 0,5,6 |
| s019,s119,s219 |  | 3,4,5 | 1,3,4 | 0,5,7 |
| s020,s120,s220 |  |  | 1,3,5 | 0,6,7 |
| s021,s121,s221 |  |  | 1,3,6 | 1,2,3 |
| s022,s122,s222 |  |  | 1,4,5 | 1,2,4 |
| s023,s123,s223 |  |  | 1,4,6 | 1,2,5 |
| s024,s124,s224 |  |  | 1,5,6 | 1,2,6 |
| s025,s125,s225 |  |  | 2,3,4 | 1,2,7 |
| s026,s126,s226 |  |  | 2,3,5 | 1,3,4 |
| s027,s127,s227 |  |  | 2,3,6 | 1,3,5 |
| s028,s128,s228 |  |  | 2,4,5 | 1,3,6 |
| s029,s129,s229 |  |  | 2,4,6 | 1,3,7 |
| s030,s130,s230 |  |  | 2,5,6 | 1,4,5 |
| s031,s131,s231 |  |  | 3,4,5 | 1,4,6 |
| s032,s132,s232 |  |  | 3,4,6 | 1,4,7 |
| s033,s133,s233 |  |  | 3,5,6 | 1,5,6 |
| s034,s134,s234 |  |  | 4,5,6 | 1,5,7 |
| s035,s135,s235 |  |  |  | 1,6,7 |
| s036,s136,s236 |  |  |  | 2,3,4 |
| s037,s137,s237 |  |  |  | 2,3,5 |
| s038,s138,s238 |  |  |  | 2,3,6 |
| s039,s139,s239 |  |  |  | 2,3,7 |
| s040,s140,s240 |  |  |  | 2,4,5 |
| s041,s141,s241 |  |  |  | 2,4,6 |
| s042,s142,s242 |  |  |  | 2,4,7 |
| s043,s143,s243 |  |  |  | 2,5,6 |
| s044,s144,s244 |  |  |  | 2,5,7 |
| s045,s145,s245 |  |  |  | 2,6,7 |
| s046,s146,s246 |  |  |  | 3,4,5 |
| s047,s147,s247 |  |  |  | 3,4,6 |
| s048,s148,s248 |  |  |  | 3,4,7 |
| s049,s149,s249 |  |  |  | 3,5,6 |
| s050,s150,s250 |  |  |  | 3,5,7 |
| s051,s151,s251 |  |  |  | 3,6,7 |
| s052,s152,s252 |  |  |  | 4,5,6 |
| s053,s153,s253 |  |  |  | 4,5,7 |
| s054,s154,s254 |  |  |  | 4,6,7 |
| s055,s155,s255 |  |  |  | 5,6,7 |

Table 7.3.1.1.2-31: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14 | 0,1,2,3 |
|  |  |  |  | 15 | reserved |

Table 7.3.1.1.2-31A: Second SRI indication for non-codebook based PUSCH transmission,
if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, Lmax=4

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0,1 | 2 | 2 | 2 | 2 |
| 1 | 2 layers: reserved | 3 | 1 layer: reserved | 3 | 3 |
|  |  | 0 | 0,1 | 4-7 | 1 layer: reserved |
|  |  | 1 | 0,2 | 0 | 0,1 |
|  |  | 2 | 1,2 | 1 | 0,2 |
|  |  | 3 | 2 layers: reserved | 2 | 0,3 |
|  |  | 0 | 0,1,2 | 3 | 1,2 |
|  |  | 1-3 | 3 layers: reserved | 4 | 1,3 |
|  |  |  |  | 5 | 2,3 |
|  |  |  |  | 6-7 | 2 layers: reserved |
|  |  |  |  | 0 | 0,1,2 |
|  |  |  |  | 1 | 0,1,3 |
|  |  |  |  | 2 | 0,2,3 |
|  |  |  |  | 3 | 1,2,3 |
|  |  |  |  | 4-7 | 3 layer: reserved |
|  |  |  |  | 0 | 0,1,2,3 |
|  |  |  |  | 1-7 | 4 layers: reserved |

Table 7.3.1.1.2-31B: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=4

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30-31 | reserved | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
|  |  | 56-63 | reserved | 97 | s034,s134,s234,s334 | … | ... |
|  |  |  |  | 98-127 | reserved | 161 | s069,s169,s269,s369 |
|  |  |  |  |  |  | 162-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1 and Table 7.3.1.1.2-31B-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31B-1: SRI combinations for 4 layers, NSRS>4

|  | SRI(s) for 4 layers, NSRS=5 | SRI(s) for 4 layers, NSRS=6 | SRI(s) for 4 layers, NSRS=7 | SRI(s) for 4 layers, NSRS=8 |
| --- | --- | --- | --- | --- |
| s00,s10,s20,s30 | 0,1,2,3 | 0,1,2,3 | 0,1,2,3 | 0,1,2,3 |
| s01,s11,s21,s31 | 0,1,2,4 | 0,1,2,4 | 0,1,2,4 | 0,1,2,4 |
| s02,s12,s22,s32 | 0,1,3,4 | 0,1,2,5 | 0,1,2,5 | 0,1,2,5 |
| s03,s13,s23,s33 | 0,2,3,4 | 0,1,3,4 | 0,1,2,6 | 0,1,2,6 |
| s04,s14,s24,s34 | 1,2,3,4 | 0,1,3,5 | 0,1,3,4 | 0,1,2,7 |
| s05,s15,s25,s35 |  | 0,1,4,5 | 0,1,3,5 | 0,1,3,4 |
| s06,s16,s26,s36 |  | 0,2,3,4 | 0,1,3,6 | 0,1,3,5 |
| s07,s17,s27,s37 |  | 0,2,3,5 | 0,1,4,5 | 0,1,3,6 |
| s08,s18,s28,s38 |  | 0,2,4,5 | 0,1,4,6 | 0,1,3,7 |
| s09,s19,s29,s39 |  | 0,3,4,5 | 0,1,5,6 | 0,1,4,5 |
| s010,s110,s210,s310 |  | 1,2,3,4 | 0,2,3,4 | 0,1,4,6 |
| s011,s111,s211,s311 |  | 1,2,3,5 | 0,2,3,5 | 0,1,4,7 |
| s012,s112,s212,s312 |  | 1,2,4,5 | 0,2,3,6 | 0,1,5,6 |
| s013,s113,s213,s313 |  | 1,3,4,5 | 0,2,4,5 | 0,1,5,7 |
| s014,s114,s214,s314 |  | 2,3,4,5 | 0,2,4,6 | 0,1,6,7 |
| s015,s115,s215,s315 |  |  | 0,2,5,6 | 0,2,3,4 |
| s016,s116,s216,s316 |  |  | 0,3,4,5 | 0,2,3,5 |
| s017,s117,s217,s317 |  |  | 0,3,4,6 | 0,2,3,6 |
| s018,s118,s218,s318 |  |  | 0,3,5,6 | 0,2,3,7 |
| s019,s119,s219,s319 |  |  | 0,4,5,6 | 0,2,4,5 |
| s020,s120,s220,s320 |  |  | 1,2,3,4 | 0,2,4,6 |
| s021,s121,s221,s321 |  |  | 1,2,3,5 | 0,2,4,7 |
| s022,s122,s222,s322 |  |  | 1,2,3,6 | 0,2,5,6 |
| s023,s123,s223,s323 |  |  | 1,2,4,5 | 0,2,5,7 |
| s024,s124,s224,s324 |  |  | 1,2,4,6 | 0,2,6,7 |
| s025,s125,s225,s325 |  |  | 1,2,5,6 | 0,3,4,5 |
| s026,s126,s226,s326 |  |  | 1,3,4,5 | 0,3,4,6 |
| s027,s127,s227,s327 |  |  | 1,3,4,6 | 0,3,4,7 |
| s028,s128,s228,s328 |  |  | 1,3,5,6 | 0,3,5,6 |
| s029,s129,s229,s329 |  |  | 1,4,5,6 | 0,3,5,7 |
| s030,s130,s230,s330 |  |  | 2,3,4,5 | 0,3,6,7 |
| s031,s131,s231,s331 |  |  | 2,3,4,6 | 0,4,5,6 |
| s032,s132,s232,s332 |  |  | 2,3,5,6 | 0,4,5,7 |
| s033,s133,s233,s333 |  |  | 2,4,5,6 | 0,4,6,7 |
| s034,s134,s234,s334 |  |  | 3,4,5,6 | 0,5,6,7 |
| s035,s135,s235,s335 |  |  |  | 1,2,3,4 |
| s036,s136,s236,s336 |  |  |  | 1,2,3,5 |
| s037,s137,s237,s337 |  |  |  | 1,2,3,6 |
| s038,s138,s238,s338 |  |  |  | 1,2,3,7 |
| s039,s139,s239,s339 |  |  |  | 1,2,4,5 |
| s040,s140,s240,s340 |  |  |  | 1,2,4,6 |
| s041,s141,s241,s341 |  |  |  | 1,2,4,7 |
| s042,s142,s242,s342 |  |  |  | 1,2,5,6 |
| s043,s143,s243,s343 |  |  |  | 1,2,5,7 |
| s044,s144,s244,s344 |  |  |  | 1,2,6,7 |
| s045,s145,s245,s345 |  |  |  | 1,3,4,5 |
| s046,s146,s246,s346 |  |  |  | 1,3,4,6 |
| s047,s147,s247,s347 |  |  |  | 1,3,4,7 |
| s048,s148,s248,s348 |  |  |  | 1,3,5,6 |
| s049,s149,s249,s349 |  |  |  | 1,3,5,7 |
| s050,s150,s250,s350 |  |  |  | 1,3,6,7 |
| s051,s151,s251,s351 |  |  |  | 1,4,5,6 |
| s052,s152,s252,s352 |  |  |  | 1,4,5,7 |
| s053,s153,s253,s353 |  |  |  | 1,4,6,7 |
| s054,s154,s254,s354 |  |  |  | 1,5,6,7 |
| s055,s155,s255,s355 |  |  |  | 2,3,4,5 |
| s056,s156,s256,s356 |  |  |  | 2,3,4,6 |
| s057,s157,s257,s357 |  |  |  | 2,3,4,7 |
| s058,s158,s258,s358 |  |  |  | 2,3,5,6 |
| s059,s159,s259,s359 |  |  |  | 2,3,5,7 |
| s060,s160,s260,s360 |  |  |  | 2,3,6,7 |
| s061,s161,s261,s361 |  |  |  | 2,4,5,6 |
| s062,s162,s262,s362 |  |  |  | 2,4,5,7 |
| s063,s163,s263,s363 |  |  |  | 2,4,6,7 |
| s064,s164,s264,s364 |  |  |  | 2,5,6,7 |
| s065,s165,s265,s365 |  |  |  | 3,4,5,6 |
| s066,s166,s266,s366 |  |  |  | 3,4,5,7 |
| s067,s167,s267,s367 |  |  |  | 3,4,6,7 |
| s068,s168,s268,s368 |  |  |  | 3,5,6,7 |
| s069,s169,s269,s369 |  |  |  | 4,5,6,7 |

Table 7.3.1.1.2-31C: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=5

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62-63 | reserved | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  |  |  | 119-127 | reserved | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  |  |  | 218-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1 and Table 7.3.1.1.2-31C-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31C-1: SRI combinations for 5 layers, NSRS>4

|  | SRI(s) for 5 layers, NSRS=6 | SRI(s) for 5 layers, NSRS=7 | SRI(s) for 5 layers, NSRS=8 |
| --- | --- | --- | --- |
| s00,s10,s20,s30,s40 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 |
| s01,s11,s21,s31,s41 | 0,1,2,3,5 | 0,1,2,3,5 | 0,1,2,3,5 |
| s02,s12,s22,s32,s42 | 0,1,2,4,5 | 0,1,2,3,6 | 0,1,2,3,6 |
| s03,s13,s23,s33,s43 | 0,1,3,4,5 | 0,1,2,4,5 | 0,1,2,3,7 |
| s04,s14,s24,s34,s44 | 0,2,3,4,5 | 0,1,2,4,6 | 0,1,2,4,5 |
| s05,s15,s25,s35,s45 | 1,2,3,4,5 | 0,1,2,5,6 | 0,1,2,4,6 |
| s06,s16,s26,s36,s46 |  | 0,1,3,4,5 | 0,1,2,4,7 |
| s07,s17,s27,s37,s47 |  | 0,1,3,4,6 | 0,1,2,5,6 |
| s08,s18,s28,s38,s48 |  | 0,1,3,5,6 | 0,1,2,5,7 |
| s09,s19,s29,s39,s49 |  | 0,1,4,5,6 | 0,1,2,6,7 |
| s010,s110,s210,s310,s410 |  | 0,2,3,4,5 | 0,1,3,4,5 |
| s011,s111,s211,s311,s411 |  | 0,2,3,4,6 | 0,1,3,4,6 |
| s012,s112,s212,s312,s412 |  | 0,2,3,5,6 | 0,1,3,4,7 |
| s013,s113,s213,s313,s413 |  | 0,2,4,5,6 | 0,1,3,5,6 |
| s014,s114,s214,s314,s414 |  | 0,3,4,5,6 | 0,1,3,5,7 |
| s015,s115,s215,s315,s415 |  | 1,2,3,4,5 | 0,1,3,6,7 |
| s016,s116,s216,s316,s416 |  | 1,2,3,4,6 | 0,1,4,5,6 |
| s017,s117,s217,s317,s417 |  | 1,2,3,5,6 | 0,1,4,5,7 |
| s018,s118,s218,s318,s418 |  | 1,2,4,5,6 | 0,1,4,6,7 |
| s019,s119,s219,s319,s419 |  | 1,3,4,5,6 | 0,1,5,6,7 |
| s020,s120,s220,s320,s420 |  | 2,3,4,5,6 | 0,2,3,4,5 |
| s021,s121,s221,s321,s421 |  |  | 0,2,3,4,6 |
| s022,s122,s222,s322,s422 |  |  | 0,2,3,4,7 |
| s023,s123,s223,s323,s423 |  |  | 0,2,3,5,6 |
| s024,s124,s224,s324,s424 |  |  | 0,2,3,5,7 |
| s025,s125,s225,s325,s425 |  |  | 0,2,3,6,7 |
| s026,s126,s226,s326,s426 |  |  | 0,2,4,5,6 |
| s027,s127,s227,s327,s427 |  |  | 0,2,4,5,7 |
| s028,s128,s228,s328,s428 |  |  | 0,2,4,6,7 |
| s029,s129,s229,s329,s429 |  |  | 0,2,5,6,7 |
| s030,s130,s230,s330,s430 |  |  | 0,3,4,5,6 |
| s031,s131,s231,s331,s431 |  |  | 0,3,4,5,7 |
| s032,s132,s232,s332,s432 |  |  | 0,3,4,6,7 |
| s033,s133,s233,s333,s433 |  |  | 0,3,5,6,7 |
| s034,s134,s234,s334,s434 |  |  | 0,4,5,6,7 |
| s035,s135,s235,s335,s435 |  |  | 1,2,3,4,5 |
| s036,s136,s236,s336,s436 |  |  | 1,2,3,4,6 |
| s037,s137,s237,s337,s437 |  |  | 1,2,3,4,7 |
| s038,s138,s238,s338,s438 |  |  | 1,2,3,5,6 |
| s039,s139,s239,s339,s439 |  |  | 1,2,3,5,7 |
| s040,s140,s240,s340,s440 |  |  | 1,2,3,6,7 |
| s041,s141,s241,s341,s441 |  |  | 1,2,4,5,6 |
| s042,s142,s242,s342,s442 |  |  | 1,2,4,5,7 |
| s043,s143,s243,s343,s443 |  |  | 1,2,4,6,7 |
| s044,s144,s244,s344,s444 |  |  | 1,2,5,6,7 |
| s045,s145,s245,s345,s445 |  |  | 1,3,4,5,6 |
| s046,s146,s246,s346,s446 |  |  | 1,3,4,5,7 |
| s047,s147,s247,s347,s447 |  |  | 1,3,4,6,7 |
| s048,s148,s248,s348,s448 |  |  | 1,3,5,6,7 |
| s049,s149,s249,s349,s449 |  |  | 1,4,5,6,7 |
| s050,s150,s250,s350,s450 |  |  | 2,3,4,5,6 |
| s051,s151,s251,s351,s451 |  |  | 2,3,4,5,7 |
| s052,s152,s252,s352,s452 |  |  | 2,3,4,6,7 |
| s053,s153,s253,s353,s453 |  |  | 2,3,5,6,7 |
| s054,s154,s254,s354,s454 |  |  | 2,4,5,6,7 |
| s055,s155,s255,s355,s455 |  |  | 3,4,5,6,7 |

Table 7.3.1.1.2-31D: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=6

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62 | 0,1,2,3,4,5 | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  | 63 | reserved | 119 | s00,s10,s20,s30,s40,s50 | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  | … | ... | 218 | s00,s10,s20,s30,s40,s50 |
|  |  |  |  | 125 | s06,s16,s26,s36,s46,s56 | … | ... |
|  |  |  |  | 126-127 | reserved | 245 | s027,s127,s227,s327,s427,s527 |
|  |  |  |  |  |  | 246-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1, Table 7.3.1.1.2-31C-1 and Table 7.3.1.1.2-31D-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31D-1: SRI combinations for 6 layers, NSRS>6

|  | SRI(s) for 6 layers, NSRS=7 | SRI(s) for 6 layers, NSRS=8 |
| --- | --- | --- |
| s00,s10,s20,s30,s40,s50 | 0,1,2,3,4,5 | 0,1,2,3,4,5 |
| s01,s11,s21,s31,s41,s51 | 0,1,2,3,4,6 | 0,1,2,3,4,6 |
| s02,s12,s22,s32,s42,s52 | 0,1,2,3,5,6 | 0,1,2,3,4,7 |
| s03,s13,s23,s33,s43,s53 | 0,1,2,4,5,6 | 0,1,2,3,5,6 |
| s04,s14,s24,s34,s44,s54 | 0,1,3,4,5,6 | 0,1,2,3,5,7 |
| s05,s15,s25,s35,s45,s55 | 0,2,3,4,5,6 | 0,1,2,3,6,7 |
| s06,s16,s26,s36,s46,s56 | 1,2,3,4,5,6 | 0,1,2,4,5,6 |
| s07,s17,s27,s37,s47,s57 |  | 0,1,2,4,5,7 |
| s08,s18,s28,s38,s48,s58 |  | 0,1,2,4,6,7 |
| s09,s19,s29,s39,s49,s59 |  | 0,1,2,5,6,7 |
| s010,s110,s210,s310,s410,s510 |  | 0,1,3,4,5,6 |
| s011,s111,s211,s311,s411,s511 |  | 0,1,3,4,5,7 |
| s012,s112,s212,s312,s412,s512 |  | 0,1,3,4,6,7 |
| s013,s113,s213,s313,s413,s513 |  | 0,1,3,5,6,7 |
| s014,s114,s214,s314,s414,s514 |  | 0,1,4,5,6,7 |
| s015,s115,s215,s315,s415,s515 |  | 0,2,3,4,5,6 |
| s016,s116,s216,s316,s416,s516 |  | 0,2,3,4,5,7 |
| s017,s117,s217,s317,s417,s517 |  | 0,2,3,4,6,7 |
| s018,s118,s218,s318,s418,s518 |  | 0,2,3,5,6,7 |
| s019,s119,s219,s319,s419,s519 |  | 0,2,4,5,6,7 |
| s020,s120,s220,s320,s420,s520 |  | 0,3,4,5,6,7 |
| s021,s121,s221,s321,s421,s521 |  | 1,2,3,4,5,6 |
| s022,s122,s222,s322,s422,s522 |  | 1,2,3,4,5,7 |
| s023,s123,s223,s323,s423,s523 |  | 1,2,3,4,6,7 |
| s024,s124,s224,s324,s424,s524 |  | 1,2,3,5,6,7 |
| s025,s125,s225,s325,s425,s525 |  | 1,2,4,5,6,7 |
| s026,s126,s226,s326,s426,s526 |  | 1,3,4,5,6,7 |
| s027,s127,s227,s327,s427,s527 |  | 2,3,4,5,6,7 |

Table 7.3.1.1.2-31E: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=7

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62 | 0,1,2,3,4,5 | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  | 63 | reserved | 119 | s00,s10,s20,s30,s40,s50 | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  | … | ... | 218 | s00,s10,s20,s30,s40,s50 |
|  |  |  |  | 125 | s06,s16,s26,s36,s46,s56 | … | ... |
|  |  |  |  | 126 | 0,1,2,3,4,5,6 | 245 | s027,s127,s227,s327,s427,s527 |
|  |  |  |  | 127 | reserved | 246 | 0,1,2,3,4,5,6 |
|  |  |  |  |  |  | 247 | 0,1,2,3,4,5,7 |
|  |  |  |  |  |  | 248 | 0,1,2,3,4,6,7 |
|  |  |  |  |  |  | 249 | 0,1,2,3,5,6,7 |
|  |  |  |  |  |  | 250 | 0,1,2,4,5,6,7 |
|  |  |  |  |  |  | 251 | 0,1,3,4,5,6,7 |
|  |  |  |  |  |  | 252 | 0,2,3,4,5,6,7 |
|  |  |  |  |  |  | 253 | 1,2,3,4,5,6,7 |
|  |  |  |  |  |  | 254-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1, Table 7.3.1.1.2-31C-1 and Table 7.3.1.1.2-31D-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31F: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=8

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62 | 0,1,2,3,4,5 | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  | 63 | reserved | 119 | s00,s10,s20,s30,s40,s50 | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  | … | ... | 218 | s00,s10,s20,s30,s40,s50 |
|  |  |  |  | 125 | s06,s16,s26,s36,s46,s56 | … | ... |
|  |  |  |  | 126 | 0,1,2,3,4,5,6 | 245 | s027,s127,s227,s327,s427,s527 |
|  |  |  |  | 127 | reserved | 246 | 0,1,2,3,4,5,6 |
|  |  |  |  |  |  | 247 | 0,1,2,3,4,5,7 |
|  |  |  |  |  |  | 248 | 0,1,2,3,4,6,7 |
|  |  |  |  |  |  | 249 | 0,1,2,3,5,6,7 |
|  |  |  |  |  |  | 250 | 0,1,2,4,5,6,7 |
|  |  |  |  |  |  | 251 | 0,1,3,4,5,6,7 |
|  |  |  |  |  |  | 252 | 0,2,3,4,5,6,7 |
|  |  |  |  |  |  | 253 | 1,2,3,4,5,6,7 |
|  |  |  |  |  |  | 254 | 0,1,2,3,4,5,6,7 |
|  |  |  |  |  |  | 255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1, Table 7.3.1.1.2-31C-1 and Table 7.3.1.1.2-31D-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-32: SRI indication or Second SRI indication, for codebook based PUSCH transmission, if ul-FullPowerTransmission is not configured, or ul-FullPowerTransmission = fullpowerMode1, or ul-FullPowerTransmission = fullpowerMode2, or ul-FullPowerTransmission = fullpower and NSRS=2

| Bit field mapped to index | SRI(s), |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

Table 7.3.1.1.2-32A: SRI indication or Second SRI indication, for codebook based PUSCH transmission, if ul-FullPowerTransmission = fullpowerMode2 and NSRS=3

| Bit field mapped to index | SRI(s), NSRS=3 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | Reserved |

Table 7.3.1.1.2-32B: SRI indication or Second SRI indication, for codebook based PUSCH transmission, if ul-FullPowerTransmission = fullpowerMode2 and NSRS=4

| Bit field mapped to index | SRI(s), NSRS=4 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

Table 7.3.1.1.2-33: Joint indication of minimum applicable scheduling offset K0/K2

| Bit field mapped to index | Minimum applicable K0 for the active DL BWP, if minimumSchedulingOffsetK0 is configured for the DL BWP | Minimum applicable K2 for the active UL BWP, if minimumSchedulingOffsetK2 is configured for the UL BWP |
| --- | --- | --- |
| 0 | The first value configured by minimumSchedulingOffsetK0 for the active DL BWP | The first value configured by minimumSchedulingOffsetK2 for the active UL BWP |
| 1 | The second value configured by minimumSchedulingOffsetK0 for the active DL BWP if the second value is configured; 0 otherwise | The second value configured by minimumSchedulingOffsetK2 for the active UL BWP if the second value is configured; 0 otherwise |

Table 7.3.1.1.2-34: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 2 |

Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1/0_3 and DCI format 0_2, configured by
higher layer parameter ul-AccessConfigListDCI-0-1 and ul-AccessConfigListDCI-0-2, respectively,
in frequency range 1

| Entry index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, 38.211] | CAPC |
| --- | --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 0 | 1 |
| 1 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 0 | 2 |
| 2 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 0 | 3 |
| 3 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 0 | 4 |
| 4 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 2 | 1 |
| 5 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 2 | 2 |
| 6 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 2 | 3 |
| 7 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 2 | 4 |
| 8 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 0 | 1 |
| 9 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 0 | 2 |
| 10 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 0 | 3 |
| 11 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 0 | 4 |
| 12 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 2 | 1 |
| 13 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 2 | 2 |
| 14 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 2 | 3 |
| 15 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 2 | 4 |
| 16 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 0 | 1 |
| 17 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 0 | 2 |
| 18 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 0 | 3 |
| 19 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 0 | 4 |
| 20 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 1 | 1 |
| 21 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 1 | 2 |
| 22 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 1 | 3 |
| 23 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 1 | 4 |
| 24 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 3 | 1 |
| 25 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 3 | 2 |
| 26 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 3 | 3 |
| 27 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 3 | 4 |
| 28 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 0 | 1 |
| 29 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 0 | 2 |
| 30 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 0 | 3 |
| 31 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 0 | 4 |
| 32 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 1 | 1 |
| 33 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 1 | 2 |
| 34 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 1 | 3 |
| 35 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 1 | 4 |
| 36 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 2 | 1 |
| 37 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 2 | 2 |
| 38 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 2 | 3 |
| 39 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 2 | 4 |
| 40 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 3 | 1 |
| 41 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 3 | 2 |
| 42 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 3 | 3 |
| 43 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 3 | 4 |

Table 7.3.1.1.2-35A: Allowed entries for DCI format 0_1, DCI format 0_2 and DCI format 0_3, configured by higher layer parameter ul-AccessConfigListDCI-0-1 in frequency range 2-2

| Entry index | Channel Access Type |
| --- | --- |
| 0 | Type 1 channel access defined in clause 4.4.1 of TS 37.213 [14] |
| 1 | Type 2 channel access defined in clause 4.4.2 of TS 37.213 [14] |
| 2 | Type 3 channel access defined in clause 4.4.3 of TS 37.213 [14] |

Table 7.3.1.1.2-36: SRS resource set indication

| Bit field mapped to index | SRS resource set indication |
| --- | --- |
| 0 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved; If there are two indicated joint/UL TCI states, the first indicated joint/UL TCI state is applied to the corresponding PUSCH transmission occasions. |
| 1 | SRS resource indicator field and Precoding information and number of layers field are associated with the second SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved; If there are two indicated joint/UL TCI states, the second indicated joint/UL TCI state is applied to the corresponding PUSCH transmission occasions. |
| 2 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set; If there are two indicated joint/UL TCI states, the first indicated joint/UL TCI state is applied to the PUSCH transmission occasions/antenna ports associated with the first SRS resource set, and the second indicated joint/UL TCI state is applied to the PUSCH transmission occasions/antenna ports associated with the second SRS resource set. |
| 3 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set; If there are two indicated joint/UL TCI states, the first indicated joint/UL TCI state is applied to the PUSCH transmission occasions associated with the first SRS resource set, and the second indicated joint/UL TCI state is applied to the PUSCH transmission occasions associated with the second SRS resource set. If multipanelSchemeSDM or multipanelSchemeSFN is configured, this row is reserved. |
| NOTE 1: The first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook' if txConfig=nonCodebook or 'codebook' if txConfig=codebook. When only one SRS resource set is configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook' respectively, the first SRS resource set is the SRS resource set. The association of the first and second SRS resource sets to PUSCH repetitions for each bit field index value is as defined in Clause 6.1.2.1 of TS 38.214 [6]. NOTE 2: For DCI format 0_2, the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook', respectively, except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'. |  |

Table 7.3.1.1.2-37: SRS offset indicator

| Bit field mapped to index | Available slot offset, K=2 | Bit field mapped to index | Available slot offset, K=3 | Bit field mapped to index | Available slot offset, K=4 |
| --- | --- | --- | --- | --- | --- |
| 0 | The 1st entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 0 | The 1st entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 0 | The 1st entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |
| 1 | The 2nd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 1 | The 2nd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 1 | The 2nd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |
|  |  | 2 | The 3rd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 2 | The 3rd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |
|  |  | 3 | Reserved | 3 | The 4th entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |

Table 7.3.1.1.2-38: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 1 | 8 |
| 7 | 1 | 9 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-39: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4 | 1 | 8,9 |
| 5 | 2 | 8,9 |
| 6 | 2 | 10,11 |
| 7 | 2 | 9,11 |
| 8-15 | Reserved | Reserved |

Table 7.3.1.1.2-40: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelSchemeSDM is not configured, dmrs-TypeEnh is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | 8-10 |
| 2 | 1 | 0,1,8 |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-40A: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelSchemeSDM is configured, dmrs-TypeEnh is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | 8-10 |
| 2 | 1 | 0,1,8 |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5 | 2 | 0,2,3 |
| 6-15 | Reserved | Reserved |

Table 7.3.1.1.2-41: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 2 | 8-11 |
| 2 | 1 | 0,1,8,9 |
| 3 | 2 | 0,1,8,9 |
| 4 | 2 | 2,3,10,11 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-42: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-43: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,10 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-44: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-45: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10,11 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-46: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| 28 | 1 | 0 | 2 |
| 29 | 1 | 1 | 2 |
| 30 | 1 | 8 | 2 |
| 31 | 1 | 9 | 2 |

Table 7.3.1.1.2-47: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | 2 | 8,9 | 2 |
| 14 | 2 | 10,11 | 2 |
| 15 | 2 | 12,13 | 2 |
| 16 | 2 | 14,15 | 2 |
| 17 | 2 | 9,11 | 1 |
| 18 | 2 | 1,3 | 1 |
| 19 | 2 | 0,2 | 2 |
| 20 | 2 | 1,3 | 2 |
| 21 | 2 | 4,6 | 2 |
| 22 | 2 | 5,7 | 2 |
| 23 | 2 | 8,10 | 2 |
| 24 | 2 | 9,11 | 2 |
| 25 | 2 | 12,14 | 2 |
| 26 | 2 | 13,15 | 2 |
| 27 | 1 | 0,1 | 2 |
| 28 | 1 | 8,9 | 2 |
| 29 | 1 | 4,5 | 2 |
| 30 | 1 | 12,13 | 2 |
| 31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-48: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelSchemeSDM is not configured, dmrs-TypeEnh is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 9-11 | 1 |
| 4 | 2 | 8,9,12 | 2 |
| 5 | 2 | 10,11,14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9 | 2 | 0,1,8 | 2 |
| 10 | 2 | 4,5,12 | 2 |
| 11 | 2 | 2,3,10 | 2 |
| 12 | 2 | 6,7,14 | 2 |
| 13 | 2 | 5,8,9 | 2 |
| 14 | 2 | 7,10,11 | 2 |
| 15 | 2 | 7,12,13 | 2 |
| 16-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-48A: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelSchemeSDM is configured, dmrs-TypeEnh is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 9-11 | 1 |
| 4 | 2 | 8,9,12 | 2 |
| 5 | 2 | 10,11,14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9 | 2 | 0,1,8 | 2 |
| 10 | 2 | 4,5,12 | 2 |
| 11 | 2 | 2,3,10 | 2 |
| 12 | 2 | 6,7,14 | 2 |
| 13 | 2 | 5,8,9 | 2 |
| 14 | 2 | 7,10,11 | 2 |
| 15 | 2 | 7,12,13 | 2 |
| 16 | 2 | 0,2,3 | 1 |
| 17-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-49: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4 | 2 | 8-11 | 1 |
| 5 | 2 | 8,9,12,13 | 2 |
| 6 | 2 | 10,11,14,15 | 2 |
| 7 | 2 | 1,3,5,7 | 2 |
| 8 | 1 | 0,1,8,9 | 1 |
| 9 | 2 | 0,1,8,9 | 1 |
| 10 | 2 | 2,3,10,11 | 1 |
| 11 | 1 | 0,1,8,9 | 2 |
| 12 | 1 | 4,5,12,13 | 2 |
| 13 | 2 | 0,1,8,9 | 2 |
| 14 | 2 | 4,5,12,13 | 2 |
| 15 | 2 | 2,3,10,11 | 2 |
| 16 | 2 | 6,7,14,15 | 2 |
| 17-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-50: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1 | 2 | 0,1,2,3,8 | 1 |
| 2 | 1 | 0,1,4,5,8 | 2 |
| 3 | 2 | 0,1,4,5,8 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-51: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1,4,5,8,12 | 2 |
| 3 | 2 | 0,1,4,5,8,12 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-52: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1 | 2 | 0,1,2,3,8,9,10 | 1 |
| 2 | 1 | 0,1,4,5,8,9,12 | 2 |
| 3 | 2 | 0,1,4,5,8,9,12 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-53: Antenna port(s), transform precoder is disabled, dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 2 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 3 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-54: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12 | 1 | 12 |
| 13 | 1 | 13 |
| 14 | 2 | 12 |
| 15 | 2 | 13 |
| 16 | 2 | 14 |
| 17 | 2 | 15 |
| 18 | 3 | 12 |
| 19 | 3 | 13 |
| 20 | 3 | 14 |
| 21 | 3 | 15 |
| 22 | 3 | 16 |
| 23 | 3 | 17 |
| 24-31 | Reserved | Reserved |

Table 7.3.1.1.2-55: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7 | 1 | 12,13 |
| 8 | 2 | 12,13 |
| 9 | 2 | 14,15 |
| 10 | 3 | 12,13 |
| 11 | 3 | 14,15 |
| 12 | 3 | 16,17 |
| 13 | 3 | 13,15 |
| 14 | 2 | 13,15 |
| 15-31 | Reserved | Reserved |

Table 7.3.1.1.2-56: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelSchemeSDM is not configured, dmrs-TypeEnh is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 1 | 0,1,12 |
| 4 | 2 | 0,1,12 |
| 5 | 2 | 2,3,14 |
| 6 | 3 | 0,1,12 |
| 7 | 3 | 2,3,14 |
| 8 | 3 | 4,5,16 |
| 9 | 3 | 13,15,17 |
| 10-31 | Reserved | Reserved |

Table 7.3.1.1.2-56A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelSchemeSDM is configured, dmrs-TypeEnh is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 1 | 0,1,12 |
| 4 | 2 | 0,1,12 |
| 5 | 2 | 2,3,14 |
| 6 | 3 | 0,1,12 |
| 7 | 3 | 2,3,14 |
| 8 | 3 | 4,5,16 |
| 9 | 3 | 13,15,17 |
| 10 | 2 | 0,2,3 |
| 11-31 | Reserved | Reserved |

Table 7.3.1.1.2-57: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2 | 1 | 0,1,12,13 |
| 3 | 2 | 0,1,12,13 |
| 4 | 2 | 2,3,14,15 |
| 5 | 3 | 0,1,12,13 |
| 6 | 3 | 2,3,14,15 |
| 7 | 3 | 4,5,16,17 |
| 8-31 | Reserved | Reserved |

Table 7.3.1.1.2-58: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1 | 2 | 0,1,2,3,12 |
| 2 | 3 | 0,1,2,3,12 |
| 3-31 | Reserved | Reserved |

Table 7.3.1.1.2-59: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1 | 2 | 0,1,2,3,12,14 |
| 2 | 3 | 0,1,2,3,12,14 |
| 3-31 | Reserved | Reserved |

Table 7.3.1.1.2-60: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3,12-14 |
| 1 | 3 | 0-3,12-14 |
| 2-31 | Reserved | Reserved |

Table 7.3.1.1.2-61: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3,12-15 |
| 1 | 3 | 0-3,12-15 |
| 2-31 | Reserved | Reserved |

Table 7.3.1.1.2-62: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28 | 1 | 12 | 1 |
| 29 | 1 | 13 | 1 |
| 30 | 2 | 12 | 1 |
| 31 | 2 | 13 | 1 |
| 32 | 2 | 14 | 1 |
| 33 | 2 | 15 | 1 |
| 34 | 3 | 12 | 1 |
| 35 | 3 | 13 | 1 |
| 36 | 3 | 14 | 1 |
| 37 | 3 | 15 | 1 |
| 38 | 3 | 16 | 1 |
| 39 | 3 | 17 | 1 |
| 40 | 3 | 12 | 2 |
| 41 | 3 | 13 | 2 |
| 42 | 3 | 14 | 2 |
| 43 | 3 | 15 | 2 |
| 44 | 3 | 16 | 2 |
| 45 | 3 | 17 | 2 |
| 46 | 3 | 18 | 2 |
| 47 | 3 | 19 | 2 |
| 48 | 3 | 20 | 2 |
| 49 | 3 | 21 | 2 |
| 50 | 3 | 22 | 2 |
| 51 | 3 | 23 | 2 |
| 52 | 1 | 12 | 2 |
| 53 | 1 | 13 | 2 |
| 54 | 1 | 18 | 2 |
| 55 | 1 | 19 | 2 |
| 56-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-63: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19 | 1 | 12,13 | 1 |
| 20 | 2 | 12,13 | 1 |
| 21 | 2 | 14,15 | 1 |
| 22 | 3 | 12,13 | 1 |
| 23 | 3 | 14,15 | 1 |
| 24 | 3 | 16,17 | 1 |
| 25 | 3 | 12,13 | 2 |
| 26 | 3 | 14,15 | 2 |
| 27 | 3 | 16,17 | 2 |
| 28 | 3 | 18,19 | 2 |
| 29 | 3 | 20,21 | 2 |
| 30 | 3 | 22,23 | 2 |
| 31 | 1 | 12,13 | 2 |
| 32 | 1 | 18,19 | 2 |
| 33 | 2 | 12,13 | 2 |
| 34 | 2 | 14,15 | 2 |
| 35 | 2 | 18,19 | 2 |
| 36 | 2 | 20,21 | 2 |
| 37 | 3 | 13,15 | 1 |
| 38 | 2 | 13,15 | 1 |
| 39-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-64: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelSchemeSDM is not configured, dmrs-TypeEnh is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 1 | 0,1,12 | 1 |
| 7 | 2 | 0,1,12 | 1 |
| 8 | 2 | 2,3,14 | 1 |
| 9 | 3 | 0,1,12 | 1 |
| 10 | 3 | 2,3,14 | 1 |
| 11 | 3 | 4,5,16 | 1 |
| 12 | 3 | 7,12,13 | 2 |
| 13 | 3 | 9,14,15 | 2 |
| 14 | 3 | 11,16,17 | 2 |
| 15 | 3 | 9,18,19 | 2 |
| 16 | 3 | 18,19,20 | 2 |
| 17 | 3 | 21,22,23 | 2 |
| 18 | 3 | 13,15,17 | 1 |
| 19-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-64A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelSchemeSDM is configured, dmrs-TypeEnh is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 1 | 0,1,12 | 1 |
| 7 | 2 | 0,1,12 | 1 |
| 8 | 2 | 2,3,14 | 1 |
| 9 | 3 | 0,1,12 | 1 |
| 10 | 3 | 2,3,14 | 1 |
| 11 | 3 | 4,5,16 | 1 |
| 12 | 3 | 7,12,13 | 2 |
| 13 | 3 | 9,14,15 | 2 |
| 14 | 3 | 11,16,17 | 2 |
| 15 | 3 | 9,18,19 | 2 |
| 16 | 3 | 18,19,20 | 2 |
| 17 | 3 | 21,22,23 | 2 |
| 18 | 3 | 13,15,17 | 1 |
| 19 | 2 | 0,2,3 | 1 |
| 20-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-65: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5 | 3 | 12,13,18,19 | 2 |
| 6 | 3 | 14,15,20,21 | 2 |
| 7 | 3 | 16,17,22,23 | 2 |
| 8 | 1 | 0,1,12,13 | 1 |
| 9 | 2 | 0,1,12,13 | 1 |
| 10 | 2 | 2,3,14,15 | 1 |
| 11 | 3 | 0,1,12,13 | 1 |
| 12 | 3 | 2,3,14,15 | 1 |
| 13 | 3 | 4,5,16,17 | 1 |
| 14-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-66: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 2 | 2 | 0,1,2,3,12 | 1 |
| 3 | 3 | 0,1,2,3,12 | 1 |
| 4 | 1 | 0,1,6,7,12 | 2 |
| 5 | 2 | 0,1,6,7,12 | 2 |
| 6 | 3 | 0,1,6,7,12 | 2 |
| 7-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-67: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2 | 2 | 0-3,12,14 | 1 |
| 3 | 3 | 0-3,12,14 | 1 |
| 4 | 1 | 0,1,6,7,12,18 | 2 |
| 5 | 2 | 0,1,6,7,12,18 | 2 |
| 6 | 3 | 0,1,6,7,12,18 | 2 |
| 7-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-68: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1 | 2 | 0-3,12-14 | 1 |
| 2 | 3 | 0-3,12-14 | 1 |
| 3 | 1 | 0,1,6,7,12,13,18 | 2 |
| 4 | 2 | 0,1,6,7,12,13,18 | 2 |
| 5 | 3 | 0,1,6,7,12,13,18 | 2 |
| 6-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-69: Antenna port(s), transform precoder is disabled, dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1 | 2 | 0-3,12-15 | 1 |
| 2 | 3 | 0-3,12-15 | 1 |
| 3 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 4 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 5 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 6-63 | Reserved | Reserved | Reserved |

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator - 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-0-2, as defined in Clause 10.1 of [5, TS38.213]. This field is reserved when this format is carried by PDCCH on the primary cell and the UE is configured for scheduling on the primary cell from an SCell, with the same number of bits as that in this format carried by PDCCH on the SCell for scheduling on the primary cell.

-	UL/SUL indicator - 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator - 0, 1 or 2 bits as determined by the number of UL BWPs nBWP, RRC configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as log2(nBWP) bits, where

-	nBWP=nBWP, RRC+1 if nBWP, RRC≤3, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP=nBWP, RRC, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment - number of bits determined by the following:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K1NRBG, K1+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K1NRBG, K1+1/2, NRBG+1 bits if resourceAllocationDCI-0-2-r16 is configured as 'dynamicSwitch', where NRBG, K1=NRBUL, BWP+NUL, BWPstartmodK1/K1, NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter resourceAllocationType1GranularityDCI-0-2. If the higher layer parameter resourceAllocationType1GranularityDCI-0-2 is not configured, K1 is equal to 1.

-	If resourceAllocationDCI-0-2-r16 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K1NRBG, K1+1/2 LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetListsDCI-0-2 contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetListsDCI-0-2 contains four offset values

-	log2NRBG, K1NRBG, K1+1/2-NUL_hop bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	log2NRBG, K1NRBG, K1+1/2 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-0-2-r16 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment - 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationListDCI-0-2 if the higher layer parameter is configured, or I is the number of entries in the higher layer parameter PUSCH-TimeDomainResourceAllocationList if the higher layer parameter PUSCH-TimeDomainResourceAllocationList is configured and the higher layer parameter pusch-TimeDomainAllocationListDCI-0-2 is not configured; otherwise I is the number of entries in the default table.

-	Frequency hopping flag - 0 or 1 bit:

-	0 bit if the higher layer parameter frequencyHoppingDCI-0-2 is not configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme -5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-0-2

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

-	Transform precoder indicator - 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderFieldPresenceDCI-0-2 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_2 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the bit is reserved.

-	0 bit otherwise.

-	HARQ process number - number of bits determined by the following:

-	5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2-v1700 if configured;

-	otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2

-	Downlink assignment index - 0, 1, 2 or 4 bits

-	0 bit if the higher layer parameter downlinkAssignmentIndexDCI-0-2 is not configured;

-	1, 2, 3, 4, 5 or 6 bits otherwise,

-	1st downlink assignment index - 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook for unicast and multicast if pdsch-HARQ-ACK-Codebook = semiStatic is configured for both unicast and multicast and the higher layer parameter fdmed-ReceptionMulticast is not configured; otherwise for semi-static HARQ-ACK codebook for unicast;

-	2 bits for dynamic HARQ-ACK codebook for unicast.

-	2nd downlink assignment index - 0 or 2 bits

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks for unicast;

-	0 bit otherwise.

-	3rd downlink assignment index - 0, 1 or 2 bits

-	1 bit for semi-static HARQ-ACK codebook for multicast if the higher layer parameter fdmed-ReceptionMulticast is configured;

-	2 bits for the dynamic HARQ-ACK codebook for multicast;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the 1st or 2 nd Downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the 1st or 2 nd Downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st or 2 nd Downlink assignment index until the bit width of the 1st or 2 nd Downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the 3rd downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the 3rd downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 3rd downlink assignment index until the bit width of the 3rd downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	Second TPC command for scheduled PUSCH - 2 bits as defined in Clause 7.1.1 of [5, TS38.213] if higher layer parameter SecondTPCFieldDCI-0-2 is configured; 0 bit otherwise.

-	SRS resource set indicator - 0 or 2 bits

-	2 bits according to Table 7.3.1.1.2-36 if

-	txConfig = nonCodeBook, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with the usage of value 'nonCodeBook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, or

-	txConfig=codebook, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided;

-	0 bit otherwise.

-	SRS resource indicator - number of bits determined by the following: '

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B if the higher layer parameter txConfig = nonCodebook, where

-	NSRS, 0_2 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel,

-	otherwise NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 and associated with the higher layer parameter usage of value 'nonCodeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set, or in the SRS resource set with lower srs-ResourceSetId of two SRS resources sets, configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList',

and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by max{maxMIMO-LayersDCI-0-2, maxMIMO-LayersforSDM-DCI-0-2} if maxMIMO-LayersforSDM-DCI-0-2 is configured

-	Lmax is given by max{maxMIMO-LayersDCI-0-2, maxMIMO-LayersforSFN-DCI-0-2} if maxMIMO-LayersforSFN-DCI-0-2 is configured

-	Lmax is given by maxMIMO-LayersDCI-0-2 otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2NSRS, 0_2  bits according to Tables 7.3.1.1.2-32/32A/32B if the higher layer parameter txConfig = codebook, where

-	NSRS, 0_2 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel,

-	otherwise NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 and associated with the higher layer parameter usage of value 'codeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook' or 'nonCodeBook', the first SRS resource set is associated with coresetPoolIndex value 0 and the second SRS resource set is associated with coresetPoolIndex value 1, where the first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets, and the first and second SRS resource sets are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook', respectively, except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'.

-	Second SRS resource indicator - number of bits determined by the following:

-	log2(maxk∈{1,2,…,min⁡{Lmax,NSRS,0_2}}NSRS,0_2k)  bits according to Tables 7.3.1.1.2-28/29A/30A/31A with the same number of layers indicated by SRS resource indicator field if the higher layer parameter txConfig = nonCodebook, the higher layer parameter maxMIMO-LayersforSDM-DCI-0-2 is not configured, and SRS resource set indicator field is present, where NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set, and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by maxMIMO-LayersforSFN-DCI-0-2 if maxMIMO-LayersforSFN-DCI-0-2 is configured

-	Lmax is given by maxMIMO-LayersDCI-0-2 otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/29 if the higher layer parameter txConfig = nonCodebook, the higher layer parameter maxMIMO-LayersforSDM-DCI-0-2 is configured and SRS resource set indicator field is present, where NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set, where the second SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set, or in the SRS resource set with higher srs-ResourceSetId of two SRS resources sets, configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList', and Lmax is given by maxMIMO-LayersforSDM-DCI-0-2.

-	log2NSRS, 0_2  bits according to Tables 7.3.1.1.2-32/32A/32B if the higher layer parameter txConfig = codebook and SRS resource set indicator field is present, where  NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set.

-	0 bit otherwise.

-	Precoding information and number of layers - number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 2 if multipanelSchemeSFN is configured or max{maxRanDCI-0-2k, maxRankSDM-DCI-0-2} = 2 if multipanelSchemeSDM is configured, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 1 if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 2 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 2 if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2}=1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2}=1 if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2.

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports by replacing maxRank-n8 with maxRankDCI-0-2, if CodebookTypeUL=codebook1, transform precoder is disabled, maxRankDCI-0-2 =4, and according to maxRankDCI-0-2;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook1, transform precoder is enabled or maxRankDCI-0-2 =1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRankDCI-0-2;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRankDCI-0-2;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

-	5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2.

-	Second Precoding information - number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=2 if multipanelSchemeSFN is configured, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=1 if multipanelSchemeSFN is configured, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=1 if multipanelSchemeSFN is configured, and the value of higher layer parameter codebookSubsetDCI-0-2.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Second Precoding information field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Second Precoding information field for the case with smaller bit width until the bit width of the Second Precoding information field for the two cases are the same.

-	Antenna ports - number of bits determined by the following:

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-0-2 is not configured;

-	2, 3, 4, 5 or 6 bits otherwise,

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by 7.3.1.1.2-6A, if transform precoder is enabled, and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled, and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/10A/11 according to the value of rank, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is not configured, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/14A/15 according to the value of rank, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is not configured, and maxLength=2;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/18A/19 according to the value of rank, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/22A/23 according to the value of rank, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, and maxLength=2.

-	4 bits as defined by Tables 7.3.1.1.2-38/39/40/40A/41, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-46/47/48/48A/49, if transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is configured, and maxLength=2;

-	5 bits as defined by Tables 7.3.1.1.2-54/55/56/56A/57, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, and maxLength=1;

-	6 bits as defined by Tables 7.3.1.1.2-62/63/64/64A/65, if transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, and maxLength=2.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively, and the value of rank is

-	the sum of the value determined according to the SRS resource indicator field and the value determined according to the second SRS resource indicator field, if txConfig = nonCodebook and multipanelSchemeSDM is configured and SRS resource set indicator field equals "10"

-	the sum of the value determined according to the Precoding information and number of layers field and the value determined according to the Second Precoding information, if txConfig = codebook and multipanelSchemeSDM is configured and SRS resource set indicator field equals "10"

-	determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and multipanelSchemeSDM is not configured, , or if the higher layer parameter txConfig = nonCodebook, multipanelSchemeSDM is configured and SRS resource set indicator field equals "00" or “01”

-	determined according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook and multipanelSchemeSDM is not configured, or if the higher layer parameter txConfig = codebook, multipanelSchemeSDM is configured and SRS resource set indicator field equals "00" or "01".

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2  and dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 and is configured with antennaPortsFieldPresenceDCI-0-2, the bitwidth of this field equals maxxA,xB, where xA  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-0-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23.

When the Transform precoder indicator field is present, if the bit width of the Antenna ports field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Antenna ports field for the case with smaller bit width until the bit width of the Antenna ports field for the two cases are the same.

-	SRS request - 0, 1, 2 or 3 bits

-	0 bit if the higher layer parameter srs-RequestDCI-0-2 is not configured;

-	1 bit as defined by Table 7.3.1.1.3-1 if higher layer parameter srs-RequestDCI-0-2 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	2 bits if higher layer parameter srs-RequestDCI-0-2 = 1 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second bit is defined by Table 7.3.1.1.3-1;

-	2 bits as defined by Table 7.3.1.1.2-24 if higher layer parameter srs-RequestDCI-0-2 = 2 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	3 bits if higher layer parameter srs-RequestDCI-0-2 = 2 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24;

-	SRS offset indicator - 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CSI request - 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSizeDCI-0-2.

-	PTRS-DMRS association - number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRankDCI-0-2=1 and maxRankSFN-DCI-0-2=1, or if maxRankDCI-0-2=1 and maxRankSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPortsforSdm;

-	2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or “01” and maxRankDCI-0-2<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" or “11”, maxRankDCI-0-2=3 or 4 and  neither multipanelSchemeSDM nor multipanelSchemeSFN is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" or "11" and maxRankDCI-0-2=2 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	When two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	When one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same.

-	Second PTRS-DMRS association - 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator - 0 bit if the higher layer parameter betaOffsetsDCI-0-2 = semiStaticDCI-0-2; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList or by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization - 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured, or if transform precoder is enabled by higher layers and the Transform precoder indicator field is not present;

-	1 bit if transform precoder is disabled by higher layers and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured, or if the Transform precoder indicator field is present and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured. If the Transform precoder indicator field is present and set to '0', the bit is reserved.

-	UL-SCH indicator - 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_2 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to recerive a DCI format 0_2 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s).

-	ChannelAccess-CPext-CAPC - 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-2 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or the number of entries in the high layer parameter ul-AccessConfigListDCI-0-1 for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ul-AccessConfigListDCI-0-2 in frequency range 1. One or more entries from Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1 in frequency range 2-2.

-	Open-loop power control parameter set indication - 0 or 1 or 2 bits.

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator is present in the DCI format 0_2;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-2 if SRS resource indicator is not present in the DCI format 0_2;

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-0-2 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Invalid symbol pattern indicator - 0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-2 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214].

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

A UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_2 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_2 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 0_2 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 0_2 with smaller size until the payload size is the same.

-	If application of step 4B in clause 7.3.1.0 results in additional zero padding for DCI format 0_2 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 0_2 monitored on the primary cell and DCI format 0_2 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.1.3-1: 1 bit SRS request in DCI format 0_2 and DCI format 1_2

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_2 and 1_2 |
| --- | --- |
| 0 | No aperiodic SRS resource set triggered |
| 1 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 |

##### 7.3.1.1.4	Format 0_3

DCI format 0_3 is used for the scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell.

The following information is transmitted by means of the DCI format 0_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Scheduled cell set indicator - log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter mc-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter mc-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter scheduledCellComboListDCI-0-3 for the scheduled cell set is not configured;

-	otherwise log2IUL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.1.4-2, where IUL is the number of entries in the higher layer parameter scheduledCellComboListDCI-0-3. If only one entry is configured in the higher layer parameter scheduledCellComboListDCI-0-3, the scheduled cells are the cells configured by higher layer parameter scheduledCellComboListDCI-0-3.

-	Bandwidth part indicator - 0, 1 or 2 bits determined as log2nBWP, max, where

-	nBWP, max=nBWP,RRCmax+1 if nBWP,RRCmax≤3, nBWP,RRCmax is the maximum number of UL BWPs configured by higher layers, excluding the initial UL bandwidth part, across all the cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP, max=nBWP,RRCmax, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

The field is only applicable to a scheduled cell with the number of configured UL BWPs larger than 1, including the initial UL bandwidth part, and is applied to the applicable scheduled cells in the scheduled cell set independently. If a UE does not support active BWP change via DCI, the UE ignores this bit field. If this field indicates a code point that does not correspond to a configured BWP of a scheduled cell, the UE ignores this bit field for the scheduled cell, and operates on the active BWP of the scheduled cell.

-	Frequency domain resource assignment - number of bits determined by the following, where NRBUL, BWP is the size of the active UL bandwidth part:

-	block number 1, block number 2,…, block number NcellUL

If scheduledCellComboListDCI-0-3 for the scheduled cell set is configured with more than one entry, NcellUL is the number of scheduled cells indicated by Scheduled cells indicator field; if scheduledCellComboListDCI-0-3 for the scheduled cell set is configured with only one entry, NcellUL is the number of cells configured by higher layer parameter scheduledCellComboListDCI-0-3; otherwise, NcellUL is the number of cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set. Each block corresponds to the frequency domain resource assignment for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the frequency domain resource assignment for the cell with the smallest serving cell index. Each block is defined by the following fields:

-	If higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is not configured

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 6.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K1NRBG, K1+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K1NRBG, K1+1/2, NRBG+1 bits if resourceAllocationDCI-0-3 is configured as 'dynamicSwitch', where NRBG, K1=NRBUL, BWP+NUL, BWPstartmodK1/K1, NRBUL, BWP is the size of the active UL bandwidth part, NUL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K1 is given by higher layer parameter resourceAllocationType1GranularityDCI-0-3. If the higher layer parameter resourceAllocationType1GranularityDCI-0-3 is not configured, K1 is equal to 1.

-	If resourceAllocationDCI-0-3 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K1NRBG, K1+1/2 LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	NUL_hop MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where NUL_hop=1 if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and NUL_hop=2 if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	log2NRBG, K1NRBG, K1+1/2-NUL_hop bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	log2NRBG, K1NRBG, K1+1/2 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-0-3 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured

-	5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

-	6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

The value of Y is determined by log2NRB-set,ULBWPNRB-set,ULBWP+12  where NRB-set,ULBWP  is the number of RB sets contained in the active UL BWP as defined in clause 7 of [6, TS38.214].

If the higher layer parameter scheduledCellComboListDCI-0-3 for the scheduled cell set is not configured, each block is also used to indicate whether the corresponding cell is scheduled or not as follows:

-	if all bits of a block are set to 0 for resource allocation type 0, or set to 1 for resource allocation type 1, or set to 0 or 1 for dynamic switch resource allocation type, or set to 0 for resource allocation type 2 with μ=1, or set to 1 for resource allocation type 2 with μ=0, the cell corresponding to the block is not scheduled;

-	otherwise, the cell corresponding to the block is scheduled.

-	Time domain resource assignment - log2(ITDRA) bits, where  ITDRA is the number of entries in the higher layer parameter tdra-FieldIndexListDCI-0-3. This field is used to indicate an entry in the higher layer parameter tdra-FieldIndexListDCI-0-3 according to Table 7.3.1.1.4-3. Each entry in the higher layer parameter tdra-FieldIndexListDCI-0-3 contains the ‘Time domain resource assignment’ index for each BWP of each cell in the scheduled cell set, where the ‘Time domain resource assignment’ indexes for all the cells are placed according to an ascending order of a serving cell index, and the 'Time domain resource assignment' indexes for all the BWPs of a cell are placed according to an ascending order of the higher layer parameter BWP-Id.

-	Frequency hopping flag - 0 or 1 bit

-	0 bit if the higher layer parameter frequencyHopping is not configured for any cell configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

The field is only applicable to a scheduled cell configured with frequencyHopping, and is applied to the applicable scheduled cells independently.

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the modulation and coding scheme for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the new data indicator for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the redundancy version for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-0-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

-	HARQ process number - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the HARQ process number for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the HARQ process number for the cell with the smallest serving cell index. Each block is 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-3 configured for the cell corresponding to the block.

-	1st downlink assignment index - 1 or 2 bits

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-3 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_3 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_3 for the two HARQ-ACK codebooks are the same.

-	2nd downlink assignment index - 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-3 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_3 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd  downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_3 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the TPC command for the scheduled PUSCH for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the TPC command for the scheduled PUSCH for the cell with the smallest serving cell index. Each block is 2 bits as defined in Clause 7.1.1 of [5, TS38.213].

-	SRS resource indicator - number of bits determined by the following:

-	If sri-DCI0-3= type1a is configured by higher layer,

-	maxr∈{1,2,…,NcellUL,2}Msr bits applying to the scheduled cells with Msr>0 independently, where NcellUL, 2 is the number of cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set, r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Msr is defined below.

-	If sri-DCI0-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the SRS resource indicator for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the SRS resource indicator for the cell with the smallest serving cell index. Each block is defined below.

Msr above for the case of sri-DCI0-3= type1a or each block above for the case of sri-DCI0-3= type2 is defined by the following:

-	log2k=1minLmax, NSRSNSRSk bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where NSRS is the number of configured SRS resources in the first SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, and associated with the higher layer parameter usage of value 'nonCodeBook' and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2NSRS bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook, where NSRS is the number of configured SRS resources in the first SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, and associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers - number of bits determined by the following:

-	If tpmi-DCI0-3= type1a is configured by higher layer,

-	 maxr∈{1,2,…,NcellUL,2}Mpr bits applying to the scheduled cells with Mpr>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mpr is defined below.

-	If tpmi-DCI0-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the precoding information and number of layers for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the precoding information and number of layers for the cell with the smallest serving cell index. Each block is defined below.

Mpr above for the case of tpmi-DCI0-3= type1a or each block above for the case of tpmi-DCI0-3= type2 is defined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the value of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRank=1, and the value of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRank=1, and the value of higher layer parameter codebookSubset.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in an SRS resource set with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

-	Antenna ports - number of bits determined by the following:

-	If antennaPortsDCI0-3= type1a is configured by higher layer,

-	maxr∈{1,2,…,NcellUL,2}MAr bits applying to the scheduled cells independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and MAr is defined below.

-	If antennaPortsDCI0-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the Antenna ports information for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the Antenna ports information for the cell with the smallest serving cell index. Each block is defined below.

MAr above for the case of antennaPortsDCI0-3= type1a or each block above for the case of antennaPortsDCI0-3= type2 is defined by the following:

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by Tables 7.3.1.1.2-6A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in clause 6.4.1.1.1.2, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in clause 6.4.1.1.1.2, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB.

-	SRS request - log2(ISRS) bits, where ISRS is the number of entries in the higher layer parameter srs-RequestListDCI-0-3, or 0 bit if the higher layer parameter srs-RequestListDCI-0-3 is not configured. This field is used to indicate an entry in the higher layer parameter srs-RequestListDCI-0-3 according to Table 7.3.1.1.4-4. Each entry in the higher layer parameter srs-RequestListDCI-0-3 contains the ‘SRS request’ index for each cell in the scheduled cell set, where the ‘SRS request’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS request’ index is defined by the following:

-	2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - log2(Ioffset) bits, whereIoffset is the number of entries in the higher layer parameter srs-OffsetListDCI-0-3, or 0 bit if the higher layer parameter srs-OffsetListDCI-0-3 is not configured. This field is used to indicate an entry in the higher layer parameter srs-OffsetListDCI-0-3 according to Table 7.3.1.1.4-5. Each entry in the higher layer parameter srs-OffsetListDCI-0-3 contains the ‘SRS offset indicator’ index for each cell in the scheduled cell set, where the ‘SRS offset indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS offset indicator’ index is defined by the following:

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CSI request - 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize. This field is applied to the cell with the smallest serving cell index among the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	PTRS-DMRS association - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellUL

Each block corresponds to the PTRS-DMRS association information for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the PTRS-DMRS association information for the cell with the smallest serving cell index. Each block is defined by the following:

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	beta_offset indicator  -  0 or 2 bits

-	0 bit if betaOffsets = semiStatic is configured in uci-OnPUSCH-ListDCI-0-3 for all the cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set;

-	otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-3 is configured, if the bit width of the beta_offset indicator in DCI format 0_3 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_3 for the two HARQ-ACK codebooks are the same.

The field is only applicable to a scheduled cell configured with betaOffsets = dynamic in uci-OnPUSCH-ListDCI-0-3, and is applied to the applicable scheduled cells independently.

-	DMRS sequence initialization - 1 bit if transform precoder is disabled at least for one cell configured by higher layer parameter ScheduledCellListDCI-0-3 in the scheduled cell set; otherwise, 0 bit.

This field is independently applied to all the scheduled cells with transform precoder disabled, and indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	UL-SCH indicator - 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. A UE is not expected to receive a DCI format 0_3 with UL-SCH indicator of "0" and CSI request of all zero(s). This field is applied to the cell with the smallest serving cell index among the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	ChannelAccess-CPext-CAPC - maxr∈{1,2,…,Ncell2}Mcr bits applying to the scheduled cells with Mcr>0 independently, where Ncell2 is the number of cells configured by higher layer parameter scheduledCellListDCI-0-3 in the scheduled cell set, r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mcr is defined by the following:

-	0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

-	Open-loop power control parameter set indication - maxr∈{1,2,…,NcellUL,2}Mor bits applying to the scheduled cells with Mor>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mor is defined by the following:

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured for a serving cell associated to index r;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator block number r is present in the DCI format 0_3 when SRI-DCI0-3= type2 or if Msr>0 when SRI-DCI0-3= type1a;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator block number r is not present in the DCI format 0_3 when SRI-DCI0-3= type2 or if Msr=0 when SRI-DCI0-3= type1a.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-0-3 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213]. This field is applied to all the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0DCI-0-3 is not configured;

-	1 bit otherwise. The 1 bit indication is used to determine the minimum applicable K2 for the active UL BWP and the minimum applicable K0 for the active DL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP for each scheduled cell shall be the same as the minimum applicable K0.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyDCI-0-3 or dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4, or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	0 bit if higher layer parameter pdcchMonAdaptDCI-0-3 is not enabled;

-	otherwise,

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

If scheduledCellComboListDCI-0-3 for the cell set is configured, zeros shall be appended to DCI format 0_3 if needed until the payload size equals the size of DCI format 0_3 that is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-0-3.

If an SCell within the scheduled cell set is deactivated, the UE determines the bitwidth of the fields in DCI format 0_3 based on a UL BWP provided by firstActiveUplinkBWP-Id for the SCell.

Table 7.3.1.1.4-1: Scheduled cell set indicator in DCI format 0_3 and DCI format 1_3

| Bit field mapped to index | Scheduled cell set |
| --- | --- |
| 0 | The cell set with the smallest set ID configured by mc-DCI-SetofCellsToAddModList |
| 1 | The cell set with the 2nd smallest set ID configured by mc-DCI-SetofCellsToAddModList |
| 2 | The cell set with the 3rd smallest set ID configured by mc-DCI-SetofCellsToAddModList, if any |
| 3 | The cell set with the 4th smallest set ID configured by mc-DCI-SetofCellsToAddModList, if any |

Table 7.3.1.1.4-2: Scheduled cells indicator in DCI format 0_3

| Bit field mapped to index | Scheduled cells |
| --- | --- |
| 0 | The cells configured by the 1st entry in scheduledCellComboListDCI-0-3 |
| 1 | The cells configured by the 2nd entry in scheduledCellComboListDCI-0-3 |
| 2 | The cells configured by the 3rd entry in scheduledCellComboListDCI-0-3, if any |
| 3 | The cells configured by the 4th entry in scheduledCellComboListDCI-0-3, if any |
| 4 | The cells configured by the 5th entry in scheduledCellComboListDCI-0-3, if any |
| 5 | The cells configured by the 6th entry in scheduledCellComboListDCI-0-3, if any |
| 6 | The cells configured by the 7th entry in scheduledCellComboListDCI-0-3, if any |
| 7 | The cells configured by the 8th entry in scheduledCellComboListDCI-0-3, if any |
| 8 | The cells configured by the 9th entry in scheduledCellComboListDCI-0-3, if any |
| 9 | The cells configured by the 10th entry in scheduledCellComboListDCI-0-3, if any |
| 10 | The cells configured by the 11th entry in scheduledCellComboListDCI-0-3, if any |
| 11 | The cells configured by the 12th entry in scheduledCellComboListDCI-0-3, if any |
| 12 | The cells configured by the 13th entry in scheduledCellComboListDCI-0-3, if any |
| 13 | The cells configured by the 14th entry in scheduledCellComboListDCI-0-3, if any |
| 14 | The cells configured by the 15th entry in scheduledCellComboListDCI-0-3, if any |
| 15 | The cells configured by the 16th entry in scheduledCellComboListDCI-0-3, if any |

Table 7.3.1.1.4-3: Time domain resource assignment in DCI format 0_3

| Bit field mapped to index | Indicated time domain resource allocation |
| --- | --- |
| 0 | The 1st entry in tdra-FieldIndexListDCI-0-3 |
| 1 | The 2nd entry in tdra-FieldIndexListDCI-0-3 |
| 2 | The 3rd entry in tdra-FieldIndexListDCI-0-3, if any |
| … | … |
| i | The (i+1)th entry in tdra-FieldIndexListDCI-0-3, if any |
| … | … |

Table 7.3.1.1.4-4: SRS request in DCI format 0_3

| Bit field mapped to index | Triggered aperiodic SRS resource set(s) |
| --- | --- |
| 0 | The 1st entry in srs-RequestListDCI-0-3 |
| 1 | The 2nd entry in srs-RequestListDCI-0-3 |
| 2 | The 3rd entry in srs-RequestListDCI-0-3, if any |
| … | … |
| 15 | The 16th entry in srs-RequestListDCI-0-3, if any |

Table 7.3.1.1.4-5: SRS offset indicator in DCI format 0_3

| Bit field mapped to index | Available slot offset |
| --- | --- |
| 0 | The 1st entry in srs-OffsetListDCI-0-3 |
| 1 | The 2nd entry in srs-OffsetListDCI-0-3 |
| 2 | The 3rd entry in srs-OffsetListDCI-0-3, if any |
| … | … |
| 7 | The 8th entry in srs-OffsetListDCI-0-3, if any |

#### 7.3.1.2	DCI formats for scheduling of PDSCH

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment -  bits where  is given by Clause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator - 1 bit.

-	If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the candidate cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	Otherwise, this field is reserved.

-	SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUL-SyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUL-SyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUL-SyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity.

-	PRACH association indicator - 0 or 1 bit

-	1bit if the UE is provided with tag2-Id, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. This field is reserved if the cell indicated by Cell indicator field is a candidate cell.

-	This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states.

-	This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format.

-	0 bit otherwise.

-	PRACH retransmission indicator - 0 or 1 bit

-	1bit if the UE is configured with higher layer parameter EarlyUL-SyncConfig. This field indicates initial transmission or retransmission of PRACH according to Table 7.3.1.2.1-3 if the cell indicated by Cell indicator field is a candidate cell, and this field is reserved if the cell indicated by Cell indicator field is a serving cell but not a candidate cell.

-	0 bit otherwise.

-	Reserved bits - a number of bits as determined by the following:

-	(12 - Y1 - Y2) bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2;

-	(10 - Y1 - Y2) bits otherwise;

where,

-	Y1=0 if the UE is not configured with higher layer parameter EarlyUL-SyncConfig; Y1=log2C+1+1 otherwise.

-	Y2=0 if the "PRACH association indicator" field is not present in this DCI format; Y2=1 otherwise.

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bits otherwise

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured; 0 bits otherwise.

-	Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by SI-RNTI:

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	System information indicator - 1 bit as defined in Table 7.3.1.2.1-2

-	Reserved bits -  17 bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; otherwise 15 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by RA-RNTI or MsgB-RNTI:

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0 if CORESET 0 is configured for the cell and  is the size of initial DL bandwidth part if CORESET 0 is not configured for the cell

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]

-	LSBs of SFN - 2 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI as defined in Clause 8.2A of [5, TS 38.213] if msgB-responseWindow is configured to be larger than 10 ms; or 2 bits for the DCI format 1_0 with CRC scrambled by RA-RNTI as defined in Clause 8.2 of [5, TS 38.213] for operation in a cell with shared spectrum channel access if ra-ResponseWindow or ra-ResponseWindow-v1610 is configured to be larger than 10 ms; 0 bit otherwise

-	Reserved bits - (16 - A) bits for operation in a cell without shared spectrum access in frequency range 1 and frequency range 2-1, (18 - A) for operation in a cell with shared spectrum access in frequency range 1 or for operation in a cell in frequency range 2-2, where the value of A is the number of bits for the field of 'LSBs of SFN' as defined above

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits

-	2 bits indicating the number of repetitions for PUCCH as defined in clause 9.2.6 of [5, TS38.213] according to Table 7.3.1.2.1-4, if the higher layer parameter numberOfMsg4HARQ-ACK-Repetitions is configured with at least two repetition factors and the UE has indicated capability of PUCCH repetition on common PUCCH resource [8, TS38.321];

-	otherwise, reserved.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; otherwise 0 bit

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

Table 7.3.1.2.1-1: Short Message indicator

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI |
| 10 | Only short message, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI |
| 11 | Both scheduling information for Paging, TRS availability indication if trs-ResourceSetConfig is configured and short message are present in the DCI |

Table 7.3.1.2.1-2: System information indicator

| Bit field | System information indicator |
| --- | --- |
| 0 | SIB1 [9, TS38.331, Clause 5.2.1] |
| 1 | SI message [9, TS38.331, Clause 5.2.1] |

Table 7.3.1.2.1-3: PRACH retransmission indicator

| Bit field | PRACH retransmission indicator |
| --- | --- |
| 0 | Initial transmission of PRACH |
| 1 | Retransmission of PRACH |

Table 7.3.1.2.1-4: Number of repetitions NPUCCHrepeat as a function of 2 bits of Downlink assignment index field

| Bit field | NPUCCHrepeat |
| --- | --- |
| 00 | First repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions |
| 01 | Second repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions |
| 10 | Third repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions if provided, otherwise reserved |
| 11 | Fourth repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions if provided, otherwise reserved |

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator - 0 or 3 bits as defined in Clause 10.1 of [5, TS 38.213]. This field is reserved when this format is carried by PDCCH on the primary cell and the UE is configured for scheduling on the primary cell from an SCell, with the same number of bits as that in this format carried by PDCCH on the SCell for scheduling on the primary cell.

-	Bandwidth part indicator - 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment - number of bits determined by the following, where  is the size of the active DL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	bits if only resource allocation type 1 is configured, or

-	 bits if resourceAllocation is configured as 'dynamicSwitch'.

-	If resourceAllocation is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocation is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment - 0, 1, 2, 3, 4, 5 or 6 bits

-	If the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH is not configured and if the higher layer parameter pdsch-TimeDomainAllocationList is configured, 0, 1, 2, 3 or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured;

-	if the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH;

-	otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping - 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured by high layers;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6  of [4, TS 38.211].

-	PRB bundling size indicator - 0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'staticBundling', or 1 bit if the higher layer parameter prb-BundlingType is set to 'dynamicBundling' according to Clause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator - 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger - 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where  is the number of aperiodic ZP CSI-RS resource sets configured by higher layer.

For transport block 1:

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator - 1 bit if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	Redundancy version - number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator - 1 bit if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	Redundancy version - number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number - 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-1-1 is configured; otherwise 4 bits

-	Downlink assignment index - number of bits as defined in the following

-	6 bits if more than one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included is configured. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group.

-	4 bits if only one serving cell is configured in the DL and the higher layer parameter nfi-TotalDAI-Included is configured. The 2 MSB bits are the counter DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group;

-	4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic, and nfi-TotalDAI-Included is not configured, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	4 bits if one serving cell is configured in the DL, and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16=enhancedDynamic, and nfi-TotalDAI-Included is not configured, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 2 bits are the counter DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, the number of serving cells is determined within a PUCCH group.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the Downlink assignment index in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	Second TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213] if higher layer parameter SecondTPCFieldDCI-1-1 is configured; 0 bit otherwise.

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDyn is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with the two cells are the same.

If the UE is configured with a PUCCH-SCell, pucch-sSCellDyn is replaced by pucch-sSCellDynSecondaryPUCCHgroup for the secondary PUCCH group.

-	One-shot HARQ-ACK request - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedback-r16 or pdsch-HARQ-ACK-EnhType3ToAddModList is configured;

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryToAddModList for the secondary PUCCH group.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-EnhType3DCI-Field is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-EnhType3ToAddModList.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3DCI-Field is replaced by pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup for the secondary PUCCH group, and pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryList for the secondary PUCCH group.

-	PDSCH group index - 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic;

-	0 bit otherwise.

-	New feedback indicator - 0, 1 or 2 bits.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic and the higher layer parameter nfi-TotalDAI-Included is not configured;

-	2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic and the higher layer parameter nfi-TotalDAI-Included=true; the MSB corresponds to the scheduled PDSCH group, and the LSB corresponds to the non-scheduled PDSCH group, as defined in [TS38.213] clause 9.1.3.3

-	0 bit otherwise.

-	Number of requested PDSCH group(s) - 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic;

-	0 bit otherwise.

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-Retx is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Retx is replaced by pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup for the secondary PUCCH group.

-	Antenna port(s) - 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE not configured with dl-OrJointTCI-StateList receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, or when a UE configured with dl-OrJointTCI-StateList is having two indicated TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10. The UE can receive an entry with DMRS ports equals to 1000, 1002, 1003 when two the UE is not configured with dl-OrJointTCI-StateList and TCI states are indicated in a codepoint of DCI field 'Transmission Configuration Indication', or when the UE configured with dl-OrJointTCI-StateList is having two indicated TCI states to be applied to PDSCH.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication - 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_1,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	TCI selection - 0 bit if higher layer parameter tci-SelectionPresentInDCI is not configured; otherwise 2 bits according to Table 7.3.1.2.2-11.

-	SRS request - 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell  and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CBG transmission information (CBGTI) - 0 bit if higher layer parameter PDSCH-CodeBlockGroupTransmission for PDSCH is not configured, otherwise, 2, 4, 6, or 8 bits as defined in Clause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG transmission information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG transmission information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG transmission information until the bit width of the CBG transmission information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	CBG flushing out information (CBGFI) - 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", 0 bit otherwise.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG flushing out information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG flushing out information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG flushing out information until the bit width of the CBG flushing out information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization - 1 bit.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-1-1 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	ChannelAccess-CPext - 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0 is not configured;

-	1 bit if higher layer parameter minimumSchedulingOffsetK0 is configured. The 1 bit indication is used to determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 value for the active UL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP shall be the same as the minimum applicable K0 value.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

If one-shot HARQ-ACK request is not present or set to '0', and all bits of frequency domain resource assignment are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type, this field is reserved and the following fields among the fields above are used for SCell dormancy indication, where each bit corresponds to one of the configured SCell(s), with MSB to LSB of the following fields concatenated in the order below corresponding to the SCell with lowest to highest SCell index

-	Modulation and coding scheme of transport block 1

-	New data indicator of transport block 1

-	Redundancy version of transport block 1

-	HARQ process number

-	Antenna port(s)

-	DMRS sequence initialization

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

-	PUCCH Cell indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDyn is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pucch-sSCellDyn is replaced by pucch-sSCellDynSecondaryPUCCHgroup for the secondary PUCCH group.

-	Co-scheduled UE information – 0 or 3 bits

-	3 bits as defined in Table 7.3.1.2.2-12 if higher layer parameter advReceiver-MU-MIMO-DCI-1-1 is configured. This field is reserved if two codewords are scheduled by this DCI format 1_1.

-	0 bit otherwise.

If DCI formats 1_1 are monitored in multiple search spaces associated with multiple CORESETs in a BWP for scheduling the same serving cell, zeros shall be appended until the payload size of the DCI formats 1_1 monitored in the multiple search spaces equal to the maximum payload size of the DCI format 1_1 monitored in the multiple search spaces.

If the number of information bits in DCI format 1_1 scheduling a single PDSCH prior to padding is not equal to the number of information bits in DCI format 1_1 scheduling multiple PDSCHs for the same serving cell, zeros shall be appended to the DCI format 1_1 with smaller size until the payload size is the same for scheduling a single PDSCH and multiple PDSCHs.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 1_1 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 1_1 with smaller size until the payload size is the same:

-	If application of step 4C in clause 7.3.1.0 results in additional zero padding for DCI format 1_1 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 1_1 monitored on the primary cell and DCI format 1_1 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.2.2-1: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 2 |
| 6 | 2 | 3 |
| 7 | 2 | 0,1 |
| 8 | 2 | 2,3 |
| 9 | 2 | 0-2 |
| 10 | 2 | 0-3 |
| 11 | 2 | 0,2 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.2.2-1A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 2 |
| 6 | 2 | 3 |
| 7 | 2 | 0,1 |
| 8 | 2 | 2,3 |
| 9 | 2 | 0-2 |
| 10 | 2 | 0-3 |
| 11 | 2 | 0,2 |
| 12 | 2 | 0,2,3 |
| 13-15 | Reserved | Reserved |

Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4-31 | reserved | reserved | reserved |
| 5 | 2 | 2 | 1 |  |  |  |  |
| 6 | 2 | 3 | 1 |  |  |  |  |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-2A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4-31 | reserved | reserved | reserved |
| 5 | 2 | 2 | 1 |  |  |  |  |
| 6 | 2 | 3 | 1 |  |  |  |  |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | 2 | 0,2,3 | 1 |  |  |  |  |

Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2-31 | reserved | reserved |
| 3 | 2 | 0 |  |  |  |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-3A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2-31 | reserved | reserved |
| 3 | 2 | 0 |  |  |  |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 2 | 0,2,3 |  |  |  |
| 25-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6-63 | Reserved | Reserved | Reserved |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 3 | 0 | 1 |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58-63 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-4A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6-63 | Reserved | Reserved | Reserved |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 3 | 0 | 1 |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58 | 2 | 0,2,3 | 1 |  |  |  |  |
| 59-63 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-5: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1/1_3 and DCI format 1_2, configured by higher layer parameter ul-AccessConfigListDCI-1-1 and ul-AccessConfigListDCI-1-2, respectively, in frequency range 1

| Entry index | Channel Access Type | The CP extension Text index defined in Clause 5.3.1 of [4,TS 38.211] |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 0 |
| 1 | Type2C-ULChannelAccess defined in clause 4.2.1.2.3 in TS 37.213 [14] | 2 |
| 2 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 0 |
| 3 | Type2B-ULChannelAccess defined in clause 4.2.1.2.2 in TS 37.213 [14] | 2 |
| 4 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 0 |
| 5 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 1 |
| 6 | Type2A-ULChannelAccess defined in clause 4.2.1.2.1 in TS 37.213 [14] | 3 |
| 7 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 0 |
| 8 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 1 |
| 9 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 2 |
| 10 | Type1-ULChannelAccess defined in clause 4.2.1.1 in TS 37.213 [14] | 3 |

Table 7.3.1.2.2-6A: Allowed entries for DCI format 1_1, DCI format 1_2 and DCI format 1_3, configured by higher layer parameter ul-AccessConfigListDCI-1-1 in frequency range 2-2

| Entry index | Channel Access Type |
| --- | --- |
| 0 | Type 1 channel access defined in clause 4.4.1 of TS 37.213 [14] |
| 1 | Type 2 channel access defined in clause 4.4.2 of TS 37.213 [14] |
| 2 | Type 3 channel access defined in clause 4.4.3 of TS 37.213 [14] |

Table 7.3.1.2.2-7: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 | 4-31 | Reserved | Reserved |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 2 | 0,2 |  |  |  |
| 12 | 1 | 8 |  |  |  |
| 13 | 1 | 9 |  |  |  |
| 14 | 1 | 8,9 |  |  |  |
| 15 | 2 | 8 |  |  |  |
| 16 | 2 | 9 |  |  |  |
| 17 | 2 | 10 |  |  |  |
| 18 | 2 | 11 |  |  |  |
| 19 | 2 | 8,9 |  |  |  |
| 20 | 2 | 10,11 |  |  |  |
| 21 | 1 | 0,1,8 |  |  |  |
| 22 | 1 | 0,1,8,9 |  |  |  |
| 23 | 2 | 0,1,8 |  |  |  |
| 24 | 2 | 0,1,8,9 |  |  |  |
| 25 | 2 | 2,3,10 |  |  |  |
| 26 | 2 | 2,3,10,11 |  |  |  |
| 27-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-7A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 | 4-31 | Reserved | Reserved |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 2 | 0,2 |  |  |  |
| 12 | 1 | 8 |  |  |  |
| 13 | 1 | 9 |  |  |  |
| 14 | 1 | 8,9 |  |  |  |
| 15 | 2 | 8 |  |  |  |
| 16 | 2 | 9 |  |  |  |
| 17 | 2 | 10 |  |  |  |
| 18 | 2 | 11 |  |  |  |
| 19 | 2 | 8,9 |  |  |  |
| 20 | 2 | 10,11 |  |  |  |
| 21 | 1 | 0,1,8 |  |  |  |
| 22 | 1 | 0,1,8,9 |  |  |  |
| 23 | 2 | 0,1,8 |  |  |  |
| 24 | 2 | 0,1,8,9 |  |  |  |
| 25 | 2 | 2,3,10 |  |  |  |
| 26 | 2 | 2,3,10,11 |  |  |  |
| 27 | 2 | 0,2,3 |  |  |  |
| 28-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-8: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0,1,2,3,8 | 1 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,8,9,10 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-4 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,4,6 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,4,5,6 | 2 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 1 | 0,1,4,5,8,12 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 1 | 8 | 1 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 1 | 9 | 1 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 1 | 8,9 | 1 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 8 | 1 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 9 | 1 | 16~127 | Reserved | Reserved | Reserved |
| 17 | 2 | 10 | 1 |  |  |  |  |
| 18 | 2 | 11 | 1 |  |  |  |  |
| 19 | 2 | 8,9 | 1 |  |  |  |  |
| 20 | 2 | 10,11 | 1 |  |  |  |  |
| 21 | 1 | 0,1,8 | 1 |  |  |  |  |
| 22 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 23 | 2 | 0,1,8 | 1 |  |  |  |  |
| 24 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 25 | 2 | 2,3,10 | 1 |  |  |  |  |
| 26 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 27 | 2 | 0 | 2 |  |  |  |  |
| 28 | 2 | 1 | 2 |  |  |  |  |
| 29 | 2 | 2 | 2 |  |  |  |  |
| 30 | 2 | 3 | 2 |  |  |  |  |
| 31 | 2 | 4 | 2 |  |  |  |  |
| 32 | 2 | 5 | 2 |  |  |  |  |
| 33 | 2 | 6 | 2 |  |  |  |  |
| 34 | 2 | 7 | 2 |  |  |  |  |
| 35 | 2 | 0,1 | 2 |  |  |  |  |
| 36 | 2 | 2,3 | 2 |  |  |  |  |
| 37 | 2 | 4,5 | 2 |  |  |  |  |
| 38 | 2 | 6,7 | 2 |  |  |  |  |
| 39 | 2 | 0,4 | 2 |  |  |  |  |
| 40 | 2 | 2,6 | 2 |  |  |  |  |
| 41 | 2 | 0,1,4 | 2 |  |  |  |  |
| 42 | 2 | 2,3,6 | 2 |  |  |  |  |
| 43 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 44 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 45 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 46 | 2 | 8 | 2 |  |  |  |  |
| 47 | 2 | 9 | 2 |  |  |  |  |
| 48 | 2 | 10 | 2 |  |  |  |  |
| 49 | 2 | 11 | 2 |  |  |  |  |
| 50 | 2 | 12 | 2 |  |  |  |  |
| 51 | 2 | 13 | 2 |  |  |  |  |
| 52 | 2 | 14 | 2 |  |  |  |  |
| 53 | 2 | 15 | 2 |  |  |  |  |
| 54 | 2 | 8,9 | 2 |  |  |  |  |
| 55 | 2 | 10,11 | 2 |  |  |  |  |
| 56 | 2 | 12,13 | 2 |  |  |  |  |
| 57 | 2 | 14,15 | 2 |  |  |  |  |
| 58 | 2 | 0,1,8 | 2 |  |  |  |  |
| 59 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 60 | 2 | 4,5,12 | 2 |  |  |  |  |
| 61 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 62 | 2 | 2,3,10 | 2 |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 64 | 2 | 6,7,14 | 2 |  |  |  |  |
| 65 | 2 | 6,7,14,15 | 2 |  |  |  |  |
| 66-127 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-8A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1,
dmrs-TypeEnh is configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0,1,2,3,8 | 1 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,8,9,10 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-4 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,4,6 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,4,5,6 | 2 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 1 | 0,1,4,5,8,12 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 1 | 8 | 1 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 1 | 9 | 1 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 1 | 8,9 | 1 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 8 | 1 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 9 | 1 | 16~127 | Reserved | Reserved | Reserved |
| 17 | 2 | 10 | 1 |  |  |  |  |
| 18 | 2 | 11 | 1 |  |  |  |  |
| 19 | 2 | 8,9 | 1 |  |  |  |  |
| 20 | 2 | 10,11 | 1 |  |  |  |  |
| 21 | 1 | 0,1,8 | 1 |  |  |  |  |
| 22 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 23 | 2 | 0,1,8 | 1 |  |  |  |  |
| 24 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 25 | 2 | 2,3,10 | 1 |  |  |  |  |
| 26 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 27 | 2 | 0 | 2 |  |  |  |  |
| 28 | 2 | 1 | 2 |  |  |  |  |
| 29 | 2 | 2 | 2 |  |  |  |  |
| 30 | 2 | 3 | 2 |  |  |  |  |
| 31 | 2 | 4 | 2 |  |  |  |  |
| 32 | 2 | 5 | 2 |  |  |  |  |
| 33 | 2 | 6 | 2 |  |  |  |  |
| 34 | 2 | 7 | 2 |  |  |  |  |
| 35 | 2 | 0,1 | 2 |  |  |  |  |
| 36 | 2 | 2,3 | 2 |  |  |  |  |
| 37 | 2 | 4,5 | 2 |  |  |  |  |
| 38 | 2 | 6,7 | 2 |  |  |  |  |
| 39 | 2 | 0,4 | 2 |  |  |  |  |
| 40 | 2 | 2,6 | 2 |  |  |  |  |
| 41 | 2 | 0,1,4 | 2 |  |  |  |  |
| 42 | 2 | 2,3,6 | 2 |  |  |  |  |
| 43 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 44 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 45 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 46 | 2 | 8 | 2 |  |  |  |  |
| 47 | 2 | 9 | 2 |  |  |  |  |
| 48 | 2 | 10 | 2 |  |  |  |  |
| 49 | 2 | 11 | 2 |  |  |  |  |
| 50 | 2 | 12 | 2 |  |  |  |  |
| 51 | 2 | 13 | 2 |  |  |  |  |
| 52 | 2 | 14 | 2 |  |  |  |  |
| 53 | 2 | 15 | 2 |  |  |  |  |
| 54 | 2 | 8,9 | 2 |  |  |  |  |
| 55 | 2 | 10,11 | 2 |  |  |  |  |
| 56 | 2 | 12,13 | 2 |  |  |  |  |
| 57 | 2 | 14,15 | 2 |  |  |  |  |
| 58 | 2 | 0,1,8 | 2 |  |  |  |  |
| 59 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 60 | 2 | 4,5,12 | 2 |  |  |  |  |
| 61 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 62 | 2 | 2,3,10 | 2 |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 64 | 2 | 6,7,14 | 2 |  |  |  |  |
| 65 | 2 | 6,7,14,15 | 2 |  |  |  |  |
| 66 | 2 | 0,2,3 | 1 |  |  |  |  |
| 67-127 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,12 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,12,14 |
| 4 | 2 | 1 | 4 | 2 | 0-3,12-14 |
| 5 | 2 | 2 | 5 | 2 | 0-3,12-15 |
| 6 | 2 | 3 | 6 | 3 | 0,1,2,3,12 |
| 7 | 2 | 0,1 | 7 | 3 | 0,1,2,3,12,14 |
| 8 | 2 | 2,3 | 8 | 3 | 0-3,12-14 |
| 9 | 2 | 0-2 | 9 | 3 | 0-3,12-15 |
| 10 | 2 | 0-3 | 10~63 | Reserved | Reserved |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| 33 | 3 | 12 |  |  |  |
| 34 | 3 | 13 |  |  |  |
| 35 | 3 | 14 |  |  |  |
| 36 | 3 | 15 |  |  |  |
| 37 | 3 | 16 |  |  |  |
| 38 | 3 | 17 |  |  |  |
| 39 | 3 | 12,13 |  |  |  |
| 40 | 3 | 14,15 |  |  |  |
| 41 | 3 | 16,17 |  |  |  |
| 42 | 1 | 0,1,12 |  |  |  |
| 43 | 1 | 0,1,12,13 |  |  |  |
| 44 | 2 | 0,1,12 |  |  |  |
| 45 | 2 | 0,1,12,13 |  |  |  |
| 46 | 2 | 2,3,14 |  |  |  |
| 47 | 2 | 2,3,14,15 |  |  |  |
| 48 | 3 | 0,1,12 |  |  |  |
| 49 | 3 | 0,1,12,13 |  |  |  |
| 50 | 3 | 2,3,14 |  |  |  |
| 51 | 3 | 2,3,14,15 |  |  |  |
| 52 | 3 | 4,5,16 |  |  |  |
| 53 | 3 | 4,5,16,17 |  |  |  |
| 54-63 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-9A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,12 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,12,14 |
| 4 | 2 | 1 | 4 | 2 | 0-3,12-14 |
| 5 | 2 | 2 | 5 | 2 | 0-3,12-15 |
| 6 | 2 | 3 | 6 | 3 | 0,1,2,3,12 |
| 7 | 2 | 0,1 | 7 | 3 | 0,1,2,3,12,14 |
| 8 | 2 | 2,3 | 8 | 3 | 0-3,12-14 |
| 9 | 2 | 0-2 | 9 | 3 | 0-3,12-15 |
| 10 | 2 | 0-3 | 10~63 | Reserved | Reserved |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| 33 | 3 | 12 |  |  |  |
| 34 | 3 | 13 |  |  |  |
| 35 | 3 | 14 |  |  |  |
| 36 | 3 | 15 |  |  |  |
| 37 | 3 | 16 |  |  |  |
| 38 | 3 | 17 |  |  |  |
| 39 | 3 | 12,13 |  |  |  |
| 40 | 3 | 14,15 |  |  |  |
| 41 | 3 | 16,17 |  |  |  |
| 42 | 1 | 0,1,12 |  |  |  |
| 43 | 1 | 0,1,12,13 |  |  |  |
| 44 | 2 | 0,1,12 |  |  |  |
| 45 | 2 | 0,1,12,13 |  |  |  |
| 46 | 2 | 2,3,14 |  |  |  |
| 47 | 2 | 2,3,14,15 |  |  |  |
| 48 | 3 | 0,1,12 |  |  |  |
| 49 | 3 | 0,1,12,13 |  |  |  |
| 50 | 3 | 2,3,14 |  |  |  |
| 51 | 3 | 2,3,14,15 |  |  |  |
| 52 | 3 | 4,5,16 |  |  |  |
| 53 | 3 | 4,5,16,17 |  |  |  |
| 54 | 2 | 0,2,3 |  |  |  |
| 55-63 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,12 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0-3,12,14 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-3,12-14 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0-3,12-15 | 1 |
| 6 | 2 | 3 | 1 | 6 | 3 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 3 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 3 | 0-3,12-14 | 1 |
| 9 | 2 | 0-2 | 1 | 9 | 3 | 0-3,12-15 | 1 |
| 10 | 2 | 0-3 | 1 | 10 | 2 | 0,1,2,3,6 | 2 |
| 11 | 3 | 0 | 1 | 11 | 2 | 0,1,2,3,6,8 | 2 |
| 12 | 3 | 1 | 1 | 12 | 2 | 0,1,2,3,6,7,8 | 2 |
| 13 | 3 | 2 | 1 | 13 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 14 | 3 | 3 | 1 | 14 | 1 | 0,1,6,7,12 | 2 |
| 15 | 3 | 4 | 1 | 15 | 1 | 0,1,6,7,12,18 | 2 |
| 16 | 3 | 5 | 1 | 16 | 1 | 0,1,6,7,12,13,18 | 2 |
| 17 | 3 | 0,1 | 1 | 17 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 18 | 3 | 2,3 | 1 | 18 | 2 | 0,1,6,7,12 | 2 |
| 19 | 3 | 4,5 | 1 | 19 | 2 | 0,1,6,7,12,18 | 2 |
| 20 | 3 | 0-2 | 1 | 20 | 2 | 0,1,6,7,12,13,18 | 2 |
| 21 | 3 | 3-5 | 1 | 21 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 22 | 3 | 0-3 | 1 | 22 | 3 | 0,1,6,7,12 | 2 |
| 23 | 2 | 0,2 | 1 | 23 | 3 | 0,1,6,7,12,18 | 2 |
| 24 | 1 | 12 | 1 | 24 | 3 | 0,1,6,7,12,13,18 | 2 |
| 25 | 1 | 13 | 1 | 25 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 26 | 1 | 12,13 | 1 | 26~255 | Reserved | Reserved | Reserved |
| 27 | 2 | 12 | 1 |  |  |  |  |
| 28 | 2 | 13 | 1 |  |  |  |  |
| 29 | 2 | 14 | 1 |  |  |  |  |
| 30 | 2 | 15 | 1 |  |  |  |  |
| 31 | 2 | 12,13 | 1 |  |  |  |  |
| 32 | 2 | 14,15 | 1 |  |  |  |  |
| 33 | 3 | 12 | 1 |  |  |  |  |
| 34 | 3 | 13 | 1 |  |  |  |  |
| 35 | 3 | 14 | 1 |  |  |  |  |
| 36 | 3 | 15 | 1 |  |  |  |  |
| 37 | 3 | 16 | 1 |  |  |  |  |
| 38 | 3 | 17 | 1 |  |  |  |  |
| 39 | 3 | 12,13 | 1 |  |  |  |  |
| 40 | 3 | 14,15 | 1 |  |  |  |  |
| 41 | 3 | 16,17 | 1 |  |  |  |  |
| 42 | 1 | 0,1,12 | 1 |  |  |  |  |
| 43 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 44 | 2 | 0,1,12 | 1 |  |  |  |  |
| 45 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 46 | 2 | 2,3,14 | 1 |  |  |  |  |
| 47 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 48 | 3 | 0,1,12 | 1 |  |  |  |  |
| 49 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 50 | 3 | 2,3,14 | 1 |  |  |  |  |
| 51 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 52 | 3 | 4,5,16 | 1 |  |  |  |  |
| 53 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| 54 | 3 | 0 | 2 |  |  |  |  |
| 55 | 3 | 1 | 2 |  |  |  |  |
| 56 | 3 | 2 | 2 |  |  |  |  |
| 57 | 3 | 3 | 2 |  |  |  |  |
| 58 | 3 | 4 | 2 |  |  |  |  |
| 59 | 3 | 5 | 2 |  |  |  |  |
| 60 | 3 | 6 | 2 |  |  |  |  |
| 61 | 3 | 7 | 2 |  |  |  |  |
| 62 | 3 | 8 | 2 |  |  |  |  |
| 63 | 3 | 9 | 2 |  |  |  |  |
| 64 | 3 | 10 | 2 |  |  |  |  |
| 65 | 3 | 11 | 2 |  |  |  |  |
| 66 | 3 | 0,1 | 2 |  |  |  |  |
| 67 | 3 | 2,3 | 2 |  |  |  |  |
| 68 | 3 | 4,5 | 2 |  |  |  |  |
| 69 | 3 | 6,7 | 2 |  |  |  |  |
| 70 | 3 | 8,9 | 2 |  |  |  |  |
| 71 | 3 | 10,11 | 2 |  |  |  |  |
| 72 | 3 | 0,1,6 | 2 |  |  |  |  |
| 73 | 3 | 2,3,8 | 2 |  |  |  |  |
| 74 | 3 | 4,5,10 | 2 |  |  |  |  |
| 75 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 76 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 77 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 78 | 1 | 0 | 2 |  |  |  |  |
| 79 | 1 | 1 | 2 |  |  |  |  |
| 80 | 1 | 6 | 2 |  |  |  |  |
| 81 | 1 | 7 | 2 |  |  |  |  |
| 82 | 1 | 0,1 | 2 |  |  |  |  |
| 83 | 1 | 6,7 | 2 |  |  |  |  |
| 84 | 2 | 0,1 | 2 |  |  |  |  |
| 85 | 2 | 2,3 | 2 |  |  |  |  |
| 86 | 2 | 6,7 | 2 |  |  |  |  |
| 87 | 2 | 8,9 | 2 |  |  |  |  |
| 88 | 3 | 12 | 2 |  |  |  |  |
| 89 | 3 | 13 | 2 |  |  |  |  |
| 90 | 3 | 14 | 2 |  |  |  |  |
| 91 | 3 | 15 | 2 |  |  |  |  |
| 92 | 3 | 16 | 2 |  |  |  |  |
| 93 | 3 | 17 | 2 |  |  |  |  |
| 94 | 3 | 18 | 2 |  |  |  |  |
| 95 | 3 | 19 | 2 |  |  |  |  |
| 96 | 3 | 20 | 2 |  |  |  |  |
| 97 | 3 | 21 | 2 |  |  |  |  |
| 98 | 3 | 22 | 2 |  |  |  |  |
| 99 | 3 | 23 | 2 |  |  |  |  |
| 100 | 3 | 12,13 | 2 |  |  |  |  |
| 101 | 3 | 14,15 | 2 |  |  |  |  |
| 102 | 3 | 16,17 | 2 |  |  |  |  |
| 103 | 3 | 18,19 | 2 |  |  |  |  |
| 104 | 3 | 20,21 | 2 |  |  |  |  |
| 105 | 3 | 22,23 | 2 |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 2 | 0,1,12 | 2 |  |  |  |  |
| 117 | 2 | 0,1,12,13 | 2 |  |  |  |  |
| 118 | 2 | 6,7,18 | 2 |  |  |  |  |
| 119 | 2 | 6,7,18,19 | 2 |  |  |  |  |
| 120 | 2 | 2,3,14 | 2 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 2 |  |  |  |  |
| 122 | 2 | 8,9,20 | 2 |  |  |  |  |
| 123 | 2 | 8,9,20,21 | 2 |  |  |  |  |
| 124 | 3 | 0,1,12 | 2 |  |  |  |  |
| 125 | 3 | 0,1,12,13 | 2 |  |  |  |  |
| 126 | 3 | 6,7,18 | 2 |  |  |  |  |
| 127 | 3 | 6,7,18,19 | 2 |  |  |  |  |
| 128 | 3 | 2,3,14 | 2 |  |  |  |  |
| 129 | 3 | 2,3,14,15 | 2 |  |  |  |  |
| 130 | 3 | 8,9,20 | 2 |  |  |  |  |
| 131 | 3 | 8,9,20,21 | 2 |  |  |  |  |
| 132 | 3 | 4,5,16 | 2 |  |  |  |  |
| 133 | 3 | 4,5,16,17 | 2 |  |  |  |  |
| 134 | 3 | 10,11,22 | 2 |  |  |  |  |
| 135 | 3 | 10,11,22,23 | 2 |  |  |  |  |
| 136-255 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-10A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2,
dmrs-TypeEnh is configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,12 | 1 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0-3,12,14 | 1 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0-3,12-14 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0-3,12-15 | 1 |
| 6 | 2 | 3 | 1 | 6 | 3 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 3 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 3 | 0-3,12-14 | 1 |
| 9 | 2 | 0-2 | 1 | 9 | 3 | 0-3,12-15 | 1 |
| 10 | 2 | 0-3 | 1 | 10 | 2 | 0,1,2,3,6 | 2 |
| 11 | 3 | 0 | 1 | 11 | 2 | 0,1,2,3,6,8 | 2 |
| 12 | 3 | 1 | 1 | 12 | 2 | 0,1,2,3,6,7,8 | 2 |
| 13 | 3 | 2 | 1 | 13 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 14 | 3 | 3 | 1 | 14 | 1 | 0,1,6,7,12 | 2 |
| 15 | 3 | 4 | 1 | 15 | 1 | 0,1,6,7,12,18 | 2 |
| 16 | 3 | 5 | 1 | 16 | 1 | 0,1,6,7,12,13,18 | 2 |
| 17 | 3 | 0,1 | 1 | 17 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 18 | 3 | 2,3 | 1 | 18 | 2 | 0,1,6,7,12 | 2 |
| 19 | 3 | 4,5 | 1 | 19 | 2 | 0,1,6,7,12,18 | 2 |
| 20 | 3 | 0-2 | 1 | 20 | 2 | 0,1,6,7,12,13,18 | 2 |
| 21 | 3 | 3-5 | 1 | 21 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 22 | 3 | 0-3 | 1 | 22 | 3 | 0,1,6,7,12 | 2 |
| 23 | 2 | 0,2 | 1 | 23 | 3 | 0,1,6,7,12,18 | 2 |
| 24 | 1 | 12 | 1 | 24 | 3 | 0,1,6,7,12,13,18 | 2 |
| 25 | 1 | 13 | 1 | 25 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 26 | 1 | 12,13 | 1 | 26~255 | Reserved | Reserved | Reserved |
| 27 | 2 | 12 | 1 |  |  |  |  |
| 28 | 2 | 13 | 1 |  |  |  |  |
| 29 | 2 | 14 | 1 |  |  |  |  |
| 30 | 2 | 15 | 1 |  |  |  |  |
| 31 | 2 | 12,13 | 1 |  |  |  |  |
| 32 | 2 | 14,15 | 1 |  |  |  |  |
| 33 | 3 | 12 | 1 |  |  |  |  |
| 34 | 3 | 13 | 1 |  |  |  |  |
| 35 | 3 | 14 | 1 |  |  |  |  |
| 36 | 3 | 15 | 1 |  |  |  |  |
| 37 | 3 | 16 | 1 |  |  |  |  |
| 38 | 3 | 17 | 1 |  |  |  |  |
| 39 | 3 | 12,13 | 1 |  |  |  |  |
| 40 | 3 | 14,15 | 1 |  |  |  |  |
| 41 | 3 | 16,17 | 1 |  |  |  |  |
| 42 | 1 | 0,1,12 | 1 |  |  |  |  |
| 43 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 44 | 2 | 0,1,12 | 1 |  |  |  |  |
| 45 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 46 | 2 | 2,3,14 | 1 |  |  |  |  |
| 47 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 48 | 3 | 0,1,12 | 1 |  |  |  |  |
| 49 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 50 | 3 | 2,3,14 | 1 |  |  |  |  |
| 51 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 52 | 3 | 4,5,16 | 1 |  |  |  |  |
| 53 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| 54 | 3 | 0 | 2 |  |  |  |  |
| 55 | 3 | 1 | 2 |  |  |  |  |
| 56 | 3 | 2 | 2 |  |  |  |  |
| 57 | 3 | 3 | 2 |  |  |  |  |
| 58 | 3 | 4 | 2 |  |  |  |  |
| 59 | 3 | 5 | 2 |  |  |  |  |
| 60 | 3 | 6 | 2 |  |  |  |  |
| 61 | 3 | 7 | 2 |  |  |  |  |
| 62 | 3 | 8 | 2 |  |  |  |  |
| 63 | 3 | 9 | 2 |  |  |  |  |
| 64 | 3 | 10 | 2 |  |  |  |  |
| 65 | 3 | 11 | 2 |  |  |  |  |
| 66 | 3 | 0,1 | 2 |  |  |  |  |
| 67 | 3 | 2,3 | 2 |  |  |  |  |
| 68 | 3 | 4,5 | 2 |  |  |  |  |
| 69 | 3 | 6,7 | 2 |  |  |  |  |
| 70 | 3 | 8,9 | 2 |  |  |  |  |
| 71 | 3 | 10,11 | 2 |  |  |  |  |
| 72 | 3 | 0,1,6 | 2 |  |  |  |  |
| 73 | 3 | 2,3,8 | 2 |  |  |  |  |
| 74 | 3 | 4,5,10 | 2 |  |  |  |  |
| 75 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 76 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 77 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 78 | 1 | 0 | 2 |  |  |  |  |
| 79 | 1 | 1 | 2 |  |  |  |  |
| 80 | 1 | 6 | 2 |  |  |  |  |
| 81 | 1 | 7 | 2 |  |  |  |  |
| 82 | 1 | 0,1 | 2 |  |  |  |  |
| 83 | 1 | 6,7 | 2 |  |  |  |  |
| 84 | 2 | 0,1 | 2 |  |  |  |  |
| 85 | 2 | 2,3 | 2 |  |  |  |  |
| 86 | 2 | 6,7 | 2 |  |  |  |  |
| 87 | 2 | 8,9 | 2 |  |  |  |  |
| 88 | 3 | 12 | 2 |  |  |  |  |
| 89 | 3 | 13 | 2 |  |  |  |  |
| 90 | 3 | 14 | 2 |  |  |  |  |
| 91 | 3 | 15 | 2 |  |  |  |  |
| 92 | 3 | 16 | 2 |  |  |  |  |
| 93 | 3 | 17 | 2 |  |  |  |  |
| 94 | 3 | 18 | 2 |  |  |  |  |
| 95 | 3 | 19 | 2 |  |  |  |  |
| 96 | 3 | 20 | 2 |  |  |  |  |
| 97 | 3 | 21 | 2 |  |  |  |  |
| 98 | 3 | 22 | 2 |  |  |  |  |
| 99 | 3 | 23 | 2 |  |  |  |  |
| 100 | 3 | 12,13 | 2 |  |  |  |  |
| 101 | 3 | 14,15 | 2 |  |  |  |  |
| 102 | 3 | 16,17 | 2 |  |  |  |  |
| 103 | 3 | 18,19 | 2 |  |  |  |  |
| 104 | 3 | 20,21 | 2 |  |  |  |  |
| 105 | 3 | 22,23 | 2 |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 2 | 0,1,12 | 2 |  |  |  |  |
| 117 | 2 | 0,1,12,13 | 2 |  |  |  |  |
| 118 | 2 | 6,7,18 | 2 |  |  |  |  |
| 119 | 2 | 6,7,18,19 | 2 |  |  |  |  |
| 120 | 2 | 2,3,14 | 2 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 2 |  |  |  |  |
| 122 | 2 | 8,9,20 | 2 |  |  |  |  |
| 123 | 2 | 8,9,20,21 | 2 |  |  |  |  |
| 124 | 3 | 0,1,12 | 2 |  |  |  |  |
| 125 | 3 | 0,1,12,13 | 2 |  |  |  |  |
| 126 | 3 | 6,7,18 | 2 |  |  |  |  |
| 127 | 3 | 6,7,18,19 | 2 |  |  |  |  |
| 128 | 3 | 2,3,14 | 2 |  |  |  |  |
| 129 | 3 | 2,3,14,15 | 2 |  |  |  |  |
| 130 | 3 | 8,9,20 | 2 |  |  |  |  |
| 131 | 3 | 8,9,20,21 | 2 |  |  |  |  |
| 132 | 3 | 4,5,16 | 2 |  |  |  |  |
| 133 | 3 | 4,5,16,17 | 2 |  |  |  |  |
| 134 | 3 | 10,11,22 | 2 |  |  |  |  |
| 135 | 3 | 10,11,22,23 | 2 |  |  |  |  |
| 136 | 2 | 0,2,3 | 1 |  |  |  |  |
| 137-255 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-11: TCI selection

| Bit field mapped to index | TCI selection |
| --- | --- |
| 0 | The first indicated joint/DL TCI state is applied to the scheduled PDSCH |
| 1 | The second indicated joint/DL TCI state is applied to the scheduled PDSCH |
| 2 | Both indicated joint/DL TCI states are applied to the scheduled PDSCH |
| 3 | Reserved |

Table 7.3.1.2.2-12: Co-scheduled UE information

| Bit field mapped to index | Co-scheduled UE information |
| --- | --- |
| 0 | In all the PRBs allocated to the UE, there is no co-scheduled UE or there is co-scheduled UE but with a different root DMRS sequence |
| 1 | In all the PRBs allocated to the UE, all the co-scheduled UE(s), if any, which have the same root DMRS sequence as the UE, are scheduled with modulation scheme QPSK |
| 2 | In all the PRBs allocated to the UE, all the co-scheduled UE(s), if any, which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 16QAM |
| 3 | In all the PRBs allocated to the UE, all the co-scheduled UE(s), if any, which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 64QAM |
| 4 | In all the PRBs allocated to the UE, all the co-scheduled UE(s), if any, which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 256QAM |
| 5 | In all the PRBs allocated to the UE, all the co-scheduled UE(s), if any, which have the same root DMRS sequence as the UE, are scheduled with modulation scheme 1024QAM |
| 6 | In each individual PRB allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with the same modulation scheme, except the cases corresponding to index 0~5 |
| 7 | All cases not covered above |
| Note: Root DMRS sequence is rn as defined in clause 7.4.1.1.1 of [4, TS 38.211] |  |

##### 7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format.

-	Carrier indicator - 0, 1, 2 or 3 bits determined by higher layer parameter carrierIndicatorSizeDCI-1-2, as defined in Clause 10.1 of [5, TS38.213]. This field is reserved when this format is carried by PDCCH on the primary cell and the UE is configured for scheduling on the primary cell from an SCell, with the same number of bits as that in this format carried by PDCCH on the SCell for scheduling on the primary cell.

-	Bandwidth part indicator - 0, 1 or 2 bits as determined by the number of DL BWPs nBWP, RRC configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as log2(nBWP) bits, where

-	nBWP=nBWP, RRC+1 if nBWP, RRC≤3, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP=nBWP, RRC, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment - number of bits determined by the following:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS 38.214];

-	log2NRBG, K2NRBG, K2+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K2NRBG, K2+1/2, NRBG+1 bits if resourceAllocationDCI-1-2-r16 is configured as 'dynamicSwitch', where NRBG, K2=NRBDL, BWP+NDL, BWPstartmodK2/K2, NRBDL, BWP is the size of the active DL bandwidth part, NDL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K2 is determined by higher layer parameter resourceAllocationType1GranularityDCI-1-2. If the higher layer parameter resourceAllocationType1GranularityDCI-1-2 is not configured, K2 is equal to 1.

-	If resourceAllocationDCI-1-2-r16 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K2NRBG, K2+1/2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-1-2-r16 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment - 0, 1, 2, 3, or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationListDCI-1-2 if the higher layer parameter is configured, or I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter pdsch-TimeDomainAllocationList is configured when the higher layer parameter pdsch-TimeDomainAllocationListDCI-1-2 is not configured; otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping - 0 or 1 bit:

-	0 bit if the higher layer parameter vrb-ToPRB-InterleaverDCI-1-2 is not configured;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211].

-	PRB bundling size indicator - 0 bit if the higher layer parameter prb-BundlingTypeDCI-1-2 is not configured or is set to 'static', or 1 bit if the higher layer parameter prb-BundlingTypeDCI-1-2 is set to 'dynamic' according to Clause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator - 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1DCI-1-2 and rateMatchPatternGroup2DCI-1-2, where the MSB is used to indicate rateMatchPatternGroup1DCI-1-2 and the LSB is used to indicate rateMatchPatternGroup2DCI-1-2 when there are two groups.

-	ZP CSI-RS trigger - 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as log2(nZP+1) bits, where nZP is the number of aperiodic ZP CSI-RS resource sets configured by higher layer parameter aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2.

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-1-2

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

-	HARQ process number - number of bits determined by the following:

-	0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-2-v1700 if configured;

-	otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-2

-	Downlink assignment index - 0, 1, 2 or 4 bits

-	0 bit if the higher layer parameter downlinkAssignmentIndexDCI-1-2 is not configured;

-	1, 2 or 4 bits determined by higher layer parameter downlinkAssignmentIndexDCI-1-2 otherwise,

-	4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI

-	4 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI.

-	1 or 2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 1 bit or 2 bits are the counter DAI.

If the UE is configured with a PUCCH-SCell, the number of serving cells is determined within a PUCCH group.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-2 is configured, if the bit width of the Downlink assignment index in DCI format 1_2 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_2 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	Second TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213] if higher layer parameter SecondTPCFieldDCI-1-2 is configured; 0 bit otherwise.

-	PUCCH resource indicator - 0 or 1 or 2 or 3 bits determined by higher layer parameter numberOfBitsForPUCCH-ResourceIndicatorDCI-1-2

If higher layer parameter pucch-sSCellPattern or pucch-sSCellDynDCI-1-2 is configured, if the bit width of the PUCCH resource indicator in DCI format 1_2 associated with one cell for PUCCH transmission is not equal to that of the PUCCH resource indicator in DCI format 1_2 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PUCCH resource indicator until the bit width of the PUCCH resource indicator in DCI format 1_2 associated with the two cells for PUCCH transmissions are the same.

If the UE is configured with a PUCCH-SCell, pucch-sSCellPattern is replaced by pucch-sSCellPatternSecondaryPUCCHgroup for the secondary PUCCH group.

-	PDSCH-to-HARQ_feedback timing indicator - 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter DL-DataToUL-ACK-DCI-1-2.

If higher layer parameter priorityIndicatorDCI-1-2 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDynDCI-1-2 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_2 associated with the two cells are the same.

-	One-shot HARQ-ACK request -0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 or pdsch-HARQ-ACK-EnhType3DCI-1-2 is configured;

-	0 bit otherwise.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-EnhType3DCI-Field-1-2 is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-EnhType3ToAddModList.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryToAddModList for the secondary PUCCH group.

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retxDCI-1-2 is configured.

-	0 bit otherwise.

-	Antenna port(s) - 0, 4, 5, 6, 7 or 8 bits

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-1-2 is not configured;

-	Otherwise 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE not configured with dl-OrJointTCI-StateList receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, or when a UE configured with dl-OrJointTCI-StateList is having two indicated TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10.

-	If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and is configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4/7/8/9/10.

-	Transmission configuration indication - 0 bit if higher layer parameter tci-PresentDCI-1-2 is not configured; otherwise 1 or 2 or 3 bits determined by higher layer parameter tci-PresentDCI-1-2 as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentDCI-1-2 is not configured for the CORESET used for the PDCCH carrying the DCI format 1_2,

-	the UE assumes tci-PresentDCI-1-2 is not configured for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentDCI-1-2 is configured for all CORESETs in the indicated bandwidth part with the same value configured for the CORESET used for the PDCCH carrying the DCI format 1_2.

-	TCI selection - 0 bit if higher layer parameter tci-SelectionPresentInDCI is not configured; otherwise 2 bits according to Table 7.3.1.2.2-11.

-	SRS request - 0, 1, 2 or 3 bits

-	0 bit if the higher layer parameter srs-RequestDCI-1-2 is not configured;

-	1 bit as defined by Table 7.3.1.1.3-1 if the higher layer parameter srs-RequestDCI-1-2 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	2 bits if the higher layer parameter srs-RequestDCI-1-2 = 1 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second bit is defined by Table 7.3.1.1.3-1;

-	2 bits as defined by Table 7.3.1.1.2-24 if the higher layer parameter srs-RequestDCI-1-2 = 2 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	3 bits if the higher layer parameter srs-RequestDCI-1-2 = 2 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24;

-	SRS offset indicator - 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	DMRS sequence initialization - 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-1-2 is not configured;

-	1 bit otherwise.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-1-2 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

-	ChannelAccess-CPext - 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-2 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or the number of entries in the high layer parameter ul-AccessConfigListDCI-1-1 for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 are configured by the higher layer parameter ul-AccessConfigListDCI-1-2 in frequency range 1. One or more entries from Table 7.3.1.1.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1 in frequency range 2-2.

-	PUCCH Cell indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDynDCI-1-2 is configured.

-	0 bit otherwise.

If DCI formats 1_2 are monitored in multiple search spaces associated with multiple CORESETs in a BWP for scheduling the same serving cell, zeros shall be appended until the payload size of the DCI formats 1_2 monitored in the multiple search spaces equal to the maximum payload size of the DCI format 1_2 monitored in the multiple search spaces.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 1_2 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 1_2 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 1_2 with smaller size until the payload size is the same.

-	If application of step 4B in clause 7.3.1.0 results in additional zero padding for DCI format 1_2 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 1_2 monitored on the primary cell and DCI format 1_2 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 1_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 1_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.2.3-1: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 3 |

##### 7.3.1.2.4	Format 1_3

DCI format 1_3 is used for the scheduling of one PDSCH in one cell, or multiple PDSCHs in multiple cells with one PDSCH per cell.

The following information is transmitted by means of the DCI format 1_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Scheduled cell set indicator -  log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter mc-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter mc-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter scheduledCellComboListDCI-1-3 for the scheduled cell set is not configured;

-	otherwise log2IDL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.2.4-1, where IDL is the number of entries in the higher layer parameter scheduledCellComboListDCI-1-3. If only one entry is configured in the higher layer parameter scheduledCellComboListDCI-1-3, the scheduled cells are the cells configured by higher layer parameter scheduledCellComboListDCI-1-3.

-	Bandwidth part indicator - 0, 1 or 2 bits determined as log2nBWP, max, where

-	nBWP, max=nBWP,RRCmax+1 if nBWP,RRCmax≤3,  nBWP,RRCmax is the maximum number of DL BWPs configured by higher layers, excluding the initial DL bandwidth part, across all the cells configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set, in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise nBWP, max=nBWP,RRCmax, in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

The field is only applicable to a scheduled cell with the number of configured DL BWPs larger than 1, including the initial DL bandwidth part, and is applied to the applicable scheduled cells in the scheduled cell set independently. If a UE does not support active BWP change via DCI, the UE ignores this bit field. If this field indicates a code point that does not correspond to a configured BWP of a scheduled cell, the UE ignores this bit field for the scheduled cell, and operates on the active BWP of the scheduled cell.

-	Frequency domain resource assignment - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

If scheduledCellComboListDCI-1-3 for the scheduled cell set is configured with more than one entry, NcellDL is the number of scheduled cells indicated by Scheduled cells indicator field; if scheduledCellComboListDCI-1-3 for the scheduled cell set is configured with only one entry, NcellDL is the number of cells configured by higher layer parameter scheduledCellComboListDCI-1-3; otherwise, NcellDL is the number of cells in the scheduled cell set configured by higher layer parameter scheduledCellListDCI-1-3. Each block corresponds to the frequency domain resource assignment for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the frequency domain resource assignment for the cell with the smallest serving cell index. Each block is defined by the following fields:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS 38.214]

-	log2NRBG, K2NRBG, K2+1/2 bits if only resource allocation type 1 is configured, or max log2NRBG, K2NRBG, K2+1/2, NRBG+1 bits if resourceAllocationDCI-1-3 is configured as 'dynamicSwitch', where NRBG, K2=NRBDL, BWP+NDL, BWPstartmodK2/K2, NRBDL, BWP is the size of the active DL bandwidth part, NDL, BWPstart is defined as in clause 4.4.4.4 of [4, TS 38.211] and K2 is given by higher layer parameter resourceAllocationType1GranularityDCI-1-3. If the higher layer parameter resourceAllocationType1GranularityDCI-1-3 is not configured, K2 is equal to 1.

-	If resourceAllocationDCI-1-3 is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2NRBG, K2NRBG, K2+1/2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocationDCI-1-3 is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

If the higher layer parameter scheduledCellComboListDCI-1-3 for the scheduled cell set is not configured, each block is also used to indicate whether the corresponding cell is scheduled or not as follows:

-	if all bits of a block are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type, the cell corresponding to the block is not scheduled;

-	otherwise, the cell corresponding to the block is scheduled.

-	Time domain resource assignment - log2(ITDRA)  bits, where ITDRA is the number of entries in the higher layer parameter tdra-FieldIndexListDCI-1-3. This field is used to indicate an entry in the higher layer parameter tdra-FieldIndexListDCI-1-3 according to Table 7.3.1.2.4-2. Each entry in the higher layer parameter tdra-FieldIndexListDCI-1-3 contains the ‘Time domain resource assignment’ index for each BWP of each cell in the scheduled cell set, where the ‘Time domain resource assignment’ indexes for all the cells are placed according to an ascending order of a serving cell index, and the 'Time domain resource assignment' indexes for all the BWPs of a cell are placed according to an ascending order of the higher layer parameter BWP-Id.

-	VRB-to-PRB mapping - 0 or 1 bit

-	0 bit if the higher layer parameter vrb-ToPRB-Interleaver is not configured for any cell configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211].

The field is only applicable to a scheduled cell configured with vrb-ToPRB-Interleaver, and is applied to the applicable scheduled cells independently.

-	PRB bundling size indicator - 0 or 1 bit

-	0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'staticBundling' for any cell configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set;

-	1 bit according to Clause 5.1.2.3 of [6, TS 38.214] otherwise.

The field is only applicable to a scheduled cell configured with prb-BundlingType set to 'dynamicBundling', and is applied to the applicable scheduled cells independently.

-	Rate matching indicator - log2(IRM) bits, where IRM is the number of entries in the higher layer parameter rateMatchListDCI-1-3, or 0 bit if the higher layer parameter rateMatchListDCI-1-3 is not configured. This field is used to indicate an entry in the higher layer parameter rateMatchListDCI-1-3 according to Table 7.3.1.2.4-3. Each entry in the higher layer parameter rateMatchListDCI-1-3 contains the ‘Rate matching indicator’ index for each cell configured with rateMatchPatternGroup1 or rateMatchPatternGroup2 on at least one DL BWP in the scheduled cell set, where the ‘Rate matching indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘Rate matching indicator’ index is defined by the following:

-	0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger - log2(ICSIRS) bits, where ICSIRS is the number of entries in the higher layer parameter zp-CSI-RSListDCI-1-3, or 0 bit if the higher layer parameter zp-CSI-RSListDCI-1-3 is not configured. This field is used to indicate an entry in the higher layer parameter zp-CSI-RSListDCI-1-3 according to Table 7.3.1.2.4-4. Each entry in the higher layer parameter zp-CSI-RSListDCI-1-3 contains the ‘ZP CSI-RS trigger’ index for each cell configured with aperiodicZP-CSI-RS-ResourceSetsToAddModList on at least one DL BWP in the scheduled cell set, where the ‘ZP CSI-RS trigger’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘ZP CSI-RS trigger’ index is defined by the following:

-	0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as log2(nZP+1) bits, where nZP is the number of aperiodic ZP CSI-RS resource sets configured by higher layer parameter aperiodicZP-CSI-RS-ResourceSetsToAddModList.

For transport block 1:

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the modulation and coding scheme for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the new data indicator for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the redundancy version for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-1-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

For transport block 2:

-	Modulation and coding scheme - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL,3

If scheduledCellComboListDCI-1-3 for the scheduled cell set is configured with more than one entry, NcellDL,3 is the number of scheduled cells indicated by Scheduled cells indicator field and configured with maxNrofCodeWordsScheduledByDCI = 2; if scheduledCellComboListDCI-1-3 for the scheduled cell set is configured with only one entry, NcellDL,3 is the number of cells configured by higher layer parameter scheduledCellComboListDCI-1-3 and configured with maxNrofCodeWordsScheduledByDCI = 2; otherwise, NcellDL,3 is the number of cells configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set and configured with maxNrofCodeWordsScheduledByDCI = 2. Each block corresponds to the modulation and coding scheme for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the modulation and coding scheme for the cell with the smallest serving cell index. Each block is 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL,3

Each block corresponds to the new data indicator for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the new data indicator for the cell with the smallest serving cell index. Each block is 1 bit.

-	Redundancy version - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL,3

Each block corresponds to the redundancy version for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the redundancy version for the cell with the smallest serving cell index. Each block is 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-1-3 configured for the cell corresponding to the block,

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number - number of bits determined by the following:

-	block number 1, block number 2,…, block number NcellDL

Each block corresponds to the HARQ process number for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the HARQ process number for the cell with the smallest serving cell index. Each block is 0, 1, 2, 3, 4 or 5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-1-3 configured for the cell corresponding to the block.

-	Downlink assignment index - number of bits as defined in the following

-	4 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-3 is configured, if the bit width of the Downlink assignment index in DCI format 1_3 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_3 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_3 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dL-DataToUL-ACK.

If higher layer parameter priorityIndicatorDCI-1-3 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDynDCI-1-3 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_3 associated with the two cells are the same.

-	One-shot HARQ-ACK request - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3 or pdsch-HARQ-ACK-enhType3DCI-1-3 is configured;

-	0 bit otherwise.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3 is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-EnhType3ToAddModList.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryList for the secondary PUCCH group.

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-retxDCI-1-3 is configured.

-	0 bit otherwise.

-	Antenna ports - number of bits determined by the following:

-	If antennaPortsDCI-1-3= type1a is configured by higher layer,

-	maxr∈{1,2,…,NcellDL,2}MAr bits applying to the scheduled cells independently, where NcellDL,2 is the number of cells configured by higher layer parameter scheduledCellListDCI-1-3 in the scheduled cell set,  r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and MAr is defined below.

-	If antennaPortsDCI-1-3= type2 is configured by higher layer,

-	block number 1, block number 2,…, block number  NcellDL

Each block corresponds to the Antenna ports information for a cell, and the blocks are placed according to an ascending order of a serving cell index, with block number 1 corresponding to the Antenna ports information for the cell with the smallest serving cell index. Each block is defined below.

MAr above for the case of antennaPortsDCI-1-3= type1A or each block above for the case of antennaPortsDCI-1-3= type2 is defined by the following:

-	4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

-	Transmission configuration indication - number of bits determined by the following:

-	0 bit if higher layer parameter tci-PresentInDCI is not enabled or if higher layer parameter tci-ListDCI-1-3 is not configured;

-	otherwise log2(ITCI) bits, where ITCI is the number of entries in the higher layer parameter tci-ListDCI-1-3. This field is used to indicate an entry in the higher layer parameter tci-ListDCI-1-3 according to Table 7.3.1.2.4-5. Each entry in the higher layer parameter tci-ListDCI-1-3 contains the ‘Transmission configuration indication’ index for each cell in the scheduled cell set, where the ‘Transmission configuration indication’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘Transmission configuration indication’ index is 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_3,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	SRS request - log2(ISRS) bits, where ISRS is the number of entries in the higher layer parameter srs-RequestListDCI-1-3, or 0 bit if the higher layer parameter srs-RequestListDCI-1-3 is not configured. This field is used to indicate an entry in the higher layer parameter srs-RequestListDCI-1-3 according to Table 7.3.1.2.4-6. Each entry in the higher layer parameter srs-RequestListDCI-1-3 contains the ‘SRS request’ index for each cell in the scheduled cell set, where the ‘SRS request’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS request’ index is defined by the following:

-	2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - log2(Ioffset) bits, whereIoffset is the number of entries in the higher layer parameter srs-OffsetListDCI-1-3, or 0 bit if the higher layer parameter srs-OffsetListDCI-1-3 is not configured. This field is used to indicate an entry in the higher layer parameter srs-OffsetListDCI-1-3 according to Table 7.3.1.2.4-7. Each entry in the higher layer parameter srs-OffsetListDCI-1-3 contains the ‘SRS offset indicator’ index for each cell in the scheduled cell set, where the ‘SRS offset indicator’ indexes for all the cells are placed according to an ascending order of a serving cell index. Each ‘SRS offset indicator’ index is defined by the following:

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	DMRS sequence initialization - 1 bit. This field is applied to all the scheduled cells indicated by Scheduled cells indicator field or Frequency domain resource assignment field independently.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-1-3 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	ChannelAccess-CPext - 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0DCI-1-3 is not configured;

-	1 bit otherwise. The 1 bit indication is used to determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 for the active UL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP for each scheduled cell shall be the same as the minimum applicable K0.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyDCI-1-3 or dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4, or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to the last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

If the “One-shot HARQ-ACK request” field is not present or set to '0', and if the “HARQ-ACK retransmission indicator” field is not present or set to ‘0’, and if all bits of the corresponding block(s) of the frequency domain resource assignment field are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type for one or more cells in the scheduled cell set, this field is reserved and the following fields, corresponding to the cell with smallest serving cell index among the one or more cell(s), among the fields above are used for SCell dormancy indication, where each bit corresponds to one of the configured SCell(s), with MSB to LSB of the following fields concatenated in the order below corresponding to the SCell with lowest to highest SCell index

-	Modulation and coding scheme of transport block 1

-	New data indicator of transport block 1

-	Redundancy version of transport block 1

-	HARQ process number

-	Antenna port(s) if antennaPortsDCI1-3= type2 is configured by higher layer.

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	0 bit if higher layer parameter pdcchMonAdaptDCI-1-3 is not enabled;

-	otherwise,

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	PUCCH Cell indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDynDCI-1-3 is configured.

-	0 bit otherwise.

If scheduledCellComboListDCI-1-3 for the cell set is configured, zeros shall be appended to DCI format 1_3 if needed until the payload size equals the size of DCI format 1_3 that is determined by the configuration of the corresponding active bandwidth part(s) of the scheduled cells in the entry which results in the largest size among the entries in the higher layer parameter scheduledCellComboListDCI-1-3.

If an SCell within the scheduled cell set is deactivated and the firstActiveDownlinkBWP-Id corresponding to the SCell is not set to dormant BWP, the UE determines the bitwidth of the fields in DCI format 1_3 based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of an SCell within the scheduled cell set is a dormant DL BWP, or if an SCell within the scheduled cell set is deactivated and the firstActiveDownlinkBWP-Id corresponding to the SCell is set to dormant BWP, the UE determines the bitwidth of the fields in DCI format 1_3 based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.2.4-1: Scheduled cells indicator in DCI format 1_3

| Bit field mapped to index | Scheduled cells |
| --- | --- |
| 0 | The cells configured by the 1st entry in scheduledCellComboListDCI-1-3 |
| 1 | The cells configured by the 2nd entry in scheduledCellComboListDCI-1-3 |
| 2 | The cells configured by the 3rd entry in scheduledCellComboListDCI-1-3, if any |
| 3 | The cells configured by the 4th entry in scheduledCellComboListDCI-1-3, if any |
| 4 | The cells configured by the 5th entry in scheduledCellComboListDCI-1-3, if any |
| 5 | The cells configured by the 6th entry in scheduledCellComboListDCI-1-3, if any |
| 6 | The cells configured by the 7th entry in scheduledCellComboListDCI-1-3, if any |
| 7 | The cells configured by the 8th entry in scheduledCellComboListDCI-1-3, if any |
| 8 | The cells configured by the 9th entry in scheduledCellComboListDCI-1-3, if any |
| 9 | The cells configured by the 10th entry in scheduledCellComboListDCI-1-3, if any |
| 10 | The cells configured by the 11th entry in scheduledCellComboListDCI-1-3, if any |
| 11 | The cells configured by the 12th entry in scheduledCellComboListDCI-1-3 if any |
| 12 | The cells configured by the 13th entry in scheduledCellComboListDCI-1-3, if any |
| 13 | The cells configured by the 14th entry in scheduledCellComboListDCI-1-3, if any |
| 14 | The cells configured by the 15th entry in scheduledCellComboListDCI-1-3, if any |
| 15 | The cells configured by the 16th entry in scheduledCellComboListDCI-1-3, if any |

Table 7.3.1.2.4-2: Time domain resource assignment in DCI format 1_3

| Bit field mapped to index | Indicated time domain resource allocation |
| --- | --- |
| 0 | The 1st entry in tdra-FieldIndexListDCI-1-3 |
| 1 | The 2nd entry in tdra-FieldIndexListDCI-1-3 |
| 2 | The 3rd entry in tdra-FieldIndexListDCI-1-3, if any |
| … | … |
| i | The (i+1)th entry in tdra-FieldIndexListDCI-1-3, if any |
| … | … |

Table 7.3.1.2.4-3: Rate matching indicator

| Bit field mapped to index | Indicated rate matching pattern |
| --- | --- |
| 0 | The 1st entry in rateMatchListDCI-1-3 |
| 1 | The 2nd entry in rateMatchListDCI-1-3 |
| 2 | The 3rd entry in rateMatchListDCI-1-3, if any |
| … | … |
| 15 | The 16th entry in rateMatchListDCI-1-3, if any |

Table 7.3.1.2.4-4: ZP CSI-RS trigger

| Bit field mapped to index | Triggered aperiodic ZP CSI-RS |
| --- | --- |
| 0 | The 1st entry in zp-CSI-RSListDCI-1-3 |
| 1 | The 2nd entry in zp-CSI-RSListDCI-1-3 |
| 2 | The 3rd entry in zp-CSI-RSListDCI-1-3, if any |
| … | … |
| 7 | The 8th entry in zp-CSI-RSListDCI-1-3, if any |

Table 7.3.1.2.4-5: Transmission configuration indication

| Bit field mapped to index | Indicated transmission configuration indication |
| --- | --- |
| 0 | The 1st entry in tci-ListDCI-1-3 |
| 1 | The 2nd entry in tci-ListDCI-1-3 |
| 2 | The 3rd entry in tci-ListDCI-1-3, if any |
| … | … |
| 15 | The 16th entry in tci-ListDCI-1-3, if any |

Table 7.3.1.2.4-6: SRS request in DCI format 1_3

| Bit field mapped to index | Triggered aperiodic SRS resource set(s) |
| --- | --- |
| 0 | The 1st entry in srs-RequestListDCI-1-3 |
| 1 | The 2nd entry in srs-RequestListDCI-1-3 |
| 2 | The 3rd entry in srs-RequestListDCI-1-3, if any |
| … | … |
| 15 | The 16th entry in srs-RequestListDCI-1-3, if any |

Table 7.3.1.2.4-7: SRS offset indicator in DCI format 1_3

| Bit field mapped to index | Available slot offset |
| --- | --- |
| 0 | The 1st entry in srs-OffsetListDCI-1-3 |
| 1 | The 2nd entry in srs-OffsetListDCI-1-3 |
| 2 | The 3rd entry in srs-OffsetListDCI-1-3, if any |
| … | … |
| 7 | The 8th entry in srs-OffsetListDCI-1-3, if any |

#### 7.3.1.3	DCI formats for other purposes

< Unchanged parts are omitted >

##### 7.3.1.3.10	Format 2_9

DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs.

The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE.

If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block:

-	Cell DTX/DRX indication - number of bits determined by the following:

-	If higher layer parameter cellDTX-DRX-L1activation is configured

-	2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigType is configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration;

-	1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigType is configured to either dtx or drx for the associated serving cell of the block;

-	0 bit otherwise.

-	NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise.

The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

< Unchanged parts are omitted >

### 8.3.1	1st-stage SCI formats

The fields defined in each of the 1st-stage SCI formats below are mapped to the information bits a0 to aA-1 as follows:

Each field is mapped in the order in which it appears in the description, with the first field mapped to the lowest order information bit a0 and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to a0.

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority - 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Frequency resource assignment - number of bits determined by the following:

-	If higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured or configured to ‘contiguousRB’

-	log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’

-	X + Y bits provide the frequency domain resource allocation according to Clause 8.1.5 of [6, TS 38.214], where the X MSBs provide the RB set allocation and the Y LSBs provide the sub-channel allocation,

-	the value of X is determined by log2(NRBsetNRBset + 12)  when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2, or determined by log2(NRBsetNRBset + 12NRBset + 16) when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, where NRBset is the number of RB sets in a resource pool

-	the value of Y is determined by log2(N subChannel SLN subChannel SL + 12)  when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2, or determined by log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Time resource assignment - 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Resource reservation period -log2Nrsv_period bits as defined in clause 16.4 of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern - log2Npattern bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], where Npattern is the number of DMRS patterns configured by higher layer parameter sl-PSSCH-DMRS-TimePatternList.

-	2nd-stage SCI format - 2 bits as defined in Table 8.3.1.1-1.

-	Beta_offset indicator - 2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-2.

-	Number of DMRS port - 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme - 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Additional MCS table indicator - as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl-Additional-MCS-Table; 0 bit otherwise.

-	PSFCH overhead indication - 1 bit as defined in clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

-	Reserved - a number of bits as determined by the following:

-	Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter sl-IndicationUE-B is not configured, or if higher layer parameter sl-IndicationUE-B is configured to 'disabled', and if higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured;

-	(Nreserved-2) bits if higher layer parameter sl-IndicationUE-B is configured to 'enabled', and if higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured, with value set to zero.

-	(Nreserved-1) bits otherwise, with value set to zero.

-	COT sharing flag – 0 or 1 bit

-	1 bit if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured;

-	0 bit otherwise.

-	Conflict information receiver flag - 0 or 1 bit

-	1 bit if higher layer parameter sl-IndicationUE-B is configured to 'enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213];

-	0 bit otherwise.

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 | SCI format 2-A |
| 01 | SCI format 2-B; or reserved if higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured |
| 10 | SCI format 2-C; or reserved if higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured and the COT sharing flag field is set to '1' |
| 11 | SCI format 2-D; or reserved if higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured |

Table 8.3.1.1-2: Mapping of Beta_offset indicator values to indexes in Table 9.3-2 of [5, TS38.213]

| Value of Beta_offset indicator | Beta_offset index in Table 9.3-2 of TS 38.213 [5] |
| --- | --- |
| 00 | 1st index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 01 | 2nd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 10 | 3rd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 11 | 4th index provided by higher layer parameter sl-BetaOffsets2ndSCI |

Table 8.3.1.1-3: Number of DMRS port(s)

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

### 8.4.1	2nd-stage SCI formats

< Unchanged parts are omitted >

#### 8.4.1.3	SCI format 2-C

SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast.

The following information is transmitted by means of the SCI format 2-C:

-	HARQ process number - 4 bits

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	Source ID - 8 bits as defined in clause 8.1 of [6, TS 38.214]

-	Destination ID - 16 bits as defined in clause 8.1 of [6, TS 38.214]

-	HARQ feedback enabled/disabled indicator - 1 bit as defined in clause 16.3 of [5, TS 38.213]

-	CSI request - 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214]

-	Providing/Requesting indicator - 1 bit, where value 0 indicates SCI format 2-C is used for providing inter-UE coordination information and value 1 indicates SCI format 2-C is used for requesting inter-UE coordination information

If the 'Providing/Requesting indicator' field is set to 0, all the remaining fields are set as follows:

-	Resource combinations - number of bits determined by the following:

-	If higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured or configured to 'contiguousRB'

-	2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y bits as defined in Clause 8.1.5A of [6, TS 38.214];

-	If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'

-	2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+log2(NRBsetNRBset + 12NRBset + 16)+9+Y bits as defined in Clause 8.1.5A of [6, TS 38.214];

where

-	Y=log2Nrsv_period and Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise.

-	N subChannel SL is provided by the higher layer parameter sl-NumSubchannel as defined in Clause 8.1.5 of [6, TS 38.214].

-	NRBset is the number of RB sets in a resource pool.

-	First resource location - 8 bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	Reference slot location - (10+log2(10∙2μ)) bits as defined in Clause 8.1.5A of [6, TS 38.214], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211].

-	Resource set type - 1 bit, where value 0 indicates preferred resource set and value 1 indicates non-preferred resource set.

-	Lowest subChannel indices - 2∙log2N subChannel SL bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	Lowest RB set indices - 2∙log2NRBset bits as defined in Clause 8.1.5A of [6, TS 38.214] if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'; 0 bit otherwise.

If the 'Providing/Requesting indicator' field is set to 1, all the remaining fields are set as follows:

-	Priority - 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Number of subchannels - log2N subChannel SL bits as defined in Clause 8.1.4A of [6, TS 38.214].

-	Number of RB sets - log2NRBset bits as defined in Clause 8.1.4A of [6, TS 38.214] if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to 'interlaceRB'; 0 bit otherwise.

-	Resource reservation period - log2Nrsv_period bits as defined in Clause 8.1.4A of [6, TS 38.214], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	Resource selection window location - 2∙10+log2(10∙2μ) bits as defined in Clause 8.1.4A of [6, TS 38.214], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211].

-	Resource set type - 1 bit, where value 0 indicates a request for inter-UE coordination information providing preferred resource set and value 1 indicates a request for inter-UE coordination information providing non-preferred resource set, if higher layer parameter sl-DetermineResourceType is configured to 'ueb'; otherwise, 0 bit.

-	Padding bits.

For operation in a same resource pool, zeros shall be appended to SCI format 2-C of which 'Providing/Requesting indicator' field is set to 1 until the payload size equals that of SCI format 2-C of which 'Providing/Requesting indicator' field is set to 0.

#### 8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool.

The following information is transmitted by means of the SCI format 2-D:

-	SL PRS resource ID - log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared SL PRS resource pool and provided by the higher layer parameter sl-PRS-ResourcesSharedSL-PRS-RP.

-	SL PRS request – 1 bit as defined in clause 8.4.4 of [6, TS 38.214] when the higher layer parameter sl-SCI-basedSL-PRS-TxTriggerSCI2-D is provided; 0 bit otherwise.

-	Embedded SCI format - 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1.

-	Embedded SCI format payload - number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

Table 8.4.1.4-1: Embedded SCI format and payload

| Value of the Embedded SCI format field | Embedded SCI format | Embedded SCI format payload |
| --- | --- | --- |
| 00 | SCI format 2-A | Set to all fields included in SCI format 2-A. Padding bits, if necessary, are appended to the 'Embedded SCI format payload' field untill the bitwidth equals the larger payload size of SCI format 2-A and SCI format 2-B. |
| 01 | SCI format 2-B | Set to all fields included in SCI format 2-B. Padding bits, if necessary, are appended to the 'Embedded SCI format payload' field untill the bitwidth equals the larger payload size of SCI format 2-A and SCI format 2-B. |
| 10 | Reserved | Reserved |
| 11 | Reserved | Reserved |

< Unchanged parts are omitted >