3GPP TSG-RAN WG1 Meeting #118-bis 	R1-2409138

Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0629 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TCI activation for mDCI based mTRP in 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, New H3C, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 supports two mDCI (multi-DCI) based mTRP (multi-DCI) modes: - mDCI based intra-cell mTRP: two coresetPoolIndex values are configued and SSB-MTC-AddtionalPCI is not configured; - mDCI based inter-cell mTRP: two coresetPoolIndex values are configued and SSB-MTC-AddtionalPCI is configured. TCI activation rule for mDCI based mTRP is given in 38.214. However, the condition “If the UE is configured with SSB-MTC-AddtionalPCI” makes the rule only applied for mDCI based inter-cell mTRP. This is incorrect as the rule should be applied for both inter-cell and intra-cell case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refine the condition of TCI activation rule for mDCI based mTRP, so that the TCI activation rule can be applied for both mDCI based intra-cell mTRP and mDCI based inter-cell mTRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI activation rules for mDCI apply only to inter-cell mTRP, and not to intra-cell mTRP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.5	Antenna ports quasi co-location

<Unchanged parts are omitted>

If the UE is configured with PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE receives an activation command associated with each coresetPoolIndex, as described in clause 6.1.3.14 of [10, TS 38.321] or 6.1.3.47 of [10, TS 38.321], used to map up to 8 TCI states and/or pairs of TCI states, with one TCI state for DL channels/signals and/or one TCI state for UL channels/signals to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP. When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex is associated with the serving cell physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID, if the UE is further configured with SSB-MTC-AddtionalPCI.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

When a UE configured with dl-OrJointTCI-StateList supports tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18, the UE may receive an activation command, as described in clause 6.1.3.70 of [10, TS 38.321], the activation command is used to map up to 8 sets of TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs, where each set is comprised of up to two TCI state(s) for DL and UL signals/channels.

When a UE configured with dl-OrJointTCI-StateList supports tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18, the UE may receive an activation command, as described in clause 6.1.3.71 of [10, TS 38.321], the activation command is used to map up to 8 sets of TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' for one or for a set of CCs/DL BWPs, and if applicable, for one or for a set of CCs/UL BWPs, where each set is comprised of up to two TCI state(s) for DL channels/signals and up to two TCI state(s) for UL channels/signals.

<Unchanged parts are omitted>