3GPP TSG RAN WG1 #118bis                                                                          R1-2409318

Hefei, China, October 14th - 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0634 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on timeline for determining activated TCI state(s) in unified TCI framework in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE), ZTE Corporation, Sanechips, Ericsson, MediaTek, Samsung, NTT DOCOMO, Google, New H3C, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core, NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the existing specification, it is unclear that which activated TCI states should be used for the determination of the indicated TCI state when activated TCI states are updated by a new MAC CE, e.g., the indicated TCI state should be based on the activated TCI states from old TCI state activation MAC CE or new TCI state activated MAC CE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the indicated TCI-State(s) or TCI-UL-State(s) is/are based on the activated TCI states in each slot where the UE applies the indicated TCI-State(s) or TCI-UL-State(s) to DL channel(s)/signal(s) or UL channel(s)/signal(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear for UE behavior on how to determine the indicated TCI state when activated TCI states are updated by a MAC CE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is not only the shadow CR of the Rel-17 Cat F CR in R1-2409267, but also serves to accommodate the MTRP operation for the unified TCI state extension in Rel-18. With the above considerations, the category of the CR for Rel-18 should be changed from Cat A in R1-2409268 to Cat F. |  |  |  |  |  |  |  |  |

### 5.1.5	Antenna ports quasi co-location

<Unchanged part is omitted>

When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to one or more PDSCH(s) scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI, for the corresponding coresetPoolIndex value when applicable, in time corresponding to positive HARQ-ACK value is applied. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. The indicated TCI-State(s) or TCI-UL-State(s) is/are based on the activated TCI states in each slot where the UE applies the indicated TCI-State(s) or TCI-UL-State(s) to DL channel(s)/signal(s) or UL channel(s)/signal(s).

<Unchanged part is omitted>