3GPP TSG-RAN WG1 Meeting #119	R1-2410814

Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0648 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PDSCH reception scheduled by CORESET not following the unified TCI state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (MediaTek. Inc), ZTE Corporation, Sanechips, Samsung, Ericsson, NTT DOCOMO, OPPO, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification for unified TCI framework, the UE behavior on how to receive PDSCH is clearly defined for the following two cases: - For scheduling offset < timeDurationforQCL, PDSCH reception behaviors are defined in TS 38.214 Clause 5.1.5, regardless of configuration of followUnifiedTCI-State and tci-PresentInDCI/tci-PresentDCI-1-2 - For scheduling offset >= timeDurationforQCL, according to TS 38.213 Clause 10.1, if a PDSCH is scheduled by a CORESET following the unified TCI state, the UE shall apply unified TCI state to the scheduled PDSCH as well However, for the case if scheduling offset >= timeDurationforQCL, and if a PDSCH is scheduled by a CORESET NOT following the unified TCI state, the UE behavior of PDSCH reception is unclear in the specifificaiton. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When a UE is configured with unified TCI framework, clafify in the specification for the case if scheduling offset >= timeDurationforQCL, and if a PDSCH is scheduled by a CORESET NOT following unified TCI state, the scheduled PDSCH reception should follow the unified (indicated) TCI state, regardless of the presence of TCI field in the scheduling DCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When a UE is configured with unified TCI framework, for scheduling offset >= timeDurationforQCL, the UE behavior of PDSCH reception scheduled by a CORESET not following the unified TCI state is unclear in the specifificaiton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of this CR. |  |  |  |  |  |  |  |  |

5.1.5	Antenna ports quasi co-location

<Unchanged parts are omitted>

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State,

-	if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception.

-	if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the indicated TCI state is applied to the PDSCH of the serving cell.

<Unchanged parts are omitted>