3GPP TSG RAN WG1 #119	R1-2410858

Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0063 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on transmission attempt from the 2nd starting symbol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (NTT DOCOMO, INC.), OPPO, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When UE fails to use the first starting symbol due to LBT failure, the second starting symbol is available to perform the failed transmission, after successful channel access. For the detailed behavior, there are two bullets: UE shall attempt to transmit from the second starting symbol when type 1 LBT is performed, and UE may attempt to do that when type 2 LBT is performed. However, UE behavior should be aligned between these cases using type 1 LBT and using type 2 LBT. The ‘may’ for the 2nd bullet is updated to “shall” since transmission should always be attempted regardless of channel access type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the second starting symbol, ‘may’ is updated to ‘shall’ to make UE attempt transmission regardless of channel access type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may skip transmission from the 2nd starting symbol by using type 2 channel access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.5	Sidelink channel access procedures

<Unchanged parts omitted>

For SL transmission(s) with multiple starting positions in a slot, the following are applicable:

-	If a UE intends to transmit PSCCH/PSSCH in sidelink resource allocation mode 1 or mode 2 using a Type 1 SL channel access procedure, and if the UE cannot access the channel for the transmission from the 1st starting symbol of a slot, the UE shall attempt to transmit PSCCH/PSSCH from the 2nd starting symbol in the same slot according to Type 1 SL channel access procedure. There is no limit on the number of attempts the UE can make using Type 1 SL channel access procedure.

-	If a UE intends to transmit PSCCH/PSSCH in sidelink resource allocation mode 1 or mode 2 using a Type 2 SL channel access procedure, and if the UE cannot access the channel for the transmission from the 1st starting symbol of a slot, the UE shall attempt to transmit PSCCH/PSSCH from the 2nd starting symbol in the same slot and according to Type 2A SL channel access procedure.