3GPP TSG RAN WG1 #118bis	R1-2409251

# Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0632 | rev | --- | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI processing criteria of CSI-ReportConfig containing a list of L sub-configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2024-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current definition of Ksi is ambiguous if a CSI-ReportConfig contains a list of L sub-configurations. Current spec does not make it clear whether the number of CSI-RS resources corresponding to the i-th sub-configuration refers to CSI-RS resources for channel measurement or to CSI-RS resources for interference measurement (e.g. when resourcesForChannelMeasurement and nzp-CSI-RS-ResourcesForInterference are both configured). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ksi is the total number of CSI-RS resources in the CSI-RS resource set for channel measurement corresponding to the i-th sub-configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of Ksi corresponding to the i-th sub-configuration is not clear in the spec, i.e. whether it is total number of CSI-RS resources in the CSI-RS resource set for channel measurements, CSI-RS resources for interference measurements, or both the above two sets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.1.6	CSI processing criteria

< Unchanged parts are omitted >

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured

-	OCPU=1  for a CSI report with LTM-CSI-ReportConfig or a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index ' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured)

-	OCPU=Y+1⋅X, for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'tdcp' and with number of delays Y configured by higher layer parameter Y, where the value of X∈{1, 2} is reported by UE capability.

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if max{ µPDCCH, µCSI-RS, µUL} ≤ 3, and if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	if a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to mTRP-CSI-numCPU-r17 and M is defined in clause 5.2.1.4.2,

-	if a CSI-ReportConfig contains a list of L sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList,

-	OCPU=i=1 LKsi for periodic CSI reporting, where Ksi is the total number of CSI-RS resources in the CSI-RS resource set for channel measurement corresponding to the i-th sub-configuration.

-	OCPU=i=1NKsi for aperiodic and semi-persistent CSI reporting, where Ksi is the total number of CSI-RS resources in the CSI-RS resource set for channel measurement corresponding to the i-th sub-configuration, and where the i-th sub-configuration is from N indicated sub-configurations out of L sub-configurations contained in a CSI-ReportConfig, where N≤L and N≥1.

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 1<NTRP≤4 resources, OCPU=ceil(X⋅NTRP), where X∈{1, 1.5, 2} is reported by UE capability indication,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18',

-	if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with K CSI-RS resources,  OCPU=8 for K=12 and OCPU=Y1⋅K for K<12, where Y1∈{1, 2, 3} is reported by UE capability indication,

-	if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with a single CSI-RS resource, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{1, 2, 3} is reported by UE capability indication,

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

< Unchanged parts are omitted >