3GPP TSG RAN meeting #106									RP-242946

Madrid, Spain, December 9th - 12th , 2024

## Status Report to TSG

Agenda item:			9.4.1.6

| WI / SI Name | Introduction of Ku Bands for NR NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ku_bands |  |  |  |  |
| Unique ID | 1040135 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241690 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 30% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Manook Soghomonian |
|  | Company | Intelsat |
|  | Email | Manook.soghomonian@intelsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

Summary

One of the main topics that was identified at the outset of the Ku band normative work was the selection of suitable Numerology, either FR1 or FR2, to cater for the wide-ranging applications and requirements.

To investigate feasible numerology, the following technical criteria were used:

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

Based on the analysis and discussion in [70] and [78], it was concluded that both FR1 numerology and FR2 numerology are feasible, and there are use cases that favour either FR1 numerology or FR2 numerology.

Consequently, it was greed to define two sets of bands, one for FR1 numerology and the other for FR2 numerology and RAN4 will further discuss SCS options.

An LS in [81] was sent to RAN1 and RAN2 cc RAN to check there are no technical consequences for the above numerology decision. A modification to the WID will be submitted to this meeting in R4-242981.

The detailed agreements are provided in the next section.

#### 2.1.1	Agreements

Core part:

RAN WG4 Meeting #112bis, Hefei, China, 14th – 18th Oct, 2024:

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_General in [40] were approved:

1 	WF for NTN Ku band General and work plan

Sub-topic 1-2: Region 3 band definition

Consider the following two bands for the NR NTN Ku band for the studied regulations (i.e. Taiwan, Japan, Australia) in the region 3:

- Band nX: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 14 – 14.5 GHz (Earth-to-Space)

- Band nY: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 13.75 – 14 GHz (Earth-to-Space) with priority for nX.

Sub-topic 1-4: Inclusion of US in WI scope

The US is in scope for the WI using the current allocation of the Ku band.

Sub-topic 1-5: Region 2 band definition

Agree the band 10.7-12.7 GHz (S-to-E) and 14.0-14.5 GHz (E-to-S) for North America.

2	WF for NTN Ku band Coexistence study based on ITU regulations

Sub-topic 2-1: SAN equivalent antenna aperture

Use 9.1 m DL, 7.14 UL (GEO ), 0.91 m DL, 0.714 UL (LEO DL 600 & 1200)

Sub-topic 2-2: SAN antenna pattern

Use section 6.4.1 from 38.811

Sub-topic 2-3: SAN Tx antenna maximum gain

Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200)

Sub-topic 2-4: SAN Rx antenna maximum gain

Use 58.5 dB (GEO), 38.5 dB (LEO 600 & 1200)

Sub-topic 2-5: SAN antenna beamwidth

Use 0.177 (GEO), 1.768 (LEO 600 & 1200)

Sub-topic 2-7: SAN max EIRP

43 dBw/MHz (GEO), 13 dBw/MHz (LEO 1200), 7 dBW/MHz (LEO 600) using 100 MHz bandwidth

Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

Sub-topic 2-9: VSAT equivalent antenna aperture

Use 60 cm

Sub-topic 2-10: VSAT polarization

Assume circular first and if necessary derive the linear result

Sub-topic 2-11: VSAT max Tx antenna gain

Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W)

Sub-topic 2-12: VSAT max Rx antenna gain

34.3 dBi

Sub-topic 2-15: VSAT noise figure

Type 1: 2.5 dB for VSAT LEO and GEO

Type 2: 6 dB for VSAT to LEO

Other options are not precluded

Sub-topic 2-16: VSAT max Tx power

40 dBm (10 W)

Sub-topic 2-21: Channel model

38.811 is the channel model  for satellite used for Ka. It can be scaled for Ku.

For TN use 38.863 and change the frequency to 11 & 14 GHz

Sub-topic 2-22: TN parameters

NTN to NTN won’t be considered.

Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation.

Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

3	WF for NTN Ku band System parameters

Sub-topic 3-4: Phase noise target SNR

Further analysis on the impact of phase noise on FR1/FR2 numerology selection should assume a maximum of 12 dB SNR and maximum 16QAM modulation. This is just for analysis and is not intended to limit support of higher MCS.

Sub-topic 3-5: Efficient use of spectrum for FR1 – native bandwidths

Assume 10 MHz and 25 MHz native bandwidths are included in the analysis of spectrum use efficiency .

Sub-topic 3-12: SCS channel raster

SCS-based channel raster is assumed.

Sub-topic 3-13: Flexible TX Rx spacing

Flexible TX Rx spacing is assumed.

Sub-topic 3-14: Sync raster

The existing sync raster design is reused for Ku band(s).

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [36] were approved:

1	WF for NTN Ku band UE RF

General issues for Ku band UE RF requirements

Issue 1-1-1: The Ku-band UE RF requirements structures

- At least the OTA requirement is considered for the Ku band, FFS on whether the conductive requirement is considered to allow efficient testing.

Issue 1-1-3: Regulations to be considered

- For the NTN Ku band 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S) for region 1

  - Consider the requirements based on ETSI Harmonized Standards EN 301 428, EN 302 448, EN 302 977, EN 303 980, EN 303 981, EN 302 340, EN 301 430 at least

  - The ECC Decision(18)04, ECC Decision(18)05, ECC Decision(19)04, ECC Decision(05)10, ECC Decision(17)04, ECC Decision(03)04, ECC Decision(06)03, ERC Decision(00)08 will also be considered at least

- For the NTN Ku band in other regions:

  - The requirements based on FCC Title 47 Chapter I Subchapter B Part 25 can also be considered.

  - Further investigate other regulations to be covered and the relation to the band plan

Sub-topic 1-2: Specific topics of detailed UE requirements for the NTN Ku band

Issue 1-2-3: Requirements of the antenna performance

- The antenna sidelobe pattern from ITU-R recommendation S.465-6 and S.580-6 will not be specified as a mandatory limit in the specification for the NTN Ku band, however, other regulation might be considered when defining the antenna performance requirement if there is any.

Issue 1-2-4: Off-axis EIRP density limits

- For the UL band 14.0 - 14.5 GHz (E-to-S) for region 1:

  - The off-axis EIRP requirements standardized by the ETSI should be considered for Ku-Band VSAT UE.

    - Fixed VSAT based on EN 301 428

    - Mobile VSAT based on EN 302 340, EN 302 977

Issue 1-2-5: On-axis spurious and off-axis spurious requirement

For the UL band 14.0 - 14.5 GHz (E-to-S) for region 1:

Specify the same on-axis spurious limits in “Emissions disabled” and “Carrier-off” states requirement as Ka band n512

General issues for Ku band SAN RF requirements:

Issue 2-1-1: Supported SAN type for NR NTN Ku band

At least the OTA type of requirements will be specified, FFS on whether conducted type of requirements is considered

RAN WG4 Meeting #113, Orlando, USA, 18th – 22nd Nov, 2024:

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_General in [82] were approved:

1 	WF for NTN Ku band General and work plan

### Sub-topic 1-1: Defining bands by regulatory authority

Agree the following band definition

Table 1: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

2	WF for NTN Ku band Coexistence study based on ITU regulations

Sub-topic 2-1: VSAT antenna pattern

Agree for VSAT antenna model, to progress the co-existence study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel.

Agree to use 38.921 unless there is a need to use sub array then 38.803

The following parameters are agreed:

Table 2 VSAT phased array antenna characteristics

| Ku band VSAT phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ku band VSAT phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 2) | 0 or 3.5 ? |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 4) | 45x45 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.5 lambda |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | [4.44 dBm] TBA |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward X+ axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element).

Note 4:	All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane.

Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration  (Y axis × Z axis).

Agree 3.5 dB element gain

Sub-topic 2-2: SAN noise figure

Agree 3 dB

Sub-topic 2-3: SAN G/T

Agree 30.9 dB/K and 10.9 dB/K based on 3 dB NF

Sub-topic 2-4: VSAT Max Tx antenna gain

Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type.

The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

Sub-topic 2-5: VSAT noise figure

Agreed at previous meeting (see also THALES R4-2419724) to have 2.5 dB NF and 6 dB NF (as we did for Ka-band).

Sub-topic 2-8: SAN antenna pattern

Agreed at previous meeting to use section 6.4.1 from TR 38.811 which corresponds to the same proposal. See also the SAN corresponding antenna beamwidth computed based on this assumption.

Sub-topic 2-9: TN BS/UE antenna patterns for 11 GHz

Agree to use TN BS and UE antenna and beam forming pattern modelling in TS 38.921 for 11GHz carrier frequency for Ku band coexistence study

Sub-topic 2-10: TN BS antenna pattern for 14 GHz

Agree to use TN BS antenna and beam forming pattern modelling in TS 38.922 for 14GHz carrier frequency for Ku band coexistence study. For TN UE, the isotropic antenna model will be used as mentioned in TS 38.922.

Sub-topic 2-17: VSAT max Tx power

Based on sub-topic 2-4, the agreement is 50 W for GSO and 10 W for NGSO.

3	WF for NTN Ku band System parameters

Sub-topic 3-9: FR1 FR2 down selection

Overall observation is that both FR1 numerology and FR2 numerology are feasible.

There are use cases that favour either FR1 numerology or FR2 numerology

Some of the disadvantages of numerology combinations can be mitigated with implementation choices

WF on numerology:

Agree  to define two sets of bands, one for FR1 numerology and the other for FR2 numerology.

RAN4 will further discuss SCS options.

An LS on Ku band numerology to RAN1 and RAN 2 cc RAN was approved in [81]

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [77] were approved:

1	WF for NTN Ku band UE RF

Sub-topic 1-1: General issues for Ku band UE RF requirements

Issue 1-1-2: Antenna Models to be considered

- The 20cm x 20cm, 40cm x 40cm and 60cm x 60cm size of the VSAT antenna models can be considered for the VSAT types with electronic steering antenna in the Ku band normative work

  - Detail parameters can be further checked and FFS on mapping to the 38.101-5 VSAT types and conducting the coexistence study

Issue 1-1-3: Common antenna type vs. separate antenna type

- For full duplex FDD mode under the band plan #1 (DL: 10.7~12.75GHz, UL: 14~14.5GHz), the following can be considered

  - The common Tx/Rx antenna with duplexer can be assumed for mechanical steered antenna type (Parabolic antenna)

  - Usually separate Tx/Rx antenna are used for the phased array antenna type, however there is also new types with common Tx/Rx antenna with duplexer

- Companies are encouraged to provide the details of the latter new types.

Issue 1-1-4: Regulations to be considered

- Aligned with the result in thread [313], RAN4 will work on the following band plans to cover all regions with the concerned regulation at least

  - Downlink 10.70 – 12.75 GHz (Space-to-Earth) and Uplink 14 – 14.5 GHz (Earth-to-Space) concerning requirements from ECC regulation and ETSI standards

  - Downlink 10.70 – 12.7 GHz (Space-to-Earth) and Uplink 14 – 14.5 GHz (Earth-to-Space) concerning FCC requirements.

  - FFS for other agreed band plans

Sub-topic 1-2: General issues for Ku band UE RF requirements

Issue 1-2-2: Maximum EIRP and TRP requirement

- The following aspects will be considered if setting the maximum EIRP & TRP of the NTN Ku band in 38.101-5

  - An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separating consideration for non-GSO scenarios

  - Upper EIRP limits in the regulation

  - Enable flexibility for different applications and scenarios

Issue 1-2-3: Maximum EIRP requirements and the minimum antenna size for the CEPT Ku band

- For the CEPT Ku band, the following requirements from ECC regulation will be considered, FFS on how to reflect to the specifications and the VSAT types.

  - NTN VSAT operating with GSO	14.0 – 14.25 GHz	Max EIRP: 60 dBW (from ECC Decision(17)04)

  - NTN VSAT operating with GSO	14.25 – 14.5 GHz	Max EIRP: 50 dBW (from ECC Decision(03)04)

  - NTN VSAT operating with GSO	14.0 – 14.25 GHz	Max EIRP: 60 dBW (from ECC Decision(06)03)

  - Mobile VSAT / Land ESIM	 14.0 – 14.5 GHz		Max EIRP: 54.5 dBW (from ECC Decision(18)04)

  - Mobile VSAT / Maritime ESIM	  14.0 – 14.5 GHz	minimum antenna size: 0.6 m (from ECC Decision (05)10)

Issue 1-2-4: Off-axis EIRP requirements for the CEPT Ku band

- For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards:

  - Specify the off-axis EIRP density limit based on EN 301 428, EN 302 340 and EN 302 977 as a starting point, the ECC decision can be further checked whether additional update is needed.

Issue 1-2-5: On-axis spurious and off-axis spurious requirement for the CEPT Ku band

- For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards:

  - Specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981 as the starting point

  - Specify the same requirement as Ka band for the general and off-axis spurious limits in “Emissions disabled” state as starting point.

  - FFS on off-axis spurious limits in “Carrier-on” and “Carrier-off” state

Issue 1-2-7: Requirements for the FCC Ku band

- For the Ku band with uplink 14 – 14.5 GHz (Earth-to-Space) concerning FCC requirements, consider the following as the starting point:

  - The current additional spectrum emission mask requirement for n511, n510 can be applied to the Ku band

  - Consider the co-polarization & off-axis cross-polarization gain limit in 38.101-5 based on the Table 1-2-7.1~1-2-7.4 in [77].

  - Consider the off-axis EIRP density limits requirement in 38.101-5 based on the Table 1-2-7.5~1-2-7.7. in [77].

#### Performance part:

Not started.

#### 2.1.2	Remaining Open issues

#### Core part:

#### Define additional bands for Ku

#### Completion of co-existence study

Define System parameters

Based on the agreement of defining two sets of bands, choose supported SCS and channel bandwidths for FR1 numerology and FR2 numerology

Specify RF requirements

SAN RF requirements

UE RF Requirements

Specify RRM requirements

#### Performance part:

- RRM performance requirements and test cases

- UE demodulation and CSI reporting requirements

- Satellite access node demodulation requirements

- Satellite access node conformance tests

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

#### Not applicable.

## 4.	References

Contributions submitted to RAN4#112bis:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [1] | R4-2415079 | Discussion on coexistence evaluations for Ku-band for NR NTN | CATT | noted |
| [2] | R4-2415080 | Discussion on system parameters for Ku-band for NR NTN | CATT | noted |
| [3] | R4-2415081 | Discussion on SAN RF core requirements for Ku-band for NR NTN | CATT | noted |
| [4] | R4-2415276 | Discussions on VSAT parameters for Ku Band coexistence testing | Sharp | noted |
| [5] | R4-2415284 | Feasibility analysis of FR1 and FR2 Numerology | Eutelsat Group | noted |
| [6] | R4-2415379 | Analysis of NTN channel bandwidths for optimal efficiency | Intelsat | noted |
| [7] | R4-2415380 | Regulatory Landscape and Satellite Use Cases in the Ku Band | Intelsat | noted |
| [8] | R4-2415381 | Mobile VSAT Motivation and Market Growth | Intelsat | noted |
| [9] | R4-2415436 | Discussions on system parameters of Ku Band for NR NTN | MediaTek inc. | noted |
| [10] | R4-2415679 | Ku-band VSAT UE RF requirement analysis | SKY Perfect JSAT Corporation | noted |
| [11] | R4-2415680 | Regulation Analysis for Region 3 Ku-band Band Definitions | SKY Perfect JSAT Corporation | noted |
| [12] | R4-2415695 | System parameters for Ku band | SAMSUNG R&D INSTITUTE JAPAN | noted |
| [13] | R4-2415822 | Discussion on coexistence study based on ITU regulations for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [14] | R4-2415823 | Discussion on system parameters for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [15] | R4-2415824 | Discussion on UE RF requirements for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [16] | R4-2415825 | Discussion on SAN RF requirements for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [17] | R4-2415976 | Discussion on NR NTN system parameters for Ku band | Nokia | noted |
| [18] | R4-2415977 | Discussion on NR NTN UE RF requirement for Ku band | Nokia | noted |
| [19] | R4-2415978 | Discussion on NR NTN SAN requirement for Ku band | Nokia | noted |
| [20] | R4-2416013 | NR NTN Ku band plan for region 3 | CHTTL | noted |
| [21] | R4-2416014 | Initial analysis on the regulations for the UE RF requirements of the NR NTN Ku band | CHTTL | noted |
| [22] | R4-2416052 | Draft CR for TR 38.863 to introduce some regulatory information | Huawei, HiSilicon | revised |
| [23] | R4-2416053 | Discussion on VSAT requirements for NR NTN Ku band | Huawei, HiSilicon | noted |
| [24] | R4-2416054 | Discussion on coexistence study for NR NTN Ku Band | Huawei, HiSilicon | noted |
| [25] | R4-2416067 | Discussion on NR NTN Ku Band SCS and CBW | LG Electronics Finland | noted |
| [26] | R4-2416140 | NTN Ku-band - Coexistence | Ericsson | noted |
| [27] | R4-2416141 | NTN Ku-band - FR1-NTN vs FR2-NTN | Ericsson | noted |
| [28] | R4-2416142 | NTN Ku-band - Regulatory aspects and bands definition | Ericsson | noted |
| [29] | R4-2416143 | NTN Ku-band - UE RF requirements | Ericsson | noted |
| [30] | R4-2416298 | On Ku-band coexsitence studies | Samsung | noted |
| [31] | R4-2416299 | On Ku-band UE RF requirements | Samsung | noted |
| [32] | R4-2416491 | Discussion on the Choice of Numerology for Ku-Band | THALES | noted |
| [33] | R4-2416503 | On the VSAT and Satellite Parameters for Ku-band Coexistence | THALES | noted |
| [34] | R4-2416534 | Topic summary for [112bis][313] NR_NTN_Ku_Band_General | Moderator (Eutelsat) | noted |
| [35] | R4-2416535 | Topic summary for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL ) | noted |
| [36] | R4-2416563 | Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | approved |
| [37] | R4-2416567 | Draft CR for TR 38.863 to introduce some regulatory information | Huawei, HiSilicon | endorsed |
| [38] | R4-2416568 | Ad-hoc meeting minutes for [112bis][313] NR_NTN_Ku_Band_General | Eutelsat | noted |
| [39] | R4-2416569 | Way Forward for [112bis][313] NR_NTN_Ku_Band_General | Eutelsat | revised |
| [40] | R4-2416593 | Way Forward for [112bis][313] NR_NTN_Ku_Band_General | Eutelsat | approved |
| [41] | R4-2417544 | Discussions on VSAT antenna patterns | SHARP | noted |

Contributions submitted to RAN4#113:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [42] | R4-2417547 | Ku Band Numerology analysis | Eutelsat Group, European Space Agency, Airbus, Intelsat | noted |
| [43] | R4-2417828 | Further discussion on coexistence evaluations for Ku-band for NR NTN | CATT | noted |
| [44] | R4-2417829 | Further discussion on system parameters for Ku-band for NR NTN | CATT | noted |
| [45] | R4-2417830 | Further discussion on SAN RF core requirements for Ku-band for NR NTN | CATT | noted |
| [46] | R4-2417904 | Discussions on system parameters of Ku Band for NR NTN | MediaTek inc. | noted |
| [47] | R4-2418029 | Ku Band Link Budget Analysis | Intelsat | noted |
| [48] | R4-2418030 | Ku Band: Analysis of the Timing Alignment and Cyclic Prefix | Intelsat | noted |
| [49] | R4-2418031 | Characterisation of Ku Band Mobile VSAT Antennas for GSO and NGSO applications | Intelsat | noted |
| [50] | R4-2418032 | Phase Noise and its Impact on Ku Band Satellites Performance | Intelsat | noted |
| [51] | R4-2418252 | Discussion on NR NTN system parameters for Ku band | Nokia | noted |
| [52] | R4-2418253 | Discussion on NR NTN UE RF requirement for Ku band | Nokia | noted |
| [53] | R4-2418254 | Discussion on NR NTN SAN requirement for Ku band | Nokia | noted |
| [54] | R4-2418364 | Discussion on band definition for NR NTN Ku band | ZTE Corporation, Sanechips | noted |
| [55] | R4-2418365 | Discussion on coexistence study based on ITU regulations for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [56] | R4-2418366 | Discussion on system parameters for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [57] | R4-2418367 | Discussion on UE RF requirements for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [58] | R4-2418368 | Discussion on SAN RF requirements for NTN Ku band | ZTE Corporation, Sanechips | noted |
| [59] | R4-2418835 | Discussion on potential options for the system parameters of NTN Ku band | CHTTL | noted |
| [60] | R4-2418837 | Discussion on UE RF requirements of the NR NTN Ku band for all regions | CHTTL | noted |
| [61] | R4-2418910 | NTN Ku-band - Coexistence - simulation assumptions | Ericsson | noted |
| [62] | R4-2418911 | NTN Ku-band - FR1-NTN vs FR2-NTN | Ericsson | noted |
| [63] | R4-2418912 | NTN Ku-band - UE RF requirements | Ericsson | noted |
| [64] | R4-2419069 | Discussion on VSAT requirements for NR NTN Ku band | Huawei, HiSilicon | noted |
| [65] | R4-2419070 | Discussion on coexistence study for NR NTN Ku Band | Huawei, HiSilicon | noted |
| [66] | R4-2419183 | Discussion on NR NTN Ku-Band system parameters | LG Electronics Finland | noted |
| [67] | R4-2419556 | Discussion on VSAT UE parameter for Ku-band co-existence study | SKY Perfect JSAT Corporation | noted |
| [78] | R4-2419557 | Ku band Efficient channel utilization analysis | SKY Perfect JSAT Corporation | noted |
| [89] | R4-2419558 | Ku band Maximum TRP and EIRP Analysis | SKY Perfect JSAT Corporation | noted |
| [70] | R4-2419600 | Topic summary for [113][313] NR_NTN_Ku_Band_General | Moderator (Eutelsat) | noted |
| [71] | R4-2419601 | Topic summary for [113][314] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL ) | noted |
| [72] | R4-2419625 | System parameters for Ku band | SAMSUNG R&D INSTITUTE JAPAN | noted |
| [73] | R4-2419647 | On Ku-band co-ex study | Samsung | noted |
| [74] | R4-2419648 | On Ku-band UE RF requirements | Samsung | noted |
| [75] | R4-2419724 | Updates on the VSAT and Satellite Parameters for Ku-band Coexistence | THALES | noted |
| [76] | R4-2419727 | Updates for the Discussion on the Choice of Numerology for NTN Ku-band | THALES | noted |
| [77] | R4-2419790 | Way Forward for [113][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | approved |
| [78] | R4-2419800 | Ad-hoc meeting minutes for [113][313] NR_NTN_Ku_Band_General | Eutelsat | noted |
| [79] | R4-2419801 | Way Forward for [113][313] NR_NTN_Ku_Band_General | Eutelsat | revised |
| [80] | R4-2419901 | Way Forward for [112][313] NR_NTN_Ku_Band_General | Moderator (Eutelsat Group) | revised |
| [81] | R4-2419902 | LS on Ku band numerology | Eutelsat | approved |
| [82] | R4-2419909 | Way Forward for [112][313] NR_NTN_Ku_Band_General | Moderator (Eutelsat Group) | approved |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template