3GPP TSG RAN Meeting #106								                                                       RP-242964

Madrid, Spain, December 9-12, 2024

Source: 	CMCC

Title:	Discussion on the check point of Rel-19 Non-Terrestrial Networks (NTN) for NR Phase 3

Agenda item:	9.3.2.2

Document for:	Discussion & Decision

Introduction

In RAN plenary #104 meeting, the WID of Rel-19 Non-Terrestrial Networks (NTN) for NR Phase 3 was updated. RAN1 was assigned to report the list of targeted physical channels/signals for the link level enhancements and system-level enhancements. And RAN1 should report on the backward compatibility issues if there is any [1].

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this contribution, we provide the latest RAN1’s conclusion on the enhancement of link level and system level enhancements, and also our views on the backward compatibility of extended SSB periodicity.

Discussion

Link level enhancements

For the link level enhancements, RAN1 has following conclusions.

Agreement (RAN1#119) For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement (RAN1#119) For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. Agreement (RAN1#118bis) Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3. - The following techniques are for further study: - PDCCH repetition, including: - Option 1: Intra-slot PDCCH repetition - Option 2: Inter-slot PDCCH repetition - CORESET length (i.e. number of OFDM symbols) extension - DCI format optimization (e.g. size reduction, etc) - Note: the same technique is intended to apply to all search space types targeted for link level enhancements - For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques: - Configuration - Backward compatibility and UE behaviour of legacy UE - Linked Search Space - Blocking probability - DCI format size budget - Resource overhead - Impact on CORESET0 - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS Agreement (RAN1#119) For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

RAN1 has concluded that at least PDCCH CSS (except Type-3), PDSCH for SIB1 and Msg4 needs enhancements under the satellite parameter set 1-3 in which scenario the transmit power of the satellite is lower with 8dB compared with the other two parameter sets. In the scenario of parameter set 1-3, the target CNR for NTN-NR DL coverage is assumed to be -8dB which is derived based on the required SINR of PSS. During the discussion, SIB19 was discussed but with no conclusions. But RAN1 can continue the relative discussions in the following working group meetings.

Observation 1:

- For the link level enhancements, PDCCH CSS(except Type 3), PDSCH for SIB1 and Msg4 are supported for further enhancement to solve the coverage problem.

- SIB19 are still under RAN1’s discussion and may be supported in the following meetings.

System level enhancements

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

In RAN1#118bis meeting, it was agreed that at least 160ms is supported for the SSB periodicity as the additional default value apart from the existing 20ms values. Whether 320ms can be supported as the maximum value of the additional default value instead of or in addition to 160ms needs more discussion.

Though the maximum default value of SSB periodicity is extended at least to 160ms, the enhancements to related channels and signals were not thoroughly discussed and concluded. Due to the extended SSB periodicity, the periodicity of the common control channels, e.g. SIB1, SIB19 and also RACH occasions may be impacted. And since the beams of the satellite in this scenario may not always illuminate over one beam footprint in a full-time manner, there could be some periods during which no transmission or reception happened. It will also impact the UE behavior and the assumptions of transmission and receptions, which need more RAN1’s discussion.

Observation 2:

- For the system level enhancement, the maximum of the additional default value is extended to at least 160ms. And whether 320ms can be supported as the maximum of additional default value in addition to or instead of 160ms is still under discussion.

- The enhancements of channels and signals due to the extended periodicity of SSB needs to be introduced but still under RAN1’s discussion.

In RAN1#117 meeting, RAN1 has following observations that only with 320ms SSB periodicity, the coverage ratio can be achieved to 96.8%. If only 160ms is supported, then the coverage ratio can only be 48.4%. Almost half of the beam footprints cannot be covered by the SSB and the SIBs. Considering such a coverage ratio loss, RAN1 needs to further discuss the possibility of extension of the SSB periodicity to 320ms. No RAN plenary action is needed related to 320ms SSB periodicity.

Observation (RAN1#117) Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs. ======other parts are omitted ==============

Observation 3:

With 160ms periodicity of SSBs, the coverage ratio can only be 48.4% in the Set 1-2 scenarios due to the limited simultaneous active beams.

Observation 4:

No RAN plenary action is needed related to 320ms SSB periodicity.

Conclusions

In this contribution, we provide the latest RAN1’s conclusion on the enhancement of link level and system level enhancements, and also our views on the backward compatibility of extended SSB periodicity. The observations and proposals are as below.

Observation 1:

- For the link level enhancements, PDCCH CSS(except Type 3), PDSCH for SIB1 and Msg4 are supported for further enhancement to solve the coverage problem.

- SIB19 are still under RAN1’s discussion and may be supported in the following meetings.

Observation 2:

- For the system level enhancement, the maximum of the additional default value is extended to at least 160ms. And whether 320ms can be supported as the maximum of additional default value in addition to or instead of 160ms is still under discussion.

- The enhancements of channels and signals due to the extended periodicity of SSB needs to be introduced but still under RAN1’s discussion.

Observation 3:

With 160ms periodicity of SSBs, the coverage ratio can only be 48.4% in the Set 1-2 scenarios due to the limited simultaneous active beams.

Observation 4:

No RAN plenary action is needed related to 320ms SSB periodicity.

References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, 3GPP TSG RAN Meeting #104