3GPP TSG RAN Meeting #106	RP-242966

Madrid, Spain, December 9-12, 2024

Agenda item:	9.2.2

Source:	CMCC

Title:	Views on Ambient IoT for Rel-19 and onwards

Document for:	Discussion

# Introduction

The RAN WG SI for Ambient IoT has been completed. And a follow up RAN WG WI for Ambient IoT solutions is expected according to the guidance of the WI outcome. This contribution addresses the scopes that need to be considered in the RAN WG WI.

# Discussion

## Overview of Ambient IoT SI progress

From RAN perspective, the overall progress of Ambient-IoT is 100%. RAN 1/2/3/4 noticed the following respectively,

RAN 1 conclusions

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR.

(1) The following techniques are recommended in the first normative phase:

- PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

- Modulations of OOK-1 and OOK-4 in R2D

- Multiplexing in time domain for R2D

- Convolutional FEC in D2R and no FEC in R2D

- R2D and D2R signal(s)

(2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release:

- Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

- D1T1 and/or D2T2

  - Scenario A1 and/or A2 and/or B and/or C

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D

- OOK and/or BPSK and/or MSK modulation for D2R

- Carrier-wave waveform 1 and/or 2, and transmission case(s)

- Device (un)availability direction 1 and/or 2

- Proximity determination solution 1 and/or 2

RAN 2 conclusions

From RAN2 point of view, the SI is considered complete.

The TR will recommend that RAN2 will have to support paging, RACH, data transmission functionality in MAC.  RAN2 has discussed topology 1 and topology 2 and has concluded it is feasible, from RAN2 perspective. RAN2 will discuss in WI phase any additional impacts from other WGs.

RAN 3 conclusions

Logical system architectures have been studied.

Impacts on CN-RAN interface, signaling and procedures have been studied to support A-IoT Inventory operation and Command operation for topology 1 and topology 2 respectively are studied.

Solutions for locating an Ambient IoT device with minimal specification impact are identified.

No evaluation and down selection for T2 solutions have been performed which requires coordination among WGs.

RAN 4 conclusions

RAN4 has performed adjacent channel co-existence studies between ambient IOT and NR for the scenario that NR macro BS outdoor and ambient IOT system indoor. RAN4 evaluated a total of 9 deployment scenarios as described in Table 6.6.2-1. For each deployment scenario, a total of 4 cases were performed as described in Table 6.6.2-1. The co-existence cases are described in Table 6.6.2-2. The performance metrics for NR system impacted by ambient IOT are throughput loss at the cell edge and cell average. The performance metric for ambient IOT system impacted by NR system is SINR degradation for inter-system interference impact analysis. Meanwhile, in addition to SINR degradation, ambient IOT intra-system performance evaluated by link level simulation is also one of the key aspects affecting ambient IOT system performance, which can be further analyzed in WI phase. The conclusion of co-existence evaluation can be found in section 6.6.5.

RAN4 has studied the RF requirements impact for ambient IOT, including system parameters, ambient IoT BS, intermediate UE, devices (device 1, 2a and 2b) and CW node, which can be found in section 6.7.1, 6.7.2, 6.7.3 and 6.7.4.

RAN4 has also studied the feasibility of CW interference cancellation for both ambient IoT BS and intermediate UE, and the feasible interference cancellation values can be found in section 6.7.5.

Moreover, RAN4 also studied the testability issue for ambient IOT devices, which can be found in section 6.7.4.4.

## Overview of Ambient IoT WI scopes

| Features | Status | Normative phase |  | Justification (refer to section xx) |
| --- | --- | --- | --- | --- |
|  |  | First release (R19) | Second release (R20) |  |
| General aspects |  |  |  |  |
| Deployment scenarios and topology, DxTx-A1/A2/B/C | RAN1/2/3/4 has finished studies for both D1T1 and D2T2. RAN WG identifies A1/A2/B/C scenarios | D1T1-B and D2T2-B, i.e., indoor scenarios | D4T1, D4T2, i.e., outdoor scenarios | 2.3.1 2.3.2 |
| Device 1/2a/2b | All devices are studied. | Device 1/2a with RF-ED [Device 2b Rf-ED] | Device 2b or device C with IF/ZIF | 2.3.5 |
| CW transmission case for D2R backscattering | The followings have been studied: - Case 1-1/1-2/1-4 for topology 1 - Case 2-2/2-3/2-4 for topology 2 | Case 1-4 for topology 1 Case 2-4 for topology 2 | / | 2.3.4 |
| Use case | Rel-19 SI only covers indoor inventory and command use cases | Indoor inventory and command | - Sensor - Positioning (study or down-selection needed) |  |
| Traffic type | The study has focused on DO-DTT and DT. DO-A has been concluded not to be supported in Rel-19. | DO-DTT and DT | DO-A |  |
| Device (un)availability | Direction1 and 2 have been studied but no recommendations. | Direction 1 | Continuation work expected. | 2.3.9 |
| RAN1 related aspects |  |  |  |  |
| R2D Modulations, OOK-1/4 | RAN1 recommends modulations of OOK-1 and OOK-4 in R2D in first release. | Both OOK-1 and OOK-4 | For release 20, OOK is considered as baseline. Further consider improvement if coverage is not satisfied for release 20 outdoor scenario. |  |
| D2R Modulation schemes, e.g., OOK/BPSK/MSK | Many sources provide evaluation results shows that OOK/BPSK can achieve coverage target. Only one source provide coverage evaluation using MSK. | Both OOK and BPSK for D2R | For release 20, BPSK is considered as baseline. Further consider improvement if coverage is not satisfied for release 20 outdoor scenario. |  |
|  | 2SB modulation is feasible for D2R for all devices. Feasibility and necessity of 1SB modulation depends on several issues. | 2SB for D2R | 2SB as baseline. Further consider improvement if coverage is not satisfied in outdoor scenario. |  |
| R2D signals | RAN1 recommends R2D and D2R signal(s). RAN1 has studied R2D/D2R preamble, midamble, postamble with potential purposes involving start indication, timing acquisition/tracking, calibration/frequency synchronization at the device and the reader, channel or interference estimation at the reader and indicating the end of R2D/D2R transmission | R2D preamble including start-indicator part and clock-acquisition part. R2D postamble | R2D preamble and postamble as baseline. Further consider additional R2D signal(s) to improve coverage in outdoor scenario. | 2.3.6 |
| D2R signals |  | D2R preamble D2R midamble D2R postamble | D2R preamble, midamble and postamble as baseline. Further consider additional D2R signal(s) to improve coverage in outdoor scenario. | 2.3.7 |
| R2D/D2R channel | RAN1 recommends PRDCH and PDRCH as respectively the single physical channel for R2D and D2R. | PRDCH for R2D PDRCH for D2R | PRDCH and PDRCH as baseline for R2D and D2R, respectively. Further consider if new physical channel(s) is needed for R2D and/or D2R with initial access enhancement. |  |
| SFO issues | RAN1 has studied the purposes for which clock(s) may be used by different devices 1/2a/2b. For sampling purpose, with < 1uW power consumption, two options (105 ppm and 104 ppm) of accuracy after clock sync have identified. RAN1 has no consensus on whether or not 104 ppm can be achieved. | 105 ppm for device 1 (device 2a/2b can apply the same implementation) 104 ppm for device 2a/2b. | Further consider improvement of the SFO assumptions is needed. |  |
| CFO issues | RAN1 has studied clock purpose for carrier frequency for up/down conversion for device 2b. RAN1 also studied signal for CFO calibration to reduce the guard band for D2R transmission. | / | Further consider improvement on CFO calibration if coverage is not satisfied in outdoor scenario. |  |
| Large frequency shift | RAN1 does not conclude on feasibility of large frequency shift for device 2a. | No LFS for device 2a | / |  |
| Line code D2R | RAN1 has identified Manchester, Miller, and no line coding for D2R. | Both Manchester and Miller. | Further consider improvement if coverage is not satisfied in outdoor scenario. | 2.3.8 |
| Line code R2D | RAN1 has identified Manchester and PIE for R2D. | Both Manchester and PIE. | Further consider improvement if coverage is not satisfied in outdoor scenario. | 2.3.8 |
| Multiple access D2R | RAN1 have studied TDMA, FDMA, CDMA. Whether CDMA is feasible and necessary for all devices is FFS. | TDMA and FDMA | TDMA and FDMA is baseline. Further consider improvement, e.g., CDMA, with larger coverage in outdoor scenario. |  |
| Multiple access R2D | RAN1 recommends multiplexing in time domain for all devices. RAN1 also studied multiplexing in frequency domain for device 2b with IF/ZIF receiver. | TDM | TDM is baseline. FDM is also considered for device with IF/ZIF receiver. |  |
| FEC | RAN1 recommends convolutional FEC in D2R and no FEC in R2D. | Convolutional code for D2R. No FEC for R2D. | Further consider improvement for R2D FEC and/or D2R FEC if coverage is not satisfied in outdoor scenario. |  |
| Carrier-wave waveform | Waveform 1 (without frequency hopping and with frequency hopping) and Waveform 2 have been studied | Waveform 1 (without hopping) as baseline, waveform 2 transparent to device can be supported. | / |  |
| Proximity determination | Solution 1 and Solution 2 have been studied. | Solution 1 | / | 2.3.10 |
| Topology 2 resource allocation | Option 1 and option 2 have been studied. | Minimize RAN1 impact for topology 2 resource allocation. |  | 2.3.2 |
| RAN2 related aspects |  |  |  |  |
| A-IoT Paging | Recommended for normative work. | Specify paging message and procedures, including, inventory and command message, RA type selection, resource allocation, paging identifier. | Corresponding enhancement for supporting DO-A |  |
| A-IoT RACH | Recommended for normative work. | Specify RACH related message, procedures to support 3-step CBRA and CFRA procedures. | Corresponding enhancement for supporting DO-A, e.g., DO-A RACH procedure and configuration. |  |
| A-IoT MAC layer data transmission functionality | Recommended for normative work. | Specify A-IoT MAC layer function and procedures, including R2D&D2R messages, MAC PDU, segmentation. | Corresponding enhancement for supporting DO-A |  |
| Topology 2 specific design | RAN2 has discussed topology resource allocation for RRC ACTIVE/INACTIVE/IDLE. RAN2 deprioritized RRC IDLE for topology 2. | Support topology 2 A-IoT operation in UE RRC CONNECTED only. A-IoT operation in UE RRC INACTIVE/IDLE are not supported. | Continuation work if any | 2.3.2 |
| RAN3 related aspects |  |  |  |  |
| A-IoT RAN architecture | RAN3 has studied the A-IoT protocol stack and logical system architecture. | Specify A-IoT protocol stack for topology 1 and topology 2 | Continuation work if any |  |
| A-IoT inventory and command procedures and signalling | RAN3 has studied the signaling and procedures to support A-IoT Inventory operation and Command operation. | Specify A-IoT inventory and command procedures and signalling between RAN and CN. | Continuation work if any |  |
| Topology 2 options | RAN3 has discussed UP/RRC/NAS solutions. | RRC based solution and UP based solution are recommended for normative work. | Continuation work if any | 2.3.2 |
| RAN4 related aspects |  |  |  |  |
| Deployment scenario | In Rel-19 SI, RAN4 has performed adjacent channel co-existence studies between ambient IOT and NR for the scenario that NR macro BS outdoor and ambient IOT system indoor. The RF requirement for ambient IoT BS is studied based on the assumption that ambient IoT BS only transmits and receives ambient IoT signal. | legacy NR macro BS outdoor, Ambient IOT reader indoor | Ambient IOT reader outdoor | 2.3.1 |
| Spectrum | The followings have been studied: For D1T1, R2D: FDD DL, D2R/CW2D: FDD UL and DL For D1T1, R2D: FDD UL, D2R/CW2D: FDD UL For D2T2, R2D: FDD UL, D2R/CW2D: FDD UL and DL | For D1T1, R2D: FDD DL, D2R/CW2D: FDD UL For D2T2, R2D: FDD UL, D2R/CW2D: FDD UL | Other spectrum usage if necessary, considering the ambient IOT outdoor deployment | 2.3.3 |
| Device type | In Rel-19, the Tx and Rx RF requirements have been studied for all device types | device 1 and 2a | device 2b | 2.3.5 |

## Discussion of Ambient IoT WI scopes

### Deployment scenarios and topology, DxTx-A1/A2/B/C

In Rel-19, RAN4 has studied the co-existence between ambient IOT and NR for the scenario that NR macro BS outdoor and ambient IOT system indoor, and has studied the RF requirement for ambient IoT BS based on the assumption that ambient IoT BS only transmits and receives ambient IoT signal. The main reason is that legacy FDD network deployment is mainly outdoor macro. Hence, we propose to focus on this deployment scenario in Rel-19 WI.

Proposal 1-1: Deployment scenario of legacy NR macro BS outdoor and Ambient IOT reader indoor are recommended for Rel-19 WI.

In Rel-19, RAN4 has studied co-existence for no legacy NR UE indoor and with legacy NR UE indoor. For scenarios that with legacy NR UE indoor, either significant NR throughput degradation or significant SINR degradation for ambient IOT system is observed. It had also been captured in TR that some existing mechanisms based on gNB implementation can be considered to avoid/mitigate the interference. Considering the limited time in Rel-19 normative phase, it is recommended not to consider any optimizations to mitigate the interference.

Proposal 1-2: No optimization to mitigate the interference for the scenario that legacy NR UE indoor in Rel-19 WI.

From RAN1 perspective, for D2R backscattering, RAN1 has studied both A1/A2/B cases including CW node inside and outside topology. Considering the limited time in Rel-19 normative phase, CW node outside topology (case B) is at least considered. Case B is flexible to control the CW node.

Proposal 1-3: CW node outside topology (Case B) is considered for Rel-19 WI.

### Topology 2

- Topology

RAN1/2/3/4 have studied both topology 1 and topology 2 during SI. RAN2 has concluded that both are feasible. RAN 1 also studied schemes for topology resource allocation. Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, we propose both topology 1 and topology 2 are recommended to normative work.

Proposal 1-4: Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, both topology 1 and topology 2 are recommended for normative work.

- Resource allocation for Topology 2

RAN1 have studied two options for resource allocation and/or controlling of the intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE:

- Option 1: Higher-layer signalling is used.

- Option 2: L1 and higher-layer signalling are used.

Due to the limited time in the normative work, for topology 2 resource allocation and/or controlling, RAN1 specification impact should be minimized, and Option 1 is preferred.

Proposal 1-5: For resource allocation and/or controlling of the intermediate UE in topology 2, minimized RAN1 specification impact is pursued.

- A-IoT architecture

RAN2 and RAN3 have studied the architecture and protocol stack for support of Ambient IoT embedded in the overall 5G system architecture for topology 1 and topology 2.

For topology 2, NAS based solution has been ruled out by SA2, so no need to further discuss in RAN WGs. We propose RAN WGs focus on RRC based solution and UP based solution.

Benefit to support RRC based solution for topology 2:

- Topology 1 is based on control plane only. After operator deploy topology 1, it is easier for operator to extend control plane to further support topology 2.

- With RRC based topology 2, gNB can be responsible to select the UE reader. gNB can better select the UE reader than core network, because gNB has the camping beam related information for CONNECTED mode UE readers, with which information, gNB can select the UE readers to perform inventory with less inter-UE interference than core network selection.

- Suitable for 2B market. Better to support server triggered inventory. UE reader is deployed to compensate the coverage hole of topology 1.

Benefit to support user plane based solution for topology 2:

- The specification impact to RAN WGs is minimized.

- Suitable for 2C market. Better to support UE triggered inventory and command.

From the analysis above, we see the benefit for both RRC based and UP based solution and propose to recommend both into normative work.

Proposal 1-6: For topology 2, RRC based solution and UP based solution are recommended for normative work.

- UE state for A-IoT operation

Regarding to the UE state during A-IoT operation, RAN2 has studied the following options and captured in the TR:

- Reader UE performing AIOT operation in RRC_CONNECTED

- Reader UE performing AIOT operation in RRC_INACTIVE

- Reader UE performing AIOT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation

- Reader UE performing AIOT operation in RRC_IDLE (down-prioritized by RAN2)

Analysis on energy consumption: The main motivation to support RRC_INACTIVE mode A-IoT operation is to save UE power. However, the inventory procedure only lasts for a few seconds, and after the completion of A-IoT inventory, the device needs to resume to RRC_CONNECTED mode again. The state transition will cause extra energy consumption. In RRC_CONNECTED mode A-IoT operation, network can configure C-DRX to save UE power.

Observation 1: Considering inventory procedure only lasts for a few seconds, the energy saving gain is limited for suspending intermediate UE to RRC_INACTIVE mode during the short period and then resuming to RRC_CONNECTED mode compring with C-DRX configured in RRC_CONNECTED mode.

Analysis on commercial deployment of RRC_INACTIVE: Even though 5G has been deployed for 5 years, RRC_INACTIVE is still not commercialized in 5G. The reasons are: 1) At early stage of 5G, large amount of UEs don't support RRC_INACTIVE mode. 2) Limited benefit for RRC_INACTIVE mode. RRC_INACTIVE mode has no energy consumption benefit comparing with RRC_IDLE. The only benefit for RRC_INACTIVE is lower state transition delay comparing with RRC_IDLE. However, even Industrial IoT don’t have the requirement of state transition delay, I-IoT only care about the delay for each packet, so the Industrial-IoT are always kept in RRC_CONNECTED mode. The delay requirement for A-IoT is much lower than I-IoT. 3) High complexity for network maintenance. RRC_INACTIVE require network to deploy Ran Notification Area for inactive mode in addition to Tracking Area for idle mode. The network planning of RNA and Xn interface for UE context fetch is much more complex than idle mode. With the limited benefit and high complexity, the RRC_INACTIVE is not widely deployed in the real network.

Observation 2: The limited commercialization of RRC_INACTIVE mode unfortunately reduce the possibility of the deployment of A-IoT operation in RRC_INACTIVE mode, which requires additional specification efforts.

Based on above analysis, we don’t see the urgency to support RRC_INACTIVE mode A-IoT operation in the first release of A-IoT.

Proposal 1-7: for intermediate UE in topology 2 scenario, considering limited power consumption gain for RRC_INACTIVE mode during inventory procedure, and low commercial deployments of RRC_INACTIVE, RRC_INACTIVE mode A-IoT operation for topology 2 is not recommended for normative work in Rel-19.

For A-IoT operation in RRC_IDLE mode, RAN2 has already agree to down-prioritize. In RRC_IDLE mode, the Uu interface and NG interface are both released. Considering the inventory procedure only takes a few seconds, it will take much more time for the intermediate UE to re-setup the Uu and NG interface after the UE has finish the inventory. Therefore, we proposal to stick to RAN2's agreement.

Proposal 1-8: RRC_IDLE mode A-IoT operation is not recommended for normative work in Rel-19.

### Spectrum

In Rel-19 study item, RAN4 peformed co-existence evaluation for the following spectrum usage:

- For D1T1, R2D: FDD DL, CW2D/D2R: FDD UL

- For D1T1, R2D: FDD DL, CW2D/D2R: FDD DL

- For D1T1, R2D: FDD UL, CW2D/D2R: FDD UL

- For D2T2, R2D: FDD UL, CW2D/D2R: FDD UL

- For D2T2, R2D: FDD UL, CW2D/D2R: FDD DL

For D1T1, R2D is transmitted from BS. Following the existing regulation, BS should only transmit in FDD DL spectrum. Hence it is recommended to only consider R2D using FDD DL spectrum for D1T1. For D2T2, only R2D transmitted in FDD UL had been studied. Hence it is recommended to only consider R2D using FDD UL spectrum for D2T2.

For CW2D and D2R, CW outside topology is recommended for Rel-19 WI as discussed in section 2.3.1. According to the CW transmission cases studied in RAN1, CW outside topology only supports CW transmission in FDD DL for topology 1 (i.e. case 1-4). For topology 2, CW outside topology supports CW transmission  using FDD UL or FDD DL (i.e. case 2-3, 2-4), considering the workload in Rel-19, it is recommended only consider CW2D in FDD UL Considering CW outside topology is recommended for Rel-19 WI.

Proposal 1-9: It is recommended to consider following spectrum for Rel-19 WI.

- For D1T1, R2D: FDD DL, CW2D/D2R: FDD UL

- For D2T2, R2D: FDD UL, CW2D/D2R: FDD UL

### CW transmission case

During the SI, the followings have been studied:

- Case 1-1/1-2/1-4 for topology 1

- Case 2-2/2-3/2-4 for topology 2

Considering the spectrum usage, further down-selection can be considered,

- Case 1-4 for topology 1

- Case 2-4 for topology 2

Proposal 1-10: The following CW cases are considered in the Rel-19 WI.

- Case 1-4 for topology 1

- Case 2-4 for topology 2

### Device types

RAN1 has conclusion device 2a RF-ED is prioritized. And device 2a IF/ZIF is deprioritized. For device 1, only RF-ED is considered.

For device 2b, if IF/ZIF is supported in Rel-19 WI, there should be additional specification impact for device to determine the initial access carrier frequency.

For Rel-20 target for outdoor, Device 2b IF/ZIF should be considered for better coverage.

Proposal 1-11: A harmonized solution for device 1 and device 2a for R2D is considered for Rel-19 WI.

Proposal 1-12: For Rel-20 WI, Device 2b with IF/ZIF device architecture for R2D should be considered for better coverage.

### R2D signals

For R2D signals, RAN1 has studied time acquisition signal (i.e., R2D preamble), midamble, and postamle.

For R2D time acquisition signal, RAN1 reached consensus that it is included at least for timing acquisition and for indicating the start of the R2D trasnmission in time domain.

For R2D midamble, RAN1 has concluded that if Manchester encoding is used, without midamble is the baseline. The controversial part is whether midabmle is needed if PIE is applied. Same as Manchester, PIE is also a self-clocking line coding method. RAN1 also agreed that the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. Therefore, midamble is not necessary for R2D transmission.

For R2D postamble, RAN1 has studied the purpose to indicate the termination of the R2D transmission. When compared to the option based on the R2D control information to derive the end of R2D transmission, it is flexible and cost less indication overhead especially in case of large payload size. Therefore, it is benefit to consider R2D postamble in Rel-19 normative work.

Proposal 1-13: For R2D signal(s), R2D time acquisition signal (i.e., R2D preamble) and R2D postamble are recommended for Rel-19 A-IoT normative work.

### D2R signals

For D2R signal(s), RAN1 have studied preamble, midamble and postamble for multiple potential purposes involving timing acquisition/tracking, calibration/frequency synchronization at the reader, channel or interference estimation at the reader, and indicating the end of a D2R transmission.

D2R preamble, midamble and postamble provide important functionalities for coherent detection and satisfactory decoding performance, and should be considered for normative work:

- Timing tracking: For D2R transmission, even with line coding, the reader is difficult to perform edge detection which is very sensitvie to noise and interferencec. In contrast, sample averaging detection is more appropriate to achieve better D2R decoding performance. However, the detection performance highly relies on accurate timing estimation and correction. In this sense, as the SFO at the device side can be as large as 105 ppm, timing tracking and precise SFO estimation based on preamble and midamble/postamble at the reader side is essential for satisfactory D2R decoding performance. By using preamble, the reader can first acquire a coarse SFO estimation (e.g., a residual SFO around 1%). Then, based on the timing peaks of both preamble and midamble/postamble, the reader can further estimate and correct the SFO with residual error less than 0.03%.

- Channel estimation: D2R preamble, midamble and postamble can serve as reference signals for channel estimation. For midamble, especially for large payload size (e.g., 400 bits), the D2R transmission time can last for several hundred milliseconds with a low data rate. In such a case, the transmission duration can be longer than the coherent time (assuming 3 km/h velocity), and hence the channel estimation performance using only preamble and postamble may not be good enough. Inserting several midambles can assist channel estimation and improve the link performance.

Proposal 1-14: For D2R signal(s), D2R preamble, D2R midamble and D2R postamble are recommended for Rel-19 A-IoT normative work.

### Line code

For R2D, RAN1 have studied Manchester encoding and PIE. Based on the study, Manchester encoding and PIE provide different advantages. Manchester encoding achieves better link performance, while PIE has a higher ratio of high to low voltage, which enables it to provide stable instantaneous RF energy to the device. Hence, we propose that both can be recommended in the normative phase.

Proposal 1-15: For R2D line code, both Manchester encoding and PIE are recommended for Rel-19 A-IoT normative work.

For D2R, RAN1 have studied Manchester encoding, Miller encoding, FM0 encoding, and no line coding. RAN1 has concluded that FM0 encoding is deprioritized. In our views, similar to RFID, where multiple D2R line codes are specified, A-IoT can also support both Manchester encoding and Miller encoding, which provide different gains. Manchester encoding provides better link performance, on the other hand, Miller encoding has more concentrated bandwidth, involving less noise and interference and hence can provide benefits in link budget calculation and spectrum usage for D2R multiplexing in the frequency domain.

Proposal 1-16: For D2R line code, both Manchester and Miller are recommended for Rel-19 A-IoT normative work.

### Device (un)availability

RAN1 have studied two directions for potential impact of device unavailability due to energy harvesting:

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

Based on the study, Direction 1 is sufficient for Device 1 and therefore Direction 1 should at least be considered. For Device 2, Direction 2 provides some gain over Direction 1. However, no consensus has been achieved for Device 2 relying on RF-EH only. In this sense, Direction 2 can be de-prioritized in the first release.

Proposal 1-17: Direction 1 of potential impact of device unavailability due to energy harvesting is recommended in the Rel-19 A-IoT normative work.

### Proximity determination

RAN1 have studied two solutions for proximity determination at reader side and concluded both as feasible:

- Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

- Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Solution 1 is purely reader implementation, and no specification impact is expected. On the contrary, solution 2 is dependent on measurements at the reader side, and therefore further discussion and specification work on the measurement may be required. Due to the limited time for Rel-19 normative work, Solution 1 is recommended for Rel-19 A-IoT normative work.

Proposal 1-18: Solution 1 for proximity determination is recommended for Rel-19 A-IoT normative work.

## Timeline

Figure 1. Rel-19/20 Ambient IoT WI timeline

Ambient IoT is an important 5G-A technology covers both indoor and outdoor solutions. Considering Rel-19 SI are mainly focuesed on indoor scenarios, thus the Rel-19 WI mainly focus on indoor scenarios. For outdoor scenarios, which gains interests in the future, this can be considerd in Rel-20 WI. The step-by-step release of specifications for A-IoT effectively supports market utilization.

Meanwhile, during SI, it is concluded that ambient IoT device will not have RRC layer. Hence, RAN1 work does not need to take RRC parameters preparation for RAN2 into consideration, which allows RAN1 and RAN2 finish work at same time.

Proposal 2:

- Rel-19 Ambient WI from 25Q1 to 25Q3 (9 month), mainly focused on indoor scenarios

- Rel-20 Ambient WI from 25Q4 to 26Q4 (15 month), mainly focused on outdoor scenarios

## Recommended scopes

Based on the discussion from previous sections, the following recommended scopes for Rel-19 Ambient IoT WI is proposed,

Proposal 3:

A harmonized Ambient IoT solution towards indoor scenarios including,

- Device 1 and Device 2a (without larger frequency shift)

- Harmonized design with up to 105 ppm SFO for R2D and D2R.

- Deployment scenarios and topology: D1T1-B and D2T2-B

- DO-DTT and DT for indoor inventory and command use cases

Recommended RAN1 scopes

- The following physical layer aspects are recommended for Rel-19 A-IoT normative work:

  - Modulations of OOK-1 and OOK-4 in R2D

  - OOK and BPSK, and 2SB modulations in D2R

  - Manchester encoding and PIE in R2D

  - Manchester encoding and Miller encoding in D2R

  - Convolutional FEC in D2R and no FEC in R2D

  - Multiplexing in time domain for R2D

  - Multiplexing in time domain and frequency domain for D2R

  - PRDCH and PDRCH, as respectively the single physical channel for R2D and D2R

  - R2D time acquisition signal (i.e., preamble) and R2D postamble

  - D2R preamble, D2R midamble and D2R postamble

  - CW waveform 1 (without hopping) as baseline, waveform 2 transparent to device can be supported.

  - Device (un)availablity direction 1

  - Proximity determination solution 1

  - Intermediate UE resource allocation and/or controlling for topology 2 with minimized RAN1 specification impact.

Recommended RAN2 scopes

- Support for “inventory only“ and “inventory and command” cases. Pending on further studies to decide whether “command only” cases are included or not.

- Specify paging related R2D messages and procedures, including

  - •Inventory and command message, device id, device group id or mask

  - RA type selection

  - Resource allocation indication applicable for D2R TDMA /FDMA among different devices.

  - Paging identifier to avoid duplicated response and support device re-access

  - And other necessary messages and procedures if needed.

- Specify RACH related message, procedure and data transmission

  - Specify necessary message / procedure to support of CBRA (i.e., 3-step RA) and CFRA procedure

  - support of NACK response (msg4) during RACH procedure to enable device re-access

- Specify MAC layer protocol

  - Design of AIOT MAC PDU framework, including definitions of R2D and D2R messages

  - Segmentation of D2R message considering D2R buffer-free, in the control of reader’s indication/scheduling information.

  - Note: no RRC / retransmission /mobility is assumed.

- Other Functionality design

  - AS ID, if needed

  - Device population estimation, if needed

  - Potential solution of energy harvesting on device availability for Tx/Rx procedures, if needed

  - Energy status reporting, if needed

  - Message size reporting, if needed

- Topo 2 specific design

  - Uu interface signalling

  - A-IoT UE reader resource allocation

  - UE reader selection, if needed

  - Support topology 2 A-IoT operation in UE RRC CONNECTED only.

Recommended RAN3 scopes

- Specify-RAN architecture aspects for support of A-IoT

- Specify A-IoT inventory and command procedures and signalling between RAN and CN.

- Topo 2 specific design

  - Reader authorizationon

  - UE reader selection;

  - A-IoT radio resource control;

Recommended RAN4 scopes

- Deployment scenario: legacy NR macro BS outdoor, Ambient IOT reader indoor

- R2D spectrum and D2R spectrum assumptions:

  - D1T1, R2D: DL, D2R/CW2D: UL

  - D2T2, R2D: UL, D2R/CW2D: UL

- Specify core requirement including:

  - System parameter

  - RF requirements for: Ambient IOT micro BS, intermediate UE, CW node, prioritize Device 1 and 2a

  - RRM requirements if needed

- Performance part including:

  - Demodulation requirements

  - BS conformance test

  - Study testability for ambient IOT device

Moreover, ambient IoT specification impacts have different physical, higher layer, architecture and RF requirements compared to legacy NR designs. A new specification is expected to describe the ambient IoT solutions.

Proposal 4: New specification TR39.XXX is expected to describe the ambient IoT solutions.

# Conclusion

This contribution discusses on design for Rel-19 Ambient IoT, and provides the following observations and proposals:

Observation 1: Considering inventory procedure only lasts for a few seconds, the energy saving gain is limited for suspending intermediate UE to RRC_INACTIVE mode during the short period and then resuming to RRC_CONNECTED mode compring with C-DRX configured in RRC_CONNECTED mode.

Observation 2: The limited commercialization of RRC_INACTIVE mode unfortunately reduce the possibility of the deployment of A-IoT operation in RRC_INACTIVE mode, which requires additional specification efforts.

Proposal 1-1: Deployment scenario of legacy NR macro BS outdoor and Ambient IOT reader indoor are recommended for Rel-19 WI.

Proposal 1-2: No optimization to mitigate the interference for the scenario that legacy NR UE indoor in Rel-19 WI.

Proposal 1-3: CW node outside topology (Case B) is considered for Rel-19 WI.

Proposal 1-4: Considering topology 1 and topology 2 are targeting to different deployment scenarios both of which are essential, both topology 1 and topology 2 are recommended for normative work.

Proposal 1-5: For resource allocation and/or controlling of the intermediate UE in topology 2, minimized RAN1 specification impact is pursued.

Proposal 1-6: For topology 2, RRC based solution and UP based solution are recommended for normative work.

Proposal 1-7: for intermediate UE in topology 2 scenario, considering limited power consumption gain for RRC_INACTIVE mode during inventory procedure, and low commercial deployments of RRC_INACTIVE, RRC_INACTIVE mode A-IoT operation for topology 2 is not recommended for normative work in Rel-19.

Proposal 1-8: RRC_IDLE mode A-IoT operation is not recommended for normative work in Rel-19.

Proposal 1-9: It is recommended to consider following spectrum for Rel-19 WI.

- For D1T1, R2D: FDD DL, CW2D/D2R: FDD UL

- For D2T2, R2D: FDD UL, CW2D/D2R: FDD UL

Proposal 1-10: The following CW cases are considered in the Rel-19 WI.

- Case 1-4 for topology 1

- Case 2-4 for topology 2

Proposal 1-11: A harmonized solution for device 1 and device 2a for R2D is considered for Rel-19 WI.

Proposal 1-12: For Rel-20 WI, Device 2b with IF/ZIF device architecture for R2D should be considered for better coverage.

Proposal 1-13: For R2D signal(s), R2D time acquisition signal (i.e., R2D preamble) and R2D postamble are recommended for Rel-19 A-IoT normative work.

Proposal 1-14: For D2R signal(s), D2R preamble, D2R midamble and D2R postamble are recommended for Rel-19 A-IoT normative work.

Proposal 1-15: For R2D line code, both Manchester encoding and PIE are recommended for Rel-19 A-IoT normative work.

Proposal 1-16: For D2R line code, both Manchester and Miller are recommended for Rel-19 A-IoT normative work.

Proposal 1-17: Direction 1 of potential impact of device unavailability due to energy harvesting is recommended in the Rel-19 A-IoT normative work.

Proposal 1-18: Solution 1 for proximity determination is recommended for Rel-19 A-IoT normative work.

Proposal 2:

- Rel-19 Ambient WI from 25Q1 to 25Q3 (9 month), mainly focused on indoor scenarios

- Rel-20 Ambient WI from 25Q4 to 26Q4 (15 month), mainly focused on outdoor scenarios

Proposal 3:

A harmonized Ambient IoT solution towards indoor scenarios including,

- Device 1 and Device 2a (without larger frequency shift)

- Harmonized design with up to 105 ppm SFO for R2D and D2R.

- Deployment scenarios and topology: D1T1-B and D2T2-B

- DO-DTT and DT for indoor inventory and command use cases

Recommended RAN1 scopes

- The following physical layer aspects are recommended for Rel-19 A-IoT normative work:

  - Modulations of OOK-1 and OOK-4 in R2D

  - OOK and BPSK, and 2SB modulations in D2R

  - Manchester encoding and PIE in R2D

  - Manchester encoding and Miller encoding in D2R

  - Convolutional FEC in D2R and no FEC in R2D

  - Multiplexing in time domain for R2D

  - Multiplexing in time domain and frequency domain for D2R

  - PRDCH and PDRCH, as respectively the single physical channel for R2D and D2R

  - R2D time acquisition signal (i.e., preamble) and R2D postamble

  - D2R preamble, D2R midamble and D2R postamble

  - CW waveform 1 (without hopping) as baseline, waveform 2 transparent to device can be supported.

  - Device (un)availablity direction 1

  - Proximity determination solution 1

  - Intermediate UE resource allocation and/or controlling for topology 2 with minimized RAN1 specification impact.

Recommended RAN2 scopes

- Support for “inventory only“ and “inventory and command” cases. Pending on further studies to decide whether “command only” cases are included or not.

- Specify paging related R2D messages and procedures, including

  - •Inventory and command message, device id, device group id or mask

  - RA type selection

  - Resource allocation indication applicable for D2R TDMA /FDMA among different devices.

  - Paging identifier to avoid duplicated response and support device re-access

  - And other necessary messages and procedures if needed.

- Specify RACH related message, procedure and data transmission

  - Specify necessary message / procedure to support of CBRA (i.e., 3-step RA) and CFRA procedure

  - support of NACK response (msg4) during RACH procedure to enable device re-access

- Specify MAC layer protocol

  - Design of AIOT MAC PDU framework, including definitions of R2D and D2R messages

  - Segmentation of D2R message considering D2R buffer-free, in the control of reader’s indication/scheduling information.

  - Note: no RRC / retransmission /mobility is assumed.

- Other Functionality design

  - AS ID, if needed

  - Device population estimation, if needed

  - Potential solution of energy harvesting on device availability for Tx/Rx procedures, if needed

  - Energy status reporting, if needed

  - Message size reporting, if needed

- Topo 2 specific design

  - Uu interface signalling

  - A-IoT UE reader resource allocation

  - UE reader selection, if needed

  - Support topology 2 A-IoT operation in UE RRC CONNECTED only.

Recommended RAN3 scopes

- Specify-RAN architecture aspects for support of A-IoT

- Specify A-IoT inventory and command procedures and signalling between RAN and CN.

- Topo 2 specific design

  - Reader authorizationon

  - UE reader selection;

  - A-IoT radio resource control;

Recommended RAN4 scopes

- Deployment scenario: legacy NR macro BS outdoor, Ambient IOT reader indoor

- R2D spectrum and D2R spectrum assumptions:

  - D1T1, R2D: DL, D2R/CW2D: UL

  - D2T2, R2D: UL, D2R/CW2D: UL

- Specify core requirement including:

  - System parameter

  - RF requirements for: Ambient IOT micro BS, intermediate UE, CW node, prioritize Device 1 and 2a

  - RRM requirements if needed

- Performance part including:

  - Demodulation requirements

  - BS conformance test

  - Study testability for ambient IOT device

Proposal 4: New specification TR39.XXX is expected to describe the ambient IoT solutions.

# References

- TR 22.840 Study on Ambient power-enabled Internet of Things (Release 19) V1.2.0, 2023-06; 3GPP SA1, S1-231333

- TR 38.848 Study on Ambient IoT (Internet of Things) in RAN (Release 18) V0.2.0, 2023-06; 3GPP RAN

- RP-240156, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC

- RP-242967	Views on Rel-20 5G-Advanced	CMCC

- RP-242965	Status report for Study on solutions for Ambient IoT (Internet of Things) in NR	CMCC