3GPP TSG Meeting #106	RP-242989

Madrid, EU, 9th to 12th December 2024

Source:	Ericsson

Title:	Motivation for new revision of BS RF evolution WID

Agenda item:	9.3.4.2

Document for:	Information

# Introduction

At the last RAN4 meeting (RAN4#113 in Orlando) the discussions related to Work Item (WI) for Rel-19 BS RF evolution (NR_BS_RF_req_evo) continued with full speed. According to current Work Item Description (WID) [1], the work is split up in several work packages with independent objectives, including improvements of FR1 and FR2 requirements in the BS RF core specification (TS 38.104) and the BS conformance specification (TS 38.141-2). More specifically the WID currently have following objectives:

- Introduction of protection requirement of Fixed Satellite System (FSS) UL within band n104 in TS 38.104 and TS 38.141-2 for BS type 1-H and BS type 1-O.

- OTA test enhancement (This objective consists of 3 parts):

  - Identify excessive BS type 1-O test cases for Transmitter Intermodulation (TX IM) requirement and Receiver out-of-band blocking requirement in TS 38.141-2.

  - Investigate whether the existing requirement and test approach can be improved and if feasible improve co-location requirements and test methods for BS type 1-O in TS 38.104 and TS 38.141-2.

  - Investigate and if possible, improve the description of Total Radiated Power (TRP) test methods applicable for BS type 1-O and BS type 2-O in TS 38.141-2, Annex I.

The outcome for each work package objective from RAN4#113 is captured in separate way-forward contributions in [2, 3, 4].

According to the WI workplan the first objective related to introduction of protection of FSS UL for band n104 was scheduled to end at RAN4#113 in Orlando. The work related to BS RF core specification is finalized and captured in draft CR to TS 38.104 in [5], while more work is needed for BS conformance test specification TS 38.141-2. It is expected that RAN4 will finalize the work at RAN4#114 in Athens.

For the other objectives minor improvements have been identified.

As a consequence, an updated version of the WID is required to capture the new finalization date for the introduction of the protection requirement, improvements of objectives, consider check point from RAN#103 in Maastricht to consider including objective related to BS RF specification improvements and reacting to requests from external groups.

In this contribution we provide a high-level summary of work progress status for the induvial objectives. The intention with this contribution is to further stimulate the work to update the WID to include feedback from the RAN4 discussions and requests from external groups.

# Discussion

Due to the Rel-19 BS RF evolution WI organization with multiple parallel work packages the RAN4 discussion is running in parallel with multiple meeting agendas and multiple discussion threads during RAN4 meetings. All work packages were not started at the same time. Initially, the work package related to introduction of a protection requirement for FSS UL in band n104 was started with highest priority. Later, work related to other objectives was also initiated in parallel.

To make the outcome of the WI successful a clear workplan and structured topic summaries are essential for each discussion thread.

Currently, RAN4 have good progress in many of the defined work packages following defined WID objectives.

As complement to the WI Status Report (SR) more details are provided for each specific objective:

- The work related to the introduction of a new requirement for protection of FSS UL within band n104 in TS 38.104 and TS 38.141-2 is progressing. The requirement is based on a new spatial parameter referred to as Expected EIRP (EEIRP). After extensive discussions (including pre-meeting, on-line sessions, and off-line discussions) the work is progressing. Currently, a draft CR to TR 38.104 have been created in [5]. With this draft CR the requirement can be introduced in BS RF core specification when Rel-19 specifications are created. The work continues to create the corresponding draft CR to TS 38.141-2 and finalize technical background information to be captured in TR 38.908. 

In current WID the finalization date was set to December 2024. To be able to conclude remaining technical open issues related to conformance testing the finalization date is proposed to be moved to March 2025. It is expected that Rel-19 CRs will be created when Rel-19 specifications are available.

- The work related to improvements of co-location requirement and associated test method for BS type 1-O progresses. Several technical aspects related to the impact due to beamforming, array size, alternative Co-Location Reference Antenna (CLTA), etc. have been discussed. Further work is needed before RAN4 can conclude on feasible improvements to be included in specifications. The improvements related to the CLTA concept is essential for bands in upper region of FR1 and new bands above FR1. The outcome of the work will be used to improve the co-location requirement and associated test method and to prepare specifications for future bands allocated above FR1.

- The discussion related to OTA test improvements for TX IM requirement and receiver out-of-band blocking requirement progresses according to workplan. The current test scope for mentioned requirements is excessive and can be reduced maintaining relevance of the test requirement. RAN4 need to decide on principles on how to reduce the test scope without losing relevance of test requirement.

- The discussion related to TRP test method improvements in TS 38.141-2, Annex I and corresponding section is MSR AAS specification have just started. It has been noticed that the objectives in the WID require some fine tuning to clarify what sections of the specification to be improved [3]. The outcome of this work is aimed to facilitate feedback from ETSI harmonized standard to reduce multiple options and alternatives in normative description of TRP test methods in an anechoic chamber test environment.

A revised WID have been created in a separate contribution in [8].

In addition, some new objectives need careful considerations:

- At the RAN#103 meeting in Maastricht a check point was captured in the chairman meeting minutes for including of an objective to improve BS RF specification structure with respect to co-existence and co-location requirements. In current specifications, the requirements are captured in very extensive tables which would require a new entry every time a new band is introduced. These tables are captured in all node specifications. More details can be found in [11].

- RAN and RAN4 have received and Liaison Statement (LS) from ETSI MSG TFES [6] with a request to re-evaluate co-existence transmitter spurious emission requirement levels for BS. RAN4 have acknowledged the LS in [10]. The current requirement levels were originally determined based on old assumptions summarized in contribution [7] presented at RAN4#113. These assumptions were relevant for UTRA operating at 2 GHz before the concept of BS classes was introduced. The re-evaluation would include new aspects such as new bands at higher frequencies and AAS BS to be considered. Proper and relevant requirement levels are essential to guarantee operation of multiple IMT networks in the same geographical area with reasonable complexity and cost.

Further details on how to handle additional work in RAN4 is captured in a separate contribution [9].

Since the work related to EEIRP is expected to soon be finalized, a new objective related to improvement of specification structure and re-evaluation of co-existence requirement levels can be introduced with small increase of Time Unit (TU) allocation to the running Rel-19 WI.

The work to improve the specification structure for co-existence and co-location spurious emission requirements is closely related to the request to re-evaluate requirement limits for co-existence spurious emission requirement levels. If RAN4 discover that requirement limits need to be updated, it would be beneficial to do so at the same time the specification structure is improved.

Eventually, the work related to specification structure improvements and re-evaluation of co-existence requirement levels can be consolidated into one common objective to be included in running Rel-19 BS RF evolution WI.

# Conclusion

In this contribution we present an overview and status for the work included in the Rel-19 BS RF evolution WI. The work included in the BS RF evolution WI to evolve BS RF specifications progresses. Based on the work progress indicated in the SR at this point in time and RAN checkpoint in December and additional request from external groups some minor updates of the WID will be required.

The work package objective related to specification structure improvements should be updated to include the work related to the request from ETSI/TFES to re-evaluate co-existence transmitter spurious emission limits. In this way RAN4 will benefit of synergies with the work related to improving transmitter co-existence spurious emission requirements. If RAN decides not to include new objective in current Rel-19 BS RF evolution WI for specification improvements and re-evaluation of co-existence requirement levels, other means to prepare and respond to external request will be needed.

Proposal: To facilitate effective work in RAN4, we propose to include work related to the request from ETSI/TFES into the ongoing Rel-19 BS RF evolution WI.

# References

[1]	RP-241284, “NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE

[2]	R4-2419827, “Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP”, ZTE

[3]	R4-2419797, “Way Forward for [113][303] NR_BS_RF_Part2_CLTA”, Huawei

[4]	R4-2419828, “Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP”, ZTE

[5]	R4-2419823, “Draft CR to TS 38.104 for introduction of expected EIRP mask core requirement”, Nokia, ZTE, Ericsson, Huawei, Samsung, Qualcomm, CATT, Spark NZ

[6]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[7]	R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson

[8]	RP- 242830, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC

[9]	RP-242941, “On request from ETSI MGS TFES on co-existence requirement limits”, Huawei, HiSilicon, Ericsson, Nokia, NEC, ZTE

[10]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[11]	RP-242767, “On new objective under NR_BS_RF_req_evo”, Nokia, Ericsson, ZTE, Huawei, HiSilicon