3GPP TSG RAN meeting #106										     RP-242996

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			10.3.1

| WI / SI Name | Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_Ph3 |  |  |  |  |
| Unique ID | 1020096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242397 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 50% RAN1: 60% RAN2: 45% RAN3: 25% RAN4: 50% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Chun-Fan (Felix) Tsai |
|  | Company | MediaTek Inc. |
|  | Email | Chun-Fan.tsai@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

###### RAN1#119, Nov’24

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode.

Agreement

For single tone 15kHz SCS Slot-level OCC for NPUSCH format 1, OCC length larger than 2 is not supported.

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, RAN1 studies at least the following options for CDM DMRS with legacy pattern for down-selection:

- Option 1: DMRS symbols are spread before the OCC is applied, e.g. according to the formula:

Where: M is the OCC length, q is the assigned OCC codeword for the UE and  is the reference signal sequence defined in TS36.211 section 10.1.4.1.1

- Option 2: DMRS symbols are not spread before the OCC is applied.

  - Option 2_1: OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2, e.g. according to the formula:

  - Option 2_2: OCC is applied to the complex-valued DMRS symbol used in slot 1 and slot 2. Depending on the OCC codeword, different DMRS sequence is used.

- Option 3: DMRS symbols are not spread and OCC is not applied.

  - Legacy complex-valued DMRS symbol is used in slots corresponding to an OCC codeword of NPUSCH. Different DMRS sequences are used for multiplexed UEs.

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot.

- Note: whether RU length is extended or not after applying OCC is a separate discussion.

Agreement

For support of single-tone OCC for NPUSCH format 1 for connected mode, the parameters that need to be signalled are:

- OCC sequence index

- Enabling of OCC feature

- FFS: whether signaling is explicit or implicit

###### RAN1#118bis, Oct’24

Agreement

At least the following schemes are supported for single-tone:

- For 3.75kHz SCS OCC for NPUSCH format 1:

  - OCC length 2, Symbol-level

    - FFS: DMRS pattern(s)

- For 15kHz SCS OCC for NPUSCH format 1:

  - OCC length 2, Slot-level, CDM DMRS with legacy pattern

    - FFS: CDM details, e.g. with or without spreading

Agreement

For NPRACH transmission, inter-symbol group OCC is not further studied.

Agreement

For support of single-tone OCC for NPUSCH format 1, RAN1 studies:

- The parameters that need to be signalled, considering the following:

  - OCC codeword

  - Enabling of OCC feature

  - FFS: other parameters

- For dynamic grant in RRC CONNECTED, study which and whether any parameters are signalled via DCI and which and whether any parameters are signalled by RRC

- FFS: whether/how to support cases other than dynamic grant in RRC CONNECTED, e.g. Msg3, PUR, CB Msg3 EDT

###### RAN1#118, Aug’24

Agreement

RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity

- UL gaps for synchronization (from Rel-13)

- Gaps around NPRACH occasions

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

Agreement

The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1:

- Option 1: OCC2, Symbol-level, TDM DMRS

- Option 2: OCC2, Symbol-level, CDM DMRS with new pattern

- Option 3: OCC2, Slot-level, TDM DMRS

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 6: OCC4, Symbol-level, CDM DMRS with new pattern

The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1:

- Option 1: OCC2, Symbol-level, TDM DMRS

- Option 3: OCC2, Slot-level, TDM DMRS

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 5: OCC4, Symbol-level, TDM DMRS

- Option 7: OCC4, Slot -level, TDM DMRS

- Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern

Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS.

Note 2: Companies to report DMRS sequence applied.

Agreement

For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

###### RAN1#117, May’24

Agreement

For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

Agreement

Inter-repetition OCC for NPRACH is not studied further in RAN1.

Agreement

- For the time-domain DMRS pattern (including blanked DMRS, if any):

  - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern

  - For 3.75kHz single-tone

    - RAN1 studies

      - Rel-17 DMRS pattern

      - A new DMRS pattern

  - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17

Agreement

The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

###### RAN1#116bis, Apr’24

Agreement

For the NPUSCH evaluation assumptions, update the DMRS configuration, as follows:

| DMRS configuration | For baseline evaluations: OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz For OCC evaluations: Up to proponent | For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Up to proponent |
| --- | --- | --- |

Agreement

At least the following NPRACH OCC schemes are considered by RAN1 for study:

- Intra-symbol group OCC

- Inter-symbol group(s) OCC

- Inter-repetition OCC

Agreement

The study of OCC for NPRACH does not consider NPRACH format 2.

Agreement

The following evaluation assumptions are used for the study of OCC for NPRACH:

|  | Parameter | value |
| --- | --- | --- |
| Scenario | Orbit and elevation angle | GEO at 12.5 degrees; LEO600 at 30 degrees |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |
|  | Power imbalance | Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value |
| Transmitter | NPRACH format | 1 or 0 |
|  | MIMO scheme | SISO |
|  | Number of repetitions (Nrep) | Up to proponent |
|  | OCC length | Up to proponent |
|  | OCC sequence | Up to proponent |
|  | Number of UE | Up to proponent |
|  | Velocity of UE | 3km/h |
|  | Total NPRACH time / frequency resource utilisation | To be reported by proponent. |
| KPI | Target detection probability | 99% |
|  | Target false alarm probability | 0.1% |
|  | SNR operating point | Report SNR where target detection probability and false alarm probability are reached for baseline and OCC schemes |

Agreement

OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS.

Agreement

For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

Agreement

For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study:

- TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs.

  - FFS: Detailed mapping

- CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs.

  - FFS: Detailed mapping

- Other schemes are not precluded, including combinations of the above

Agreement

For the NPUSCH evaluation assumptions, update the frequency error assumption, as follows.

| Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. For GEO, the same frequency error is applied to each subframe of a transport block. For LEO, the same frequency error is applied to each subframe of a segment (if applied in the evaluation). Companies to report their assumption on frequency error across segments. |
| --- | --- |

###### RAN1#116, Feb’24

Agreements on 9.11.4 IoT-NTN uplink capacity/throughput enhancement

Agreement

For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study:

- Time domain OCC where OCC spreads across:

  - Symbol-level

  - Slot-level

  - Repetition-level

  - RV-level

For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study:

- Time domain OCC where OCC spreads across:

  - Symbol-level

  - Slot-level (including Nslot level)

  - Repetition-level

  - RV-level

- Intra-symbol pre-DFT spreading OCC

Agreement

The following evaluation assumptions are used for the study of OCC for NPUSCH format 1:

|  | Parameter | value |  |  |
| --- | --- | --- | --- | --- |
| scenario | orbit | GEO | LEO600 |  |
|  | Elevation angle | 12.5 degree | 30degree |  |
| Channel and impairments | carrier frequency | 2GHz |  |  |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |  |  |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |  |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |  |  |
|  | Power imbalance | Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value |  |  |
| transmitter | SCS | 3.75KHz and 15KHz | 15kHz |  |
|  | Number of tones | Single tone | Single tone and multi tone up to 12 tones |  |
|  | Waveform | DFT-s-OFDM |  |  |
|  | Frequency hopping | w/o frequency hopping |  |  |
|  | MIMO scheme | SISO |  |  |
|  | DMRS configuration | For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |  | For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |
|  | Number of resource unit (NRU) | Up to proponent |  | Up to proponent |
|  | Modulation order (Qm) | Up to proponent |  | Up to proponent |
|  | TBS (ITBS) | Up to proponent |  | Up to proponent |
|  | Number of repetitions (Nrep) | Up to proponent |  |  |
|  | OCC length | Up to 4 |  |  |
|  | OCC sequence | Up to proponent |  |  |
|  | Number of UE | Up to 4 |  |  |
|  | Velocity of UE | 3km/h |  |  |
| receiver | Receiver algorithm | MMSE |  |  |
|  | Channel estimation | Real channel estimation |  |  |
| KPI | SNR at 10% BLER | Report for baseline and OCC schemes |  |  |
|  | Aggregated throughput | Total throughput of up to 4 UEs multiplexed |  |  |

#### 2.1.2	Remaining Open issues

Definition of necessary physical layer features enabling

- IoT-NTN uplink capacity/throughput enhancement

## 2.2	RAN2

#### 2.2.1	Agreements

###### RAN2#128, Nov’24

Support of Store & Forward

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

Support of PWS Reception

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

###### RAN2#127bis, Oct’24

Support of Store & Forward

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

Support of PWS Reception

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

###### RAN2#127, Aug’24

Support of Store & Forward

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

###### RAN2#126, May’24

Support of Store & Forward

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

###### RAN2#125bis, April’24

Support of Store & Forward

RAN2 assumptions:

- S&F implies that at least the full eNB will be onboard

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

3.	The S&F satellite operation is common for NB-IoT and eMTC.

4.	The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

#### 2.2.2	Remaining Open issues

- RAN2 impact due to S&F operation

- Detail procedure for EDT-like transmissions without msg1/ RAR

- Detail procedure for PWS reception for NB-IoT NTN

## 2.3	RAN3

#### 2.3.1	Agreements

###### RAN3#126, Nov’24

N/A

###### RAN3#125, Aug’24

Agreement:

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later

#### 2.3.2	Remaining Open issues

- Potential standard impact of Split MME architecture and Full CN on board architecture

- Potential standard impact of eNB as regenerative payload without S&F satellite operation

## 2.4	RAN4

#### 2.4.1	Agreements

###### RAN4#113, Nov’24

#### Issue 1-1: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with symbol-level OCC2

- Agreement:

  - Keep following the RAN4#112 agreement of “No UE RF requirement impact from symbol-level for NPUSCH”.

#### Issue 1-2: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with slot-level OCC2/OCC4

- Agreement:

  - RAN4 stops evaluating UE RF requirement impacts for slot-level OCC2/OCC4 of 3.75 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove the options.

#### Issue 1-3: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC4

- Agreement:

  - RAN4 stops evaluating UE RF requirement impacts for slot-level OCC4 of 15 kHz single-tone NPUSCH format 1 since RAN1 agreed to remove this option.

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Pending on RAN1 progress of slot-level OCC2 resource mapping.

#### Issue 1-5: Whether there would be UE RF requirement impact when OCC shemes would cross gap in UL transmission.

- Agreement:

  - Whether to study the case that gap between OCC slots may affect RF requirement pending on RAN1.

#### Issue 1-6: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

RF

Issue 1-1-1: RRM impact from CB-msg3 EDT

Agreement:

- Clarify the “N_TA” mentioned in the agreement R4-2416867 refers to “N_TA_Ref” as specified in clause 7.20A.2 and 7.24A.2 of TS 36.133 v18.7.0.

RRM

###### RAN4#112bis, Oct’24

#### Issue 1-1: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with symbol-level OCC4

- Agreement:

  - Encourage companies to reconfirm whether previous agreement would be still valid.

    - RAN4#112 R4-2413530: “Agreement: No UE RF requirement impact from symbol-level for NPUSCH”

#### Issue 1-2: UE RF requirement impact from 3.75 kHz single-tone NPUSCH with slot-level OCC2/OCC4

- Agreement:

  - Wait for further progress in RAN1 before further work on inter-slot OCC, if any.

#### Issue 1-3: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - Encourage companies to study the potential RF impact.

#### Issue 1-4: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC4

- Agreement:

  - Encourage companies to study the potential RF impact.

#### Issue 1-5: UE RF requirement impact from NPRACH with OCC2/OCC4 feature

- Agreement:

  - Wait for further progress in RAN1 before further work on inter-symbol/inter-slot OCC, if any.

RF

RRM

Topic #2: LS reply for R1-2407548

Issue 2-1: Reply regarding the Note 1

Agreement:

- Reply to RAN1 that RAN4 assumed that N_TA is equal to 0, if there is no other procedure for NTA determination introduced by other WG. RAN4 will follow the conclusion of the other working groups in the future, if any.

Issue 2-2: Reply regarding the Note 2

Agreement:

- Confirm to RAN1 that NOTE 2 in R1-2407548 is valid.

Besides, the following LS is approved.

- R4-2416918 Reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

###### RAN4#112, Aug’24

# Topic #1: Work plan for Rel-19 IoT_NTN_Ph3

Issue 1-1: Work plan for Rel-19 IoT_NTN_Ph3

- Approve the workplan in R4-2413529

# Topic #2: RF core requirements

NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH.

#### Issue 2-1: UE RF requirement impact for NPUSCH with OCC feature

- Agreement:

  - No UE RF requirement impact from symbol-level for NPUSCH

  - FFS on UE RF requirement impact from slot-level for NPUSCH

#### Issue 2-2: UE RF requirement impact for NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions on OCC feature before RAN4 evaluates the UE RF impact.

#### Issue 2-3: SAN RF requirement impact for NPUSCH/NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions for OCC feature before RAN4 evaluates the SAN RF impact.

RF

### Sub-Topic 1-1: RRM impact

The work scope of RRM for Rel-19 IoT NTN is to discuss whether and how to define timing requirements for Msg3 transmission without msg1/ Random Access Response (RAR).

- Note: It can be revisited if the RRM impact from other objectives has been identified.

RRM

Besides, the following LS is approved.

- R4-2414114 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

#### 2.4.2	Remaining Open issues

- UE RF and SAN RF requirement impact due to OCC feature

- UE RRM requirements due to Msg3 transmission without msg1/ Random Access Response (RAR).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

List of all related Tdocs in the affected WGs since last TSG.

## 4.1	RAN1

RAN1#118bis, Oct’24:

R1-2407663	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2407724	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum Communications

R1-2407881	Discussion on IoT-NTN uplink capacity enhancement	vivo

R1-2407923	Discussion on the IoT -NTN uplink capacity/throughput enhancements	CMCC

R1-2407936	Discussion on UL capacity enhancement for IoT NTN	ZTE Corporation, Sanechips

R1-2407959	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2408036	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2408138	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2408229	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2408243	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL

R1-2408301	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2408324	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2408346	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2408425	On IoT-NTN uplink capacity enhancements	Sony

R1-2408490	Discussion on IoT-NTN Uplink Capacity Enhancement	Apple

R1-2408529	IoT NTN OCC methods for NPUSCH and NPRACH	Sharp

R1-2408555	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2408581	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2408666	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2408719	IoT-NTN uplink capacity and throughput enhancement	MediaTek Inc.

R1-2408735	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2408870	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2408877	IoT-NTN uplink capacity/throughput enhancement	Nordic Semiconductor ASA

R1-2408944	Operator views on UL Capacity Enhancements for IoT NTN	Inmarsat, Viasat

R1-2409192	FL Summary #1 for IoT-NTN	Moderator (Sony)

R1-2409193	FL Summary #2 for IoT-NTN	Moderator (Sony)

R1-2409309	Final FL Summary for IoT-NTN	Moderator (Sony)

RAN1#119, Nov’24:

R1-2409408	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2409530	Discussion on the IoT -NTN uplink capacity/throughput enhancements	CMCC

R1-2409617	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2409653	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum, UNISOC

R1-2409701	Discussion on IoT-NTN uplink capacity enhancement	vivo

R1-2409722	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2409729	Discussion on UL capacity enhancement for IoT NTN	ZTE Corporation, Sanechips

R1-2409826	Discussion on IoT-NTN Uplink Capacity Enhancement	Apple

R1-2409834	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2409839	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL

R1-2409848	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2409872	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2409886	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2409981	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2410082	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2410161	IoT NTN OCC methods and signaling for NPUSCH and NPRACH	Sharp

R1-2410240	On IoT-NTN uplink capacity enhancements	Sony

R1-2410280	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2410306	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2410365	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2410498	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2410503	Operator views on KPIs for uplink capacity enhancement for IOT- NTN	ViaSat Satellite Holdings Ltd

R1-2410506	IoT-NTN uplink capacity/throughput enhancement	Nordic Semiconductor ASA

R1-2410529	IoT-NTN uplink capacity and throughput enhancement	MediaTek Inc.

R1-2410763	FL Summary #1 for IoT-NTN	Moderator (Sony)

R1-2410764	FL Summary #2 for IoT-NTN	Moderator (Sony)

R1-2410765	Final FL Summary for IoT-NTN	Moderator (Sony)

## 4.2	RAN2

RAN2#127bis, Oct’24:

Organizational

R2-2407920	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 (R1- 2407548; contact: ZTE)	RAN1	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2, RAN4

R2-2407931	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 (R4-2414114; contact: ZTE)	RAN4	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2	Cc:RAN1

R2-2407938	LS on reply to LS on FS_5GSAT_Ph3_ARCH conclusions (S3-243533; contact: Nokia)	SA3	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:SA2	Cc:SA3-LI, RAN2

R2-2408635	Revised work Plan for Rel-19 IoT NTN	MediaTek Inc.	Work Plan	IoT_NTN_Ph3-Core	R2-2402941

R2-2409182	Introduction of IoT NTN phase 3	Ericsson	draftCR	Rel-19	36.300	18.3.0	B	IoT_NTN_Ph3-Core

Support of Store & Forward

R2-2407966	Discussion on RAN2 impacts due to the satellite ID list from MME in S&F operation	CATT	discussion	IoT_NTN_Ph3-Core

R2-2408017	RAN2 Aspect for S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408049	Remaining issues of IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408064	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2408066	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408108	Further consideration on Store and Forward	Huawei, HiSilicon, Turkcell, China Southern Power Grid	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408244	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2407487

R2-2408282	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408303	Access control and information exchange for Store and Forward operation	Lenovo	discussion	Rel-19

R2-2408333	Further consideration on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408360	Discussion on information for Store & Forward	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2406526

R2-2408389	Access Control for Store and Forward Operation	CATT, Huawei, HiSilicon, CMCC, vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408460	Access control and the S&F indication	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408501	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408591	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408620	Discussion on the support of store and forward	Xiaomi	discussion

R2-2408622	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2406821

R2-2408675	Radio Interface Aspect of Store and Forward	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408754	On RAN2 Impacts of SF Operation	Nokia, Nokia Shanghai Bell	discussion

R2-2408802	Discussion on Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408895	Discussion on S&F mode operation	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408905	Considerations on multi-satellite for S&F Satellite operation	NOVAMINT, Sateliot	discussion

R2-2408956	Considerations on Store & Forward Satellite Operation	SHARP Corporation	discussion

R2-2408971	Support of Store & Forward	Sequans Communications	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2407537

R2-2409064	Discussion on Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2409189	Support of Store and Forward	InterDigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Reduce the necessary signaling to complete an Early Data Transmission

R2-2407965	Further consideration on UL capacity enhancements	CATT	discussion	IoT_NTN_Ph3-Core

R2-2408018	Discussion on CB-Msg3 EDT Enhancement	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408050	Support Contention-based Msg3-EDT in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408065	Discussion on uplink capacity enhancement	Transsion Holdings	discussion	Rel-19

R2-2408082	Further discussion on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408109	Discussion on the mechanisms for UL capacity enhancement based on simulation results	Huawei, HiSilicon, CTCC, CATT, Nokia, Nokia Shanghai Bell, Apple, Turkcell, China Southern Power Grid	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408163	Uplink Capacity Enhancement for EDT transaction	Spreadtrum Communications	discussion	Rel-19

R2-2408304	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2408334	Comparison of solutions for UL capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408413	Consideration on UL capacity enhancement for IoT-NTN	NEC Corporation.	discussion	Rel-18	IoT_NTN_Ph3-Core

R2-2408466	Discussion on uplink capacity enhancements for IOT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408502	Discussion on enhanced EDT for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408545	Repetitions and Delay Considerations about SA and DSA	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot, Toyota ITC	discussion	Rel-19

=> Withdrawn

R2-2408547	Repetitions and Delay Considerations about SA and DSA	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot, Toyota ITC	discussion	Rel-19

R2-2408590	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408623	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2406869

R2-2408803	Procedures for uplink capacity enhancements for IoT NTN	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408831	Discussion on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408863	Discussion on throughput and delay performance of slotted ALOHA and diversity slotted ALOHA 	DLR, ESA	discussion	Rel-19

R2-2408896	Discussion on EDT enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409170	Operator views on EDT enhancements and CRDSA	Inmarsat, Viasat	discussion	Rel-19	Late

R2-2409181	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409190	EDT/PUR enhancements	InterDigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Support of PWS Reception

R2-2408019	Discussion on the Support of PWS in IoT-NTN	vivo, Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408051	Support PWS in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408083	Discussion on broadcast of PWS messages for NB-IoT	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408110	Support of PWS for NB-IoT	Huawei, HiSilicon, Turkcell, China Southern Power Grid	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408305	PWS broadcast support for NB-IoT in NTN	Lenovo	discussion	Rel-19

R2-2408335	PWS support for NB-IoT over NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408412	Consideration on PWS broadcast for NB-IoT	NEC Corporation.	discussion	Rel-18	IoT_NTN_Ph3-Core

R2-2408621	Discussion one the support of broadcast of PWS for NB-IoT	Xiaomi	discussion

R2-2408624	Discussion on supporting PWS for NB-IoT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core

R2-2408804	Impact of PWS signalling for NB-IoT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408826	Discussion on Emergency Broadcast for NB-IoT	Inmarsat, Viasat	discussion	Rel-19

R2-2408832	Support of PWS for NB-IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408897	Discussion on PWS in NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2408998	Discussion on broadcast of PWS message for NB-IoT	KT Corp.	discussion

R2-2409191	Support of PWS	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

RAN2#128, Nov’24:

Organizational

R2-2409524	Reply LS on Support of Regenerative-based Satellite Access (S2-2410918; contact: Qualcomm)	SA2	LS in	Rel-19	NR_NTN_Ph3-Core, 5GSAT_Ph3_ARCH	To:RAN3, RAN	Cc:RAN2

R2-2409529	Reply LS on FS_5GSAT_Ph3_ARCH conclusions (S2-2411250; contact: Sateliot)	SA2	LS in	Rel-19	5GSAT_Ph3_ARCH	To:SA3	Cc:RAN2, SA3-LI

R2-2409530	Reply LS on FS_5GSAT_Ph3_ARCH conclusions (s3i240703; contact: Tridea Works)	SA3-LI	LS in	Rel-18	5GSAT_Ph3_ARCH	To:SA2	Cc:RAN2, SA3

R2-2410813	RRC Runing CR for IoT NTN	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.3.1	B	IoT_NTN_Ph3-Core

R2-2410883	Draft Introduction of IoT NTN phase 3	Ericsson	draftCR	Rel-19	36.300	18.3.0	B	IoT_NTN_Ph3-Core

Support of Store & Forward

R2-2409540	Further Discussion on S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409585	Discussion on Store and Forward operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409674	Discussion on RAN2 impacts due to the satellite ID list from MME in S&F operation	CATT	discussion

R2-2409676	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2408244

R2-2409689	Further consideration on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409799	Support of Store and Forward.	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409822	Discussion on the S&F operation indication	Google Ireland Limited	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409876	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409889	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2409935	Discussion on Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2409976	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410049	Discussion on S&F mode operation	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410110	Further discussion of IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410181	Discussion on assistance information for S&F	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410270	Further considerations on S&F operation	Lenovo	discussion	Rel-19

R2-2410314	Discussion on Support of Store & Forward	TOYOTA Info Technology Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410346	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410395	Radio Interface Aspect of S&F	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410468	Radio Interface Impacts of SF operation	Nokia, Nokia Shanghai Bell	discussion

R2-2410482	Discussion on Store and Forward operation	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410591	Further consideration on Store and Forward	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410596	Considerations on multi-satellite for S&F Satellite operation	NOVAMINT, Sateliot, Thales	discussion

R2-2410599	Suspend/resume procedure for Store and Forward satellite operation	SHARP Corporation	discussion

R2-2410636	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2408622

R2-2410687	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410765	Further considerations on S&F operations	Continental Automotive	discussion	Rel-19

R2-2410854	Support of Store & Forward	Sequans Communications	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2408971

R2-2410863	Support for store and forward in IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Reduce the necessary signaling to complete an Early Data Transmission

R2-2409541	Discussion on CB-Msg3 Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409586	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409591	Discussion on the RAN2 solutions for uplink capacity enhancement	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409673	Further consideration on UL capacity enhancements	CATT	discussion

R2-2409690	Further consideration on UL capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409800	EDT/PUR enhancements.	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409877	Discussion on CB-msg3 EDT and msg4 enhancement	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409937	Further considerations on retransmission and finite buffer for DSA	NTU	discussion	Rel-19

R2-2409975	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410050	CB-Msg3 and Msg4 enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410111	Contention-based Msg3-EDT in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410271	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2410291	Consideration on UL capacity enhancement for IoT-NTN	NEC Corporation	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410296	Discussion on the use of an implicit pointer for locating DSA replicas for EDT of CB-Msg3	DLR, ESA	discussion	Withdrawn

R2-2410309	Further discussion on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410332	Considerations on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410347	Discussion on Uplink Capacity Enhancement	TOYOTA Info Technology Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410483	Procedures for contention-based Msg3	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410641	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2408623

R2-2410725	Discussion on EDT Enhancements for IOT NTN	Skylo Technologies	discussion	Rel-19

R2-2410875	Implicit pointer for locating DSA replicas for EDT of CB-Msg3	DLR, ESA, Inmarsat, Viasat	discussion

R2-2410882	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core	Late

Support of PWS Reception

R2-2409542	Further Discussion on PWS Support for NB-IoT	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409587	PWS support for NB-IoT over NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409675	Support of PWS for NB-IoT NTN UE	CATT	discussion

R2-2409691	Further consideration on PWS support for NB-IoT over NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409801	Support of PWS.	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2409896	Discussion on supporting PWS in IOT-NTN network	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410051	Discussion on PWS in NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410272	PWS broadcast support for NB-IoT in NTN	Lenovo	discussion	Rel-19

R2-2410289	Discussion on PWS for NB-IoT	Google	discussion	Rel-19

R2-2410292	Consideration on PWS broadcast for NB-IoT	NEC Corporation	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410304	Further considerations on PWS support for NB-IoT	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410310	Further discussion on support of PWS	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410333	Support of PWS messages for NB-IoT	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410484	Impact of PWS signalling for NB-IoT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2410643	Discussion on supporting PWS for NB-IoT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2408624

R2-2410864	Support for PWS in NB-IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

## 4.3	RAN3

RAN3#126, Nov’24:

R3-247008	LS on PWS support for NB-IoT NTN  	RAN2(InterDigital)	  LS in

R3-247549  Further discussion on support of store and forward satellite operation for IoT NTN   ZTE Corporation   discussion

R3-247392  Discussion for IoT NTN   CATT   discussion

R3-247318  Support of Store and Forward in IoT NTN   Xiaomi   discussion

R3-247331  Discussion on Support for IoT NTN for Regenerative Payload   Qualcomm Incorporated   discussion

R3-247346  Discussion on the support of IoT NTN   Nokia, Nokia Shanghai Bell   discussion

R3-247378  Support for IoT NTN with regenerative payload   China Telecom   discussion

R3-247401	(TP for TS 36.300) Support of store and forward for IoT NTN  	Huawe	  other

R3-247455  How to Specify Store & Forward (S&F) Architecture in RAN3   Ericsson, Sateliot, Thales, ESA, Deutsche Telekom   discussion

R3-247456  Store & Forward (S&F) Architecture - Stage 2 Impacts   Ericsson, Sateliot, Thales, ESA, Deutsche Telekom   other

R3-247457  Store & Forward (S&F) Architecture - S1 Removal   Ericsson, Sateliot, Thales, ESA, Deutsche Telekom   other

R3-247458  Store & Forward (S&F) Architecture - S1AP Impacts   Ericsson, Sateliot, Thales, ESA, Deutsche Telekom   other

R3-247531  PWS Support for NB-IoT NTN   Ericsson LM   discussion

R3-247595  Further discussions on support of Store and Forward operations in NTN   THALES, Sateliot, Novamint

discussion

R3-247609  Discussion on support of regenerative payload for IoT NTN   Samsung   discussion

R3-247676  Discussion on support of IoT NTN   CMCC   discussion

## 4.4	RAN4

RAN4#112bis, Oct’24:

R4-2416075	Discussion on reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 Ericsson

R4-2415584	OCC scheme on the RF requirements for IoT NTN	Sony

R4-2415740	Discussion on RF requirement impact for IoT NTN phase 3 Mediatek India Technology Pvt.

R4-2416464	IoT NTN UL capacity enhancements	Qualcomm Inc.

R4-2415353	Impact on RRM core requirement Qualcomm Incorporated

R4-2415829	Discussion on RRM impacts for R19 IoT NTN	Huawei, HiSilicon

R4-2415957	Discussion on RRM requirements for IoT NTN phase 3 MediaTek inc.

R4-2416074	Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 Ericsson

R4-2416188	On the reply LS to RAN1 on contention based Msg3 transmission.	Nokia

R4-2415670	Topic summary for [112bis][223] IoT_NTN_Ph3	Moderator (MediaTek)

R4-2416536	Topic summary for [112bis][315] IoT_NTN_Ph3	Moderator (MediaTek)

R4-2416565	Way Forward for [112bis][315] IoT_NTN_Ph3	MediaTek

R4-2416867	WF on RRM requirements for IoT_NTN_Ph3	MediaTek

R4-2416918	Reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 ZTE Corporation, Sanechips

RAN4#113, Nov’24:

R4-2418038	OCC scheme on the RF requirements for IoT NTN	Sony

R4-2419496	On IoT NTN UE RF requirment	Ericsson

R4-2419524	Discussion on RF requirement impact for IoT NTN phase 3	Mediatek India Technology Pvt.

R4-2418436	Discussion on the RRM requirement for IOT NTN phase3	CMCC

R4-2418903	Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 Ericsson

R4-2419633	On the specification impact of CB-Msg3 transmission without Msg1 and 2.	Nokia

R4-2418290	Topic summary for [113][232] IoT_NTN_Ph3	Moderator (MediaTek)

R4-2419602	Topic summary for [113][315] IoT_NTN_Ph3	Moderator (MediaTek)

R4-2419899	Way Forward for [113][315] IoT_NTN_Ph3	MediaTek

R4-2420106	WF on RRM requirements for IoT_NTN_Ph3	MediaTek