3GPP TSG-RAN Meeting #106                                                RP-243012

Madrid, Spain, December 9 - 12, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the scope of 6Rx

Agenda Item:	9.3.4.1

Document for:	Discussion

Introduction

Per the WID on UE RF enhancement Phase 4 [1], RAN4 is tasked to develop the 6Rx RF requirements applied to TDD bands.

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we would like to explore the possibility for extending the WI scope to cover some popular FDD bands based on what we observed from spectrum allocation and deployment.

Discussion

FDD frequencies play irreplaceable carrier on both coverage and capacity, because of their low path loss which is a natural advantage and the wide frequency span especially for those middle FDD bands. The following table demonstrates how rich and diverse the FDD frequency resources are all around the world.

Table 1. Global frequency allocations for FDD and TDD

|  | Frequency/Band |  | Europe/Africa/Middle East/Central Asia | Asia Pacific | Latin America |
| --- | --- | --- | --- | --- | --- |
| FDD | 450MHz | B31/N72 | 2*5MHz | 2*5MHz | 2*5MHz |
|  | 600MHz | B71/N71 | 2*35MHz | 2*40MHz | 2*35MHz |
|  | 700MHz | B28/N28 | 2*30MHz | 2*45MHz | 2*45MHz |
|  | 800MHz | B20/N20 | 2*30MHz |  |  |
|  | 850MHz | B5/N5 |  | 2*10MHz | 2*25MHz |
|  | 900MHz | B8/N8 | 2*35MHz | 2*35MHz | 2*10MHz |
|  | L-Band | B32/N75 | 1*85MHz | 1*85MHz | 1*85MHz |
|  | 1700MHz | B3/N3 | 2*75MHz | 2*75MHz |  |
|  | PCS | B2/N2 |  |  | 2*60MHz |
|  | AWS | B4/N66 |  |  | 2*70MHz |
|  | 2100MHz | B1/N1 | 2*60MHz | 2*60MHz |  |
|  | 2600MHz | B7/N7 | 2*70MHz | 2*70MHz | 2*70MHz |
| TDD | 2.3GHz | B40/N40 | 100MHz | 100MHz | 100MHz |
|  | 2.6GHz | B41/N41 | 196MHz | 196MHz |  |
|  | 2.6GHz | B38/N38 | 50MHz | 50MHz | 50MHz |
|  | C-band | B43/B43/N77/N78 | 500MHz | 500MHz | 500MHz |
|  | 4.9GHz | N79 | 200MHz | 200MHz | 200MHz |

For a more intuitive perception, we present the following information which shows that the FDD network deployment actually shares quite a proportion that cannot be ignored.

Table 2. Information gathered from current deployment

| Proportion of FDD network in current TDD & FDD deployment | ~46% |
| --- | --- |
| Proportion of network for >1GHz FDD bands in current FDD deployment | ~57% |

The above table shows that the proportion of FDD network in current deployment is virtually identical to the one of TDD network. Further, for those >1GHz FDD bands, where a good balance between coverage performance (related to the frequency per concerned band) and capacity potentiality (associated with the available bandwidth) can be achieved, the proportion of the network in current FDD deployment exceeds 50%.

Observation 1: The value of FDD network in continuously meeting operator demands on both coverage and capacity is self-evident.

- The proportion of FDD network in current deployment is virtually identical to the one of TDD network;

- For those >1GHz FDD bands, where a good balance between coverage performance (related to the frequency per concerned band) and capacity potentiality (associated with the available bandwidth) can be achieved, the proportion of the network in current FDD deployment exceeds 50% as well.

As the FDD network is evolving towards more Tx number or even massive MIMO, it should be noted that the system performance cannot be fully improved without any enhancement at UE side. As captured in WF [2], the RF requirements including REFSENS and relaxation on Tx power for AS-SRS for 6Rx in those example TDD bands are defined based on companies’ inputs. We think there could be a chance to discuss on introducing 6Rx as optional feature for FDD bands like n1 and n3 since they are adjacent to the band n41 which is covered by the example bands.

Observation 2: As the FDD network is evolving towards more Tx number or even massive MIMO, the overall DL performance can be further improved when UE side enhancements happen.

For instance, one can start with considering 6Rx for band n1 and n3 in order to collect diversity gain especially for cell edge.

Proposal: To discuss whether the support of 6Rx can be extended to FDD bands, e.g. n1 and n3.

If there is consensus for extension of 6Rx scope based on the discussion, the possible changes of the WID could be:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz for TDD bands and > 1.8GHz for FDD bands) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n1, n3, n41, n77/n78, n79, n104 - Support 4 MIMO layers for TDD bands and FDD bands at least, and study the gain and feasibility and if feasible, support 6 MIMO layers for TDD bands - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Conclusion

In this contribution we discussed on the motivation for extending the Rel-19 6Rx WI scope to > 1GHz FDD bands.

Observation 1: The value of FDD network in continuously meeting operator demands on both coverage and capacity is self-evident.

- The proportion of FDD network in current deployment is virtually identical to the one of TDD network;

- For those >1GHz FDD bands, where a good balance between coverage performance (related to the frequency per concerned band) and capacity potentiality (associated with the available bandwidth) can be achieved, the proportion of the network in current FDD deployment exceeds 50% as well.

Observation 2: As the FDD network is evolving towards more Tx number or even massive MIMO, the overall DL performance can be further improved when UE side enhancements happen.

- For instance, one can start with considering 6Rx for band n1 and n3 in order to collect diversity gain especially for cell edge.

Proposal: To discuss whether the support of 6Rx can be extended to FDD bands, e.g. n1 and n3.

The paper aims to gather companies' views on 6Rx for FDD bands.

References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei. HiSilicon, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17-20, 2024.

- R4-2417209, “WF on requirements for 6Rx UE requirements”, AT&T, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.