3GPP TSG RAN Meeting #106						RP-243131

Madrid, Spain, 9th -12th December, 2024

Agenda item:	16

Source:	CATT

Title:	Consideration and proposals on KPIs for IMT-2030

Document for:	Discussion and Decision

Background

High-level considerations for 6G timeline were provided and endorsed in TSG#102 referring to [1]. Additional consideration for 6G timeline was discussed and endorsed in TSG#103 referring to [2]. IMT-2030 discussion was expected in RAN from Sep. to Dec. 2024 and RP SI Rel-20 was expected approval in Dec. 2024, starting with focus on ITU-IMT-2030.

In the last meeting, the contribution from CATT [3] provided overall views on IMT-2030 KPI and possible work plan to be considered. We also found that an LS [4] from ITU-R WP5D was received by 3GPP. In this contribution, CATT provided considerations on the SI and interworking activities with ITU based on [4] and also propose 6G KPIs in more details. It is noted that the discussions on 6G KPIs in this contribution are focusing on ITU IMT-2030, i.e. terrestrial perspective.

Discussion

Considerations on the SI and interworking activities with ITU

A SI will set up and start during this 3GPP RP as planned in [2]. This SI will start with focus on ITU-IMT-2030 terrestrial perspective.

The LS [4] from ITU-R WP5D invited activities as below:

Working Party 5D kindly invites External Organizations to provide their inputs on the minimum technical performance requirements for IMT-2030 radio interface technologies, including but not limited to,

– Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (4-13 February 2025).

– Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)

We believe that it is valuable to present the voice of industry on the understanding of minimum requirements as well as preparation of such requirements in 3GPP itself for the following 6G study. It is noted that planned WGs SI would start next August, which needs those requirements as part of guidance, as we did in 5G. There should be a good match between 3GPP and ITU in case that 3GPP considers submitting its 6G technology and self-evaluation to ITU in future. The relevant timetable is summarized for reference.

Table 1 Meeting schedule in 3GPP and contribution deadline in ITU

| 3GPP |  | ITU |  |
| --- | --- | --- | --- |
| Plenary RAN meeting | Planned meeting dates | ITU-R WP5D meeting | Deadline for contributions |
| RAN#106 | 9-12 December 2024 | WP5D #48, 4-13 February 2025 | 23 January 2025 (1600 hours UTC) |
| RAN#107 | 12-14 March 2025 | - | - |
| RAN#108 | 9-13 June 2025 | WP5D #49, 24 June – 3 July 2025 | 12 June 2025 (1600 hours UTC) |

Proposal 1. Approval of SI as planned. With start of work on KPIs focused on ITU IMT-2030, to send the aligned items as proposal to ITU and prepare the corresponding target values, from this plenary meeting. To complete target values proposal preferably at RAN#107 and guarantee that by RAN#108.

Discussions on KPIs focused on ITU IMT-2030

It is clear to know that technical performance requirements to be defined in ITU is “minimum requirement” to evaluate whether a candidate technology can be a qualified 6G technology or not, rather than a complete set of requirements for the whole 6G system and all services. KPIs focused on IMT-2030 should represent the most basic elements to be supported in IMT-2030 technologies. Additional KPIs or service requirements can be further discussed as a part of 6G general requirements study.

Possible KPIs in 3 evolved usage scenarios

Three usage scenarios include Immersive Communication(IC), Massive Communication(MC) and Hyper Reliable and Low-Latency Communication(HRLLC), evolved from familiar eMBB, mMTC and URLLC. Following above underlined sentence, most well defined KPIs in 38.913[5] (mostly aligned with M.2410 [7]) can still be applied to IMT-2030. Those items and definitions are quite stable and some of which were even used since 4G to evaluate radio interface technologies. With those, on one hand, those KPIs are iconic and familiar to industry. On the other hand, it is easy to tell the advantage and improvements by applying 6G technologies compared with 5G. Within this group, only the definition to Energy efficiency is newly proposed and others follow [7] and [5].

Proposal 2. Propose to inherit the applicable KPI items and definitions from 5G as the following Table 2 and Annex 1 (Table 5), except for Energy Efficiency, for 3 evolved usage scenarios.

Table 2 Proposed IMT-2030 KPIs inherited from IMT-2020(5G)

| Num | IMT-2020 TPR(ITU-R M.2410) | 38.913 chapter 7 KPI matching IMT-2020 TRP | Proposed IMT-2030 KPI |
| --- | --- | --- | --- |
| 1 | Peak data rate @eMBB | Peak data rate | Peak data rate @IC |
| 2 | Peak spectral efficiency @eMBB | Peak spectral efficiency | Peak spectral efficiency @IC |
| 3 | User experienced data rate @eMBB Dense Urban | User experienced data rate @eMBB DU | User experienced data rate @IC Dense Urban |
| 4 | 5th percentile user spectral efficiency @eMBB | 5th percentile user spectrum efficiency @eMBB | 5th percentile user spectral efficiency @IC |
| 5 | Average spectral efficiency @eMBB | Cell/Transmission Point/TRxP spectral efficiency @eMBB | Average spectral efficiency @IC |
| 6 | Area traffic capacity @eMBB InH | Area traffic capacity | Area traffic capacity @IC InH |
| 7 | User plane latency @eMBB, URLLC | User plane latency @eMBB, URLLC | User plane latency @IC, HRLLC |
| 8 | Control plane latency @eMBB, URLLC | Control plane latency | Control plane latency @IC, HRLLC |
| 9 | Connection density @mMTC | Connection density @mMTC | Connection density @MC |
| 10 | Energy efficiency @eMBB | UE energy efficiency & Network energy efficiency | Energy efficiency (UE & Newtowrk) @IC |
| 11 | Reliability @URLLC | Reliability @URLLC | Reliability @HRLLC |
| 12 | Mobility @eMBB | Mobility | Mobility @IC |
| 13 | Mobility interruption time @eMBB, URLLC | Mobility interruption time | Mobility interruption time @IC, HRLLC |
| 14 | Bandwidth | Bandwidth | Bandwidth |

But for sure, even if inheriting the same items and their definitions, requirements are expected higher than those of 5G. The conditions to achieve such requirements, e.g. carrier frequency, may also be reconsidered, to match the expectation of 6G.

- Spectral efficiency

- Peak spectral efficiency

According to the generic formula, modulation level, coding rate, spatial layers and overhead are key factors. Some of which are difficult to achieve much growth again though. To consider the possible improvement, e.g. modulation and number of streams, the target value is expected to reach 2.5x of IMT-2020 targets, i.e. 75bps/Hz for downlink and 37.5bps/Hz for uplink.

- Average and 5th percentile user spectral efficiency

The improvements may mostly come from larger antenna arrays and better channel rebuild. But considering cost and windward side in the practice, the number of antenna and TxRU cannot increase unlimited. In most current deployment in 5G, the popular number of TxRU is 32 at gNB and mainly 4 at UE. The number of antenna elements does not beyond 256 at gNB. So we expect that there will be improvement in 6G, but within a reasonable range. 2~3x of IMT-2020 targets are proposed for both, e.g. 0.6bps-0.9bps 5th percentile user spectral efficiency at downlink Indoor Hotpot.

Also, evaluations for Indoor Hotspot, Dense Urban and Rural for full buffer as defined environments in 5G are sufficient. New carrier frequency, e.g. U6GHz, could be considered to match 6G frequency band.

- Data rate (User experienced data rate and Area traffic capacity)

KPIs of data rate category, including User experienced data rate and Area traffic capacity, mostly could be derived from the spectral efficiency and the supported bandwidth. In 5G, there was pretty high bandwidth applied to reach the IMT-2020 targets, e.g. 800MHz, in the case that the applied spectral efficiency in calculation was actually beyond its IMT-2020 target. It was questioned even in 5G how it could reach the target in practice, particularly for user experienced data rate, which seemed impossible at all. With increase of spectral efficiency, there will be higher target of data rate, but it is not that reasonable to reply on extremely high and non-existing bandwidth/carrier aggregation in practice. The target values of these KPIs are expected to be 2~3x of IMT-2020 targets. The similar consideration comes to peak data rate.

- Latency (Control plane latency and User plane latency)

Substantial decrease in control plane latency is expected on the simplification of the access process and the enhancement of the processing capability of base station and UE, reflecting a faster transition from battery efficiency state to an active communication state. The target value is expected to be 10ms.

Not sure yet the remarkable use case to apply lower value of user plane latency because end-end latency seems more important to immersive cases while the latency only from radio interface may not be the key.

- Connection density

The connection density is valuable and unique to be defined for Massive Communication. Considering the rise of numerous IoT devices and the expansion of use cases, the requirement for connection density may also increases. This KPI is expected to reach 10~100x of IMT-2020 target.

- Energy efficiency

In IMT-2020, the energy efficiency was inspected by sleep ratio and sleep duration in unloaded case. Average spectral efficiency was instead in loaded cases. It is still workable while dimensions to reduce power consumption are not well reflected. It should be defined quantitatively. The proposed new definition can be found in Annex 1(No.14).

Proposal 3.  There are considerations on some preliminary values of proposed “inherited” KPI in Table 3.

Table 3 Proposed values of “inherited” KPI

| Num | Proposed IMT-2030 KPI as a start | Possible values in consideration |
| --- | --- | --- |
| 1 | Peak data rate @IC | 2.5~5x of IMT-2020 target |
| 2 | Peak spectral efficiency @IC | 2.5 of IMT-2020 target |
| 3 | User experienced data rate @IC DU | 2~3x of IMT-2020 target |
| 4 | 5th percentile user spectral efficiency @IC | 2~3x of IMT-2020 target |
| 5 | Average spectral efficiency @IC | 2~3x of IMT-2020 target |
| 6 | Area traffic capacity @IC InH | 2~3x of IMT-2020 target |
| 7 | User plane latency @IC, HRLLC | TBD |
| 8 | Control plane latency @IC, HRLLC | 10ms |
| 9 | Connection density @MC | 10x of IMT-2020 target |
| 10 | Energy efficiency (UE and Network) @IC | TBD (quantitative) |
| 11 | Reliability @HRLLC | TBD |
| 12 | Mobility @IC | TBD |
| 13 | Mobility interruption time @IC, HRLLC | 0ms |
| 14 | Bandwidth | TBD |

Possible KPIs in 3 new usage scenarios

Three new usage scenarios include Ubiquitous Connectivity (UC), Integrated Sensing and Communication (ISAC) and AI and Communication (AIAC). It is expected to have new KPIs, which would be based more on the selected typical services.

In ISAC, both positioning (to connected devices) and sensing (to unconnected devices) KPIs are needed.

- Positioning (Positioning accuracy)

Positioning was 5G supplementary-service related requirement in 38.913[5]. It is an existing capability supported in each of previous generations and should be quantified in IMT-2030. Requirements to accuracy of positioning performance depend on the use cases, environments including geographic assumption and deployment. It is important to define evaluation environments and channel model together with KPI of positioning accuracy, to both horizontal and vertical plane. It is noted that in M.2160, 1-10cm as a capability is required. The proposed definition can be found in Annex 2.

- Sensing

Different from Positioning, although use cases and requirements of sensing were provided by SA1 for 5G-A, there is not yet any KPI translated in RAN. In case that sensing UAV is supported in 5G-A, other typical use cases can be selected for 6G, e.g. detection and tracking in highway. That apparently comes to KPIs on detection probability, localization accuracy and velocity accuracy, while to be more basic measured elements, distance and angle accuracy can replace localization accuracy for many cases.

In AIAC, there was no discussion yet for such KPIs on AI in 3GPP. For the very first step, it is necessary to discuss whether quantitative IMT-2030 KPIs are needed. From our perspective, considering AI as a technology to improve internal communication performance, it can be well reflected in the “inherited” KPIs such as spectral efficiency in 3 evolved usage scenarios. When AI is applied in the system to provide other services, the main advantages shown by IMT-2030 would be its new functions, which could also be applied to improve internal communication performance. With that,  qualitative KPI(s) to be inspected would be more proper to present AI, rather than a non-generalization/case-by-case quantitative KPI.

Proposal 4. There are proposals for KPIs in ISAC and AIAC in Table 4 and Annex 2(Table 6).

Table 4 Proposed new IMT-2030 KPIs compared with IMT-2020(5G) by CATT

| Num | Proposed IMT-2030 KPI as a start | Evaluation environments | Possible values in consideration |
| --- | --- | --- | --- |
| 1 | Positioning @ISAC | Indoor Factory | 1~10cm for horizontal plane TBD for vertical plane |
| 2 | Sensing relative KPIs @ISAC, including , localization accuracy and velocity accuracy | Dense Urban | TBD |
| 3 | AI relative KPIs @AIAC | N.A. | Qualitative |

Proposals

Proposal 1. Approval of SI as planned. With start of work on KPIs focused on ITU IMT-2030, to send the aligned items as proposal to ITU and prepare the corresponding target values, from this plenary meeting. To complete target values proposal preferably at RAN#107 and guarantee that by RAN#108.

Proposal 2. To inherit the applicable KPI items and definitions from 5G as the following Table 2 and Annex 1, for 3 evolved usage scenarios.

Table 2 Proposed IMT-2030 KPIs inherited from IMT-2020(5G)

| Num | IMT-2020 TPR(ITU-R M.2410) | 38.913 chapter 7 KPI matching IMT-2020 TRP | Proposed IMT-2030 KPI |
| --- | --- | --- | --- |
| 1 | Peak data rate @eMBB | Peak data rate | Peak data rate @IC |
| 2 | Peak spectral efficiency @eMBB | Peak spectral efficiency | Peak spectral efficiency @IC |
| 3 | User experienced data rate @eMBB Dense Urban | User experienced data rate @eMBB DU | User experienced data rate @IC Dense Urban |
| 4 | 5th percentile user spectral efficiency @eMBB | 5th percentile user spectrum efficiency @eMBB | 5th percentile user spectral efficiency @IC |
| 5 | Average spectral efficiency @eMBB | Cell/Transmission Point/TRxP spectral efficiency @eMBB | Average spectral efficiency @IC |
| 6 | Area traffic capacity @eMBB InH | Area traffic capacity | Area traffic capacity @IC InH |
| 7 | User plane latency @eMBB, URLLC | User plane latency @eMBB, URLLC | User plane latency @IC, HRLLC |
| 8 | Control plane latency @eMBB, URLLC | Control plane latency | Control plane latency @IC, HRLLC |
| 9 | Connection density @mMTC | Connection density @mMTC | Connection density @MC |
| 10 | Energy efficiency @eMBB | UE energy efficiency & Network energy efficiency | Energy efficiency (UE & Newtowrk) @IC |
| 11 | Reliability @URLLC | Reliability @URLLC | Reliability @HRLLC |
| 12 | Mobility @eMBB | Mobility | Mobility @IC |
| 13 | Mobility interruption time @eMBB, URLLC | Mobility interruption time | Mobility interruption time @IC, HRLLC |
| 14 | Bandwidth | Bandwidth | Bandwidth |

Proposal 3.  There are considerations on some preliminary values of proposed “inherited” KPI in Table 3.

Table 3 Proposed values of “inherited” KPI

| Num | Proposed IMT-2030 KPI as a start | Possible values in consideration |
| --- | --- | --- |
| 1 | Peak data rate @IC | 2.5~5x of IMT-2020 target |
| 2 | Peak spectral efficiency @IC | 2.5 of IMT-2020 target |
| 3 | User experienced data rate @IC DU | 2~3x of IMT-2020 target |
| 4 | 5th percentile user spectral efficiency @IC | 2~3x of IMT-2020 target |
| 5 | Average spectral efficiency @IC | 2~3x of IMT-2020 target |
| 6 | Area traffic capacity @IC InH | 2~3x of IMT-2020 target |
| 7 | User plane latency @IC, HRLLC | TBD |
| 8 | Control plane latency @IC, HRLLC | 10ms |
| 9 | Connection density @MC | 10x of IMT-2020 target |
| 10 | Energy efficiency (UE and Network) @IC | TBD (quantitative) |
| 11 | Reliability @HRLLC | TBD |
| 12 | Mobility @IC | TBD |
| 13 | Mobility interruption time @IC, HRLLC | 0ms |
| 14 | Bandwidth | TBD |

Proposal 4. There are proposals for KPIs in ISAC and AIAC in Table 4 and Annex 2.

Table 4 Proposed new IMT-2030 KPIs compared with IMT-2020(5G) by CATT

| Num | Proposed IMT-2030 KPI as a start | Evaluation environments | Possible values in consideration |
| --- | --- | --- | --- |
| 1 | Positioning @ISAC | Indoor Factory | 1~10cm for horizontal plane TBD for vertical plane |
| 2 | Sensing relative KPIs @ISAC, including localization accuracy and velocity accuracy | Dense Urban | TBD |
| 3 | AI relative KPIs @AIAC | N.A. | Qualitative |

References

- RP-233985 Additional considerations for 6G timeline, TSG Chairs

- RP-240823 High-level considerations for 6G timeline	, TSG Chairs

- RP-242128 Views on IMT-2030 KPI, CATT

- RT-240015 Liaison statement to external organizations - Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R Working Parting 5D

- 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 15)

- ITU-R M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond

- ITU-R M.2410 Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- Annex 1

Table 5 Proposed definitions of KPIs in 3 evolved usages scenarios

| Num | Proposed IMT-2030 KPI | Proposed Definitions |
| --- | --- | --- |
| 1 | Peak data rate@IC | Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the user peak data rate Rp is given by: Rp = W × SEp (1) Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is R=i=1Q Wi × SEpi (2) where Wi and SEpi (i = 1,…Q) are the component bandwidths and spectral efficiencies respectively. |
| 2 | Peak spectral efficiency@IC | Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). |
| 3 | User experienced data rate @IC Dense Urban | User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time. In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by: Ruser = W × SEuser (3) In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands. Full buffer experienced user data rate is evaluated. |
| 4 | 5th percentile user spectral efficiency @IC | The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio. With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to equation (4). (4) |
| 5 | Average spectral efficiency @IC | Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio. Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to equation (5). (5) |
| 6 | Area traffic capacity @IC Indoor Hotspot | Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth. Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6). Carea = ρ × W × SEavg (6) In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands. Full buffer area traffic capacity is evaluated. |
| 7 | User plane latency @IC, HRLLC | User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. |
| 8 | Control plane latency @IC, HRLLC | Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). |
| 9 | Connection density @MC | Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2). Connection density should be achieved for a limited bandwidth and number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability. |
| 10 | Reliability @HRLLC | Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. |
| 11 | Mobility @IC | Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h) |
| 12 | Mobility interruption time @IC, HRLLC | Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions. |
| 13 | Bandwidth | Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. |
| 14 | Energy efficiency(UE and Network) @IC | Network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics. The energy consumption could be defined as average energy consumption during the observation time. E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal (in energy unit/time unit) Ei (in energy unit) is the energy computation category at network or device for transmission and/or reception in loaded and unloaded cases; As an illustration, it is assumed at least three energy consumption categories: Where, Eon is the energy consumption for active transmission and/or reception at the network or device during the observation/evaluation time. Eoff is the energy consumption at network or device when there is no transmission and/or reception Etransition is the energy consumption at network or device during transition between on and off. Ttotal (in time unit)is the total observation/evaluation time. Energy efficiency of the network and the device can relate to the support for the following two aspects: - For unloaded case, - Energy consumption - For loaded case - Energy consumption, or - Energy efficiency The energy efficiency EEloaded is given by: EEloaded=v/E Where E denotes the average energy consumption during the observation time, v denotes the communication related performance metric. |

- Annex 2

Table 6 Proposed definitions of KPIs in new usages scenarios

| Num | Proposed IMT-2030 KPI as a start | Definition |
| --- | --- | --- |
| 15 | Positioning @ISAC | Positioning is the ability to calculate the position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude). Horizontal positioning accuracy and vertical positioning accuracy are positioning error metrics in e.g. 90% of UEs/devices. |
| 16 | Sensing relative KPIs @ISAC, including distance accuracy, angle accuracy, velocity accuracy, detection probability and/or localization accuracy | Distance accuracy: the difference between the calculated distance and the actual distance of the sensing target. Angle accuracy: the difference between the calculated angles and the actual angles of the sensing target. Localization accuracy: the difference between the measured position and the actual position of the sensing target. Velocity Accuracy: the difference between the estimated velocity and the actual velocity of the sensing target. Detection probability: the probability of correctly detecting the presence of the object. |
| 17 | AI relative KPIs @AIAC | The IMT-2030 system is required to provide mechanisms and/or signalling related to functionalities that support distributed AI infrastructure/architecture for AI applications and also apply AI to improve internal-IMT communication performance. These functionalities include distributed management and data processing, distributed learning, distributed AI model inference, distributed AI computing, AI model execution, or any other functionalities which are relevant to better supporting AI enabled applications. |