3GPP TSG RAN Meeting #106	RP-243139

Madrid, Spain, December 9th - 12th, 2024

Agenda item:		9.3.1.5

Source:	Deutsche Telekom, Vodafone, Orange, Fraunhofer IIS, Fraunhofer HHI, Lenovo

Title:	Cell A to NES cell switch for OD-SIB1 procedure

Document for:		Discussion and Decision

# Introduction

In the previous RAN Plenary #105 meeting in Melbourne, during the discussion on NES checkpoint, a proposal was made by Deutsche Telekom’s delegate & RAN VC to remove restriction that Cell A and NES cell must be different (at different times). The proposal was endorsed, according to RAN Plenary’s MoMs shown below for reference, but it was felt that no WID update would be necessary.

RP-242141	On demand SIB 1 and the need for case 1 for multi-vendor RAN	Vodafone, Deutsche Telekom,

Orange, Lenovo, Google,

Fraunhofer IIS and

Fraunhofer HHI, ETRI,

CEWiT

Replaces

RAN WG1 and RAN WG2 discussed 3 scenarios for the support of on demand SIB1:

- Case 1: Standalone, where configuration to request SIB1, SIB1 request and SIB1 are delivered within the SIB1-less NES cell

- Case 2: Anchor: configuration to request SIB1 is delivered by an Anchor cell, but SIB1 request and SIB1 are delivered within the SIB1-less NES cell

- Case 3: Anchor: configuration to request SIB1, SIB1 request and SIB1 (for the SIB1-less NES cell) are delivered by the Anchor cell

Proposal 1: The co-signing companies recommend supporting Case 1 in Rel 19 in addition to case 2.

DT: some clarification is needed before we could go forward with case 1, anchor cell can be same as NES cell? a cell could work as normal cell during the day and as NES cell in the night, if these are 2 different cells, then we cannot agree to case 1

RAN chair: focussing on case 2 is acceptable?

Qualcomm: if WUS configuration for neighbours is still possible, then we would be ok

RAN chair: asks RAN1 chair to draft a way forward in RP-242353

The document was noted.

Replaced by

RP-242353	Way forward on NES checkpoint	RAN1 chair (Samsung)

The document was revised.

Replaced by RP-242384

RP-242384	Way forward on NES checkpoint	RAN1 chair (Samsung)

Replaces

RP-242353

DT: would like to remove restriction that Cell A and NES cell must be different (at different times)

Panasonic: things that this is possible

RAN chair: as you talk about different times, this is possible

conclusion: proposal is endorsed

In RAN2#128 meeting, there was unwillingness to follow the RAN#105 guidance documented in the minutes to clarify that a Cell A can switch to become a NES cell at a different point in time and reflect this in the stage 2 agreements in RAN2. The clarifying proposal for RAN2 was submitted by multiple companies in R2-2410842.

According to the MoM one company insisted that the NES WID needs to be updated first, before RAN2 can agree:

# Summary and Conclusions

RAN Plenary #106 shall agree on an update of the WID to clarify that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell. In this case, Cell A provides UL-WUS configuration, which may be used by the UE to request SIB1 of this cell once it becomes NES cell. The definition status of “Cell A” or “NES cell” as currently used in the RAN WGs will not be changed with this.

Proposal 1: Agree on the updated WID as proposed in RP-243140.

# References

[1]		“MeetingReport_RAN_105_240912_eom”

[2] 		R2-2410842, RAN2#128, “Considerations on OD-SIB1 feature”, Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS

[3]		“Draft_RAN2_128_Meeting_Report_v1” [https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_128/Report/Draft_RAN2_128_Meeting_Report_v1.zip]