3GPP TSG-RAN Meeting #106                                                RP-243145

Madrid, Spain, December 9 - 12, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussions on 6 layers feasibility for 6Rx

Agenda Item:	9.3.4.1

Document for:	Discussion

Introduction

One objective of WI NR_ENDC_RF_Ph4 [14] is the evaluation of the maximum supported MIMO layers for 6Rx. Some companies [1~3] have provided measured ECC matrixes, which makes the evaluation more practical. Many companies [4~12] have provided evaluation results. However, companies have diversity views on final simulation assumptions. In this contribution we summarize the current stage of study firstly and then provides our views on some critical issues and feasibility definition.

Discussion

Current RAN4 discussion status

In RAN4#113 meeting, interesting companies [4~12] provided link level simulation results for the comparison of 6 layers and 4 layers, some companies [4~8] show that gain of 6 layers can be observed under certain conditions, other companies [9~12] show there are no gain of 6 layers. It's worth noting that there are still many options for some key parameter such as Tx EVM, DMRS configuration, number of Tx ports and ECC matrix, and the evaluations results provided by some companies are based on their preferred assumptions. The WID [14] clearly states that RAN4 should study the gain and feasibility for support 6 MIMO layers, hence RAN4 is recommended to continue discussion on the simulation assumptions alignment among companies, otherwise, it is hard to conclude the feasibility of 6 MIMO layers for 6Rx.

Observation 1: Some key simulation parameters are still open and under discussion in RAN4 at the current stage, which should be aligned among companies before RAN4 makes conclusion on 6 MIMO layers for 6Rx.

Proposal 1: RAN4 shall continue discussion on the simulation assumptions alignment among companies in following meetings.

UE Rx EVM

All the previous simulations conducted by companies in RAN4 are based on the assumptions with baseband processing only without any Rx RF impairment/Rx EVM considered. However, same as BS Tx EVM, UE Rx RF impairment plays an important role at high SNR range and is a key factor affecting high MIMO layers scheduling. RAN4 hasn’t considered this factor yet, hence we don’t think it is reasonable make any conclusion without the consideration of this key factor.

Observation 2: UE Rx RF impairment is a very important factor affecting high layers performance, which hasn’t been discussed by RAN4. It’s hard to make conclusion for the feasibility of 6 MIMO layers without this factor considered.

Proposal 2: RAN4 shall consider UE Rx EVM impact in the evaluations for the feasibility study of 6 MIMO layers for 6 Rx in next meetings.

Antenna efficiency

RAN4 agreed to consider antenna efficiency for feasibility analysis in RAN4#112bis and RAN4#113 meeting [13], [12] provides the details on the modelling of antenna efficiency and shows that it has huge impact on the performance. However, the study on the antenna efficiency is not adequate since there is no consensus on how to model the impact of antenna efficiency in simulations and only one company models it in the simulation.

Observation 3: The study on antenna efficiency impact is not adequate.

Proposal 3: RAN4 shall continue the discussion on how to model the impact of antenna efficiency in evaluation for the feasibility study of 6 MIMO layers for 6Rx.

The completion time for this WI is June, 2025, RAN4 still has enough time to do further study.

Observation 4: RAN4 still have enough TU for the 6 MIMO layers for 6Rx evaluation.

Feasibility

Simulation results from some companies [4~8] show the gain of 6 layers based on some certain cases which are not commonly used in the real Network. Typical and practical simulation assumptions should be considered to make the feasibility study of 6 MIMO layers more meaningful. Based on previous discussion and our understanding, the following aspects should be considered for the 6 MIMO layers feasibility study and justification:

Simulations should be based on the practical simulation assumptions which are commonly used in the real Network, corner cases shall be precluded.

The SNR points where 6 layers outperforms 4 layers shall not be very high and within a reasonable SNR range

The gain of 6layers outperforms 4 layers should be high enough.

Proposal 4: The following aspects should be considered for the 6 MIMO layers feasibility study and justification:

Simulations should be based on the practical simulation assumptions which are commonly used in the real Network.

SNR points where 6layers outperforms 4layers shall be within a reasonable SNR range

The gain of 6 layers should be high enough

For those key parameters still with several options, we would like to share our views on the simulation assumption alignment to facilitate RAN4 following evaluation.

DMRS configuration

[5] proposes to use same DMRS overhead for 4layers and 6layers evaluation, to achieve this, two DMRS configurations for 6 layers are proposed: Rel-18 1+1 and Rel-15 2+0. But Rel-18 DMRS configuration is introduced in Rel-18 and has not been supported by most UE devices. 2+0 DMRS configuration will cause severe performance degradation with high UE mobility which is not a typical configuration in the real network. Rel-15 1+1 DMRS configuration is most common configuration in real deployment and should be used in the simulation. DMRS 2+2 is more practical for evaluation with layers larger than 4 which is common understanding and used for the performance requirements definition of 6 layers and 8 layers for 8Rx.

BS TxEvm

RF core spec defines 6% TxEvm for 64QAM and 3.5% TxEvm for 256QAM, it is worth noting that this is minimum requirements and it has been tightened to 3% for RAN4 demodulation requirements definition, the motivation is that 3% is more closed to the BS implementation, which is a stable number and shouldn’t be changed without any technical justification. If some BS vendor argues that the Tx EVM in their product can be 1% even 0%, it means the current Tx EVM requirements for BS in the specification is too loose and can be tightened, as part of the feasibility study of 6 MIMO layers, the study of a feasible and practical BS Tx EVM can be performed at the same time.

Number of Tx

[5] proposes to use 32Tx ports for evaluation, but large number of Tx ports is never used for TDL channel. As discussed in [8], it’s hard to see digital beamforming gain with large number of Tx ports when channel correlation is low. Additionally, support of larger number of 32Tx ports requires higher UE capability which is not supported by most UEs.

Proposal 5: RAN4 shall use following simulation assumptions which are closed to real field for 6 MIMO layers feasibility study:

- Rel-15 DMRS 1+1 for 4 layer and 2+2 for 6 layers

- 3% TxEvm

- 8Tx ports

Conclusion

In this contribution we provide our views on 6 layers feasibility for 6Rx. The observations and proposals are:

Observation 1: Some key simulation parameters are still open and under discussion in RAN4 at the current stage, which should be aligned among companies before RAN4 makes conclusion on 6 MIMO layers for 6Rx.

Observation 2: UE Rx RF impairment is a very important factor affecting high layers performance, which hasn’t been discussed by RAN4. It’s hard to make conclusion for the feasibility of 6 MIMO layers without this factor considered.

Observation 3: The study on antenna efficiency impact is not adequate.

Observation 4: RAN4 still have enough TU for the 6 MIMO layers for 6Rx evaluation.

Proposal 1: RAN4 shall continue discussion on the simulation assumptions alignment among companies in following meetings.

Proposal 2: RAN4 shall consider UE Rx EVM impact in the evaluations for the feasibility study of 6 MIMO layers for 6 Rx in next meetings.

Proposal 3: RAN4 shall continue the discussion on how to model the impact of antenna efficiency in evaluation for the feasibility study of 6 MIMO layers for 6Rx.

Proposal 4: The following aspects should be considered for the 6 MIMO layers feasibility study and justification:

Simulations should be based on the practical simulation assumptions which are commonly used in the real Network.

SNR points where 6layers outperforms 4layers shall be within a reasonable SNR range

The gain of 6 layers should be high enough

Proposal 5: RAN4 shall use following simulation assumptions which are closed to real field for 6 MIMO layers feasibility study:

- Rel-15 DMRS 1+1 for 4 layer and 2+2 for 6 layers

- 3% TxEvm

- 8Tx ports

- References

- R4-2412572 On MIMO layer evaluation for 6Rx UE. Huawei, HiSilicon.

- R4-2415681 Discussion and simulation results on MIMO layer evaluation for 6Rx UE. Samsung

- R4-2417916 R19 Simulation results of 6Layer performance. OPPO

- R4-2418011 Simulation Results for on MIMO layer evaluation for 6Rx UE. Nokia

- R4-2418437 Discussion on the demodulation performance requirements for 6Rx devices. Qualcomm.

- R4-2419165 Discussion on MIMO layer evaluation for 6Rx UE. ZTE

- R4- 2419550 Discussion on the MIMO layers for 6Rx UE. Ericsson

- R4-2419029 On MIMO layer evaluation for 6Rx UE. Huawei, HiSilicon.

- R4-2418619 Discussion and simulation results on MIMO layer evaluation for 6Rx UE. Samsung

- R4-2418595 Views on Maximum Number of MIMO Layers for 6Rx UEs. Apple

- R4-2418188 Updated simulation results of MIMO layer evaluation for 6Rx UE. Vivo

- R4-2417561 Feasibility evaluation of MIMO layer for 6Rx UE. MediaTek.

- R4-2417209 WF on 6Rx UE requirements.AT&T

- RP-241656 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC Phase 4. Huawei,HiSilicon