3GPP TSG RAN Meeting #106	                                          RP-243155

Madrid, Spain, December 9-12, 2024

Title: 	Views on Ambient IOT enhancement in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.2

Document for:	Discussion and decision

Introduction

In RAN#102, R19 Ambient IoT(A-IoT) SID was approved in [1]. The comprehensive study of A-IoT in RAN WGs has been completed to support the Rel-19 A-IoT with target scenario, topology, traffic type and device type summarized in Table 1[2].

Table 1 Summary of Rel-19 A-IoT

| Scenario | Indoor scenarios, with maximum coverage range of 10-50 m |
| --- | --- |
| Topology | D1T1(BS<-> Ambient IoT device); D2T2(BS <-> intermediate UE<-> Ambient IoT device) |
| Traffic type | DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command) |
| Device type | Device 1: ~1 µW peak power consumption, has energy storage,UL transmission is backscattered on a carrier wave provided externally. |
|  | Device 2a: a few hundred µW peak power consumption, has energy storage, UL transmission is backscattered on a carrier wave provided externally. |
|  | Device 2b: a few hundred µW peak power consumption, has energy storage, UL transmission is generated internally by the device. |

In this contribution, the potential enhancements for Rel-20 are discussed to further extend the use cases of A-IoT.

Discussion about Rel-20 A-IoT

Use case and scenarios

- Based on the RAN study in TR38.848 [3], the following 8 representative use cases (rUC) of A-IoT are formulated. Furthermore, the mapping from SA1 use cases and traffic scenarios onto RAN rUCs has been captured in TR, which can be found in the Appendix of this contribution.

-	rUC1: Indoor inventory

-	rUC2: Indoor sensors

-	rUC3: Indoor positioning

-	rUC4: Indoor command

-	rUC5: Outdoor inventory

-	rUC6: Outdoor sensors

-	rUC7: Outdoor positioning

-	rUC8: Outdoor command

- It can be observed that only a limited set of use cases has been studied in Rel-19, with focus on indoor inventory/command. In Rel-20, to support the use cases such as smart grids, automated supply chain distribution, personal belongings finding, health care/monitoring, etc., the outdoor inventory/command/sensor/positioning should be considered.

- In addition to outdoor scenarios, whether further enhancements on indoor use cases are needed or not depend on Rel-19 A-IoT WI scope, which can be further discussed after the Rel-19 WI objectives are clear.

- Rel-20 A-IoT should support outdoor inventory/command/sensor/positioning. FFS enhancements on indoor scenarios.

Device types

In Rel-19, even though three different device types with different peak power consumption, complexities and capabilities have been studied, the design principle is a harmonized air interface design with minimized differences. Therefore, the market differentiation among the three device types in Rel-19 is unclear.

The Rel-19 A-IoT harmonized design is based on device 1/2a with RF-ED receivers.

Based on the device architectures captured in TR38.769, the device 1/2a are RF-ED receivers, which may be in absence of RF filter and the corresponding envelope detection bandwidth is 20MHz. To mitigate the interference of other A-IoT readers and guarantee the reader-to-device (R2D) performance, the minimum distance between active readers is assumed as 60m in the co-existence evaluations in RAN4.

Furthermore, based on the evaluation of co-existence between A-IoT device 1/2a and NR, it is observed when the legacy NR UEs are not deployed indoor with A-IoT, the co-existence between A-IoT and NR is feasible. Meanwhile, if there are NR UEs indoor as A-IoT, NR throughput degradation or significant SINR degradation for A-IoT system is observed. Additional gNB implementation based techniques are needed to avoid/mitigate the interference.

Therefore, for device 1/2a, the co-existence between A-IoT and NR, different A-IoT systems are based on deployment (indoor/outdoor separation) and gNB implementation. It will result into issues such as low resource utilization/inventory efficiency, complicated coordination between different gNB/reader, etc.

However, in outdoor scenarios, interference from macro cells to devices becomes more serious, leading to significant degradation in R2D performance. It will be challenging to deploy A-IoT with all the restrictions to mitigate the interference caused by co-existence issues. Therefore, solutions to enable better co-existence between A-IoT and another A-IoT/NR are needed, e.g., higher device capabilities with narrower receiver bandwidth/RF filter.

For device 1/2a, the co-existence between A-IoT and NR, different A-IoT systems are based on deployment (indoor/outdoor separation) and gNB implementation.

- Solutions to enable better co-existence between A-IoT and another A-IoT/NR can be considered.

Moreover, based on the coverage evaluation results in TR38.769, the coverage of device 1/2a in D1T1/D2T2 is less than 50m, while the coverage range of device 2b can reach to hundreds of meters. Hence, the device 2b is more appropriate for outdoor scenario in Rel-20.

For device 2a, whether it can be applied in Rel-20 outdoor scenario depends on the aspects listed above, i.e., co-existence and coverage issue.

In addition, device C has been studied in Rel-18 RAN SI, which may have higher capabilities/power consumption than device 2b. The device C can be also considered in Rel-20 if the differentiation with device 2b is clarified.

- At least device 2b can be considered for outdoor scenario in Rel-20.

Enhancements for Rel-20 A-IoT

Coverage/capacity/spectral efficiency enhancements

In Rel-19, the maximum coverage for the indoor scenario is 50m. To support outdoor case, evaluation on coverage and potential enhancements are needed.

Furthermore, there are 600 A-IoT devices in D1T1 InF-DH scenarios. Multiple device inventory completion time has been evaluated and captured in TR 38.769, where the inventory completion time can be tens of seconds based on varied assumptions. For outdoor cases with large coverage, the number of devices in connection will be increased accordingly. Therefore, solutions to further increase the capacity and reduce the inventory completion time should be considered.

In addition, considering the coexistence with NR systems outdoors, higher spectral efficiency is required to reduce the frequency domain resource overhead of A-IoT and minimize the impact on existing NR systems. Therefore, solutions with higher spectral efficiency should be considered. In Rel-19, given the limited capabilities for device 1, only binary modulation, such as OOK in R2D/D2D and BPSK in D2R, is considered. And for R2D, FEC isn’t recommended in R2D. In Rel-20, the enhancements like BPSK/FEC in R2D, higher modulation order such as QPSK at least in D2R, can be considered to improve the spectral efficiency.

- Techniques to increase the A-IoT coverage, capacity, spectral efficiency should be considered in Rel-20 outdoor scenario.

Positioning enhancements

According to the RAN study, the design target of absolute positioning accuracy is several tens of meters @ 90% outdoor location. In Rel-19 SID, the following two objectives are relevant to positioning.

Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).

However, the positioning/locating led by RAN3 may aim to provide a location of reader ID granularity. While for the proximity discussion in RAN1, the following two solutions have been identified.

Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader. Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side. Proximity determination based on device-side measurements is not considered.

Therefore, there is a gap for the location/proximity discussion in Rel-19 and the positioning requirement in RAN design target. Techniques to provide the target positioning accuracy should be considered in Rel-20.

- Techniques to provide the target positioning accuracy in TR 38.848 should be considered in Rel-20.

DO-A traffic

The Rel-19 SI focuses on DO-DTT and DT traffic. It has been discussed about the feasibility to support DO-A traffic via the harmonized design for DO-DTT and DT traffic. The conclusion in RAN1 and RAN2 can be found as below.

From the RAN1 perspective, at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type: For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic.

From RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

From RAN2 perspective, DO-A use case cannot be supported with the current design discussed in the study item.

With the above evaluation and conclusion in both RAN1 and RAN2, we observe that:

DO-A use case cannot be supported with the current design in the Rel-19 AIOT SI.

While we understand the DO-A use case has been widely acknowledged for device to autonomously report environment information such as temperature, humidity, etc. Considering the limited time in Rel-19, we would like to support DO-A in Rel-20.

- Support DO-A traffic type in Rel-20 and specify the corresponding configuration and procedures.

A-IoT performance and efficiency enhancements

Random access

For contention based random access, the device randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader at least for TDMA case, which may lead to lots of conflicts and re-access or multiple rounds of inventory. To address this, we understand deterministic access occasion selection can be considered to reduce access collision in multiple devices paging or group paging cases, e.g., access occasion selection based on device identity.

- Deterministic access occasion selection can be considered in Rel-20 to reduce the access collision for multi-device access case.

Paging

The following scenarios have been acknowledged in the SI:

- Scenario 1: The reader may send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided.

- Scenario 2: Different readers may send A-IoT paging messages, which are associated with the same service request from the CN, to the same device for response.

For the above scenarios, if it is possible to avoid duplicated response from the same device for the same service request, the inventory and command efficiency can be improved while access collision can be alleviated a little bit. There has been discussion in SI phase for these while further design and detailed discussion is still needed. Considering the limited time in Rel-19 WI phase, reducing duplicated response from the same device for the same service request should be considered in Rel-20 if not specified in Rel-19.

- Reducing duplicated response from the same device for the same service request should be considered in Rel-20, if not specified in Rel-19.

Topology 2

We have proposed to specify the following aspects for TP#2 [4]:

- RRC signaling to provision radio resources at the UE reader for A-IoT operation

  - Dedicated signaling in RRC_CONNECTED state for configuring A-IoT radio resources

  - Procedures to enable the UE to release the A-IoT resources upon cell change or moving to out of coverage

  - UE assistance information from the reader to gNB for configuration of A-IoT radio resources

  - As a second priority, support operation of A-IoT whilst the UE reader is in one of RRC_INACTIVE state or RRC_IDLE state (RAN2 to down select between INACTIVE and IDLE states)

If we manage to specify all the above aspects in Rel-19, we still see room for further enhancement to improve the efficiency, e.g. to identify the inventory area or scope of each UE reader and select appropriate UE reader to page device.

- For TP#2, UE reader inventory area management for network to identify the inventory area of each UE reader and select the appropriate UE reader can be considered in Rel-20, if TP#2 is supported and specified in Rel-19.

Conclusion

Based on the above discussion about Rel-20 A-IoT scope, we have the following observations and proposals:

- Observation 1: 	The Rel-19 A-IoT harmonized design is based on device 1/2a with RF-ED receivers.

- Observation 2: 	For device 1/2a, the co-existence between A-IoT and NR, different A-IoT systems are based on deployment (indoor/outdoor separation) and gNB implementation.

- Observation 3: 	DO-A use case cannot be supported with the current design in the Rel-19 AIOT SI.

- Proposal 1:	Rel-20 A-IoT should support outdoor inventory/command/sensor/positioning. FFS enhancements on indoor scenarios.

- Proposal 2:	Solutions to enable better co-existence between A-IoT and another A-IoT/NR can be considered.

- Proposal 3:	At least device 2b can be considered for outdoor scenario in Rel-20.

- Proposal 4:	Techniques to increase the A-IoT coverage, capacity, spectral efficiency should be considered in Rel-20 outdoor scenario.

- Proposal 5:	Techniques to provide the target positioning accuracy in TR 38.848 should be considered in Rel-20.

- Proposal 6:	Support DO-A traffic type in Rel-20 and specify the corresponding configuration and procedures.

- Proposal 7:	Deterministic access occasion selection can be considered in Rel-20 to reduce the access collision for multi-device access case.

- Proposal 8:	Reducing duplicated response from the same device for the same service request should be considered in Rel-20, if not specified in Rel-19.

- Proposal 9:	For TP#2, UE reader inventory area management for network to identify the inventory area of each UE reader and select the appropriate UE reader can be considered in Rel-20, if TP#2 is supported and specified in Rel-19.

References

[1] RP-234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] TR38.769. Study on solutions for ambient IoT (Internet of Things) (Release 19)

[3] TR38.848. Study on Ambient IoT (Internet of Things) in RAN (Release 18)

Appendix

Table 4.1.1-1 in TR38.848: Mapping between RAN representative use cases and SA1 use cases

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable |
| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |