# 3GPP TSG RAN Meeting #106								                                          RP-243165

# Madrid, Spain, December 9-12, 2023

Agenda Item:	15.2

Source:	ZTE Corporation, Sanechips

Title:	Views on SON/MDT in 5G-A Rel-20

Document for:		Discussion and Decision

1. Introduction

The data collection for SON&MDT support for NG-RAN has been specified from Rel-16 to Rel-19[1][2][3][4]. In this contribution, the motivation and scope of the proposed new work item towards Rel-20 is elaborated.

- Discussion

2.1 Enhancement of Rel-20 topics

- MRO for inter-CU LTM

To develop the Rel-20 5G-A SON/MDT functions with a focus on real commercial interests, mobility robustness remains the most critical feature in SON. The new mobility enhancement features introduced in Rel-19, such as inter-CU LTM and conditional LTM, could be further refined to improve MRO. However, given the shorter timeline compared to Rel-19, we believe that only the most essential feature should be prioritized for Rel-20 MRO. As a result, the inter-CU LTM feature will be the sole focus for further enhancement in Rel-20 MRO, while MRO for the conditional LTM function can be deferred to a later release.

For Rel-19, the normative work on mobility enhancement concentrated on intra-CU LTM and the F1 interface between the CU and DU, which was significantly impacted. For inter-CU LTM in Rel-20, however, the Xn interface should be considered to enhance mobility robustness, particularly in scenarios involving L1/L2-triggered mobility events.

Proposal 1: MRO optimization for inter-CU LTM optimization can be considered for normative work in Rel-20.

- Support of SON/MDT enhancements for XR

XR is one of the most significant 5G media applications currently being explored in the industry. The 3GPP has completed a study on XR [5], covering aspects such as capacity, power consumption, and coverage. The findings of this study are detailed in [6]. During the Rel-18 WI phase [7], enhancements related to power saving, capacity improvements, and XR awareness were introduced. The ongoing Rel-19 WI [9] includes additional enhancements, such as enabling transmission/reception during gaps or restrictions and uplink (UL) scheduling to support high XR capacity.

With regard to XR awareness, in addition to the PDU set concept defined earlier, 3GPP has also introduced multiple enhancements at the Layer 2 level to optimize uplink resource scheduling. After two releases, XR functionalities have become more mature and stable, generating increased interest in deploying these technologies. Operators may also be interested in evaluating the quality of XR services in specific areas. To support this, SON/MDT can be utilized to evaluate XR performance in Rel-20.

Proposal 2: SON/MDT enhancements for XR can be considered for normative work in Rel-20.

- Support of SON/MDT enhancements for RACH optimization

The RACH optimization function has evolved over several releases within the scope of SON/MDT topics. Using information from RA reports, the network can optimize the configuration of RACH resources. In Rel-19, SBFD (Sub-band Full Duplex) introduced a new partitioning of RA resources based on the SBFD mode. RACH resources dedicated to SBFD mode cannot be utilized if the SBFD function is not supported by the UE.

Additionally, since the calculation of RACH occasions differs from traditional RACH configurations, further RA optimization could be developed specifically for the dedicated SBFD RA configuration.

Proposal 3: RACH optimization for dedicated SBFD considered for normative work in Rel-20.

3. Conclusion

In this document, the motivation and the potential topics for data collection for SON&MDT in NR and MR-DC towards Rel-20 are described and the proposals as follows:

Proposal 1: MRO optimization for inter-CU LTM optimization can be considered for normative work in Rel-20.

Proposal 2: SON/MDT enhancements for XR can be considered for normative work in Rel-20.

Proposal 3: RACH optimization for dedicated SBFD can be considered for normative work in Rel-20.

Proposal 4: RAN plenary to consider the following objectives for SON/MDT in 5G-A Rel-20.

Objectives:

- MRO enhancement for R19 mobility mechanisms, including Inter-CU LTM [RAN3, RAN2]:

  - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

  - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

- Support of SON/MDT enhancements for the following features [RAN3, RAN2]:

  - XR (e.g  SON/MDT enhancements to support PDU set related measurements)

  - RA optimization (e.g SBFD dedicate RACH resource optimization)

- Support of the leftovers in Rel-19 SON/MDT, if any

If needed, co-operate with RAN1, SA2, SA5, CT

4. Reference

[1]  RP-192610, WID on SON/MDT support for NR

[2]  RP-201281, WID on enhancement of data collection for SON/MDT in NR and EN-DC

[3]  RP-221825, WID on further enhancement of data collection for SON/MDT in NR and MR-DC

[4] RP-234038, Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[5] RP-210460 SID on XR Evaluations for NR

[6] TR 38.838 Study on XR (Extended Reality) Evaluations for NR

[7] RP-230786 R18 WID on XR Enhancements for NR

[8] RP-241771 R19 WID on XR (eXtended Reality) for NR Phase 3