3GPP TSG RAN Meeting #106						   RP-243174

Madrid, Spain, December 9-12, 2024

Agenda Item:	9.3.2.2

Source:	Apple

Title:	On Rel-19 NR NTN

Document for:	Discussion/Decision

# Introduction

The Release-19 NR NTN enhancement work item was approved in RAN #102 meeting [1] and updated in RAN #105 meeting [2].

One objective of Release-19 NR NTN is downlink coverage enhancements, with the following scope:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

It is clear that the check point for this objective is at RAN# 106 meeting.

In this contribution, we present our views on the topics of downlink coverage enhancements.

# Discussion

One of the objectives in Release-19 NR NTN [2] is to study and if needed specify solutions for downlink coverage enhancement. This includes link level enhancements for FR1-NTN and/or system level enhancements for FR1-NTN and/or FR2-NTN.

In RAN1 discussions, it was agreed [3], [4] that

RAN1# 118bis Agreement Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3. - The following techniques are for further study: - PDCCH repetition, including: - Option 1: Intra-slot PDCCH repetition - Option 2: Inter-slot PDCCH repetition - CORESET length (i.e. number of OFDM symbols) extension - DCI format optimization (e.g. size reduction, etc) - Note: the same technique is intended to apply to all search space types targeted for link level enhancements - For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques: - Configuration - Backward compatibility and UE behaviour of legacy UE - Linked Search Space - Blocking probability - DCI format size budget - Resource overhead - Impact on CORESET0 - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS RAN1 #119 Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. RAN1 #119 Agreement For link level enhancement of PDSCH with SIB1: - Support PDSCH repetitions within 20 ms duration - The number of repetitions is fixed to 2 repetitions - Further discuss the specification impact for at least the following: - Procedure and signaling (enabling repetitions, associated time resource determination, etc.) - Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size. - Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. - Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern. RAN1 #119 Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. - FFS: intra-slot and/or inter-slot

It is clear that RAN1 has made very good progress on the study of link level enhancements for FR1-NTN. Based on the above agreements, PDCCH CSS (except for Type-3) coverage enhancements via PDCCH repetition needs to be specified, where inter-slot PDCCH repetition and/or intra-slot PDCCH repetition could be considered. Also, the coverage of PDSCH with Msg4 needs to be enhanced via PDSCH repetition. Finally, PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration needs to be specified. Hence, we have the following proposal.

Proposal 1: NR NTN Rel-19 to specify the link level coverage enhancement for the following physical channels:

  - PDCCH CSS (except for Type-3) via PDCCH repetition

  - PDSCH with Msg4 via PDSCH repetition

  - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

In RAN1 discussions, it was agreed [3], [4] that

RAN1 #118bis Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms RAN1 #119 Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

It is clear that RAN1 has made very good progress on the study of system level enhancements. It was agreed in RAN1 to support SSB periodicity extension, where the maximum of additional default value is at least 160 ms.

The above observation addresses the question on backward compatibility of SSB periodicity extension. Since legacy UEs assumes a default SSB periodicity of 20 ms, it is not guaranteed for legacy UEs to access the cell when SSB periodicity is larger than 20 ms. On the other hand, increasing the default SSB periodicity will reduce cell search success rate, increase UE’s cell search complexity and latency. Hence, to avoid the impact on UEs not supporting NR NTN features, we think the SSB periodicity extension should be restricted to NTN bands only.

Based on the RAN1 agreement, we have the following proposal.

Proposal 2: NR NTN Rel-19 to specify the system level coverage enhancement of SSB periodicity extension, which is applicable to NTN bands only.

Besides SSB periodicity extension, RAN1 considered another potential system level coverage enhancement, specifically, transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint.

RAN1 #118 Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

In RAN1 study [5], a feature lead proposal on this topic was made, but not agreed. In the proposal, there are 3 options to enable the operation of wider beam for DL common channels and narrower beam for dedicated channels. The first two options have specification impact, while the third option does not have specification impact.

FL proposal in [5] : Regarding wider beam for SSB/ transmitting the DL common channels, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint, RAN1 further studies the following candidate options for enabling this operation - Option 1 -Network broadcasts a set of ROs associated with each narrow beams, and the UE chooses the RO based on the indicated information (e.g. reference Location, or elevation angle of reference Location) of the RO and its own location - Option 2- Network indicates narrow beam RS (CSI-based or NCD-SSB) and associated ROs, and the UE selects the RO based on measurements on indicated narrow beam RS - Option 3 - Network implementation based on existing specification e.g. network detection based on PRACH repetitions. No spec change.

Although the first two options may save RACH resource or reduce the collision probability of PRACH transmissions, the third option (based on network implementation without specification impact) is already able to support the operation. Given the limited RAN1 NTN TU, we slightly prefer Option 3, which does not require specification work to enable the operation.

Overall, we think RAN plenary guidance is needed on whether or not to study/specify for the operation of wider beam for DL common channels and narrower beam for dedicated channels.

Proposal 3: RAN plenary guidance is needed on whether or not to study/specify for the operation of wider beam for DL common channels and narrower beam for dedicated channels.

# Conclusion

Based on the discussions above, we have the following proposals.

Proposal 1: NR NTN Rel-19 to specify the link level coverage enhancement for the following physical channels:

  - PDCCH CSS (except for Type-3) via PDCCH repetition

  - PDSCH with Msg4 via PDSCH repetition

  - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

Proposal 2: NR NTN Rel-19 to specify the system level coverage enhancement of SSB periodicity extension, which is applicable to NTN bands only.

Proposal 3: RAN plenary guidance is needed on whether or not to study/specify for the operation of wider beam for DL common channels and narrower beam for dedicated channels.

# Reference

- RP-234078, “New WI: Non-Terrestrial Networks (NTN) for NR Phase 3,” Dec. 2023.

- RP-241798, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Sep. 2024.

- Chairman’s Notes, 3GPP TSG RAN1 #119 meeting, Nov. 2024.

- Chairman’s Notes, 3GPP TSG RAN1 #118bis meeting, Oct. 2024.

- R1-2409438, “FL Summary #4: NR-NTN downlink coverage enhancements,” Nov. 2024.