3GPP TSG RAN Meeting #106								RP-243214

Madrid, Spain, December 9-12th, 2024

Agenda Item:	9.1.2

Source:	Ericsson, AT&T, Swift Navigation, FirstNet

Title:	Introduction of Regional Coordinate Reference Systems

Document for:	Discussion, Decision

# 1	Global vs Regional Coordinate Reference Systems

A geodetic coordinate reference system defines coordinates in relation to the earth – a datum. Geodetic coordinates in 3GPP are defined in WGS 84 and the most recent versions of WGS84 are aligned to the ITRF datum. This means that they are well defined globally. However, nationally and regionally, coordinates are defined in different datums – reference surfaces that represent the specific region well - to ensure that coordinates match regional and national maps, considering tectonic plate movements horizontally and vertically.

For example, the North American Datum, NAD83, differs from ITRF by about 2 meters, and the European Terrestrial Reference Frame, ETRF89, differs from ITRF by about 1 meter. Such a difference was negligible when positioning accuracy requirements were rather loose, which was the case when 3GPP decided to represent coordinates only in WGS84/ITRF. However, today, positioning accuracy requirements are much tighter and the positioning capabilities much better – plentiful of use cases rely on ca few centimeters precision in relation to maps, e.g. TR 22.872, TR 38.857. Therefore, it is highly relevant that the coordinate representation in 3GPP becomes more flexible with alternatives of coordinates in regional coordinate reference systems to adapt to use case needs and regional maps. Indeed, it is also important to know which version (epoch) of WGS84 has been adopted in the 3GPP network implementation to enable translations through time at global and regional scales.

3GPP LPP introduced support for assistance of high accuracy Global Navigation Satellite Systems (GNSS) in Rel 15 based on RTCM SC 104 v.3.3 [1], released in October 2016, to enable centimeter precision. Since then, 3GPP Rel 16 and 17 has introduced support for decimeter precision cellular 5G positioning, and Rel 17 and 18 has introduced support for integrity to define error sources and mechanisms to assess positioning reliability.

The current 3GPP RAN support for only WGS84 CRS has the following shortcomings:

- Coordinates only defined in WGS84 without an associated epoch time are not well-defined.

- Site surveys of TRP antenna locations are typically performed in regional CRS so the network (RAN OAM or E-SMLC/LMF) needs to translate these coordinates to WGS84.

- Regional GNSS corrections providers typically provides data in regional CRS, so either they need to instead provide data in WGS84 or network needs to transform the reference station coordinates from regional CRS to WGS84.

- UEs needs to transform its estimated position in WGS84 to a regional CRS in order to align to maps.

- RTCM has the ability to align all served devices by data in a regional CRS or via a signaled CRS transformation in an eco-system, while 3GPP LPP will not be able to align devices causing a reliability risk with potentially different understandings of the CRS and relations to maps.

3GPP RAN only representing coordinates in WGS84 implies extensive coordinate transformations in different nodes, and it is not possible to remain within a regional CRS that is the established CRS in a region/nation.

Coordinate transformations in network and devices without a common set of transformation parameters implies a reliability and integrity risk

RTCM SC 104 v3.4 [2] was released in December 2023, and it features messages to represent regional and national coordinate reference systems (CRS) and parameterized coordinate transformation formulas to enable conversion between WGS 84 and regional/national CRS that are specific to countries and parts of countries that will stay relevant over long time.

RTCM has published messages for regional CRS identification and CRS transformation parameters that 3GPP RAN can leverage to introduce support for regional CRS and CRS transformations.

It is therefore timely to make sure that 3GPP RAN, leveraged by RTCM [2], also supports regional CRS and CRS transformations which are  relevant for both RAT-dependent and RAT-independent positioning methods including handling of location coordinates in LPP, LPPa, NRPPa, and RRC A short Rel 19 work item is also very suitable, ensuring support for both 4G/LTE and 5G/NR via LPP, while also being ready with the 3GPP representation to be transferred to 6G.

Benefits with regional CRS and CRS transformations support in 3GPP RAN are summarized below.

- 3GPP coordinate representations can be aligned with regional maps and avoid the meter level difference between regional maps and WGS84/ITRF

- 3GPP LPP assistance data representation will be on par with RTCM – industry standard for representing GNSS assistance data in many verticals.

- 3GPP LPP assistance data representation will also be on par with the representation of the data in regional CRS available from GNSS correction providers in a region/country.

- 3GPP distributed regional CRS or CRS transformations will align coordinate representation and avoid different understandings and assumptions of what the coordinates represent and how they should be converted on the device application layer – a difference in understanding that otherwise would be a positioning error source impacting positioning reliability.

- 3GPP regional CRS coordinate representation will mean that the mobile network operator will be able to represent base station antenna coordinates in a CRS that will remain relevant and associated to maps over long time and would require the base station antenna coordinates to be re-determined regularly causing higher OPEX.

Introduce support for regional CRS and CRS transformations in 3GPP RAN.

Expected work to introduce support for regional CRS and CRS transformations:

- A small WI led by RAN2 over three meetings (6 months)

- Impacted RAN working groups:

  - RAN2 for stage 2 (36.305, 38.305), stage 3 LPP (37.355) and stage 3 RRC (for posSIBs in 36.331, 38.331) updates

  - RAN3 for related NRPPa and LPPa updates

Therefore, we have the following proposal for RAN plenary to agree the draft WID [3] about the introduction of support for regional CRS and CRS transformations.

Approve the draft WID in [3].

# 4	Conclusion

We propose the following:

Proposal 1	Introduce support for regional CRS and CRS transformations in 3GPP RAN.

Proposal 2	Approve the draft WID in [3].

# References

- RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

- RTCM Standard 10403.4: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, December 1, 2023.

- RP-243215, Introduction of Regional Coordinate Reference Systems, Ericsson