3GPP TSG RAN Meeting #106	Draft RP-243312

Madrid, Spain, December 9-12, 2024

NOTE: RAN specific additions are added in blue.

- Source:	RAN1 Vice-chair (Huawei)

- Title:	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- Document for:	Approval

Agenda Item:	9.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Solutions for Ambient IoT (Internet of Things) in NR

Acronym: Ambient_IoT_Solutions

Unique identifier:	xxxxx

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access | X |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  | X |
| Don't know | X |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

{The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_Ambient_IoT_RAN | RAN | 970078 | Study on Ambient IoT in RAN |
| FS_ Ambient_IoT_Solutions | RAN1 | 1020085 | Study on solutions for Ambient IoT (Internet of Things) in NR |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| FS_AmbientIoT | 950004 | Study on Ambient power-enabled Internet of Things | Related SA1 study item |
| FS_AmbientIoT | 1020071 | Study on Architecture support of Ambient power-enabled Internet of Things | Related SA2 study item |
| FS_Ambient_IoT_Sec | 1030031 | Study on Security Aspect of Ambient IoT Services in 5G | Related SA3 study item |
| TBA | TBA | To be added if/when approved | Related SA2 work item, if/when approved |
| TBA | TBA | To be added if/when approved | Related SA3 work item, if/when approved |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

# 3	Justification

In recent years, IoT has attracted much attention in the wireless communication world. More ‘things’ are expected to be interconnected for improving productivity efficiency and increasing comforts of life. Further reduction of size, complexity, and power consumption of IoT devices can enable the deployment of tens or even hundreds of billion IoT devices for various applications and provide added value across the entire value chain. It is impossible to power all the IoT devices by battery that needs to be replaced or recharged manually, which leads to high maintenance cost, serious environmental issues, and even safety hazards for some use cases (e.g., wireless sensor in electric power and petroleum industry).

Most of the existing wireless communication devices are powered by battery that needs to be replaced or recharged manually. The automation and digitalization of various industries open numbers of new markets requiring new IoT technologies of supporting batteryless devices with no energy storage capability or devices with energy storage that do not need to be replaced or recharged manually. The form factor of such devices must be reasonably small to convey the validity of target use cases.

TR 22.840 and TS 22.369 have been developed by SA1 to capture use cases, traffic scenarios, device constraints of ambient power-enabled Internet of Things and identify new potential service requirements as well as new KPIs. SA1 considered devices being either battery-less or with limited energy storage capability (i.e., using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable.

Considering the limited size and complexity required by practical applications for batteryless devices with no energy storage capability or devices with limited energy storage that do not need to be replaced or recharged manually, the output power of energy harvester is typically from 1µW to a few hundreds of µW. Existing cellular devices may not work well with energy harvesting due to their peak power consumption of higher than 10mW.

An example type of application in TR 22.840 and TS 22.369 is asset identification or inventory, which presently has to resort mainly to barcode and RFID in most industries. The main advantage of these two technologies is the ultra-low complexity and small form factor of the tags. However, the limited reading range of a few meters usually requires handheld scanning which leads to labor intensive and time-consuming operations, or RFID portals/gates which leads to costly deployments. Moreover, the lack of interference management scheme results in severe interference between RFID readers and capacity problems, especially in case of dense deployment. It is hard to support large-scale network with seamless coverage for RFID.

TSG RAN has completed a Rel-18 RAN-level SI on Ambient IoT, which provides a terminological and scoping framework for future discussions of Ambient IoT. This has defined representative use cases, deployment scenarios, connectivity topologies, Ambient IoT devices, design targets, and required functionalities. For selected options among those, RAN WGs have completed a Rel-19 SI on solutions for Ambient IoT, reported in TR 38.769, which explored more-detailed radio aspects. Following those study items, it is now proposed to begin specification of Ambient IoT, according to the scope in Section 4. Since existing technologies cannot meet all the requirements of target use cases, a new IoT technology is recommended to open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies. The new IoT technology shall provide complexity and power consumption orders of magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC), and shall address use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power.

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor..

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

- Proximity determination via Solution 1 in TR 38.769 only.

- Device (un)availability via Direction 1 in TR 38.769 only.

WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope:

The following objectives are set, within the General Scope:

- RAN1 scope:

  - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

  - R2D and D2R signal(s)

  - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

  - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

  - R2D transmission supports only the Manchester line code in TR 38.769

  - D2R transmission supports:

    - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and

    - A corresponding small frequency shift method according to the options in TR 38.769.

  - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use another generator polynomial by RAN1#120bis).

  - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use another generator polynomial by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1.

  - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

- RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support at least the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in paging message, for forward compatibility purposes

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1

  - Define OTA test methodology for devices

  - Study and develop OTA test methodology for A-IoT device 1

    - Use test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

Note 1: Coordination with SA2 and SA3 is expected. Updates to the WID objectives should be considered if needed.

Note 2: This WI shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The WI shall not aim to replace existing 3GPP LPWA technologies.

### 4.2	Objective of Performance part WI

- Specify the necessary conformance testing for Ambient-IoT BS

- Specify test methodology for device 1

- Specify the necessary RRM performance requirements for device 1

- Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#109 | RAN4 Core part Note: including RF requirements and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) radio transmission and reception |  | RAN#109 | RAN4 Core part |
| New TS | TS 38.xxx | Requirements for Ambient IoT device |  | RAN#111 | RAN4 Performance part Note: including RF requirements, demodulation and RRM requirements (if any) |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) radio transmission and reception |  | RAN#111 | RAN4 Performance part |
| New TS | TS 38.xxx | Ambient IoT Base Station (BS) conformance testing |  | RAN#111 | RAN4 Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.201 | NR; Physical layer; General description | RAN#108 | Core part |
| TS 38.202 | NR; Services provided by the physical layer | RAN#108 | Core part |
| TS 38.211 | NR; Physical channels and modulation | RAN#108 | Core part. Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.212 | NR; Multiplexing and channel coding | RAN#108 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.213 | NR; Physical layer procedures for control | RAN#108 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.214 | NR; Physical layer procedures for data | RAN#108 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#109 | Core part |
| TS 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#109 | Core part Note: whether a new specification in 38 series will be used instead will be decided at RAN#107 |
| TS 38.401 | NG-RAN; Architecture description | RAN#109 | Core part |
| TS 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#109 | Core part |
| TS 38.410 | NG-RAN; NG general aspects and principles | RAN#109 | Core part |
| TR 38.870 | OTA test method | TSG #109 | Core part |
| TR 38.870 | OTA method for performance requirements | TSG #111 | Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

TBD.

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

Lead WG: RAN1

Secondary WGs: RAN2, RAN3, RAN4.

# 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

SA2: Architecture aspects

SA3: Security aspects

# 9	Supporting Individual Members

Supporting IM name