3GPP TSG RAN Meeting #106	RP-243262

Madrid, Spain, December 9-13, 2024	rev from RP-242464, 1789, 1667

- Source:	Thales, CATT

- Title:	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- Document for:	Approval

Agenda Item:	9.3.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Non-Terrestrial Networks (NTN) for NR Phase 3

Acronym: NR_NTN_Ph3

Unique identifier:	1020097

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:  Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  | X |
| Don't know | X |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
| X | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| FS_NR_nonterr_nw | 750040 | Study on NR to support non-terrestrial networks | The present WID will make use of the channel model defined by the FS_NR_nonterr_nw study |
| FS_NR_NTN_solutions | 800099 | Study on solutions for NR to support non-terrestrial networks | The present WID will leverage solutions identified in FS_NR_NTN_solutions to address some of the key issues associated to NTN |
| 5GSAT_ARCH | 890034 | Integration of satellite systems in the 5G architecture | The present WID can consider the QoS management and Satellite NG-RAN global identities solutions identified in 5GSAT_ARCH. |
| NR_NTN_solution | 860046 | Solutions for NR to support non-terrestrial networks | The present WID will use this WID as basis for the evolution. |
| FS_NR_NTN_netw_verif_UE_loc | 960091 | Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR | The present WID will use this WID as basis for the evolution. |
| NR_NTN_enh | 941006 | NR NTN (Non-Terrestrial Networks) enhancements | The present WID will use this WID as basis for the evolution. |
| NR_NTN_CBW_30MHz | 1000060 | 30 MHz Channel Bandwidth for NR NTN (non-terrestrial networks) in FR1 (frequency range 1) | The present WID will use this WID as basis for the evolution. |
| NR_NTN_LSband | 981043 | Introduction of the satellite L-/S-band for NR | The present WID will use this WID as basis for the evolution. |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

In Release 17, a work item was carried out to define solutions enabling New Radio and NG-RAN to support Non-Terrestrial Networks (NTN). Then, Release 18 introduced enhancements for NR NTN. As part of Release 19, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks in order to:

- Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS.

DL coverage enhancements can be considered at both

- Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design.

- System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

- Offer optimized capacity performance on uplink through multiplexing techniques, motivated by:

- The coverage of NTN satellites is very wide, and considering device density, it is expected that a large number of UEs will be within a satellite’s coverage. Especially for LEO, a large number of UEs in coverage must succeed in transmitting desired data during a satellite coverage which means that rapid access to and release of satellite resources is required.

- The total spectrum resources available to the network will be limited especially in the early phases of NR NTN deployments.

- Some users will require higher resources than others, depending on their traffic patterns. Therefore, further granularity of resource multiplexing can significantly improve system capacity efficiency.

- Possibly to allocate higher per-UE resources to better support VoNR/VoIP services in coverage-limited scenarios.

- MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service.

- Support non-terrestrial network architecture with 5G system functions on board the NTN vehicle (i.e. regenerative payloads).

Support of non-terrestrial network architecture with 5G system functions on board the NTN vehicle (i.e. regenerative payloads) provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. Support of regenerative payload brings some benefits on radio resource handling in Uu, and radio resource coordination between the gNBs via the ISL. For the support of real time connectivity between 2 UEs and between network and UE via the space segment with/without ISL, a regenerative payload (5G system functions on board satellite) is required.

- Address RedCap UE within FR1 NTN

The support of RedCap devices (e.g. handheld and IoT) operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices. Global coverage would clearly benefit RedCap devices. RF and RRM requirements were defined for RedCap devices only for terrestrial networks in releases 17 and 18.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions:

- GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- UEs with GNSS (Global Navigation Satellite Systems) capabilities

- In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO

- Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant

Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion).

The objectives of the work item are the following:

- tSpecify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Specify solutions, including link level enhancements for FR1-NTN  and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - Link level enhancements are to be specified for the following channels:

    - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

    - PDSCH with Msg4 via PDSCH repetition

    - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

  - System-level enhancements are to be specified for the following:

    - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

      - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms

    - Check point at RAN#107 on the below discussions

      - Whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

      - Consider (if beneficial) enhancements on Cell DTX/DRX

      - Potential DL coverage enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

  - Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used

  - Specify necessary signalling

  - Update RF requirements accordingly, if needed

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

- Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

- Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

- For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4]

- For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1]

  - Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

    - Semi-statically configured DL reception collides with semi-statically configured UL transmission

    - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

  - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution.

- Notes for this objective:

  - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

The Performance Part objectives of the work item are the following:

- Specify RRM performance requirements [RAN4]

- Specify UE demodulation requirements [RAN4]

- Specify SAN demodulation requirements [RAN4]

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | NR; Physical channels and modulation | RAN#108 | Core part |
| 38.212 | NR; Multiplexing and channel coding | RAN#108 | Core part |
| 38.213 | NR; Physical layer procedures for control | RAN#108 | Core part |
| 38.214 | NR; Physical layer procedures for data | RAN#108 | Core part |
| 38.215 | NR; Physical layer measurements | RAN#108 | Core part |
| 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#109 | Core part |
| 38.304 | NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state | RAN#109 | Core part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#109 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification: | RAN#109 | Core part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#109 | Core part |
| 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#109 | Core part |
| 38.423 | NG-RAN; NG-RAN; Xn Application Protocol (XnAP) | RAN#109 | Core part |
| 38.101-5 | NR; User Equipment (UE) radio transmission and reception, part 5: Satellite Access Radio Frequency (RF) and performance requirements | RAN#109 | Core part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#109 | Core part |
| 38.108 | NR; Satellite Access Node radio transmission and reception | RAN#109 | Core part |
| 38.101-5 | NR; User Equipment (UE) radio transmission and reception, part 5: Satellite Access Radio Frequency (RF) and performance requirements | RAN#111 | Perf part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#111 | Perf part |
| 38.181 | NR; Satellite Access Node conformance testing | RAN#111 | Perf part |
| 38.863 | NR; Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects | RAN#109 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Chuberre, Nicolas, nicolas.chuberre@thalesaleniaspace.com

Sun, Jiancheng, sunjiancheng@catt.cn

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN2

Secondary responsible Working Group(s): RAN1, RAN3, RAN4.

# 8	Aspects that involve other WGs

None so far.

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

Thales

Airbus

Apple

Baicells

BBC

CAICT

CATT

CEWiT

China Telecom

China Unicom

CHTTL

CMCC

Dell technologies

Deutsche Telekom

Dish

Ericsson

ESA

ETRI

Eutelsat Group

FGI

FirstNet

Fraunhofer IIS

Fraunhofer HHI

Fujitsu

Gatehouse Satcom

Gilat

HiSilicon

Hispasat

HONOR

Huawei

Hughes Network systems Ltd

III

Inmarsat

Intel Corporation

InterDigital

Iridium Satellite LLC

ITL

ITRI

Keysight Technologies

KT Corp.

Lenovo

LG Electronics

Ligado

MediaTek

MITRE

Motorola Mobility

NEC

New H3C

Nokia

Nokia Shanghai Bell

Novamint

NTT DOCOMO

OPPO

Panasonic

Qualcomm

Ruijie Networks

Samsung

Sanechips

Sateliot

Semtech

SK Telecom

Sharp

Sony

Spreadtrum

TCL

Tejas Networks

TOYOTA InfoTechnology Center

Transsion Holdings

Viasat

Vivo

Xiaomi

ZTE