3GPP TSG-RAN RP #106	RP-243234

Madrid, Spain, December 9-12, 2024

Agenda item:	9.3.4.1

Source: 	T-Mobile USA, Verizon, CMCC, BT plc, China Telecom, SK Telecom, Deutsche Telekom, Boost Mobile Network, NTT Docomo, Telecom Italia, AT&T

Title: 	Six Layer Support for Devices with 6Rx in Rel-19

Document for:	Discussion

# Introduction

In this paper we provide our view of 6Rx and 6L for handheld and FWA devices.

# Discussion

RAN4 discussed in several meetings whether six MIMO layers for handheld devices are feasible in downlink direction.

- Several companies provided measurements of the envelope correlation coefficients (ECC) of realistic antenna layouts in a handheld formfactor [3], [4], [5]

- Two infra vendors confirmed that for the feasibility analysis low TxEVM can be assumed [10], [11].

- Multiple companies showed in simulation results that 5 layer and 6 layer provide gains over 4 layers for 6Rx antennas for these ECC and TxEVM values at achievable SNR operation points [3], [4], [6], [7], [8].

- One company provided field results showing projected throughput gains of 6 layers and 6Rx over 4 layers [3].

In addition to this feasibility analysis of some companies, it was recommended by many companies to introduce the support of 6 layers as an optional feature in Rel-19 [1], [2], [3], [6], [9], [12]. Hence, it is left to the eco system whether six layers is supported. Based on the feasibility analysis and since 6 layers support would be an optional feature, 6 layers support for 6Rx antennas should be introduced in Rel-19 both for handheld and FWA devices. Given the status of the discussion in RAN4, no further progress can be made, and plenary should decide to introduce 6 layers support as an optional feature for handheld and FWA devices. Since in TS 38.101-4, demodulation performance requirements are already defined for 8Rx and 8 layers, there is no reason why not also 6 layers can be introduced.

Proposal 1: Introduce six layers for FWA devices as an optional feature in Rel-19.

Proposal 2: Introduce six layers for handset devices as an optional feature in Rel-19.

# Conclusions

In this paper we provided our view of 6Rx and 6L for handheld and FWA devices. Our proposals are:

Proposal 1: Introduce six layers for FWA devices as an optional feature in Rel-19.

Proposal 2: Introduce six layers for handset devices as an optional feature in Rel-19.

# References

- R4-2411525, Discussion on MIMO layer evaluation for 6Rx UE, Nokia, RAN4 #112, August 2024

- R4- 2411609, MIMO layer evaluation for 6Rx UE, Xiaomi, RAN4 #112, August 2024

- R4-2411680, Discussion on the demodulation performance requirements for 6Rx devices, Qualcomm, RAN4 #112, August 2024

- R4-2412352, R19 Simulation results of 6Layer performance, OPPO, RAN4 #112, August 2024

- R4-2412572, On MIMO layer evaluation for 6Rx UE, Huawei, RAN4 #112, August 2024

- R4-2413269, Discussion on the MIMO layers for 6Rx UE, Ericsson, RAN4 #112, August 2024

- R4-2414985, R19 Simulation results of 6Layer performance, OPPO, RAN4 #112bis, October 2024

- R4-2415647, Discussion on the demodulation performance requirements for 6Rx devices, Qualcomm, RAN4 #112bis, October 2024

- R4-2416508, 6-Layer downlink MIMO for handheld UEs, T-Mobile USA, RAN4 #112bis, October 2024

- R4-2417559, Discussion on MIMO layer evaluation for 6Rx UE, Nokia, RAN4 #113, November 2024

- R4-2419550, Discussion on the MIMO layers for 6Rx UE, Ericsson, RAN4 #113, November 2024

- R4-2418056, Six Layer Support for Devices with 6Rx in Rel-19, T-Mobile USA, Verizon, CMCC, BT plc, China Telecom, SK Telecom, Deutsche Telekom, Vodafone, Boost Mobile Network, NTT Docomo, Telecom Italia, AT&T, RAN4 #113, November 2024