3GPP TSG RAN WG1 #118bis     													                     R1-2408294

Hefei, China, October 14th – 18th, 2024

Source: 	Intel Corporation

Title:	Enhancements for asymmetric DL/UL scenarios

Agenda item:	9.2.4

Document for:	Discussion and Decision

# Introduction

In NR MIMO Phase 5 WI, the following motivation and objective have been identified [1]:

Motivation:

Heterogeneous Network can be deployed to improve UL throughput. Since the macro gNB and micro nodes differ in power rating, a UE may receive DL transmission from the macro gNB, but transmit UL to either the macro gNB or non-co-located micro nodes in order to maximize UL throughput. As an option to further reduce energy consumption, the micro nodes can, for instance, reduce or even turn off DL transmissions. To support such deployment scenario, enhancements on UL power control (PC) are needed at least. First, when pathloss RS is transmitted from the macro gNB and the UE transmits UL to the micro nodes, the pathloss measured from the pathloss RS from the macro gNB is not accurate. Therefore, it is necessary to configure the UE with pathloss offset to facilitate accurate calculation of the pathloss associated with the micro nodes. Second, an additional SRS closed-loop PC for DL CSI acquisition to the macro gNB (for DL transmission), separate from that for the SRS to the micro nodes (for UL mTRP reception) should be introduced. Therefore, there is a need for supporting two closed-loop PC adjustment states for SRS, both separate from PUSCH.

Objective:

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In the contribution, we present our views on the support for asymmetric DL/UL scenarios, focusing on the remaining issues in Rel-19.

# Enhancement topics for asymmetric DL/UL

In the following we discuss the enhancement topics for supporting asymmetric DL/UL transmission according to the scenarios and transmission schemes described above.

## Pathloss offset

Indicating PL offset for PDCCH-order PRACH:

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3: - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

In RAN1#118 it was agreed that a PL offset for PDCCH-order PRACH is indicated using Alt-3. The detailed design of the signaling is FFS. We think a 1-bit indicator in DCI is needed to select one of two indicated joint/UL TCI state would be needed. The existing PRACH association indicator can be re-purposed for this.

The agreement in RAN1#118 is for FR1 – in order to extend this agreement to FR2 we expect to discuss two additional issues – 1) UE Tx beam determination 2) Enabling 2 TA that is critical for FR2 due to shorter CP lengths.

Proposal-1: A 1-bit indicator in DCI is proposed for indicating a joint/UL TCI state for PDCCH-order PRACH. The existing PRACH association indicator can be re-purposed for this purpose.

Power control formulas for PL offset

In the current PL formulas, we expect that the pathloss term PLb,f,cqd would be adjusted due to PL offset. PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP of carrier 𝑓 of serving cell 𝑐. The exact modification of the equation (if needed) could be handled at a later date. As an example, one option is to simply specify that PLb,f,cqd is replaced by PLb,f,cqd±PLoff where PLoff is the associated PL offset in the indicated TCI-State (or TCI-UL-State) in PUSCH transmission occasion i – this can be specified without modifying the main PL formulas in the specifications.

PHR Triggering conditions

According to 38.321, a PHR is triggered if PL has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell. The path loss variation for one cell is assessed as between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. According to current RAN1 status (see conclusion for proposal 1.6 on UL PL from RAN1#117), “measured PL” applies only to DL and therefore, not impacted by PL offset. Hence, there is no need to change PHR triggering conditions.

Proposal-2: There is no need to change PHR triggering conditions since “measured PL” corresponding to PHR triggering applies only to DL and therefore not impacted by PL offset (according to conclusion for proposal 1.6 of not introducing UL PL from RAN1#117)

Joint TCI state in FR2

Agreement RAN1#116 For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1

It is agreed that a Joint TCI state mode can be configured at least for FR1. We don’t see a reason to extend the applicability of a Joint TCI state (in this context) to FR2 because clearly for a given UE, there is no use-case where an UL-only TRP is co-located with the serving DL/UL TRP.

Proposal-3: Joint TCI state mode should not be configured for FR2 (in which case separate DL/UL TCI state should be used)

## Closed loop PC for SRS

Starting bit of a block in DCI format 2_3

Agreement About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis: - Alt1: X = 45 (to be captured in RAN2 spec) - This feature is a separate UE capability and is appliable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not. - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise - Alt2: X = 44 (to be captured in RAN2 spec) - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states. - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise - Above extended value range is applied to startingBitOfFormat2-3.

Our view is that the applicability of value range extension is not just for asymmetric DL/UL usage but for all UEs monitoring DCI 2_3 with separate CLPC. Note that DCI 2_3 is size matched to DCI 1_0. The max size of DCI 1_0 is 44 bits or 46 bits (in case of shared spectrum or FR2-2). Minimum block size for asymmetric DL/UL usage is 3 bits (CLPC indicator + 2 bits TPC), minimum block size for UEs with single CLPC indicator is 2 bits. Then we observe that

X = 42, if max DCI size = 44 and UE configured with two CLPC adjustment states

X = 43, if max DCI size = 44 and UE configured with single CLPC adjustment state

X = 44, if max DCI size = 46 and UE configured with two CLPC adjustment states

X = 45, if max DCI size = 46 and UE configured with single CLPC adjustment states

Proposal-4: Regarding the starting bit of a block in DCI format 2_3 support Alt1 (value of X=45 can serve all cases)

Scenario with SRS not configured with any TCI state

Agreement - Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state - Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

The premise here is that when the SRS resource set is not provided with followUnifiedTCI-StateSRS and the SRS resource with the lowest ID in that set is not provided with a TCI state, a UE is not able to obtain PL offset or information about CLPC adjustment state. This, we believe, could be avoided by NW implementation and there is no need for additional specifications.

Proposal-5: The case of SRS not associated with any TCI state could be avoided by NW implementation with no additional specifications.

## UTCI enhancements

In heterogenous networks, it is typical to divert high load UL traffic to a LPN (or UL-only TRP) while DL traffic is flowing from the macro to the UE. It is, therefore, natural to consider that a UE dynamically switches between two modes of operation – a joint TCI mode for the macro, supporting DL channels and related UL transmissions (like SRS for DL, PUCCH) and an UL TCI mode for the LPN (or UL-only TRP) supporting PUSCH and SRS for UL. Therefore, we think it is beneficial to support a mix of joint TCI states and UL TCI states in MAC-CE activated DCI codepoints i.e., remove the restriction of RRC configured separate DL/UL TCI mode or joint TCI mode which was introduced in Rel-17.

In order to support dynamic switching of UL transmissions among multiple nodes (including UL-only nodes) with low latency it is proposed to extend Rel-17 UL beam indication capability (which was limited to DL DCI formats in Rel-17) to UL DCI formats for both intra-cell and inter-cell cases.

Proposal-6: It is beneficial to extend beam indication capability to UL DCI formats for both intra-cell and inter-cell cases. Consider supporting a mix of joint TCI states and UL-TCI states in MAC-CE activated DCI codepoints.

## TA enhancements

The following was agreed in RP-242394 in RAN#105 (addition of sub-objective b)

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In order to support UL transmissions to two TRPs with very different coverage (large difference in DL Tx power between the two nodes) a large TA difference is needed. Moreover this TA difference varies as a function of time due to movements in the environment or at the UE. Consequently it is quite difficult for the NW to adjust a single TA to order to handle UL transmissions with such TA differences. A natural solution in this case is to use 2-TA adjustment states at the UE. Note that in Rel-18 a UE is capable of supporting two UL timings (2 TAs) in the same serving cell conditioned on mDCI mTRP capability (both intra-cell and inter-cell cases are supported). The conditioning on mDCI mTRP configuration and capability is artificial and we propose to extend the 2-TA capability for sTRP and sDCI mTRP UL schemes in Rel-19

Proposal-7: Remove the requirement of multiple CORESETPoolIndex configuration for configuration of two TAGs in a serving cell

Note that the Rel-18 2-TA feature is associated with two DL timings. This is shown in DL reference timing linkage to coresetPoolIndex as per RAN4 spec (7.1.1 from 38.133)

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place(NTA+NTA offset)×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the cell associated with a coresetPoolIndex having same TAG as the uplink signal, where NTA is commanded by the network independently for each TAG [TS 38.331].

In Rel-19 we think it is desirable to have a single DL reference timing associated with two uplink timings which is a natural solution for sDCI operation.

Proposal-8: Introduce a single DL timing associated with 2 uplink timings (2 TAGs) so that a UE can determine TA for both TAG-0 and TAG-1 with respect to a single common DL timing

# Conclusions

In this contribution, we presented our views on supporting asymmetric DL/UL transmissions for MIMO. we summarize the proposals below:

Proposal-1: A 1-bit indicator in DCI is proposed for indicating a joint/UL TCI state for PDCCH-order PRACH. The existing PRACH association indicator can be re-purposed for this purpose.

Proposal-2: There is no need to change PHR triggering conditions since “measured PL” corresponding to PHR triggering applies only to DL and therefore not impacted by PL offset (according to conclusion for proposal 1.6 of not introducing UL PL from RAN1#117)

Proposal-3: Joint TCI state mode should not be configured for FR2 (in which case separate DL/UL TCI state should be used)

Proposal-4: Regarding the starting bit of a block in DCI format 2_3 support Alt1 (value of X=45 can serve all cases)

Proposal-5: The case of SRS not associated with any TCI state could be avoided by NW implementation with no additional specifications.

Proposal-6: It is beneficial to extend beam indication capability to UL DCI formats for both intra-cell and inter-cell cases. Consider supporting a mix of joint TCI states and UL-TCI states in MAC-CE activated DCI codepoints.

Proposal-7: Remove the requirement of multiple CORESETPoolIndex configuration for configuration of two TAGs in a serving cell

Proposal-8: Introduce a single DL timing associated with 2 uplink timings (2 TAGs) so that a UE can determine TA for both TAG-0 and TAG-1 with respect to a single common DL timing

# References

- RP-234007, “New WID: NR MIMO Phase 5”, December 2023