3GPP TSG RAN WG1#118bis	        	R1-2408405

Hefei, China, October 14th – 18th, 2024

Agenda Item:					    9.2.2

Source:	Sony

Title:	More views on CSI enhancements

Document for:	Discussion and decision

# Introduction

During the RAN1#118 meeting, regarding CSI enhancements for MIMO, Rel-19, RAN1 agreed the following [1]–[9] :

## Agreements on Type-I/II codebook refinement and CRI-based CSI for up to 128 CSI-RS ports

Agreement: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, reuse the legacy Rel-15 Type-I layer pairing scheme, and fixed mapping between SD basis vectors and layers:

- The k-th SD basis vector is associated with the k-th layer-group.

- Note: This should be the baseline based on joint SD basis selection/indication using the agreed combinatorial indication (logCLGN1N2  bits).

LG denotes the number of layer groups per previous agreement.

- Note: For RI=5-6, LG=3 was agreed, and the additional support for LG=4 is still FFS).

Agreement: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports,

- For Capability 1 timeline:

  - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8.

  - Active resource counting follows the legacy Rel-18 Type-II Doppler.

- For Capability 2 timeline:

  - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS.

  - Active resource counting follows the legacy Rel-18 Type-II Doppler.

Agreement: For the Rel-19 Type-I MP codebook refinements for 48, 64, and 128 CSI-RS ports, regarding OCPU, timeline, active resource counting, and CMR restriction (FDM/TDM, EPRE offset), fully reuse those for the Rel-19 Type-I SP codebook.

Agreement: For the Rel-19 Type-I MP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below:

- Note: The second column includes the location of the parameters when reported with two-part UCI.

| Parameter | UCI | Details/description |
| --- | --- | --- |
| RI | Part 1 | Same as Rel-15 Type-I SP: RI=v={1,2,3,4} |
| Wideband CQI for the first TB | Part 1 | Same as Rel-15 Type-I SP |
| Subband differential CQI for the first TB (*) | Part 1 | Same as Rel-15 Type-I SP |
| First SD basis vector selection indicator | Part 2 Wideband | One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of Rel-15 Type-I SP codebookMode=1 |
| Second SD basis vector selection indicator | Part 2 Wideband | One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I SP codebookMode=1 |
| Inter-pol co-phase selection indicator | Part 2 Wideband or Subband (**) | One (set of) indicator per each of the Ng resources: - Same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I codebookMode=1 |
| Inter-resource co-phasing | Part 2 Wideband | QPSK (2-bit) co-phasing on resource k=2,…,K relative to the first resource, layer-common |

(*): Not included when CQI reporting granularity is set to ‘wideband.’

(**): Wideband when PMI reporting is set to ‘wideband,’ Subband when PMI reporting granularity is set to ‘subband.’

Agreement: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-A for RI=5-8:

- Note: The second column includes the location of the parameters when reported with two-part UCI.

Schemes-A

| Parameter | UCI | Details/description |
| --- | --- | --- |
| First SD basis vector selection indicator | Part 2 Wideband | v=5-8: Decide (by RAN1#118) Alt1 vs Alt2 Alt1. Same as legacy Rel-15 Type-I SP, separate (i1, i2) indicators of log2N1O1 bits and log2N2O2 bits, respectively Alt2: Separate (i1, i2) indicators of log2N1 bits and log2N2 bits, respectively; and separate (q1, q2) indicators of log2O1 bits and log2O2 bits, respectively. |
| Second SD basis vector selection indicator | Part 2 Wideband | v=5-8: Freely select the other nSDBV=2 (v=5-6) or 3 (v=7-8) SD basis vectors such that they are orthogonal in at least one dimension (horizontal or vertical). Decide (by RAN1#118) the indication scheme. |
| Inter-pol co-phase selection indicator | Part 2 Wideband or Subband (**) | v=5-8: Same as legacy Rel-15 Type-I SP |

(*): Not included when CQI reporting granularity is set to ‘wideband.’

(**): Wideband when PMI reporting is set to ‘wideband,’ Subband when PMI reporting granularity is set to ‘subband.’

Agreement: For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32<P (or PCSI-RS) ≤128, for AP-CSI-RS where the K NZP CSI-RS resources are located in two consecutive slots,

- Except for codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not.

- For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not.

Agreement: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths

- Note: Per previous agreements, K is the number of aggregated NZP CSI-RS resources to attain 32<P (or PCSI-RS) ≤128.

Agreement: For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the W2 structure, clarify that:

- Inter-polarization co-phasing is based on legacy Rel-15 Type-I SP codebookMode=1 for PCSI-RS<16.

- QPSK inter-resource co-phasing alphabet is {+1,+j,-1,-j}.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI parameters when two-part UCI/CSI is used:

- Part 1: M CRI(s), M RI(s), M sets of CQI values for 1st CW.

  - When MR is configured, the CRI(s) associated with the MR resource(s) are not reported.

- Part 2: M sets of {PMI, LI (if applicable), CQI values for 2nd CW (if applicable)}.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if MR is configured, the supported value(s) of MR are as follows:

- Rel-15 Type-I SP: {1, 2}

- Rel-16 eType-II: {1}.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding CBSR and RI restriction, support resource-specific specific CBSR.

- FFS (by RAN1#118): Whether RI restriction is resource-common or resource-specific.

Agreement: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding first SD basis vector selection indicator for Scheme-A for RI=5-8, reuse the legacy Rel-15 Type-I SP scheme, i.e. separate (i1, i2) indicators of log2N1O1 bits and ⌈log2 N2 O2⌉ bits, respectively.

Agreement: At least for type 1 single panel, for the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows:

- Consecutive priority levels (higher to lower):

  - Even sub-bands (G1) associated with the first configured CMR among the non-reported MR CRIs,

  - Odd sub-bands (G2) associated with the first configured CMR among the non-reported MR CRIs,

  - …,

  - Even sub-bands (G1) associated with the last configured CMR among the non-reported MR CRIs,

  - Odd sub-bands (G2) associated with the last configured CMR among the non-reported MR CRIs,

  - Even sub-bands (G1) of the 1st reported CRI,

  - Odd sub-bands (G2) of the 1st reported CRI,

  - …,

  - Even sub-bands (G1) of the (M-MR)th reported CRI,

  - Odd sub-bands (G2) of the (M-MR)th reported CRI.

Note: For UCI fields in wideband (G0), even sub-bands (G1), and odd sub-bands (G2), legacy design is fully reused.

Agreement: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-B for RI=5-8:

- Note: The second column includes the location of the parameters when reported with two-part UCI.

Scheme-B

| Parameter | UCI | Details/description |
| --- | --- | --- |
| SD basis oversampling (rotation) factor q1, q2 | Part 2 Wideband | v=5-8: Values of q1, q2 follow Rel-16 eType-II, log2O1O2 bit indicator |
| SD basis vector selection indicator for each layer | Part 2 Wideband | v=5-8: log2N1N2LG bit indicator, where LG=3 for v=5-6, and 4 for v=7-8 |
| Inter-pol co-phase selection indicator for each layer | Part 2 Wideband or Subband (**) | v=5-8: QPSK: 2-bit indicator per layer group l=1, …, LG |

(*): Not included when CQI reporting granularity is set to ‘wideband.’

(**): Wideband when PMI reporting is set to ‘wideband,’ Subband when PMI reporting granularity is set to ‘subband.’

Conclusion: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, conditioned in UE capabilities, a UE can be configured with either the group-based hard CBSR, or the 3-bit SD basis group-based scaling factor, or both, or none of the two.

Agreement: For the 3-bit scaling scheme for Type-I SP codebook, support, in addition, (X1, X2)=(8,1).

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, support resource-specific RI restriction.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, OCPU=KS.

Agreement: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=5-8, the UCI parameters for the selection scheme for the other n_SDBV SD basis vectors (in Part 2 UCI, wideband) are as follows:

- A 1-bit beam group indicator i3∈{0,1}.

- For each of the n_SDBV other selected SD basis vectors,

  - If i3=0:

    - log2N1-bit, log2N2-bit, and log2O2-bit (q2) indicators.

      - In this case, q1=mod(i1,O1) of the first SD basis vector.

  - If i3=1:

    - log2N1-bit, log2N2-bit, and log2O1-bit (q1) indicators.

      - In this case, q2=mod(i2,O2) of the first SD basis vector.

- Note: It is up to the editor how this is captured in the specification

- Note: (q1,q2) is analogous to (q1,q2) for Type-II CSI.

Note (from previous agreement): nSDBV=2 (v= 5-6) or 3 (v= 7-8).

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-1, CSI part-1 is either reported entirely or dropped entirely, with the following UCI packing order:

- CSI part-1 associated with the first configured CMR among the non-reported MR CRIs.

- …

- CSI part-1 associated with the last configured CMR among the non-reported MR CRIs.

- CSI part-1 of the 1st reported CRI,

- …

- CSI part-1 of the (M-MR)th reported CRI.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports based on Rel-16 eType-II, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows:

- Consecutive priority levels (higher to lower):

  - G1 PMI components associated with the first configured CMR among the non-reported MR CRIs,

  - G2 PMI components associated with the first configured CMR among the non-reported MR CRIs,

  - …,

  - G1 PMI components associated with the last configured CMR among the non-reported MR CRIs,

  - G2 PMI components associated with the last configured CMR among the non-reported MR CRIs,

  - G1 PMI components of the 1st reported CRI,

  - G2 PMI components of the 1st reported CRI,

  - …,

  - G1 PMI components of the (M-MR)th reported CRI,

  - G2 PMI components of the (M-MR)th reported CRI.

Note: For UCI fields in wideband (G0), G1 PMI components (based on Rel-16 eType-II CB in Section 6.3.2.1.2, TS 38.212), and G2 PMI components, legacy design is fully reused.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding timeline:

- Multiply legacy Z’ by a factor of 2.

- Multiply legacy Z by a factor of 2.

Agreement: For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, study, for RI= ϑ >1, applying the 3-bit scaling factor(s) as agreed in RAN1#117, where a per-layer scaling factor applied to the ith selected SD basis vector is given by e.g. min{1,ϑsi2ri }, where unit scaling factor “1” is associated with the PDSCH-to-CSIRS EPRE offset “portion” contributed by the  ith selected SD basis vector without the 3-bit scaling factor si configured, e.g. si∈{1, 12, 13  ,14,16,18,112,116}   is the scaling factor associated with the  ith SD basis vector, and ri∈{1,2} is the number of layers transmitted using the  ith SD basis vector.

- Note: This feature is a separate UE capability.

- Study whether per-SD-basis-vector/layer power adjustment (including boosting) needs to be supported in addition.

Agreement: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding resource-specific CBSR,

- For Rel-16 eType-II-based: reuse Rel-16 eType-II CBSR design for each of the KS resources:

  - Just as Rel-18 Type-II CJT CBSR, decouple (N1,N2) from each of the KS CBSR IEs.

  - This implies that it is possible not to configure CBSR for any of the KS resources.

  - Only 1-bit hard CBSR is supported.

- For Rel-15 Type-I-SP-based: reuse Rel-15 Type-I SP CBSR design for each of the KS resources:

  - Just as Rel-18 Type-II CJT CBSR, decouple (N1,N2) from each of the KS CBSR IEs.

  - This implies that it is possible not to configure CBSR for any of the KS resources.

## Agreements on Rel-19 NR CJT calibration reporting

Conclusion: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR and the number of SRS ports (=PSRS) corresponding to the ‘reference UE antenna port,’ there is no consensus to support the following:

- Q>1;

- PSRS>1.

Note: This implies no support for configuring >1 SRS resource sets for associated SRS when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset).

Note: There is a source R1-2407184 showing performance benefit of using multiple UE antenna ports non-coherent with each other.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- The supported value(s) of x follows the supported configuration(s) of SRS resource for antenna switching xTyR used for reciprocity-based DL CSI acquisition.

  - Note: This doesn’t have any spec impact (no new RRC parameter is needed).

- For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of the available port(s)), decide, by RAN1#118, from the following schemes:

  - Scheme1: NW-configured via higher-layer (RRC) signalling.

  - Scheme1B: NW dynamic selection via MAC CE or A-CSI triggering (DCI).

  - Scheme2: UE-selected, the selection included in the phase offset report.

  - Scheme3: Fixed rule, e.g. the first out of the available port(s).

FFS:

- Whether/how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering.

- The supported time-domain behaviour(s) for the associated SRS resource (periodic, semi-persistent, aperiodic).

  - In case the NW configures the Q=1 associated SRS resource from an existing SP and/or AP SRS antenna switching resource configuration for DL CSI acquisition (which utilizes dynamic signalling), whether/how to identify the Q=1 associated SRS resource.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of available port(s)) is NW-configured via higher-layer (RRC) signalling.

Working assumption: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic.

Conclusion: For the Rel-19 aperiodic standalone CJT calibration reporting, there is no consensus to support the following reporting formats:

- Joint FO + wideband PO reporting

- Joint Dd + FO + wideband PO reporting.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO=256.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for delay offset reporting Dn,offset, additionally support MD={128, 256}.

Conclusion: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset and frequency offset reporting FOn, there is no consensus on supporting additional values of dynamic range.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI mapping order is as follows:

- When ReportQuantity is ‘cjtc-Dd’ (Doffset+d):

  - nref ,

  - {Dn,offset,  n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

  - {dn,  n=0, 1, …, NTRP – 1, n≠nref ordered from the lowest to highest CSI-RS resource set ID

- When ReportQuantity is ‘cjtc-F’ (FO):

  - nref,

  - {FOn,  n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

- When ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO):

  - nref1,

  - nref2,

  - {Dn,offset,  n=0, 1, …, NTRP – 1, n≠nref1} ordered from the lowest to highest CSI-RS resource set ID,

  - {dn,  n=0, 1, …, NTRP – 1, n≠nref1} ordered from the lowest to highest CSI-RS resource set ID.

  - {FOn,  n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (Σ>1):

- Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (Σ=1) phase offset reporting, reporting, in one CSI reporting instance, Φn,0, Φn, 1,…, Φn,NSB-P-1, n=0, 1, …, NTRP-1, n≠nref.

  - The alphabet for Φn,σ follows the previously agreed alphabet for Σ=1, including the ‘invalid’ state.

- FFS:

  - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {1, 2, 4, 8, 16 PRBs}.

  - Whether the UE performs measurement over the entire configured CSI reporting band WCSI.

  - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P).

  - If needed, configuration of whether the CSI-RSs are MRT-beamformed in frequency-domain or not.

Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation.

Note: There is a source R1-2407184 showing for frequency-domain-MRT-precoded CSI-RSs, inter-TRP phase has a linear rotation in frequency-domain, and is associated with a single delay component.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (Σ>1) {(Φn,0, Φn,1, …, Φn,NSB-P-1), n=0, 1, …, NTRP-1, n≠nref}:

- Supported sub-band size(s) {1, 2, 4, 8, 16} PRB.

- The NW configures, via higher-layer (RRC) signalling, which NSB-P sub-band(s) the UE reports.

Agreement: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (PO) and Σ=1:

- The UCI parameters are captured in the tables below:

| Parameter | Details/description |
| --- | --- |
| nref | Reference CSI-RS resource index, based on the ordering from RRC configuration: logNTRP bits. |
| {Φn,n=0, 1, …, NTRP-1, n≠nref} | DL/UL phase offset for CSI-RS resource n: NTRP-1logMΦ bits. |

- The UCI mapping order is as follows:

  - nref,

  - {Φn, n=0, 1, …, NTRP-1, n≠nref,} ordered from the lowest to highest CSI-RS resource ID.

Agreement: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’). And if supported, whether any of the following is additionally supported or not:

- NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report; and/or

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot).

- The delay offset value to be compensated is the latest reported DO before the DCI triggering the CJT CSI reporting.

FFS: AP-CSI-RS can be configured for the Rel-18 eType-II CJT report.

The above only applies when the CMRs do not share common QCL source for average delay indication.

Agreement: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific frequency offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’). And if supported, whether any of the following is additionally supported or not:

- NW indicates the frequency offset value to be compensated for the Rel-18 eType-II CJT CSI report; and/or

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC frequency offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot).

- The frequency offset value to be compensated is the latest reported fO before the DCI triggering the CJT CSI reporting.

FFS: AP-CSI-RS can be configured for the Rel-18 eType-II CJT report

The above only applies when the CMRs do not share common QCL source for Doppler shift indication.

Agreement: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’).

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) can be separately or jointly triggered [and carried on a same PUSCH (hence on a same slot)] following legacy joint triggering mechanism.

  - (Working Assumption) When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting.

    - FFS: whether some expiration time interval is needed.

  - (Working Assumption) When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance.

FFS: NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report.

FFS: Whether only AP-CSI-RS, or any type of CSI-RS (P, SP, or AP) can be configured as the CMR for the Rel-18 eType-II CJT reporting.

FFS: Support for SP-CSI for CJTC report .

FFS: Whether this is also applicable for Rel-19 Type-I MP codebook.

The above only applies when the CMRs do not share common QCL source for average delay indication.

The above is UE optional feature.

This contribution provides our views on CSI enhancements for UE reporting for coherent joint transmission (CJT).

# Enhancements for CJT calibration reporting

## On the linkage between SRS and DL/UL phase offset reporting

One of the open issues from the RAN1#118 meeting has to do with the linkage between SRS and DL/UL phase offset reporting:

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset),

- The supported value(s) of x follows the supported configuration(s) of SRS resource for antenna switching xTyR used for reciprocity-based DL CSI acquisition.

  - Note: This doesn’t have any spec impact (no new RRC parameter is needed).

- For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of the available port(s)), decide, by RAN1#118, from the following schemes:

  - Scheme1: NW-configured via higher-layer (RRC) signalling.

  - Scheme1B: NW dynamic selection via MAC CE or A-CSI triggering (DCI).

  - Scheme2: UE-selected, the selection included in the phase offset report.

  - Scheme3: Fixed rule, e.g. the first out of the available port(s).

FFS:

- Whether/how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering.

- The supported time-domain behaviour(s) for the associated SRS resource (periodic, semi-persistent, aperiodic).

  - In case the NW configures the Q=1 associated SRS resource from an existing SP and/or AP SRS antenna switching resource configuration for DL CSI acquisition (which utilizes dynamic signalling), whether/how to identify the Q=1 associated SRS resource.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of available port(s)) is NW-configured via higher-layer (RRC) signalling.

Working assumption: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic.

Regarding the working assumption, we do not see any compelling reason for limiting the time-domain behavior of the SRS resources associated with ‘cjtc-P’ calibration reporting. Thus, we support confirming the working assumption.

- Confirm the working assumption: When ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic.

On the question of how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering, we note the following. There seems to be no issue for aperiodic SRS as there is only one transmission occasion. For semi-persistent or periodic SRS, however, multiple SRS transmission occasions can be considered. Of particular importance are the following: i) the latest SRS transmission occasion occurring before the CSI-RS(s) used for generating the ‘cjtc-P’ report; and ii) the earliest SRS transmission occasion occurring after said CSI-RS(s). These two SRS transmission occasions are the closest to the CSI-RS(s) used for measuring the ‘cjtc-P’ report; using them minimizes the risk that DL/UL channel reciprocity deteriorates significantly. For some reports, SRS transmission occasion i) will be the closest to the CSI-RS(s) used to measure the ‘cjtc-P’ report. For others, it will be ii).

- For SP- or P-SRS, care must be taken to unambiguously identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port. Valid SRS transmit occasions exist before and after the CSI-RS(s) measured for generating the ‘cjtc-P’ report.

- For CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), when the configured associated SRS resource is periodic or semi-persistent, consider both the latest SRS transmit occasion before the CSI-RS(s) measured for generating the ‘cjtc-P’ report, and the earliest one after said CSI-RS(s). The closest of the two can be used for non-precoded CSI-RS transmission schemes to minimize the risk of DL/UL channel reciprocity deterioration.

## On sub-band DL/UL phase offset reporting

Another open issue has to do with the potentially excessive overhead incurred by sub-band PO reporting. This issue is captured in the FFS of the following RAN1#118 agreement:

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (Σ>1):

- Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (Σ=1) phase offset reporting, reporting, in one CSI reporting instance, .

  - The alphabet for Φn,σ follows the previously agreed alphabet for Σ=1, including the ‘invalid’ state.

- FFS:

  - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {1, 2, 4, 8, 16 PRBs}.

  - Whether the UE performs measurement over the entire configured CSI reporting band WCSI.

  - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P).

  - If needed, configuration of whether the CSI-RSs are MRT-beamformed in frequency-domain or not.

Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation.

Note: There is a source R1-2407184 showing for frequency-domain-MRT-precoded CSI-RSs, inter-TRP phase has a linear rotation in frequency-domain, and is associated with a single delay component.

Agreement: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (Σ>1) {(Φn,0, Φn,1, …, Φn,NSB-P-1), n=0, 1, …, NTRP-1, n≠nref}:

- Supported sub-band size(s) {1, 2, 4, 8, 16} PRB.

The NW configures, via higher-layer (RRC) signalling, which NSB-P sub-band(s) the UE reports.

The overhead associated with DL/UL phase offset sub-band reporting, i.e., Σ>1, can be calculated as

OH= NPRBSSBMΦ NTRP-1,

where NPRB is the number of PRBs configured for CSI reporting and SSB∈{1, 2, 4, 8, 18} is the configured sub-band size for ‘cjtc-P’ reporting when Σ>1. If no mechanism to limit the CSI reporting overhead is introduced, we have

NSB-P=NPRBSSB,

where NPRB is the number of sub-bands that the UE reports. From the point of view of CSI overhead, the worst case is SSB=1, in which case, NSB-P=NPRB and the CSI reporting overhead may easily exceed the 1706-bit limit of 5G NR polar codes. Thus, we think that CSI overhead capping mechanisms are necessary.

- For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) and Σ>1 (sub-band reporting), introduce mechanisms to limit the CSI reporting overhead.

The agreement suggests that the number of reported sub-bands, NSB-P, can be upper bounded to, say,  NSB-P. In our view, this would be an effective way to limit the CSI reporting overhead. Further, the upper limit NSB-P might depend on the configured resolution of the reported phase offsets, i.e., MΦ. For a fixed CSI reporting overhead, the lower the configured phase offset resolution, the larger the number of sub-bands that can be reported:

- To limit the CSI reporting overhead, the maximum number of sub-bands that the UE reports can be upper bounded. Consider whether this upper bound may depend on the configured resolution for phase-offset reporting, thereby allowing the UE to report more sub-bands if a lower resolution is configured.

Since the number of sub-bands reported by the UE may be limited, a mechanism is needed to select NSB-P≤NSB-P reported sub-bands out of NPRBSSB available. This mechanism is to be provided “via higher-layer (RRC) signaling” per the agreement. At least a couple of options are possible:

- To select NSB-P≤NSB-P reported sub-bands out of NPRBSSB available, consider the following mechanisms:

- Higher-layer (RRC) signaling provides a bitmap. In this case, NSB-P≤ NSB-P is the number of ones in the bitmap.

- Higher-layer (RRC) signaling provides a decimation factor KSB. Only every KSB-th sub-band is reported out of NPRBSSB available. In this case, NSB-P≤ NSB-P is approximately NPRBSSB/KSB.

## On UE-specific delay offset pre-compensation

We next discuss open issues about UE-specific delay-offset pre-compensation:

Agreement: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’).

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) can be separately or jointly triggered [and carried on a same PUSCH (hence on a same slot)] following legacy joint triggering mechanism.

  - (Working Assumption) When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting.

    - FFS: whether some expiration time interval is needed.

  - (Working Assumption) When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance.

FFS: NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report.

FFS: Whether only AP-CSI-RS, or any type of CSI-RS (P, SP, or AP) can be configured as the CMR for the Rel-18 eType-II CJT reporting.

FFS: Support for SP-CSI for CJTC report.

FFS: Whether this is also applicable for Rel-19 Type-I MP codebook.

The above only applies when the CMRs do not share common QCL source for average delay indication.

The above is UE optional feature.

We support confirming the working assumptions.

- For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support the two working assumptions to determine the delay offset value to be compensated when the two configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are jointly or separately triggered.

Further, we understand that an expiration timer interval is not needed. The reason is that it is not clear what the best course of action is upon the expiration of the timer. Shall the UE then skip delay-offset pre-compensation? If so, this is apt to result in considerable degradation of the associated PDSCH transmissions as some compensation of the delay offsets among the TRPs is nonetheless required. Instead, the time expiration may trigger a new ‘cjtc-Dd’ report; up-to-date delay offsets could be measured and reported. This course of action seems more appropriate. However, if this is the case, the network can keep network-side timers—it has to, if the UE and the network are to remain synchronized—and trigger a new ‘cjtc-Dd’ report as it sees fit. It follows that no explicit “expiration timer interval” is needed.

- For separately triggered configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report), an explicit expiration time interval mechanism is not needed. The network can keep track of appropriate expiration time intervals transparently.

On the other hand, we do see an issue with separately triggered configured reports. If the ‘cjtc-Dd’ report is lost or incorrectly decoded by the network, the UE and the network can get out-of-synch. More specifically, the delay offset value applied by the UE to perform PMI calculation and the delay offset used by the network to facilitate UE-specific delay offset pre-compensation on PDSCH need no longer be the same. Thus, there is a need for a mechanism that keeps the delay offsets applied by the UE and the network in synch. In our view, a network indication of the delay offset value to be compensated performs this role efficiently. Other types of network indication might also be possible and efficient.

- With separately triggered configured reports, there is a need for a mechanism that keeps the delay offsets applied by the UE and the network for pre-compensation of PMI calculation and PDSCH transmission, respectively, in synch. A network indication of the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report performs this role efficiently.

- For separately triggered configured reports (i.e. the Rel-18 eType-II CJT CSI report and the CJTC delay offset report), introduce a mechanism to synchronize pre-compensation delay offset values at the UE side, for calculation of PMI, and the network side, for UE-specific pre-compensation of PDSCH transmissions.

- Introduce an explicit network indication of the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report. This indication efficiently keeps delay-offset precompensation at the UE and network sides in synch for the case of separately triggered configured reports. Other types of network indications might also be possible and efficient.

## On UE-specific frequency offset pre-compensation

Open issues for UE-specific frequency offset pre-compensation also exist:

Agreement: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific frequency offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’). And if supported, whether any of the following is additionally supported or not:

- NW indicates the frequency offset value to be compensated for the Rel-18 eType-II CJT CSI report; and/or

- The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC frequency offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot).

- The frequency offset value to be compensated is the latest reported FO before the DCI triggering the CJT CSI reporting.

FFS: AP-CSI-RS can be configured for the Rel-18 eType-II CJT report

The above only applies when the CMRs do not share common QCL source for Doppler shift indication.

The issues highlighted in the agreement for UE-specific frequency offset pre-compensation are akin to those of UE-specific delay offset pre-compensation. For this reason, we suggest delaying the treatment of this topic until all the UE-specific delay offset pre-compensation open issues have been resolved.

- For UE-specific frequency offset pre-compensation, we are, in principle, supportive of the additional issues captured in the bullets of the agreement. Since these issues parallel those of UE-specific delay offset pre-compensation, they can be treated after the related discussion on UE-specific delay offset pre-compensation has concluded.

## On specifying “invalid” conditions

Conclusion: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the ‘out of range’ or ‘invalid’ quantization state/hypothesis, there is no consensus on specifying a condition/event for such state.

In RAN#117, the following conclusion was reached:

In the following, we derive bounds for the precision of the wideband parameters Φn and ξn. However, a similar derivation is possible for the sub-band parameters Φn,0, Φn,1, …, Φn,NSB-P-1, and identical conclusions are drawn at the end. The precision with which the parameters Φn and ξn can be estimated can, however, be quantified/approximated. This enables stringent conditions for declaring an “invalid” state.

- The precision with which Φn,0 and Γn can be estimated from (noisy) signals Yn(k) can be well approximated.

Let us define the signal Zk=Y1*(k)Y2(k), where noise with variance N0 is included in signals Yn(k). We may approximate this signal as

Zk=HTRP2k2HTRP1k2exp-j2πkΔfΦ2,0+jΓ2+W(k)

where we may approximate W(k) as complex Gaussian with variance

EW(k)2=N0(N0+HTRP1k4+HTRP2k4).

The Cramer-Rao (CR) bounds read (the Fisher information matrix is diagonal)

EΦ2,0-Φ2,02=4π2Δf2kA2kk2

EΓ2-Γ22=kA2k,

where

Ak=HTRP1k2HTRP2k2N0(N0+HTRP1k4+HTRP2k4) .

Now, the precision predicted by the CR bounds are only applicable at high SNR. At low/medium SNR the probability that the estimator produces estimates that are not in the vicinity of the true values is substantial. The Euclidean distance, normalized with the variance of W(k), between the signal set {Zk} stemming from the true values Φ2,0, Γ2 and a set {Zk} stemming from erroneous values Φ2,0'=Φ2,0+ΔΦ, Γ2'=Γ2+ΔΓ reads

D2ΔΦ,ΔΓ=kAk1-exp-i2πΔfkΔΦexp-iΔΓ2.

As an approximation to the probability of producing estimates not in the vicinity of the true values, we may consider the smallest Euclidean distance at a local minimum of D2ΔΦ,ΔΓ, where the trivial minimum at ΔΦ=ΔΓ=0 is discarded; denote this distance by Dmin2. Then we may approximate the probability of a bias in the estimates as Pbias=Q(Dmin2/2). Whenever the probability Pbias and/or the CR variances are large, the reports may be considered as invalid.

We may note that this analysis is enabled whenever the channel magnitudes |HTRPnk| are at hand. This implies that both the network and the UE can determine the precision with which the UE can estimate the parameters.

- Based on the observed channel magnitudes, the network as well as the UE can predict the accuracy of the estimations of Φn and Γn.

We also make, without proving it, the following observation

- Based on the observed channel magnitudes, the network as well as the UE can predict the accuracy of the estimations of Φn,0, Φn,1, …, Φn,NSB-P-1.

From this observation, the following proposals follow:

- For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), for wideband reporting (Σ=1) and sub-band reporting (Σ>1), the network should be able to forbid the UE from reporting invalid (as the network can determine the quality is sufficiently good).

- The UE should report invalid whenever it determines that the precision is below a given threshold.

Note that preventing the UE from reporting invalid values of the Φn and Γn parameters—or the sub-band parameters Φn,0, Φn,1, …, Φn,NSB-P-1, from which the former ones can be derived—is particularly convenient when the network intends to merge the estimates reported by several UEs. At a fundamental level, the parameters Φn,0 and Γn characterize the hardware impairments that the network wishes to compensate, namely, time and phase offsets between the DL and UL RF chains of the TRPs participating in CJT. Therefore, phase offset reports from several UEs can be combined to improve the accuracy of the estimates.

- Preventing the UE from reporting invalid values of the Φn and Γn parameters—or the sub-band parameters Φn,0, Φn,1, …, Φn,NSB-P-1, from which the former can be derived—is particularly convenient to merge, by the network, the estimates reported by several UEs. UE report combining can be used to improve the accuracy of estimates of the UE-common parameters Φn and Γn characterizing the time and phase offsets between the DL and UL RF chains of the TRPs participating in CJT.

## On wideband reporting of DL/UL phase offset

In offline discussions during the RAN1#118 meeting [5], the following proposal was made:

Proposal 3.D.2: For the Rel-19 aperiodic standalone CJT calibration reporting, support joint Dd + phase offset (PO) reporting as follows:

- Only wideband (S=1) PO is supported

- No further optimization of CSI reporting format, e.g. configurability of not reporting {dn}

- The UCI parameters are captured in the table below

When ReportQuantity is ‘cjtc-Dd-P’ (joint Doffset+d and PO)

| Parameter | Details/description |
| --- | --- |
| nref1 | Reference TRS resource set index for Doffset+d, based on the ordering from RRC configuration: logNTRP bits |
| nref2 | Reference TRS resource set index for PO, based on the ordering from RRC configuration: logNTRP bits |
| {Dn,offset, n=0, 1, …, NTRP – 1 n≠nref1} | Delay offset for CSI-RS resource set n: NTRP-1logMD bits |
| {dn, n=0, 1, …, NTRP – 1, n≠nref1 } | 1-bit inside/outside indicator for CSI-RS resource set n: NTRP-1 bits |
| {POn , n=0, 1, …, NTRP –1, n≠nref2} | Wideband phase offset for CSI-RS resource n: NTRP-1logMΦ bits |

- The UCI mapping order is as follows:

  - nref1,

  - nref2,

  - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID,

  - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID

  - {POn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource ID,

In our view, joint Dd + WB PO reporting provides a more efficient alternative to sub-band PO reporting regarding CSI reporting overhead. Moreover, contributions to RAN1#118 showed that under certain operating conditions, joint Dd + WB PO reporting performs at least as well as sub-band PO reporting. We therefore support this proposal for study in the RAN1#118-bis meeting.

- For the Rel-19 aperiodic standalone CJT calibration reporting, support joint Dd + phase offset (PO) reporting.

# Conclusions

We made the following observations and proposals:

- Confirm the working assumption: When ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic.

- For CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), when the configured associated SRS resource is periodic or semi-persistent, consider both the latest SRS transmit occasion before the CSI-RS(s) measured for generating the ‘cjtc-P’ report, and the earliest one after said CSI-RS(s). The closest of the two can be used for non-precoded CSI-RS transmission schemes to minimize the risk of DL/UL channel reciprocity deterioration.

- For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) and Σ>1 (sub-band reporting), introduce mechanisms to limit the CSI reporting overhead.

- To limit the CSI reporting overhead, the maximum number of sub-bands that the UE reports can be upper bounded. Consider whether this upper bound may depend on the configured resolution for phase-offset reporting, thereby allowing the UE to report more sub-bands if a lower resolution is configured.

- To select NSB-P≤NSB-P reported sub-bands out of NPRBSSB available, consider the following mechanisms:

- Higher-layer (RRC) signaling provides a bitmap. In this case, NSB-P≤ NSB-P is the number of ones in the bitmap.

- Higher-layer (RRC) signaling provides a decimation factor KSB. Only every KSB-th sub-band is reported out of NPRBSSB available. In this case, NSB-P≤ NSB-P is approximately NPRBSSB/KSB.

- For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support the two working assumptions to determine the delay offset value to be compensated when the two configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are jointly or separately triggered.

- For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, support the two working assumptions to determine the delay offset value to be compensated when the two configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are jointly or separately triggered.

- For separately triggered configured reports (i.e. the Rel-18 eType-II CJT CSI report and the CJTC delay offset report), introduce a mechanism to synchronize pre-compensation delay offset values at the UE side, for calculation of PMI, and the network side, for UE-specific pre-compensation of PDSCH transmissions.

- Introduce an explicit network indication of the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report. This indication efficiently keeps delay-offset precompensation at the UE and network sides in synch for the case of separately triggered configured reports. Other types of network indications might also be possible and efficient

- For UE-specific frequency offset pre-compensation, we are, in principle, supportive of the additional issues captured in the bullets of the agreement. Since these issues parallel those of UE-specific delay offset pre-compensation, they can be treated after the related discussion on UE-specific delay offset pre-compensation has concluded.

- For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), for wideband reporting (Σ=1) and sub-band reporting (Σ>1), the network should be able to forbid the UE from reporting invalid (as the network can determine the quality is sufficiently good).

- The UE should report invalid whenever it determines that the precision is below a given threshold.

- For the Rel-19 aperiodic standalone CJT calibration reporting, support joint Dd + phase offset (PO) reporting.

- For SP- or P-SRS, care must be taken to unambiguously identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port. Valid SRS transmit occasions exist before and after the CSI-RS(s) measured for generating the ‘cjtc-P’ report.

- With separately triggered configured reports, there is a need for a mechanism that keeps the delay offsets applied by the UE and the network for pre-compensation of PMI calculation and PDSCH transmission, respectively, in synch. A network indication of the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report performs this role efficiently.

- The precision with which Φn,0 and Γn can be estimated from (noisy) signals Yn(k) can be well approximated.

- Based on the observed channel magnitudes, the network as well as the UE can predict the accuracy of the estimations of Φn and Γn.

- Based on the observed channel magnitudes, the network as well as the UE can predict the accuracy of the estimations of Φn,0, Φn,1, …, Φn,NSB-P-1.

- Preventing the UE from reporting invalid values of the Φn and Γn parameters—or the sub-band parameters Φn,0, Φn,1, …, Φn,NSB-P-1, from which the former can be derived—is particularly convenient to merge, by the network, the estimates reported by several UEs. UE report combining can be used to improve the accuracy of estimates of the UE-common parameters Φn and Γn characterizing the time and phase offsets between the DL and UL RF chains of the TRPs participating in CJT.

# References

- “Draft report of 3GPP TSG RAN WG1 #118 v0.3.0,” RAN1 chair.

- R1-2406643, “Moderator summary for OFFLINE discussion on Rel-19 CSI enhancements,” Moderator (Samsung).

- R1-2406644, “Moderator summary on Rel-19 CSI enhancements,” Moderator (Samsung).

- R1-2407282, “Moderator summary of Tuesday offline on Rel-19 CSI enhancements,” Moderator (Samsung).

- R1-2407279, “Moderator summary#2 on Rel-19 CSI enhancements: Round 2,” Moderator (Samsung).

- R1-2407283, “Moderator summary of Wednesday offline on Rel-19 CSI enhancements,” Moderator (Samsung).

- R1-2407280, “Moderator summary#3 on Rel-19 CSI enhancements: Round 3,” Moderator (Samsung).

- R1-2407466, “Moderator summary of Thursday offline on Rel-19 CSI enhancements,” Moderator (Samsung)

- R1-2407281, “Moderator summary #4 on Rel-19 CSI enhancements: Round 4,” Moderator (Samsung).

- R1-2406468, “More views on CSI enhancements,” Sony.

For the sake of brevity, this section uses, without explanation, mathematical symbols and terminology introduced in earlier contributions. The reader interested in the details of this section is referred to, e.g., [10].