3GPP TSG-RAN WG1 Meeting #119	  R1-2409415

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.5.3

Source:	Huawei, HiSilicon

Title:	On common channel/signal adaptation for eNES

Document for:	Discussion and Decision

- Introduction

A new work item on network energy saving was approved in RAN plenary #102 and revised as in []. The following objective has been agreed on adaptation of common signal/channel transmissions for Rel-19 NES:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

Analyses for adaptation schemes on respective common channels are to be provided in the document.

- Adaptation of SSB

  - SSB Adaptation for Rel-19 NES-capable UE in RRC_INACTIVE mode

During the discussion on PRACH adaptation in the last meeting, DCI based signaling, majorly, DCI format 1_0 scrambled by P-RNTI, for additional PRACH resources for UEs in idle/inactive mode was proposed by many companies. From an inactive UE’s perspective, the indication method for adaptation of SSB and PRACH has commonality in terms of requirements of low latency, low detection complexity and being detectable. A common DCI design is preferrable to ease UE monitoring of PDCCH, and for gNB to control these adaptations simultaneously.

- RAN1 to consider to use a common DCI signalling design, for example, DCI format 1_0 scrambled by P-RNTI, for SSB and PRACH adaptation for RRC_INACTIVE UEs.

  - SSB Adaptation for Rel-19 NES-capable UE in SCell

The following proposal was proposed in RAN1#117 meeting.

Proposal 2.1.1-updated For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

On SSB (OD-SSBs or adaptable SSBs) in time

Specifically, the dense-periodicity SSB configuration designed for expediting SCell activation is important. In current spec, with transmission of 20ms periodicity SSB, large activation delay exists due to unknown state, especially for FR2 SCell, as is shown in the following Figure 2. Note that about 4 rounds of SSB bursts are needed per UE Rx beam in FR2, in which 2 rounds are for AGC settling, 1 round is for synchronization and 1 round is for L1-meaurements. Assuming a typical FR2 UE (equipped with 8 Rx beams), then about 32 rounds of SSB bursts are needed for the SCell activation, leading to a long activation delay of 640ms. Consequently, such huge delay will force gNB to stay active for a long time, preventing it from transmitting data as soon as possible and get into deeper sleep mode.

Figure 1 Illustration for a FR2 unknown SCell activation delay when applying 20ms-periodicity SSB

By contrast, the SCell activation delay can be largely reduced if applying dense-periodicity SSB, e.g., 2ms level. Under the same setting of an FR2 unknown cell, the latency can be reduced from 640ms to 64ms, and then NW can go into sleep earlier after completion of data transmission. Besides preserving NW energy saving gain, this is also beneficial for user-perceived transmission rate.

Figure 2 Illustration for a FR2 unknown SCell activation delay when applying dense periodicity (e.g., 2ms level)

- Reducing the periodicity of SSB burst to e.g. 2 ms level can reduce the latency of SCell activation in FR2 by several hundreds of ms. This saved time could help the SCell to enter in deeper sleeping modes once data transmission is finished.

In a word, new dense periodicities which are less than 5ms can be considered to boost the acceleration of SCell activation. In current spec, the transmission of an SSB burst is confined in a half frame (5ms) and the actual transmission window within is less than 5ms, which varies depending on the frequency band and SCS, i.e., from no more than 2ms (e.g., SCS=15kHz, sub-3GHz) to more than 4ms but less than 5ms (e.g., SCS=120kHz, FR2). The new periodicity can thus be defined, with details to be considered under different bands and SCSs.

- Support new SSB burst periodicity of values of less than 5ms for SCell SSB transmission.

There are two favourable mechanisms to support new SSB burst periodicity of values less than 5ms.

- Mechanism 1: Truncating the subframes that has no SSBs in the legacy half frame SSB burst patterns

According to [2], seven cases of SSB burst pattern have been supported in current spec, each applying to a certain SCS and frequency band. Within a half frame, candidate SS/PBCH Blocks are placed at intervals and many OFDM symbols are left with no SSB transmission. It is observed that, for most of the SSB burst patterns, there exist non-SS/PBCH Block-transmitting subframes at the end of a half frame. As illustrated in Figure 4, in Case E (applied in FR2, SCS = 240kHz), the candidate SS/PBCH blocks are only distributed in the first three subframes, leaving the last two with no SSB transmission.

Figure 3 Illustration for truncating the last two non-SSB transmitting subframes in Case E SSB burst pattern

For the illustrated Case E, it turns out that the actual transmission window (at a subframe level) is 3 subframes, and the last two subframes can be truncated in order to achieve an SSB burst transmission window of 3ms. That is, the periodicity of the new SSB burst can be also 3ms. This can be also applied to other patterns, e.g., Case A, B, C, etc.

- Mechanism 2: Mechanism 1 + New compact SSB burst patterns i.e., smaller inter-SSBs gaps

Besides truncating the last subframes with no SSB transmission, further, the gaps between two adjacent SS/PBCH blocks in an SSB burst can also be squeezed and the candidate SS/PBCH blocks can be adaptive to the number of SSB beams, leading to an even narrower transmission window (consequently, denser periodicity). In Figure 5, an example of compact SSB pattern is illustrated, where the SCS is 240 kHz for FR2 and the number of SSB beams is assumed as 64. The compact SSB pattern occupies 2ms to transmit a burst, and the periodicity is reduced to 2ms (considering rounding operation to align with the frame boundary).

Figure 4 An example of new SSB burst with compact SSB pattern

This new dense SSB periodicity does not only save network energy but also reduce UE power for SCell activation. On the other hand, shortened SSB bursts may not be applicable for PCell/idle UEs since a UE has to frequently detect a potential SSB for initial access. Therefore, restriction of this method to SCell only is reasonable, and can mitigate the implementation impact.

To enable smaller SSB burst periodicity, less than 5 ms, we propose

- Support the following two mechanisms to enable new configurable SSB burst periodicity of less than 5ms

- Mechanism 1: Truncating the subframes that has no SSBs in the legacy half frame SSB burst patterns. i.e., configurable gap between SSB bursts using RRC.

- Mechanism 2: Mechanism 1 + New compact SSB burst patterns i.e., smaller inter-SSBs gaps. i.e., configurable gap(s) between candidate SS/PBCH blocks within an on-demand SSB burst.

  - The impact of SSB adaptation on other common / dedicated signals

In current spec, SSB transmission/reception is prioritized over other signals in the resource grid. Consider that SSB transmission is dynamically adapted on a Rel-19 eNES cell, the impact on other common/dedicated signals can be revisited.

- On PRACH/PUSCH occasion validation: In TDD system, a valid PRACH Occasion (for both Type-1 and Type-2 RA) or a valid PUSCH Occasion (for Type-2 RA) needs to start at least a given number of symbols after a last SSB symbol on the PRACH/PUSCH slot, where the SSB position in time is given by SIB1 or RRCReconfiguration. Otherwise, collision is considered to happen and the invalid PRACH/PUSCH occasions would impair the RA process. Note that RA process is not only essential to PCell, but also to SCell when time alignment for a secondary TAG needs to be established upon the activation of a PUCCH SCell. On a Rel-19 eNES cell, the same validation rule can still work, on the condition that the validation is refreshed after UE receives the adapting signaling. For example, in the figure below, before receiving the adapting signaling, UE assumes the ROs are invalid due to dense SSB transmission. Yet, the adaptation to long periodicity will refresh the RO validation, and when such validation is refreshed can reach a consensus between gNB and UE.

Figure 5 SSB adaptation will impact the validation of PRACH occasions in RA procedure

- RAN1 to consider the impact of SSB adaption on other common / dedicated signals, e.g., its impact on PRACH/PUSCH occasion validation.

- On CSI-RS validation: For CSI-RS L1 measurement, especially for periodic CSI-RS, when CSI-RS is in the same OFDM symbol as SSB for other L1 measurement, UE is only required to measure one of the RS (i.e. either CSI-RS or SSB) as defined in current RAN4 spec. When gNB is going to adapt SSB for SCell activation for a particular UE, the adapted SSB may be overlapped with some periodic CSI-RS in time domain. The configured periodic CSI-RS resource may or may not be transmitted due to potential Tx beam restriction. Based on legacy UE behavior, UE may choose to measure the CSI-RS which can be dropped by gNB, and this will lead to unexpected UE behavior, e.g., inaccurate L1-RSRP reporting or a wrong beam failure report. Thus, UE shall be aware of whether the CSI-RS overlapped newly-adapted SSB is valid for measurement. One solution is to reconfigure the CSI-RS before adapting SSB. However, this may introduce unnecessary RRC latency, and gNB may need to reconfigure the CSI-RS again after SSB is further adapted.

- RAN1 to consider the impact of SSB adaptation on overlapping CSI-RS in time-domain, e.g., its impacts on whether CSI-RS is valid for UE measurement.

- Adaptation of PRACH

Working Assumption@118bis For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement@118 For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

It was agreed in last meeting to use DCI format 1_0 scrambled by P-RNTI as a working assumption for PRACH adaptation indication. We think it is feasible and useful to use the 4 less significant bits of the short message field (which are currently reserved TS 38.331 spec in Appendix). These reserved bits exist regardless if short message is sent or not making them a good choice for indicating the activation/deactivation of the additional PRACH resources. Hence, we propose to confirm this working assumption and to use these reserved bits for PRACH adaptation indication.

- Confirm the working assumption and reserved bit(s) in short message field is used for PRACH adaptation indication.

Only supporting DCI format 1_0 scrambled by P-RNTI for indicating PRACH adaptation is a catastrophic choice that RAN1 should avoid for idle/inactive UEs who support energy saving feature (can receive PEI). Because if a PO(s) is used to indicate PRACH adaptation then for these UEs the PEI need to indicate not to skip the next PO(s) wasting an opportunity for these UEs to go to deeper sleeping mode. For that reason, additionally having PEI, i.e., DCI 2_7, is a must for UEs who can monitor PEI DCI. Once these UEs see the indication of PRACH adaptation in the PEI DCI, they can skip the PO(s) monitoring as indicated in the legacy filed of PEI, e.g., there is no other reason that require them to monitor the next DCI format 1_0 scrambled by P-RNTI.

For the PEI DCI 2_7 (see Figure 1), its DCI size is configured by payloadSizeDCI-2-7 broadcasted in SIB1. the maximum possible value of payloadSizeDCI-2-7 is 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

Figure 6 Illustration of PEI DCI payload

If the number of useful information bits (Paging indication field and TRS availability indication field) in DCI 2_7 is less than payloadSizeDCI-2-7, then the remaining bits are reserved. Hence, it up to gNB implementation to decide whether DCI 2_7 can has reserved bits or not. So, we think DCI 2_7 can also be used for PRACH adaptation indication for idle/inactive UEs.

- For PRACH adaptation indication, additionally support the use of DCI 2_7 for idle/inactive UEs.

In legacy for connected mode UEs, there are many use cases that require the UE to send PRACH either on the same serving cell or on a target cell e.g., in case of a Handover or LTM. The PRACH transmission could be triggered by DCI (DCI 1_0 scrambled by C-RNTI) or MAC CE. Indicating the PRACH adaptation only in PO for connected mode UEs, will cause high latency. Because in order for the NES cell to use/activate its own additional PRACH resources, it can only indicate in PO(s) the activation of these resources. After that, it can indicates the use of these additional PRACH resources in the PDCCH order (DCI 1_0 scrambled by C-RNTI) or MAC CE order. This is clearly not good neither for NES cell energy consumption nor for the connected mode UEs latency. DCI 1_0 scrambled by C-RNTI has many reserved bits (see the appendix A). These reserved bits can be used for PRACH adaptation indication (activation/deactivation) at the same time as the DCI for PDCCH order indicating which resource to be used (legacy or additional).

- For PRACH adaptation indication, additionally support the use of DCI 1_0 with C-RNTI for connected mode UEs.

Agreement For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different - FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s)

It was agreed in last meeting that the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different, and there are two options for the detailed signalling. We didn’t see the necessity to introduce a new frequency domain parameter to configure the frequency resources for additional PRACH. The legacy msg1-FrequencyStart/msg1-FDM is already enough to avoid the overlapping between legacy PRACH resource and additional PRACH resources with minimal spec impact.

- At least msg1-FrequencyStart/msg1-FDM can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

For determining the additional PRACH resources in time-domain, the UE can obtain the time locations of additional PRACH resources using the tables specified in TS 38.211 e.g., Table 6.3.3.2-2, Table 6.3.3.2-3, etc. For Alt1 (same PRACH configuration index), full overlap in time between the additional PRACH resources and PRACH resources for legacy UEs (if any) is happening. In this case, the frequency domain parameters such as the msg1-FrequencyStart/msg1-FDM can be configured to avoid the overlapping in both time and frequency which is already agreed in last meeting. The additional mechanism for the same purpose is not needed any more to minimal the spec impact. For Alt2 (different PRACH configuration index), no/partial overlap in time between the additional PRACH resources and PRACH resources for legacy UEs (if any) could happen. E.g. assume the legacy PRACH configuration index is 0 of Table 6.3.3.2-3, then the legacy PRACH would be in subframe 9 of radio frame 1, 17 etc with 160ms period. For additional PRACH resources with 20ms period, its configuration index can be 3 in which the additional PRACH resource would be in subframe 9 of radio frame 0, 2 etc. There is no time overlap between the legacy PRACH and additional PRACH. If the configuration index is 4, the additional PRACH resource would be in subframe 9 of radio frame 1,3,5 etc. Then partial overlap would happen but this overlap can also be resolved easily by different frequency resources configuration which is already agreed in last meeting. Based on that we have the following proposal.

- (Opt 1-5) No additional mechanism is selected for both Alt 1 and Alt 2.

- Conclusions

The following observations and proposals are provided.

- RAN1 to consider to use a common DCI signalling design, for example, DCI format 1_0 scrambled by P-RNTI, for SSB and PRACH adaptation for RRC_INACTIVE UEs.

- Reducing the periodicity of SSB burst to e.g. 2 ms level can reduce the latency of SCell activation in FR2 by several hundreds of ms. This saved time could help the SCell to enter in deeper sleeping modes once data transmission is finished.

- Support new SSB burst periodicity of values of less than 5ms for SCell SSB transmission.

- Support the following two mechanisms to enable new configurable SSB burst periodicity of less than 5ms

- Mechanism 1: Truncating the subframes that has no SSBs in the legacy half frame SSB burst patterns. i.e., configurable gap between SSB bursts using RRC.

- Mechanism 2: Mechanism 1 + New compact SSB burst patterns i.e., smaller inter-SSBs gaps. i.e., configurable gap(s) between candidate SS/PBCH blocks within an on-demand SSB burst.

- RAN1 to consider the impact of SSB adaption on other common / dedicated signals, e.g., its impact on PRACH/PUSCH occasion validation.

- RAN1 to consider the impact of SSB adaptation on overlapping CSI-RS in time-domain, e.g., its impacts on whether CSI-RS is valid for UE measurement.

- Confirm the working assumption and reserved bit(s) in short message field is used for PRACH adaptation indication.

- For PRACH adaptation indication, additionally support the use of DCI 2_7 for idle/inactive UEs.

- For PRACH adaptation indication, additionally support the use of DCI 1_0 with C-RNTI for connected mode UEs.

- At least msg1-FrequencyStart/msg1-FDM can be configured separately for the additional PRACH resources at least for 4-step RACH.

- (Opt 1-5) No additional mechanism is selected for both Alt 1 and Alt 2.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), RAN#102, December 11th-15th, 2023.

- 3GPP TS 38.213, “Physical layer procedures for control”, V18.4.0, September, 2024.

# Appendix A

TS 38.331 ## 6.5 Short Message Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1). Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit. Table 6.5-1: Short Messages Bit; Short Message. 1; systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs.. 2; etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification.. 3; stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1.. 4; systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period.. 5 – 8; Not used in this release of the specification, and shall be ignored by UE if received..

TS 38.212 ##### 7.3.1.2.1 Format 1_0 <text omitted> If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. - PRACH association indicator - 0 or 1 bit - 1bit if the UE is provided with tag-Id2, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. - This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the active additional PCI. - This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format. - 0 bit otherwise. - PRACH retransmission indicator - 0 or 1 bit - 1bit if the UE is configured with higher layer parameter EarlyUlSyncConfig. This field indicates initial transmission or retransmission of PRACH according to Table 7.3.1.2.1-3 if the cell indicated by Cell indicator field is a candidate cell, and this field is reserved if the cell indicated by Cell indicator field is a serving cell but not a candidate cell. - 0 bit otherwise. - Reserved bits - a number of bits as determined by the following: - 12 bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2, and if the UE is not configured with higher layer parameter EarlyUlSyncConfig; - 11-log2C+1 bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2, and if the UE is configured with higher layer parameter EarlyUlSyncConfig; - 9-log2C+1 bits for operation in a cell without shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-1 or when the DCI format is monitored in UE-specific search space for operation in a cell in frequency range 2-2, and if the UE is configured with higher layer parameter EarlyUlSyncConfig; - 10 bits otherwise. <text omitted > The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: - Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1. - Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - is the size of CORESET 0 - Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured; 0 bits otherwise. - Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above. <text omitted> Table 7.3.1.2.1-1: Short Message indicator Bit field; Short Message indicator. 00; Reserved. 01; Only scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI. 10; Only short message, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI. 11; Both scheduling information for Paging, TRS availability indication if trs-ResourceSetConfig is configured and short message are present in the DCI.