3GPP TSG RAN WG1 #119	R1-2409445

Orlando, USA, November 11th- 15th 2024

Agenda item:		9.6.2

Title:					LP-WUS operation in IDLE/Inactive mode

Source:				Nokia

Document for:		Discussion and Decision

# Introduction

In RAN#102 meeting, a new work item was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this contribution we provide our initial views related to the LP-WUS operation in IDLE/Inactive mode.

# RRM aspects

In following we discuss some aspects related to the LR measurement quantities.

## Consideration on LR measurements quantities

In RAN1#116bis meeting it was agreed to support LP-RSRP and LP-RSRQ for LR-based measurement quantities. Furthermore, in RAN1#117 discussion for LP-SS based metric definitions was progressed with following agreements and a working assumption was taken for the LR that is capable of receiving PSS/SSS:

Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

The definition of the LR measurement quantities was further clarified in RAN1#118 as follows:

Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

It was also agreed in last meeting that there should be one to one association between LP-SS and SSB (and LP-WUS) and that there would be 4 OOK sequences for LP-SS.

As discussed on our companion paper [3], the accuracy for the LP-RSRS, may depend on LP-SS design. Firstly, if Manchester encoding is not applied, the power on ON durations may vary (for M>1) as OFDM symbols can have varying number of ONs and OFFs durations. In order to have consistent RSRP reports, this should be accounted in the receiver to have constant RSRP estimates regardless of the applied sequence. Effectively this would mean scaling ON duration energy depending on the number of ON durations in one OFDM symbol. However, this could lead to degraded SNR for the measurement (noise amplification). Thus, how and whether this needs to be accounted would need to be considered after finalization of LP-SS design. Also, depending on the approach and binary sequences selected for the LP-SS, the total number of ONs and OFFs OOK symbols could be unequal. For consistent reports, there could be need to consider this impact also in the measurement metric definition, at least if the difference can be large.

Observation: If Manchester encoding is not applied to the LP-SS and depending on the binary sequence selection for LP-SS, the current definitions might lead to inconsistent results for LP-RSRQ and for LP-RSRQ between different LP-SS sequences

Proposal: Once LP-SS sequence design has been finalized, verify if the LP-RSRP and LP-RSRQ metric definitions need to be revisited.

For NR measurements, e,g For SS-RSRP, it is defined that SS-RSRP is SSB index specific, i.e. measured quantity is associated to a particular beam:

SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity.

Correspondingly similar approach should be considered also for LP-SS measurements, so that measurement of LP-RSRP including e.g. averaging over multiple LP-SS bursts separated e.g. by 320ms, should be carried over LP-SS signals associated to same beam.

Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam.

It has been discussed in past meetings, whether LP-SS and LP-WUS need to be present in all the beams, and whether UE can be directly informed about it. In scope of ED-LR that is measuring LP-SS, it could be considered that UE can be configured which LP-SS beams to consider for measurements. This is similar as for SS-RSRP, where based on network configuration select sub-set of SSBs to be used for SS-RSRP measurement:

If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s).

Proposal: For LP-RSRP and LP-RSRQ measurements, it should be possible to network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured.

For the cases when network does not configure LP-SS threshold for the entry condition, or for OFDM-LR, LP-SS based mechanism (namely omitting LP-SS transmission) would not be able to ensure that UE does not attempt to LP-WUS monitoring in any/all SSB beam(s). While one approach is to explicitly define (via bitmap) that LP-WUS is not deployed is some SSB beams is one possible approach, this results a binary decision, which may not be desirable in all deployments. In some deployments it could be desirable to have more flexible adaptation. E.g. for both, sector edge and cell edge scenarios, fully preventing LP-WUS monitoring always may not be preferable if there are stationary UEs (IoT devices) that would benefit from low power operation in paging monitoring and would be in good conditions permanently.

Proposal: For controlling LP-WUS monitoring in SSB level, consider some flexible methods that would enable UEs in very good conditions to monitor LP-WUS even in cell edge area, and allowing more relaxed conditions for cell centre operation.

For LR that is capable of receiving PSS/SSS, it was agreed (as a working assumption) that measurement quantities from NR would be re-used. The question for this meeting is whether this working assumption can be confirmed.

The time and frequency resource of the measurement quantity is defined for SS-RSRP as follows:

SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals

Respectively for SS-RSRQ the time and frequency resource is determined as follows:

Secondary synchronization signal reference signal received quality (SS-RSRQ) is defined as the ratio of N×SS-RSRP / NR carrier RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks. NR carrier Received Signal Strength Indicator (NR carrier RSSI), comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over N number of resource blocks from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc.

Afore definitions do not strictly take any firm position whether the measurement needs to be performed in frequency or time domain. Thus, it would seem viable to reuse the existing definitions for applicable parts. As the operation only relates to IDLE/Inactive mode the applicability of the measurement quantity should be defined accordingly.

Observation: Definitions of NR SS-RSRP and SS-RSRQ can be re-used for the LR that can receive SSS. Applicability should be restricted only to IDLE/Inactive mode.

Now while there are existing measurement quantity definitions are to be re-used, reuse of same quantity naming may not be preferable. For the clarity of reporting and measurement configuration, separating the LR based quantities from MR based quantities might be clearest. Also the SS-RSRP definition contains some aspects that are not directly applicable to LR assumptions, namely the L1-RSRP and use of PBCH DMRS and CSI reference signals. Former is related to CSI reporting, which is not supported to LR operation and latter would require frequency domain processing, which is subject to UE implementation. Hence, for clarity having separate, while identical measurement for relevant parts measurement definitions for LR, would seem clearest.

Observation: For the clarity of separating MR and LR based measurement configurations, it would appear to be preferable that LR based measurement quantities, while using same definition, have their own naming.

Proposal: For LR that is capable of receiving PSS/SSS, RAN1 confirms the working assumption of re-using the measurement quantity definition but separates the definitions of measurement quantities for LR measurements based on SSB (from the MR measurement quantities) for configuration clarity.

Note, that it is also assumed that while LRs processing the signal in time domain can have different sampling rates, the requirements would be defined so that consistent UE reporting is achieved. I.e. UEs correct any bias caused by the applied L1 sampling rate.

Observation: It is expected that performance requirements will ensure consistent reporting from different UEs.

# LP-WUS procedures and content in IDLE/inactive mode

In this section we consider the LP-WUS related configurations and content.

## Aspects related to LP-WUS monitoring configuration

WID objectives determine that duty-cycled LR operation needs to be supported, thus, there would be a need to be able to configure UE with LP-WUS monitoring window. In this section we consider the aspects related to the configuration of LP-WUS monitoring to trigger the paging monitoring in IDLE/Inactive mode.

In RAN1#116 meeting following agreement was made for the definition of LP-WUS monitoring configuration to facilitate the discussion:

Agreement Multi-beam operations are supported for LP-WUS and LP-SS for idle mode Agreement LP-WUS occasions (LOs) are defined for LP-WUS monitoring. - Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs. - Different LP-WUS MOs may correspond to different beams in multi-beam operation - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs - FFS details - It is at least supported that a UE monitors LOs with a configured periodicity. - Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period. - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported) - FFS eDRX, if supported

Further details were agreed in RAN1#116bis:

Agreement For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS - the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation Agreement Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam. - Option 1: K = 1 - Option 2: K can be larger than or equal to 1 - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

In RAN1#118 it was agreed that:

Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE.

In Rel-17 PEI, the monitoring window is determined in relation to the PF. For Rel-17 UE is configured with ‘pei-FrameOffset’ giving the offset between the paging frame and PEI-frame, and with ‘firstPDCCH-MonitoringOccassionOfPEI-O’ determining the start of the PDCCH monitoring occasions where UE will monitor the DCI format 2_7 in search space defined by pei-SearchSpace. The duration of the PEI monitoring window is set to cover one PDCCH monitoring occasion per SSBs for NR.

Figure 1: Illustration of Rel-17 PEI monitoring configuration.

Hence, for LP-WUS monitoring it would be necessary to be able configure the time location and duration of the LP-WUS monitoring window. The time location of this monitoring window would be affected by the minimum distance from the paging occasion to be monitored by MR, and, possibly, the maximum distance for LP-SS. Furthermore, the duration of the LP-WUS monitoring window would be affected by the number of SSBs, assuming at least one LP-WUS occasion per SSB.

The LP-WUS monitoring periodicity, i.e. LO periodicity, would determine how frequently UE needs to monitor LP-WUS. It was agreed in last meeting that LO periodicity is same as the iDRX periodicity. Following from this approach the LO could be associated to a PF via offset, streamlining the configuration. As the offset to meet the UE wake-up time (e.g. 400ms) may need to be large, it would be preferable not to need to configure this offset individually for each LO (or LP-WUS MO). Hence the offset (configured in frames) from PF pointing an earlier reference frame would be common for all POs of the PF. Then for a LO corresponding to a PO, an additional offset from the reference frame would be configured, giving the starting time for a LO. Depending on the approach selected for LP-WUS MO configuration and LP-WUS duration (in time), this could be set either in slot level or symbol level.

Proposal: LO reference time location is defined in relation to PF. One common offset from PF to a reference frame before the PF is configured for all LOs, together with offsets for each PO-LO pair from the reference frame to indicate the start of a LO.

In RAN1#116bis it was agreed that LO would contain set of LP-WUS MOs. To enable UE selecting the LP-WUS MO to be monitored, the LP-WUS MOs should be determined explicitly in time so that UE do the selection of LP-WUS MOs it monitors e.g. one or more LP-WUS MOs associated to a particular beam should have predefined timing locations. Similar approach as done for PDCCH monitoring occasions definition via SS-set could be considered.

Proposal: Define starting symbol for each the LP-WUS MOs in LO.

## Minimum offset between LO/LP-WUS and the associated PO

Next we consider the minimum time offset between LO and LP-WUS MO(s) and the associated PO for MR transition. Following was agreed:

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE.

Firstly, from system perspective association between LP-WUS and PO would be explicit so that each LO/LP-WUS MO addresses certain PO. It was considered whether the exact PO which UE monitors upon triggered by LP-WUS would depend on the UE capability for the MR wake-up delay so that from system perspective the PO associated to LP-WUS would be UE capability dependent or whether the LP-WUS has a common association to a give PO for all UEs. In last meeting, three possible options were for the identified for the association, with an option to consider additional sub-options for UE behaviour when the configured offset(s) do not meet the UE minimum wake-up delay. As discussed in last meeting option 2A does not seem practical as it would in practise enforce network to always to configure (at least) the longest offset when accounting roaming devices. This would also mean that network has limited control over the paging delay. Therefore, option 2A does not seem preferable.

Observation: As option 2A would enforce network to always configure based on the longest supported offset and limit the network control over paging latency, it is not preferrable.

Thus two possible sub-options (under each main option) for further down selection would be:

  - Alt 1-1/2B-1: it monitors the PO associated with the offset.

  - Alt 1-2/2B-2: it monitors the first PO after its reported wake-up delay.

Now as discussed, the paging cycle determination is guided by the latency KPI (based on the supported services) to be met. E.g. if the configured paging cycle (iDRX) is shorter than the UE supported MR wake-up delay, UE should not be allowed to apply MR sleep state that would result increasing the paging delay. If this is not prohibited, LP-WUS operation could not be considered for any UEs where the service requirements mandate applying shorter iDRX cycles. As discussed in study item phase, power saving can be attained by UE with LR operation, even if the MR is in deep sleep state (not in ultra-deep sleep). Correspondingly, if the LP-WUS operation is dependent on the UE specific capability for wake-up delay, it may not be possible to deploy LP-WUS if the network configuration cannot control the resulted paging delay.

Observation: Making the paging delay UE capability dependent, may prohibit the use of LP-WUS in deployments where paging latency target requires specific iDRX configuration.

Effectively, making the paging procedure delay UE capability dependent, could result a need in RAN to handle each UE separately, complicating the system operation. This could require AMF to consider different paging policies and would imply AMF-RAN paging message information would need to contain information regarding the UE capability. Furthermore, RAN would need to handle (split and merge) possible different paging messages.

Observation: Making the paging delay UE capability dependent, would complicate the system paging procedure.

Hence, it is proposed that LP-WUS to PO mapping is explicit so that there is common LP-WUS to PO mapping to all UEs.

Proposal: LP-WUS to PO mapping is configured by network commonly so that all UEs of a LP-WUS share same PO mapping. UEs should respect the configured offset between LP-WUS and PO so that service latency requirement can be met and UE should monitors the PO associated with the configured offset.

Another (main) question was whether to support network configuration of multiple offsets or only one i.e.:

  - Option 1: gNB configures a single offset value.

  - Option 2: gNB configures one or multiple offset values.

The main argument for multiple offset values was to enable simultaneous support of UEs operating with ultra-deep sleep and UEs with deep sleep. As UE supporting wake-up capability that is shorter than the configured offset, it should be able to monitor the LP-WUS also based on longer offset, thus there is no meaningful impact from UE power saving perspective. Now as from system perspective the supported paging latency would be determined by the longest delay configured, thus unless the use of LP-WUS can be restricted service level or limited/excluded from devices that support certain type of services, the offset configuration needs to consider all service requirements/KPIs jointly. Thus, it is not clear whether the different paging delays via multiple offsets can be benefitted from system perspective.

Observation: There does not appear to be any clear benefit from UE power saving nor practical paging latency perspective to support configuration of multiple offsets.

Proposal: It is supported that gNB configures a single offset value between PO and LO.

In previous meeting it was agreed that UE can report the associated capability:

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Agreement Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Another aspect left for further discussion was the number of different possible capability values. Considering that discussion in study item phase and that the LR could assist the MR with the synchronisation [last meeting paper], together with the known LP-WUS monitoring occasions (duty cycled) design , we feel that one value for the minimum offset between LP-WUS and PO should be in range of 400ms. Also, for the UEs that support faster transition, one lower value could be considered. Higher values (beyond 400ms) were also discussed in context of the study item, to enable more complex eMBB type of devices with unoptimized MR activation strategies to be supported by LP-WUS. As eMBB latency requirements are typically driven by end-user experience it would not appear to be preferable to have long delays as they would imply increased paging latency, thus could have limited practical applications. Options of having shorter delays were also considered. This maybe attractive from system perspective, but it should be ensured that these are meaning full from the UE implementation perspective so that power saving benefit can be obtained.

Proposal: RAN1 considers at least one value in range of 400ms for the minimum offset between LP-WUS and the associated PO. One lower value could be considered in addition.

The maximum offset (between LP-WUS and associated PO) would not seem critical parameter from UE implementation perspective, and should account the possible option for mapping more POs to one LP-WUS. Corresponding to this, the LO is defined in relation to the PF.

Another issue that can affect the configuration of the LP-WUS monitoring window, is whether there needs to be some maximum time offset from LP-SS to enable the LR to do good LP-WUS detection. This depends on multiple assumptions, including LP-WUS channel design  and LR assumption for the use of LP-SS [3]. It was agreed earlier that the maximum frequency error would be 5-10ppm. Based on this, LR would also be able to tolerate the maximum drift between two LP-SS e.g. separated by 320ms, hence there would appear to need to have some tolerance build in the LR, namely reception window. Assuming 320ms delta between the two RX occasions for LP-SS, worst case UE would need to account ±3.2us (assuming 10ppm drift) window and method to correct the timing, depending on the applied M value of OOK. As LR can use the NCO to improve the timing error correction (as discussed in [3] for LP-SS) for detection window determination and for LP-WUS the applied known codepoints/codewords together with Manchester encoding can be further benefitted to adjust the timing estimate, there may not be no need to restrict the LP-WUS location in relation to LP-SS location. Also as LP-SS, LP-WUS and additional dynamic RS/preamble would imply similar processing (detecting a known sequence), the UE processing for these would be similar, and LP-WUS could be used by the UE to refine it’s timing estimate. From this perspective, there would not appear to be a need to limit the offset between LP-SS and LP-WUS. Note that it is assumed that separation between LP-WUS and SSB would be limited in most practical scenarios to much lower value than the possible time separation between LP-WUS and LP-SS.

Proposal: No maximum allowed time offset between LP-WUS and LP-SS is defined.

## Mapping options between LO, LP-WUS MO and PO

In earlier meeting it was agreed to consider the options for determining the LO to monitor from sub-grouping perspective, namely whether UE’s belonging to a different sub-groups of a PO, would monitor different LOs. In last meeting it was agreed to support at least option 1. Furthermore the LP-WUS MO design options were identified:

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered. Agreement For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO). Agreement When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between - Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - M = 1 and M > 1 is supported. - FFS: detailed UE monitoring behavior - FFS R=1 or R>= 1 - Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID. - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - FFS: detailed UE monitoring behavior - M = 1 and M > 1 is supported. - FFS R=1 or R>=1 - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

Considering the option 2 and 3 listed in RAN1#117 for PO-LO mapping, following different approaches considered:

- For option 2, where multiple POs would map to same LO, it could be done similarly as in option1, either by having one LP-WUS to carry paging monitoring triggering to subgroups of multiple POs, or that each PO would have separate LP-WUS MO in LO or that the different POs are further split so that LP-WUS corresponds to a sub-set of sub-groups of PO(s).

- Option 3, where the sub-groups of a PO are split over different LOs, or different MOs inside a LO where each LP-WUS MO would carry indication for a part of the sub-groups.

Based on earlier agreements, ‘splitting of PO’ to multiple monitoring occasions is supported in LP-WUS MO level, inside a LO. Hence, considering the option A and option B for the LP-WUS MO configuration could be illustrated as in Figure 2.

As discussed in last meeting, LP-WUS content is assumed to be repetition/duplicate over the R MOs of one of the M groups and that different information (LP-WUS content) can be sent in each M groups. The UE monitoring behaviour in relation to M groups was not defined for option A, e.g. whether UE monitors all M groups, or whether UE monitors only one of the M groups in a predefined manner. For option B it was defined that UE monitors all or some of the MOs within one of the G groups based on the UE ID. E.g. UE selects one of the G groups based on it’s UE ID. In case in Option A that UE would not monitor at least one MO in M groups, it would effectively fallback to Option B. Assuming that if in both options, UE would monitor all M MOs (or at least till it receives it’s wake-up indication), option B would allow LP-WUS to support smaller number of sub-groups, i.e. the number of sub-groups, would be Gth fraction of total number of sub-groups. For Option A, full number of sub-groups per PO would need to be supported in LP-WUS. So splitting sub-groups to multiple LP-WUS MO could be used to reduce the number of information bits needed in one LP-WUS, while allowing to support of larger total number of sub-groups per PO, and to reduce the unnecessary MR false wake-ups.

Observation: Option B would allow lower number of sub-groups per LP-WUS compared to Option A.

Proposal: Option B is adopted for LP-WUS MO configuration, and the configurable number of groups G, is determined together with the LP-WUS content design.

For the MOs within a G groups (of M*R MOs), the if UE monitors all or at least until it has received it’s wake-up indication, network could select in which group of M it would send the wake-up indication to a sub-group. Number of M groups should be configurable, and as shown in results in Section 3.5 M=2 would suffice in majority of considered scenarios, with G=2.

Observation: Based on evaluation results, M={1,2} with G={1,2} configuration could be supported for Option B.

As noted above, selecting Option B, would allow to reduce the payload in LP-WUS, thus helping to meet the coverage. Whether there would be need to support multiple repetitions, R>1, would depend on the LP-WUS design.

Figure 2. Illustration of considered LP-WUS MO configuration options.

As discussed in last meeting, option B of LP-WUS MO configuration would cover the option 3. Option 2, would allow to map multiple POs to one LO. With support of Option B/Option 3, there would not appear to be to large difference from UE perceptive to enable also support of Option 2. E.g. by defining that a sub-set of total of G groups can be assigned to a single PO, option 2 could also be supported without any change for the UE monitoring behaviour.

Proposal: Option 2 is also considered to be supported by allowing G groups to be assigned to different POs

## LP-WUS, LP-SS and SSB association

It was agreed in past meeting that there shall be one to one association between LP-SS, LP-WUS and SSB beams. As raised earlier this would be a pre-requisite for the UE to be able to select LP-WUS beams to be monitored, agreed earlier.  It was also agreed that there will be a QCL relation between the LP-WUS/LP-SS and SSB.

Agreement At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams. Agreement Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB. - FFS QCL Type A or Type C and/or Type D - FFS implicit QCL determination or some signaling is required Agreement The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO.

It was agreed in RAN1#116bis that the LP-SS is a binary sequence that can be configured and/or defined by a rule e.g. based on cell-ID to differentiate different cells (under ag. 9.6.1). In RAN1#118 it was further agreed that the number of binary sequences is 4. This effectively means that LP-SS would not be able to carry any beam indication directly. Hence it would necessary to use time location of LP-SS to associated with specific SSB beam.

Observation: UE should be able to identify the LP-SS beam association with specific SSB index via time location association.

It was agreed in RAN1#116bis that in one LO, there will be one LP-WUS MO for each beam (i.e. ‘N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS’. To complete the UE beam selection, also LP-WUS MO association would need to be determined. If the assumption is that UE can obtain power saving from the LP-WUS beam selection, the LP-WUS MOs (in time) would need to be associated in LP-SS/SSB beams.

Observation: To close the loop in UE LP-WUS beam selection, LP-WUS MOs association to LP-SS/SSB beam should be identifiable via time location association.

Proposal: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams.

It was also agreed in last meeting that each LP-WUS/LP-SS shall be QCL with one SSB and it was left for further discussion. As the assumption is that LP-SS can be used to obtain/measure RSRP, and to attain that UE needs to be provided with the transmitted power for reference. As this is already provided for SSB, it would seen viable that UE is provided with LP-SS EPRE offset to SSB. Hence, as the main purpose of LP-SS is to enable UE to evaluate different thresholds, based on LP-RSRP or LP-RSRQ, at least LP-SS and SSB should be quasi co-located with respect to average gain and QCL-type C. From FR2 perspective, it would seem relevant to support QCL-type D.

Proposal: UE can assume that LP-SS/LP-WUS and the associated SSB are quasi co-located with respect to average gain, quasi co-location 'typeC' and 'typeD', where applicable.

## LP-WUS content

The LP-WUS channel design is discussed under agenda item 9.6.1 and in our contribution [2]. In this section we consider the information content related assumptions and requirements that can be made from IDLE/Inactive mode perspective. As noted in WID, the IDLE/Inactive mode operation should prioritized on the design over Connected mode operation.

Now the WID objective defines that sub-grouping is to be supported for the IDLE/Inactive mode operation. This results direct question for the number of sub-groups that would need to be supported per PO. In Rel-17 PEI operation the number of sub-groups per PO can be configured (subgroupsNumPerPO) from 1 to 8. It was concluded in RAN1#116bis that the mapping of UE ID to sub-group ID is left to RAN2:

Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

In RAN1#117 it was further agreed to narrow down the possible range:

Agreement For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16]. - FFS the exact value of Z Agreement For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. - FFS the exact value of X.

In RAN1#118bis, the number of sub-groups was further discussed, and a working assumption was taken:

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

We evaluated the total false wake-up probability with different number of UEs per PO/LP-WUS for different paging probabilities, 0.001%, 0.01% and 0.1%, results presented in Figures 3 to 11. The user of interest is not considered to be paged, thus evaluation focuses on total false wake-ups accounting false paging, false wake-up due to noise or erroneous code word detection. In comparison to results presented in last meeting, we considered much lower erroneous code word probability (flipping) that will reduce the impact of increased number of sub-groups. The assumptions presented in Appendix of our companion contribution [3]. In short the key assumptions are as follows:

- The number of bits used for LP-WUS is either 8, 12 or 16 bits for all options. Evaluation was done for {8, 16, 32, 64, 128} subgroups.

- Number of UEs per PO considered was from 8 till 2048, with resulted total paging probability shown in square brackets in x-axis.

- It considered that UE monitors {1,2,3,4} MOs and UE ID can be sent in any of them.

- One code point is allocated for a subgroup specific wake-up and one for all subgroup wake-up (referred as N-CP1 in last meeting).

- Probability of error (per bit) against the noise, qn, was assumed to be 0.5, and assuming BLER=1% the probability of bit flipping was estimated as qb≈(1-(1 -BLER)1/B, account the code word (LP-WUS) length and amount of possible codewords.

  - Note that adjusting the probability of flipping a bit for each LP-WUS length effectively assumes better SNR per bit for longer LP-WUS lengths.

| (a) | (b) |
| --- | --- |

Figure 3. False wake-up probability for UE paging probability, pk, of 0.001% with 8 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 4. False wake-up probability for UE paging probability, pk, of 0.001% with 12 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 5. False wake-up probability for UE paging probability, pk, of 0.001% with 16 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 6. False wake-up probability for UE paging probability, pk, of 0.01% with 8 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 7. False wake-up probability for UE paging probability, pk, of 0.01% with 12 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 8. False wake-up probability for UE paging probability, pk, of 0.01% with 16 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 9. False wake-up probability for UE paging probability, pk, of 0.1% with 8 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 10. False wake-up probability for UE paging probability, pk, of 0.1% with 12 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

| (a) | (b) |
| --- | --- |

Figure 11. False wake-up probability for UE paging probability, pk, of 0.1% with 16 bits LP-WUS size for (a) 1 and 2 MOs and (b) 3 and 4 MOs.

Following notes can be made from the results presented:

- Number of sub-groups:

  - Increasing the number of sub-groups reduces the false wake-up probability more with ≥2 MOs

  - With higher paging probabilities and one MO, increasing the sub-groups has limited impact

- Number of MOs:

  - For lower (per UE) paging probability or low number of UEs per PO, there is no significant benefit in increasing the number of MOs beyond one. Increased number of MOs results slight increase in false wake-ups due to false alarms due to noise.

  - For cases with per UE higher paging probability having two MOs provides the most gain that be obtained from increased MOs.

  - In relation to discussion in Section 3.3, it can be observed that {M=2, G=2} results lower false wake-up probability than assuming one group with four MOs {G=1, M=4}.

- These evaluation results imply that LP-WUS length would have limited impact regardless on the consider number of sub-groups. As noted, this is as the flipping probability is normalized to same total codeword error probability for all lengths. However, as shown in [3] (Section 3.1), increasing number of sub-groups for a given LP-WUS length has impact to the probability of detecting code word erroneously (flipping). Effectively, it can be seen from [3] that LP-WUS length needs to be larger than 8 bits, the final value depending on the number of sub-groups.

Observation: Considering 64 sub-groups provides gain over 32 sub-groups with ≥2 MOs, further extending to 128 sub-groups provides less benefit.

Observation: At least two MOs (M=2) should be supported.

Observation: Option to configure two split LP-WUS MO’s to two groups (G=2), each with two MOs should be considered.

Proposal: Consider extending the number of supported sub-groups per PO to 64. Limit the number of sub-groups per LP-WUS to 32.

Proposal: Support LP-WUS MO configuration where MOs can be split to two groups (G=2) and support two MOs per group (M=2).

For other information in LP-WUS for IDLE/Inactive mode operation i.e. shortMessage, does not seem implicitly necessary. When SI update or ETWS is triggered, UE would need to monitor/receive with MR for a while, thus it would not seem necessary to apply any related information to LP-WUS. Moreover assuming that the LP-WUS information payload is limited (to ensure performance and footprint) it would seem preferable to prioritize paging sub-grouping related information.

Proposal: Do not consider shortMessage related information separately in LP-WUS information content.

# Other aspects for UE power saving in IDLE/Inactive mode

## False alarm cost reduction

As noted in study item phase, the key aspect driving the UE power consumption for IDLE/Inactive mode operation, is the MR transition energy cost. Hence, if UE is repeatedly woken up by LP-WUS to monitor paging with MR, the power consumption benefits are negatively impacted. While this mostly relates to number of paging occasions (PO) and paging frames and possibility to indicate sub-groups when triggering paging monitoring to reduce the false alarms, this also relates to the paging load. In many scenarios the paging load is not evenly distributed and in certain times of day the paging activity may significantly increase from the normal/average. In such situations the paging message size (due to large number of UE IDs) may result that there will be paging in successive paging occasions, increasing the probability with LP-WUS operation that MR would need to be triggered to monitor paging. This would result increased UE power consumption, due to frequent MR transitions from ultra-deep sleep to normal active mode. Therefore it would be useful to consider methods to make UE aware whether it should expect further paging for the corresponding PO in following paging cycle.

For NR-U, new short message based indication was introduced, as shown in Table 1, for informing the UE that UE may cease paging monitoring. As in shared spectrum operation the number of PDCCH monitoring occasions per SSB can be increased (to overcome LBT), this indication may shorten the PDCCH monitoring time. For operation with LP-WUS operation, especially with eDRX, adopting some method to assist the UE to avoid unnecessary MR transitions could be considered. Using the NR-U based indication or similar indication via short message to inform UE whether paging monitoring can be stopped, could allow the UE to know whether it needs to monitor the next PO. Other alternatives could also be considered, e.g. based on UE implicitly on paging message size or number of paging records whether the same paging message will be repeated, or directly indication that next message will be a repetition.

Table 1: Short Messages [TS38.331]

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period. |
| 5 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

Observation: To reduce the energy cost of repeated paging monitoring due to MR wake-ups, UE should be provided assistance information whether it should expect further paging in following paging occasion.

## eDRX support for LP-WUS

It has also been discussed in past meetings whether and how the eDRX should be accounted in LP-WUS operation/configuration. In legacy eDRX operation, UE wakes up every eDRX cycle to monitor paging during the PTW. As considered in the evaluations, the paging probability is not even during the PTW, thus depending on the overall paging load and applied eDRX cycle, there can be some congestion at the start of the PTW. Paging message can of course arrive to RAN during the PTW, thus normal IDLE mode type of operation would also need to be supported during PTW. Hence, from system perspective, in minimum it should be possible to page the UE like with normal eDRX operation during the PTW. Therefore it would seem that most straight forward LP-WUS operation with eDRX would be that if UE is configured with LP-WUS, UE can monitor the LP-WUS before each paging occasion for the PTW. This would be aligned with the Rel-17 PEI operation.

Proposal: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW.

## LP-SS and LP-WUS configuration

As discussed in section 3.1, LP-WUS monitoring configuration needs to be provided to the UE. Correspondingly the time domain configuration would need to be provided for LP-SS also. For this it can be assumed that UE has knowledge of the cell timing and associated sub-carrier spacing. Effectively , for LP-SS, it would be expected that UE would be provided with a periodicity and starting time location for the LP-SS. The start time should contain at least SFN but depending on the approach also an additional, e.g. slot level offset may be needed. If the time domain pattern of the LP-SS is fixed (as for SSB) it would not need to be separately provided, but if LP-SS time locations can be configured by network, this information would need to be separately provided. In scope of the IDLE/Inactive mode, one option to deliver this information to UE would be via broadcast, but the details can be left for RAN2.

Observation: Both LP-WUS and LP-SS time and frequency resources need to be provided for the IDLE/Inactive mode UE. Delivery method is up to RAN2.

Observation: For LP-SS the time domain configuration would need to contain periodicity and start time (SFN) possibly with additional e.g. slot level offset. Furthermore, fixed time pattern or configurable time locations for LP-SS corresponding to SSB would be needed.

In addition to time domain configuration, UE needs also to be provided information for the frequency domain physical resources for the LP-WUS and LP-SS. As for time location definition, it can be assumed that UE as already acquired necessary information regarding the frequency domain occupancy of the serving cell. Considering the frequency domain resource configuration, as UE as acquired access to the cell, the LP-WUS and LP-SS could be configured on the common resource grid based on sub-carrier spacing applied for the LP-WUS. I.e. the frequency location would be configured based on the first common RB in corresponding scsCarrier. This would follow the configuration done for example for the CSI-RS. Other possible approach would be to configure the location directly based on PointA. Furthermore one approach would be to configure the LP-WUS and LP-SS location in relation to the active BWP e.g. as for CORESET.

Observation: Frequency location of LP-WUS and LP-SS can be determined using existing frequency resource grids/references, such as common resource grid.

Related to this the question, it would need to be discussed how the LP-SS/LP-WUS need to be located in relation to the UE active BWP (e.g. initial BWP). Additional aspect is, whether LP-WUS and LP-SS need to be on (approximately) on same frequency resources or whether they can be configured independently of each other. From LR perspective, different resources being in non-overlapping resources can require the UE to change the centre frequency of the LR, thus fully independent and non-overlapping configuration may not be attractive. Furthermore, if LR is able to detect SSB, it would need to be discussed whether there are some restrictions the frequency location relation of LP-WUS and SSB.

Observation: For frequency domain configuration, it would be beneficial to agree whether LP-SS/LP-WUS configuration needs to have some relation to UE active BWP and in relation to each other and/or SSB.

# Conclusion

In this contribution we have discussed aspects related to the LP-WUS operation in IDLE/Inactive mode.

In Section 2.1 we discuss the LR measurement quantities, with following observation and proposals:-

Observation: If Manchester encoding is not applied to the LP-SS and depending on the binary sequence selection for LP-SS, the current definitions might lead to inconsistent results for LP-RSRQ and for LP-RSRQ between different LP-SS sequences

Proposal: Once LP-SS sequence design has been finalized, verify if the LP-RSRP and LP-RSRQ metric definitions need to be revisited.

Proposal: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam

Proposal: For LP-RSRP and LP-RSRQ measurements, it should be possible to network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured.

Proposal: For controlling LP-WUS monitoring in SSB level, consider some flexible methods that would enable UEs in very good conditions to monitor LP-WUS even in cell edge area, and allowing more relaxed conditions for cell centre operation.

Observation: Definitions of NR SS-RSRP and SS-RSRQ can be re-used for the LR that can receive SSS. Applicability should be restricted only to IDLE/Inactive mode.

Observation: For the clarity of separating MR and LR based measurement configurations, it would appear to be preferable that LR based measurement quantities, while using same definition, have their own naming.

Proposal: For LR that is capable of receiving PSS/SSS, RAN1 confirms the working assumption of re-using the measurement quantity definition but separates the definitions of measurement quantities for LR measurements based on SSB (from the MR measurement quantities) for configuration clarity.

Observation: It is expected that performance requirements will ensure consistent reporting from different UEs.

In Section 3.1 we consider the aspects related to the LP-WUS monitoring configuration:-

Proposal: LO reference time location is defined in relation to PF. One common offset from PF to a reference frame before the PF is configured for all LOs, together with offsets for each PO-LO pair from the reference frame to indicate the start of a LO.

Proposal: Define starting symbol for each the LP-WUS MOs in LO.

In Section 3.2 we discuss the minimum offset between LO/LP-WUS and the associated PO:-

Observation: As option 2A would enforce network to always configure based on the longest supported offset and limit the network control over paging latency, it is not preferrable.

Observation: Making the paging delay UE capability dependent, may prohibit the use of LP-WUS in deployments where paging latency target requires specific iDRX configuration.

Observation: Making the paging delay UE capability dependent, would complicate the system paging procedure.

Proposal: LP-WUS to PO mapping is configured by network commonly so that all UEs of a LP-WUS share same PO mapping. UEs should respect the configured offset between LP-WUS and PO so that service latency requirement can be met and UE should monitors the PO associated with the configured offset.

Observation: There does not appear to be any clear benefit from UE power saving nor practical paging latency perspective to support configuration of multiple offsets.

Proposal: It is supported that gNB configures a single offset value between PO and LO.

Proposal: RAN1 considers at least on value in range of 400ms for the minimum offset between LP-WUS and the associated PO. One lower value could be considered in addition.

Proposal: No maximum allowed time offset between LP-WUS and LP-SS is defined.

In Section 3.3 we the association between LO, LP-WUS and PO is considered, focusing on splitting PO sub-groups to different LP-WUS MOs or LOs:-

Observation: Option B would allow lower number of sub-groups per LP-WUS compared to Option A.

Proposal: Option B is adopted for LP-WUS MO configuration, and the configurable number of groups G, is determined together with the LP-WUS content design.

Observation: Based on evaluation results, M={1,2} with G={1,2} configuration could be supported for Option B.

Proposal: Option 2 is also considered to be supported by allowing G groups to be assigned to different POs

Section 3.4 considers the issues related to the determination of beams association:-

Observation: UE should be able to identify the LP-SS beam association with specific SSB index via time location association.

Observation: To close the loop in UE LP-WUS beam selection, LP-WUS MOs association to LP-SS/SSB beam should be identifiable via time location association.

Proposal: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams.

Proposal: UE can assume that LP-SS/LP-WUS and the associated SSB are quasi co-located with respect to average gain, quasi co-location 'typeC' and 'typeD', where applicable.

In Section 3.5 we discuss LP-WUS content presenting some analysis results and make following observations and proposals:-

Observation: Considering 64 sub-groups provides gain over 32 sub-groups with ≥2 MOs, further extending to 128 sub-groups provides less benefit.

Observation: At least two MOs (M=2) should be supported.

Observation: Option to configure two split LP-WUS MO’s to two groups (G=2), each with two MOs should be considered.

Proposal: Consider extending the number of supported sub-groups per PO to 64. Limit the number of sub-groups per LP-WUS to 32.

Proposal: Support LP-WUS MO configuration where MOs can be split to two groups (G=2) and support two MOs per group (M=2).

Proposal: Do not consider shortMessage related information separately in LP-WUS information content.

In section 4.1 we discuss about methods to reduce the false alarms for the IDLE/Inactive mode UEs:-

Observation: To reduce the energy cost of repeated paging monitoring due to MR wake-ups, UE should be provided assistance information whether it should expect further paging in following paging occasion.

Aspects related to eDRX are presented in Section 4.2:-

Proposal: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW.

The configuration of LP-WUS and LP-SS is discussed in Section 4.3:-

Observation: Both LP-WUS and LP-SS time and frequency resources need to be provided for the IDLE/Inactive mode UE. Delivery method is up to RAN2.

Observation: For LP-SS the time domain configuration would need to contain periodicity and start time (SFN) possibly with additional e.g. slot level offset. Furthermore, fixed time pattern or configurable time locations for LP-SS corresponding to SSB would be needed.

Observation: Frequency location of LP-WUS and LP-SS can be determined using existing frequency resource grids/references, such as common resource grid.

Observation: For frequency domain configuration, it would be beneficial to agree whether LP-SS/LP-WUS configuration needs to have some relation to UE active BWP and in relation to each other and/or SSB.

# References

RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

R1-2401872, Summary #4 (final) on LP-WUS operation in IDLE/INACTIVE mode, Moderator (Apple)

R1-2408537, LP-WUS and LP-SS design, Nokia

R1-2307418, Low power WUS Evaluation Methodology, Nokia