3GPP TSG RAN WG1 #119                                                                              R1-2409448

Orlando, US, November 18th – 22nd, 2024

Source:                QUECTEL

Title:	Discussion on on-demand SSB SCell operation

Agenda Item:	9.5.1

Document for:	Discussion and Decision

Introduction

In RAN1#118bits meeting, several agreements were achieved related to on-demand SSB SCell operation [1].

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not Conclusion No consensus on the support of on-demand SSB SCell operation triggered by UE. Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

In RAN1#118bit meeting, several agreements were achieved related to on-demand SSB SCell operation [2]

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4

This contribution discusses on-demand SSB SCell operation. Section 2 discusses the scenarios and cases of on-demand SSB SCell operation. Section 3 discusses contents of on-demand SSB configuration/indication. Finally, section 4 summarizes the observation and proposals with conclusions.

Discussion

Scenarios and Cases

In RAN1#116-bis meeting, one agreement was achieved related to the scenarios and cases of on-demand SSB SCell operation [4].

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

As shown in Figure 1, there are four scenarios. Scenario 2 starts from the configuration of the SCell and ends before the SCell is activated via MAC CE. Scenario 2A represents the moment when the SCell is activated via MAC CE, Scenario 3A represents the entire SCell activation duration, and Scenario 3B represents the state after the SCell activation is completed.

We first need to understand the role of the SSB during legacy SCell activation. SCell activation is completed after the UE reports a valid CSI. If the L3-RSRP has been reported within a certain period before the SCell activation command, the gNB can acquire the UE's measurement results in advance. When the UE receives the SCell activation command, this SCell is referred to as a known SCell. So after receiving the SCell activation command, the UE would use the SSB for fine time tracking.

On the other hand, if no L3-RSRP report is available before the SCell activation command, the SCell is considered an unknown SCell. After receiving the SCell activation command, the UE must perform AGC, synchronization, and L1 measurement/report based on the SSB. The gNB then activates the TCI state for data transmission according to the UE's report. As a result, a longer activation delay is required for the unknown SCell. In FR2, each UE Rx beam requires approximately 4 rounds of SSB bursts, with 2 rounds used for AGC stabilization, 1 round for synchronization, and 1 round for L1 measurement. Assuming a typical FR2 UE (equipped with 8 Rx beams), SCell activation would require approximately 32 rounds of SSB bursts, resulting in a long activation delay of about 640ms. Such a significant delay would force the gNB to remain active for an extended period, preventing it from transmitting data quickly and entering a deeper sleep mode.

Therefore, a dense periodic SSB configuration designed to accelerate SCell activation is crucial. Different SSB configurations can be used to achieve different functions, resulting in greater scheduling flexibility, improved UE experience, and increased network energy savings. Through RRC configuration messages, two or more configurations can be provided to the UE, with one being indicated via dynamic signaling. Although multiple configurations can be provided, the gNB must specify an active configuration for a particular SCell function at a certain time. For example, an SSB configuration can be set for different scenarios in Figure 1. Configurations with different periodicities can trigger different SSB modes and be indicated to UEs with varying mobility and/or traffic needs.

The three SSBs with different periodicities during the SCell configuration (addition) can be configured, as shown in the figure 2. Before SCell activation, configuration #1 (with a long periodicity, such as 160ms) is indicated, which is used for L3 measurements to evaluate the appropriate cell for a given UE to activate, while allowing the SCell to have more sleep opportunities. After the SCell is activated, configuration #2 (with a dense periodicity, such as 5ms or less) is indicated to speed up SCell activation. After SCell activation, the gNB switches to configuration #3 (with a normal periodicity, such as 20ms) to balance reliable beam management with network energy savings. Additionally, if there is an always-on SSB, and that SSB is triggered to achieve faster SCell activation, once the SCell enters active mode, the SSB can be muted (i.e., no longer transmitted) to save energy.

When configuring the SCell, three SSBs with different periodicities can be switched based on the network status and the phase of cell activation. One example is the use of three SSB configurations with different periodicities: assume that the periods for configuration #1, configuration #2, and configuration #3 are T1, T2, and T3, respectively. There can be an inherent relationship established between these three periods.

Based on network load and Quality of Service (QoS) requirements, for example, when the network load is high, the period of T3 will shorten to speed up SCell beam management; when the network load is low, the periods of T1 and T3 will increase to save energy. This newly introduced SSB adjustment feature will ensure flexibility, providing a variety of target services throughout the SCell's lifecycle, and activating specific configurations in a timely manner to respond to UE needs as functions are required. As part of the SCell configuration, candidate configurations are provided in the RRC configuration message, and L1/L2 signaling sent over the PCell can dynamically adapt (activate) one of them. The flexibility of this new SSB adjustment feature will ensure diverse service offerings during the SCell's lifecycle, and specific configurations can be activated promptly for UE when needed.

By applying densely periodic SSBs, such as a 2ms cycle, the activation delay of the SCell in FR2 can be reduced by several hundred milliseconds. This time saving can help the SCell enter a deeper sleep mode after data transmission is completed. In the same FR2 unknown cell configuration, the delay can be reduced from 640ms to 64ms, allowing the network to enter sleep mode sooner after data transmission is completed. In addition to maintaining network energy savings, this also improves the perceived transmission speed for users.

Proposal 1：The three different SSB periodicities during the SCell configuration can be configured, before SCell activation, configuration #1 (with a long periodicit) is indicated, which is used for L3 measurements to evaluate the appropriate cell for a given UE to activate, After the SCell is activated, configuration #2 (with a dense periodicity) is indicated to speed up SCell activation. After SCell activation, the gNB switches to configuration #3 (with a normal periodicity) to balance reliable beam management with network energy savings.

Proposal 2 : There can be an inherent relationship established among configuration #1, configuration #2, and configuration #3 .

- 2.2  on-demand SSB be triggered

A UL WUS signal can be triggered when the received signal strength from the reference cell associated with the SCell (determined by predefined rules or explicitly configured by higher-layer parameters) falls below a given threshold. When the base station sends SCell activation signaling, it can simultaneously trigger the on-demand SSB transmission from the SCell. On-demand SSB SCell operation triggered by UE or Network， there are pros and cons in these two circumstances.

Alt1: The on-demand SSB SCell operation is triggered by the UE, and views on the design of UL WUS using existing signals/channels are provided.

The UE can send a UL WUS (Uplink Wake-Up Signal) on an SCell (without SSB) to request SSB transmission, utilizing existing signals/channels for the UL WUS ( such as PRACH, Periodic/Semi-persistent PUCCH/PUSCH).

For PRACH, dedicated PRACH resources can be configured to trigger on-demand SSB for a specific SCell (in this case, msg3 PUSCH is not required), or PRACH resources can be associated with multiple SCells,or   For periodic/semi-persistent PUCCH/PUSCH, even if PUCCH/PUSCH resources are allocated to the UE, the UE might not use the allocated PUCCH/PUSCH resources unless on-demand SSB is triggered. In other words, once on-demand SSB transmission for the SCell is triggered, the UE will transmit SCell-related PUCCH/PUSCH, carrying the necessary information for SSB transmission on the SCell.

Alt2: The gNB CU can request the gNB DU to activate specific SSB beams by indicating the SSB index, and it can confirm whether on-demand SSB triggering is supported over the F1 interface. The UE can send a UL WUS signal to the PCell to request the SCell to send SSB, or it can directly send a UL WUS signal to the SCell to request SSB transmission. The resource configuration information for the UL WUS signal can either be allocated by the PCell or be pre-configured by the SCell.

In both UE-triggered and network-triggered on-demand SSB operations, there are pros and cons depending on the scenario and system requirements. Let's analyze the advantages and disadvantages of each trigger mechanism.

- UE-triggered on-demand SSB operation

Pros:

- Lower Latency in Non-Ideal Backhaul Scenarios:

In non-ideal backhaul carrier aggregation (CA) scenarios, where the backhaul between the Primary Cell (PCell) and the inactivated SCell is suboptimal, UE-triggered SSB can reduce latency by allowing the UE to directly request SSB transmission. This avoids delays associated with relying on the gNB's decision-making through the backhaul, especially when the gNB has incomplete or delayed information about the UE's conditions.

- Faster Recovery in Beam Failure Situations

When an SCell beam failure occurs, the UE can quickly trigger on-demand SSB, leading to faster beam failure recovery. The legacy recovery procedure relying on periodic SSB transmission from the gNB may take longer, especially if the SSB periodicity is not optimized for fast recovery. UE-initiated SSB can help reduce downtime in this case.

- Efficient SCell Activation for Uplink-Heavy Scenarios:

When the SCell is configured but not yet activated, and the UE experiences heavy uplink traffic, the UE can immediately trigger on-demand SSB for faster SCell activation, by passing the process of informing the PCell and waiting for the gNB to respond. This is more efficient than waiting for the uplink buffer status to be transmitted to the PCell, reducing overall delay.

- Accurate Timing Adjustment:

For cases where the downlink reception timing difference (RTD) between the SSB-less SCell and its associated reference cell(s) exceeds a given threshold, the UE can trigger on-demand SSB to re-align and minimize latency. The gNB may not have real-time information on DL reception timing conditions (especially in inter-band CA scenarios), making the UE-triggered option more responsive.

- Immediate Knowledge of Local Conditions:

The UE has more direct and immediate knowledge of local conditions (such as signal strength, beam failures, or uplink buffer status) compared to the gNB. This allows the UE to take proactive steps and trigger the on-demand SSB at the most appropriate time.

Cons:

- Potential for Increased Signalling Overhead:

Frequent triggering of on-demand SSB by the UE, especially in high mobility or fluctuating channel conditions, could increase uplink signaling overhead. This might lead to more frequent interruptions and an overall increase in network traffic.

- Complexity in UE Decision-Making:

The UE must be equipped with sophisticated decision-making algorithms to determine when and under what conditions to trigger on-demand SSB, which adds to the complexity of UE design.

- Possible Network Resource Misalignment:

The UE-triggered approach may result in resource allocation that is not fully aligned with the gNB’s view of network resources or load. The gNB may be better positioned to manage network-wide resource allocation efficiently.

- Network-triggered on-demand SSB operation

Pros:

- Centralized Resource Control and Management:

The gNB has a broader view of network resources, load, and performance metrics, allowing it to make more informed decisions about when to activate SSBs. This centralized control helps ensure optimal allocation of resources and avoids potential conflicts or inefficiencies arising from decentralized decision-making.

- Coordination Across Multiple Cells:

The gNB can better coordinate across multiple cells, ensuring that triggering SSB on one SCell does not negatively impact other cells or ongoing operations. This is particularly important in scenarios involving multiple SCells, where careful management is required to avoid resource contention.

- Reduced Signaling Overhead:

Since the gNB controls when to trigger the on-demand SSB, unnecessary signaling from the UE can be avoided. The gNB can minimize SSB transmission and focus on sending SSBs only when it determines that the network conditions require it, reducing overall network signaling load.

- Better Network-Level Optimization:

With knowledge of the broader network load, QoS requirements, and traffic patterns, the gNB can make more optimized decisions for triggering SSB. For example, the gNB can prioritize triggering SSB for UEs with higher priority traffic or more stringent QoS requirements.

Cons:

- Higher Latency in Non-Ideal Backhaul Scenarios:

In cases where the backhaul between the PCell and the SCell is not ideal (non-ideal CA scenarios), the gNB might have delayed or incomplete information about the UE’s conditions, leading to longer delays in triggering SSB transmissions. This can be particularly problematic for latency-sensitive applications like VoIP or video streaming.

- Slower Response to Beam Failures:

In beam failure scenarios, waiting for the gNB to detect the failure and trigger on-demand SSB can introduce additional delays, especially if the gNB relies on periodic SSB transmissions. UE-triggered SSB can address these beam failures more rapidly.

- gNB Might Lack Real-Time Knowledge of UE Conditions:

The gNB may not always have up-to-date or granular knowledge of the UE’s local conditions, such as uplink buffer load, downlink timing issues, or precise beam failures. This can lead to suboptimal timing in triggering the SSB, potentially increasing the overall delay in SCell activation.

- Less Flexibility for UE-Driven Use Cases:

In cases where the UE has more immediate and local knowledge (such as heavy UL traffic or beam failure recovery), relying solely on the gNB to trigger SSB could reduce the system’s responsiveness and flexibility, leading to missed opportunities for efficiency gains.

Therefore, UE-triggered on-demand SSB is advantageous in scenarios where the UE has more immediate and localized knowledge of conditions, such as in non-ideal backhaul scenarios, beam failure recovery, or when uplink traffic is high. This approach offers faster response times and greater flexibility in handling SCell activation, particularly in latency-sensitive or beam management situations. However, it may lead to increased signaling overhead and requires more complex decision-making logic at the UE level. Network-triggered on-demand SSB, on the other hand, provides better centralized control and resource management, which is critical for ensuring overall network efficiency and coordination. It reduces signaling overhead and enables the gNB to make more informed decisions based on network-wide conditions. However, it may result in higher latency in scenarios where the gNB lacks real-time visibility into UE-specific conditions or when backhaul is not ideal.

Ultimately, the choice between UE-triggered and network-triggered on-demand SSB should depend on the specific use case, with both approaches potentially complementing each other in a hybrid solution where the gNB dynamically allows the UE to trigger SSB when necessary while maintaining centralized control over network resources.

Proposal 3: UE-triggered and network-triggered on-demand SSB should depend on the specific use case, with both approaches potentially complementing each other in a hybrid solution .

On-demand SSB Transmission Indication

In RAN1#117 meeting, RRC based signaling and MAC-CE based signaling have been both supported to indicate on-demand SSB transmission [3]. In RAN2#127 meeting, the following agreements on RRC and MAC-CE indication were agreed:

Agreements on OD-SSB

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

In RAN1#118 meeting, RRC and MAC CE based signaling mechanisms were introduced to indicate on-demand SSB transmission. (Group-common or UE-specific) DCI also can be considered to indicate on-demand SSB transmission.

- Option 1: RRC based signaling ,RRC signaling is supported to indicate the on-demand SSB transmission on the cell, and the configuration index of one of the multiple pre-configured OD-SSB configurations for on-demand SSB transmission can be indicated.

- Option 2: MAC CE based signaling ,MAC CE signaling is supported to indicate the on-demand SSB transmission on the cell, and the index of one of the pre-configured OD-SSB configurations for on-demand SSB transmission can be indicated.

- Option 3: (Group-common or UE-specific) ,DCI signaling is supported to indicate the on-demand SSB transmission on the cell, and the index of one of the pre-configured OD-SSB configurations for on-demand SSB transmission can be indicated.

On-demand SSB can be used for L3 measurements. When there is a measurement requirement, both RRC and MAC CE signaling can indicate the on-demand SSB transmission. When a cell supports on-demand SSB transmissions, multiple configurations can be pre-set, such as OD-SSB configuration #0, OD-SSB configuration #1, etc. These configurations provide flexibility, allowing the UE to use different synchronization resources based on network conditions, user requirements, or measurement needs. The network can instruct the UE on which configuration to use via the signaling methods described above (RRC, MAC CE, or DCI).

On-demand SSB configuration by RRC should include time domain, frequency domain, spatial domain and power parameters. For Case#1 (SCell without always-on SSB), all these parameters should be configured by RRC. For Case#2 (SCell with always-on SSB), some parameters should share the same value as that of always-on SSB, e.g. frequency, SCS, and transmit power. These parameters could be omitted in on-demand SSB configuration. MAC CE-based signaling is supported to indicate on-demand SSB transmission for Scenario #2 and #2A. For cells that support on-demand SSB-SCell operations, the following information also needs to be configured, where RRC can configure multiple candidate values, and MAC CE can indicate the applicable value for on-demand SSB transmission on the cell。

Based on the previous agreement regarding contents for on-demand SSB, it will be discussed in this meeting whether other parameters are needed besides periodicity.

- Frequency for on-demand SSB

- location SSB in an on-demand SSB burst

- Periodicity of the on-demand SSB

- Subcarrier spacing of the on-demand SSB

- Physical cell ID for on-demand SSB

- Position of the on-demand SSB burst

- Downlink transmission power for on-demand SSB

- Number of SSB burst transmissions

- T-value (to determine time instance A)

- OD-SSB configuration index

- The number of SSB bursts, if supported

According to the above agreements, for RRC based signaling, OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. Similar as the parameter sCell State used for SCell activation, a new RRC parameter, e.g. OD-SSB State, could be employed for each SCell as OD-SSB transmission indication. Considering the agreement of RAN1 (i.e. this RRC also configures the SCell), OD-SSB State should be configured in SCell Config as shown in the following:

Considering the requirements of different deployment scenarios (e.g. L1/L3 measurements, SCell activation), multiple sets of on-demand SSB configurations are preferred to be available for a cell. In the last meeting, multiple candidate values for periodicity were agreed to be configured via RRC. In addition, the TX behavior of on-demand SSB may differ between L1/L3 measurements and SCell activation. For example, during L1/L3 measurements, a relatively long duration of on-demand SSB transmission is required to obtain stable channel quality. And SSB would be transmitted in beam sweeping mode to determine all possible UE positions. However, for SCell activation, a relatively short during of on-demand SSB transmission would be preferred to reduce latency, and SSB may be transmitted in specific directions based on the knowledge of UE positions. Therefore, besides periodicity, multiple candidate values can also be configured by RRC for the following parameters:

Proposal 4: More than one on-demand SSB configuration can be configured for the cell to UE.

Proposal 5: The following information for on-demand SSB should be included in on-demand SSB configuration:

- Frequency for on-demand SSB

- location SSB in an on-demand SSB burst

- Periodicity of the on-demand SSB

- Subcarrier spacing of the on-demand SSB

- Physical cell ID for on-demand SSB

- Position of the on-demand SSB burst

- Downlink transmission power for on-demand SSB

- Number of SSB burst transmissions

- T-value (to determine time instance A)

- OD-SSB configuration index

- The number of SSB bursts, if supported

As mentioned above, if multiple sets of on-demand SSB configuration are supported, on-demand SSB configuration ID should be carried by MAC-CE. In addition, considering the possible change of UE distribution due to UE movement, SSB positions within an on-demand SSB burst are also preferred to be included in the MAC-CE. Further, the same MAC-CE may also be used for the deactivation of on-demand SSB.

Conclusion

In this contribution, we discussed on-demand SSB SCell operation, and gave the following proposals:

Proposal 1：The three different SSB periodicities during the SCell configuration can be configured, before SCell activation, configuration #1 (with a long periodicit) is indicated, which is used for L3 measurements to evaluate the appropriate cell for a given UE to activate, After the SCell is activated, configuration #2 (with a dense periodicity) is indicated to speed up SCell activation. After SCell activation, the gNB switches to configuration #3 (with a normal periodicity) to balance reliable beam management with network energy savings.

Proposal 2 : There can be an inherent relationship established among configuration #1, configuration #2, and configuration #3 .

Proposal 3: UE-triggered and network-triggered on-demand SSB should depend on the specific use case, with both approaches potentially complementing each other in a hybrid solution.

Proposal 4: More than one on-demand SSB configuration can be configured for the cell to UE.

Proposal 5: The following information for on-demand SSB should be included in on-demand SSB configuration:

- Frequency for on-demand SSB

- location SSB in an on-demand SSB burst

- Periodicity of the on-demand SSB

- Subcarrier spacing of the on-demand SSB

- Physical cell ID for on-demand SSB

- Position of the on-demand SSB burst

- Downlink transmission power for on-demand SSB

- Number of SSB burst transmissions

- T-value (to determine time instance A)

- OD-SSB configuration index

- The number of SSB bursts, if supported

References

- Chair notes RAN1#118bits, 3GPP TSG RAN WG1 Meeting #118, October 14th – 18 th, 2024.

- Chair notes RAN1#118, 3GPP TSG RAN WG1 Meeting #118, August 19th – 23 th, 2024.

- Chair notes RAN1#117, 3GPP TSG RAN WG1 Meeting #117, May 20th – 24th, 2024.

- Chair notes RAN1#116-bis, 3GPP TSG RAN WG1 Meeting #116-bis, April 15th –19th, 2024.

- 3GPP TS 38.133, Requirements for support of radio resource management (Release 18), V18.4.0 (2023-12).

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN#102 Meeting, December 11th-15th, 2023.