3GPP TSG RAN WG1 #119			R1- 2409455

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	9.1.1

Source:	InterDigital, Inc.

Title:	Discussion on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new work item on AI/ML for NR Air Interface has been approved [1], that builds on the previous Rel-18 studies in FS_NR_AIML_Air [2].

For the beam management use case, the objective of the study is to address some of the outstanding issues identified during the Rel-18 Study Item, as summarized in TR 38.843 [3].

With reference to beam management, the study objectives listed in the Rel-19 WID are shown below [1].

WID: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management in NR air interface based on BM-Case1 and BM-Case2. Additionally, we provide an updated set of evaluation results for beam prediction.

# Discussions

## Response to RAN2 LS

In RAN2#127 [9], an LS on applicabile functionality reporting for UE-sided model was sent to RAN1. The LS includes several questions and we propose the following response:

Proposal 1: The following responses to the questions in RAN2 LS are adopted.

On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? - In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI. On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? - RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list. - Q3: Is NW-side additional condition functionality specific? - RAN 1 hasn’t made any conclusion on whether NW-side additional condition functionality specific or not yet. - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - Inference related parameters can be a basis to determine the applicability. For example, NW can provide candidate inference parameters (e.g., as a separate I.E. in Option 2 or a CSI report config in Option 1), so that UE can identify its applicable inference parameters among the candidate inference parameters. In another example, UE can indicate applicable inference parameters in Option 3 and so that NW can activate a CSI report configuration based on the reported applicable inference parameters. - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - It depends on the selected option. For example, inference parameters can be included in Step 3 for Option 1 and Option 2 while Option 3 does not need additional inference parameters as UE provides applicable inference configuration in Step 4. - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - Inference parameters represent applied parameters during training of AI/ML model based on NW-side additional condition. - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - According to RAN1 agreement, the inference parameters can be at least including Set A related information, Set B related information, Report content related information and Time instances related information for measurements and prediction for BM-Case 2. - Q5: What is the content of applicable functionality reporting in Step 4? - The content of applicable functionality reporting also depends on the selected option. For Option 1, UE indicates applicable CSI report config IDs while applicable inference parameter IDs are indicated in Option 2. In Option 3, UE needs to indicate applicable sets of inference configurations. - Q6: What is the content of inference configuration in Step 5? - The content of inference configuration also depends on the selected option. For Option 2 and Option 3, a CSI report configuration for inference should be configured based on the UE applicability reporting in Step 4 while Option 1 may not need Step 5. On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? - It does not activate the functionality immediately. - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? - For periodic CSI reporting, the configured CSI report configuration in Step 5 can be activated immediately upon receiving Step 5. For semi-persistent/aperiodic CSI reporting, additional activation/trigger is required. - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? - Multiple/all applicable functionalities can be activated within UE capability. - Q10: Is L1/L2 signaling for functionality activation/deactivation needed? - L1/L2 signaling for functionality activation/deactivation can be additionally considered for better flexibility.

## Associated ID

In RAN1#118 [8], the following agreement was made on associated ID including the working assumption on configuration of the associated ID within CSI framework:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

While high level principle of inference related configurations, based on CSI framework with two resource sets for Set A and Set B, respectively, is agreed, how to configure data collection related configuration was not actively discussed. However, data collection related configuration is one of the major factors for designing a framework to utilize the associated ID as the motivation of the associated ID is to ensure consistency between training and inference. In that regard, how to configure the data collection related configuration should be decided before confirming the working assumption.

Proposal 2: Clarify the definition and the detailed signaling of data collection related configuration before confirming the working assumption on the support of associated IDs in CSI framework.

According to the observations from the SI [3], it is well known that one generalized AI/ML model trained for mixed scenarios provide performance benefits for the trained scenarios. Therefore, data collection related configuration should be designed to allow different scenarios/configurations associated with one associated ID. For example, multiple sets of sub-data collection configurations (e.g., Set A and Set B) can be configured within one data collection configuration, so that one of the sub-data collection configurations can be dynamically activated.

Proposal 3: Data collection related configuration supports sub-data collection configurations, which can be dynamically activated, considering different scenarios/configurations associated with one associated ID.

## Applicability of inference for UE-side model

In RAN1#118-bis [10], the following agreement was made on applicability for inference for UE-sided model:

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

During the discussion, the representative advantage of Option 1 is that applicable CSI report config may be activated without Step 5. However, excessive configuration overhead for configuring candidate CSI report configs is expected as full CSI report configurations for all the candidate scenarios should be configured. To reduce the excessive configuration overhead, potential reduction of some IEs of CSI report configuration was also captured. However, if reduction of some IEs of CSI report configuration is applied, the applicable CSI report config cannot be activated without Step 5 and there’s no clear benefit of Option 1. In this case, it’s better to design a separate IE for inference parameters as in Option 2 or Option 3.

Observation 1: To identify applicability of UE-side model, Option 1 requires excessive configuration overhead to configure candidate CSI report configurations.

Observation 2: If reduction of some IEs is applied to Option 1 to reduce configuration overhead, Option 1 cannot activate CSI report config without Step 5. Additional RRC reconfiguration is required.

Although Option 2 provides similar functionality with Option 1, Option 2 does not require excessive overhead as multiple sets of inference parameters are configured instead of full CSI report configurations. Additional RRC configuration is essential in Step 5 for configuring actual inference configuration, but actual overhead should be still lower than Option 1 as only few CSI report configs are needed. In addition, applicability identification step can be reused for other functionalities by supporting different sets of inference parameters if similar applicability identification is supported in other functionalities.

Observation 3: Option 2 provides efficient applicability identification of UE side model without excessive configuration overhead of full CSI report configs.

Observation 4: RRC reconfiguration is Step 5 is essential for Option 2, but actual overhead should be still lower than Option 1 as only few CSI report configs are needed.

Observation 5: If similar applicability identification is supported in other functionalities, Option 2 can be reused by supporting a different sets of inference parameters which are designed for other functionalities.

While Option 2 and Option 3 share utilization of the inference parameters The major difference between Option 2 and Option 3 is source of inference parameters. That is, while gNB provides candidate inference parameters in Option 2, UE provides applicable inference parameters in Option 3. Compared to UE implementation, gNB implementation has limited flexibility, identifying applicability of UE side model based on the candidate inference parameters would be more a practical solution.

Observation 6: Identification applicability of UE side model based on gNB configured candidate inference parameters is desired considering limited flexibility of gNB implementation.

Proposal 4: Support Option 2 for applicability for inference for UE-side model.

## Beam prediction, indication and reporting

### Configuration of Set A and Set B

In RAN1#118 [8], the following agreement was made on configuration of Set A and Set B:

Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE perform measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

In RAN1#116 [5], the following conclusion was made on configuration of Set B:

Conclusion For UE sided model at least for inference, for measurement, the configuration of Set B, - take the current CSI framework as the starting point

In RAN1#116-bis [6], the following agreement was made on configurations of Set A and Set B:

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded.

Based on the above agreement, we provide the following analysis.

- Alt 1: one CSI-ResourceConfigId is configured for Set B

  - In Alt 1, key information for supporting construction of Set A is configuring additional conditions of gNB. For example, when NW-sided additional conditions such as number of beams, beam widths, beam angles and etc. are provided, the UE can construct Set A without having explicit configurations. However, feasibility of configuring proprietary NW-sided additional conditions is unclear considering the ongoing discussion, as the network vendors were severely negative on providing such information. In addition, Alt 1 is not able to support scenarios with transmission of Set A in longer periodicity.

- Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

  - There are many benefits of Alt 2. Firstly, as Alt 2 requires Set A and Set B configuration within one CSI-ResourceConfig, most of specifications in CSI framework for single TRP and/or multi-TRP can be reused. Therefore, AI/ML based beam management can be supported without huge specification impacts. Secondly, both scenarios (i.e., not transmitted Set A and transmitted Set A with larger periodicity) can be easily supported as resource Set(s) provide actual configuration.

- Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 3 is a similar approach with Alt 2. The key difference is that Alt 3 utilizes two CSI-ResourceConfigs while Alt 2 utilizes a single CSI-ResourceConfig. As CSI-ResourceConfig is designed to configure channel measurement resources, interference measurement resources, resource type and BWP ID, two CSI-ResourceConfig approach may be valid if different channel/interference measurements, resource type or BWP ID are needed. However, as Set A and Set B are designed for one gNB, motivation to support two CSI-ResourceConfigs is not clear. Also, CSI-RS resource indexes in the Set B is a subset of those in Set A, separate configurations will unnecessarily increase the signalling overhead.

- Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

  - Alt 4 is also a similar approach with Alt 2, but configuring Set A potentially in CSI-Report Config instead of CSI-ResourceConfig. We observed many disadvantages of Alt 4. For example, relatively larger specification impact is required as the design is totally against the principle of CSI framework by allowing CSI resource configuration in CSI-ReportConfig. In addition, the benefit is not clear. According to the discussion, repeated Set A configuration can be avoided in CSI-ResourceConfig by associating Set A with CSI report config. However, even for the identical Set A, different CSI-ReportConfig needs to be supported considering different Set B.

Observation 7: Feasibility of Alt 1: one CSI-ResourceConfigId is configured for Set B is doubted as the gNB vendors are severely negative on providing the NW-sided additional conditions for constructing Set A.

Observation 8: Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B is beneficial as Alt 2 supports both scenarios (i.e., not transmitted Set A and transmitted Set A with larger periodicity) with minimum specification impacts.

Observation 9: Motivation of Alt 3: two CSI-ResourceConfigIds are configured for Set A and Set B separately is not clear as different channel/interference measurements, resource type or BWP ID are not needed.

Observation 10: Benefit of Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId is not clear as different CSI-ReportConfig needs to be supported considering different Set B even for an identical Set A.

Proposal 5: Support Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B.

Proposal 6: Deprioritize configuring RS resource set for only Set B.

As stressed in RAN1#118, it is important to support both no transmission (Scenario 1) and transmission with longer periodicity (Scenario 2) for AI/ML beam management operation. For Scenario 1, RS overhead can be less than Scenario 2, however, additional handling of QCL operation for Set A resources, which are not transmitted, is needed. On the other hand, for Scenario 2, although RS overhead can be higher than Scenario 1, existing QCL operation can be reused. Therefore, both Scenario 1 and Scenario 2 should be supported for AI/ML beam management.

Proposal 7: Support Set A for both no transmission and transmission with longer periodicity.

For the configuration of Set A and Set B, the best option that we can support both Scenario 1 and Scenario 2 is to support RS resource design which allows both no transmission (Scenario 1) and transmission with longer periodicity (Scenario 2). While resources of Set A can be used only for indicating best CRIs in Scenario 1, resources of Set A should allow measurement of physical resources as well as indicating best CRIs in Scenario 2. Given the both scenarios, the RS resources configured in Set A should be designed to allow no transmission (Scenario 1) as well as physical transmission with different periodicity from Set B (Scenario 2). For Scenario 2, as the existing specification already supports independent configuration of CSI-ResourcePeriodicityAndOffset for each resource, it can be supported without additional specification support. On the other hand, for Scenario 1, the existing specification does not allow RS resources without transmission. The simplest way to achieve Scenario 1 is allowing Set A RS resources without configured parameters related to physical transmission. For example, parameters such as resourceMapping, CSI-ResourcePeriodicityAndOffset and etc. may not be configured for Set A RS resources.

Proposal 8: Support CSI resource configs with/without physical RS transmission parameters (e.g., resourceMapping, CSI-ResourcePeriodicityAndOffset and etc.).

### QCL measurements for unmeasured Set A beams.

For successful PDCCH/PDSCH reception and decoding, the UE needs to measure associated QCL parameters (e.g., Doppler spread, Doppler shift, delay spread, average delay and spatial Rx parameter) as shown in section 5.1.5 of TS 38.214 [4].

### 5.1.5 Antenna ports quasi co-location The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values: - 'typeA': {Doppler shift, Doppler spread, average delay, delay spread} - 'typeB': {Doppler shift, Doppler spread} - 'typeC': {Doppler shift, average delay} - 'typeD': {Spatial Rx parameter}

When PDCCH/PDSCH are to be transmitted via a predicted best beam which is not measured, the UE cannot acquire QCL-related parameters thereby failing to successfully decode the PDCCH/PDSCH. Therefore, a procedure to obtain QCL-related parameters for an unmeasured predicted beam is needed before PDCCH/PDSCH transmission. One possible way to acquire QCL-related parameters for an unmeasured predicted beam is utilizing neighbouring beams of the unmeasured predicted beam.

In Figure 1 and Figure 2, below, we show performance of doppler shift and delay spread estimation for an unmeasured beam based on neighbouring beams. In the Figures below, estimation of the QCL parameters for missing Tx beams is performed using measurement of QCL parameters based on adjacent beams on each side of the missing beam along the azimuth direction. The UE is configured to sweep through 64 Tx beams.

Figure 1. CDF of the absolute error in estimating doppler shift for unmeasured/missing beams based on adjacent beams on each side of the missing beams.

Figure 2. CDF of the absolute error in estimating RMS delay spread for unmeasured/missing beams based on adjacent beams on each side of the missing beams.

It can be seen from the figures above that estimation of QCL parameters for unmeasured/missing beams based on neighbouring/adjacent is possible within an acceptable margin of error. For example, Figure 1 show that using adjacent beams on each side of an unmeasured/missing beam, the doppler shift, which would be measured by the missing beam at the UE, can be estimated such that the estimation error falls below 10 Hz 90% of the time. Likewise, estimation of the RMS delay spread which would be measured by the missing beam for the UE can be managed to 15ns or less in absolute error 90% of the time.

Observation 11: When PDCCH/PDSCH are to be transmitted via a predicted best beam which is not measured, the UE cannot acquire QCL-related parameters, therefore a procedure to obtain QCL-related parameters for the unmeasured beam is needed.

Observation 12: Estimation of QCL-related parameters via neighbouring beams achieves Doppler shift estimation with error below 10 Hz for 90% of the time and RMS delay spread below 15 ns for 90% of the time.

Proposal 9: Support configuration of RS resources to estimate QCL-parameters for a RS resource without transmission in Set A.

### Pattern based beam indication

As best beams can be successfully predicted by AI/ML model in BM-Case 2, enhanced beam indication mechanism is needed to enable future beam indication based on the prediction. One possible way is updating the current beam mechanism from indication of one beam to indication multiple beams with corresponding time resources. An efficient way to enable multiple beam indication can be introducing a beam indication mechanism based on beam patterns. For example, a beam pattern and corresponding TCI states required for the indicated beam pattern can be used based on AI/ML based beam prediction.

Observation 13: Enhanced beam indication mechanism is needed to enable future beam indication based on prediction of AI/ML model in BM-Case 2.

Proposal 10: Support a beam indication mechanism with a beam pattern and corresponding TCI states required for the indicated beam pattern.

### Reporting beam measurements/predictions

In RAN1#118-bis [10], the following agreement was made on CPU mechanism for AIML-based CSI processing:

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

The benefit of a separate CPU counter for AI/ML-based CSI processing is not clear. Separately allocating CPU resources for each type of CSI-processing (i.e., legacy and AI/ML) results in an inefficient utilization of UE CSI-processing capability. For example, in a scenario when one type of CSI-processing is underutilized (i.e., idle CPUs) and the other type of CSI-processing is bottlenecked by the number of allocated CPUs. Therefore, it is beneficial to have shared CPUs between legacy and AI/ML-based processing.

Proposal 11: Support a shared CPU counter between legacy and AIML-based CSI reporting.

In TR 38.843 [3], the following potential specification impact was included for assessment.

For BM-Case1 and BM-Case2 with a NW-side AI/ML model: - L1 beam reporting enhancement for AI/ML model inference: - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

In RAN1#116 [5], the following agreement was made on beam measurement reporting:

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report

In RAN1#117 [7], the following agreements were made for inference report for NW-side model.

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Although it is beneficial to report measurement results of more than 4 beams in one UE reporting instance, however, having UE to report all AIML model input measurements for every AIML prediction instance to gNB greatly increases UE reporting overhead. Therefore, a procedure to reduce reporting overhead by UE is beneficial in some scenarios. For example, when channel condition or UE position does not change significantly, the UE can skip reporting of a subset of AIML input measurements to gNB.

Tables 1 and 2 show beam prediction accuracy considering different UE beam reporting methods. In this evaluation, we consider a sequence of spatial beam predictions for a UE moving at speed of 30 kmh. In addition, RSRP measurements are taken every ΔT (ΔT = 80 ms or 640 ms). RSRP measurements from set B, consisting of 16 Tx beams following the pattern shown below (i.e., Pattern #1), were used from set A consisting of 64 Tx beams.

|  | Beam is member of set A and set B |
| --- | --- |
|  | Beam is member of set A |

Azimuth Angles

Elevation Angles

Pattern #1

Figure 3. Beam patterns of Set B

We consider multiple cases for selecting reported RSRP measurements for set B:

- Case 1 (Baseline): Every ΔT ms, all beams of set B are measured and reported.

- Case 2: Every ΔT ms, 50% of the beams of set B are measured and reported alternatively (e.g., odd beams are measured and reported at time instant T and even beams are measured and reported at time instant T+1). For spatial beam prediction at time instant T, odd beam measurements are combined with even beam measurements from time instant T-1 as input to AIML model. Similarly, at time instant T+1, even beam measurements are combined with odd beam measurements from time instant T-1 as input to AIML model.

- Case 3: Every ΔT ms, 50% of the beams of set B, that have highest RSRP at time instant T-1, are measured and reported at time instant T. For spatial beam prediction at time instant T, best beam measurements are combined with worst beam measurements from time instant T-1 as input to AIML model.

- Case 4a/4b: Every ΔT ms, a subset of beams from set B that are within X dB (12 dB for case 4a, 18 dB for case 4b) from the highest RSRP at time instant T-1, are measured and reported at time instant T. For spatial beam prediction at time instant T, the subset of high RSRP beam measurements are combined with weakest beam measurements from time instant T-1 as input to AIML model.

Table 1: Sequential Spatial Beam Prediction Accuracy of AIML-based Classification (Reduced Overhead Reporting) – ΔT = 80 ms

| Case | Measured Beams at Time Instant T | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy |
| --- | --- | --- | --- | --- |
| Case 1 (Baseline) | All Beams | 74.58% | 92.70% | 96.52% |
| Case 2 | Odd/Even beams alternatively | 72.90% | 92.26% | 96.19% |
| Case 3 | Best 50% of beams at time instant T-1 | 74.05% | 92.53% | 96.50% |
| Case 4a | Subset of beams within 12 dB from best beam (Average number of measured beams = 6.5323) | 73.65% | 92.58% | 96.46% |
| Case 4b | Subset of beams within 18 dB from best beam (Average number of measured beams = 11.4063) | 74.47% | 92.83% | 96.50% |

Table 2: Sequential Spatial Beam Prediction Accuracy of AIML-based Classification (Reduced Overhead Reporting) – ΔT = 640 ms

| Case | Measured Beams at Time Instant T | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy |
| --- | --- | --- | --- | --- |
| Case 1 (Baseline) | All Beams | 73.25% | 92.58% | 96.15% |
| Case 2 | Odd/Even beams alternatively | 62.96% | 87.50% | 93.56% |
| Case 3 | Best 50% of beams at time instant T-1 | 67.56% | 90.14% | 94.83% |
| Case 4a | Subset of beams within 12 dB from best beam (Average number of measured beams = 6.5321) | 65.86% | 89.37% | 94.78% |
| Case 4b | Subset of beams within 18 dB from best beam (Average number of measured beams = 11.4097) | 68.93% | 91.29% | 95.69% |

For ΔT = 80 ms, as shown in Table 1, Case 2, Case 3, Case 4a and Case 4b showed marginal difference in beam prediction accuracy with 50% reduction of UE reporting overhead. Also, for ΔT = 640 ms, as shown in Table 2, minor difference in Top-3/1 and Top-5/1 beam prediction accuracy is shown with 50% reduction of UE reporting overhead. Therefore, UE beam reporting overhead can be reduced without sacrificing beam prediction accuracy. Moreover, beam prediction accuracy with variable overhead reduction (i.e., reporting beams within X dB of the best beam) is also as good as baseline. It provides finer control on the number of beams to be reported by configuring a value for X. It should be noted that, CSI reporting with variable payload sizes is already supported by spec in NR through multi-part CSI-Report.

Observation 14: The UE’s frequent reporting of AIML input measurements for inference at gNB greatly increases UE reporting overhead.

Observation 15: Beam prediction accuracy with both fixed overhead reduction (e.g., reporting odd/even beams alternatively or best 50% of beams) and variable overhead reduction (i.e., reporting beams within X dB of the best beam) is as good as baseline case (i.e., reporting all beams in Set B).

Proposal 12: Support an overhead reduction mechanism for reporting data collection for inference (e.g., based on channel conditions).

Proposal 13: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

A useful method to report beam measurements is through pattern-based reporting. In case when the number of Set B beam is large, reporting information of all the set B beams may not be efficient. In this case, supporting a pattern-based reporting enables the UE to report large number of beam measurements without indicating additional CRI overheads, thus reducing UE’s reporting overhead. Moreover, the pattern-based reporting also enables overhead reduction in reporting by the UE by associating a subset of Set B beams with pattern ID(s). When overhead reduction for reporting is enabled, the UE reports beam measurements with corresponding indication of pattern ID(s) for a subset of all pattern ID(s) associated to Set B.

Proposal 14: Support a pattern-based reporting mechanism for data collection for inference wherein the UE reports a pattern ID(s) and corresponding beam measurements of a subset of beams in Set B.

In RAN1#116-bis [6], the following agreements were made on beam measurements and inference reporting:

Agreement For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded.

In RAN1#116-bis [6], the following agreements was on CSI reporting in agenda item 9.1.3.1:

Agreement For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

In RAN1#118 [8], the following agreements were made on beam measurements and inference reporting:

Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted.

In RAN1#118-bis [10], the following agreement was made on determination of reference time for BM-Case 2:

Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

For a UE-side AI/ML model, required specification impact can be minimized as UE determines best beams based by using the UE-side AI/ML model. For example, beam(s) that is based on the output of AI/ML model inference was agreed as a potential specification support. For BM-Case 1, the current beam reporting with CRIs/SSBRIs and corresponding L1-RSRP values is enough to indicate the best beam(s) from AI/ML model inference. For BM-Case 2 with a gNB-side model, time domain characteristics of beam measurements are essential for time domain prediction. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Information about measured past instances (e.g., the time stamp) for reported beam related information (e.g., CRIs/SSBRIs/RSRPs) can be further considered.

Additionally, for BM-Case2 with a UE-sided model, providing multiple sets of predicted beam information for future time instances (e.g., future beam predictions by the UE) should be beneficial. Reporting information of multiple future time instances reduces latency as network will obtain information of multiple future time instances in advance and does not have to wait for UE reports in queue. For the design for the reporting information of multiple future time instances, the aforementioned agreements in agenda items 9.1.1 and in 9.1.3.1 (CSI Prediction) can be a starting point. For example, the choice of N for reporting BM-Case 2 inference results can be either same with or multiple of CSI reporting periodicity in CSI prediction.

For the determination of reference time for BM-Case 2 report, the benefit of Option 1 is not clear since the purpose of reference time is to determine the time of applicability of the UE report. From the perspective of a temporally predicting AI/ML model, the time at which Set B measurements are performed is the reference time or time 0. The time of applicability of a prediction made by the AI/ML model is calculated/outputted by the AI/ML model relative to the reference time (i.e., time 0) mentioned earlier. Moreover, Option 2 encompasses Option 3 since CSI reference resource (as stated in Option 2) also allows for the configuration of reference resource according to Option 3.

The temporal predictions made by an AI/ML model become less reliable, the farther in time they are especially in a rapidly changing environment (e.g., Urban moving UE). Similarly, the predictions closer to the time of obtaining AIML inputs (i.e., Set B beam measurements) are expected to be more accurate. For example, for a static or slow speed moving UE, the report containing information (e.g., beam ID and predicted RSRP) of a larger number of future time instances is beneficial as the temporal predictions are likely to be valid for a longer duration. Conversely, when a UE is moving at high speed, it is more beneficial to report for a smaller number of future time instances since predictions farther in time are less likely to be valid. Therefore, it is beneficial to have a control on the number of future time instances and the number of reported beams for each future time instance.

For report content of inference results for UE-sided model for BM-Case 1, both ‘Option A: Predicted RSRP’ and ‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement’ are useful. In a high noise scenario, a predicted RSRP, usually obtained through regression based AIML model can be closer to the ground truth than a real measurement (e.g., RSRP), which is subject to high measurement errors due to high noise. This is because regression-based prediction also acts as a denoiser for the input. On the other hand, when there’s low measurement noise, the real measurement is usually closer to ground truth than a predicted measurement. Therefore, it is beneficial to have a UE based selection procedure to report according to Option A or Option B.

Observation 16: For BM-Case 1 with a UE-side AI/ML model, the current beam reporting with CRIs/SSBRIs and corresponding L1-RSRP values is enough to indicate the best beam(s) from AI/ML model inference.

Observation 17: For BM-Case 2 with a gNB-side AI/ML model, information about time stamp on measurement instances for reported beam related information needs to be reported for time domain prediction at gNB.

Observation 18: For BM-Case 2 with a UE side AI/ML model, reporting information of multiple future time instances in one report reduces latency as gNB does not have to wait for UE reports in queue.

Proposal 15: Information about the time stamp for measurement instances should be supported.

Proposal 16: Reporting prediction results of multiple future time instances in one report should be supported. Beam inference reporting periodicity should be aligned with CSI-Reporting periodicity as a baseline.

Proposal 17: Support “Option 2: Based on the CSI reference resource corresponding to the report” for determination of reference time.

Proposal 18: Support flexible configuration of multiple future time instance reporting (e.g., based on channel conditions, gNB configuration).

Proposal 19: Support both

‘Option A: Predicted RSRP’

‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.’

Additionally, support a UE based selection procedure to report according to Option A or Option B.

In RAN1#116, whether to support beam reporting for only inference or both training and inference was discussed especially for network sided model. As how to utilize reported information, there’s no need to limit the use case of beam reporting. However, enhancement of UE reporting can be considered to allow both training and inference via UE beam reporting. For example, in BM-Case 1, current beam reporting with up to 4 CRIs/SSBRIs and corresponding L1-RSRP values is not enough for AI/ML model training, so that beam reporting up to 64 RSRPs can be considered given the maximum number of beams supported in the current specification. Compared to RSRPs, additional reporting of CRIs/SSBRIs may not be needed as the UE need to report all measured RSRPs from configured RSs (but needs an association between reported RSRPs and measured RSs). For BM-Case 2, time domain characteristics of beam measurements are essential for time domain prediction. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Information about measured past instances (e.g., time stamp) for reported beam related information should be considered.

Observation 19: No need to restrict purpose of UE beam reporting, especially for network sided model, as how to utilize reported information is up to gNB implementation.

Observation 20: For network sided model, enhancement of UE reporting can be considered to allow both training and inference via UE beam reporting.

Proposal 20: For network sided model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances.

- No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used.

- Information on measured past instances (e.g., time stamp) is supported.

### LCM and Consistency and Performance Monitoring

#### Considerations on beam patterns

In TR 38.843 [3], the following conclusion was drawn for BM-specific conditions for AIML models.

Conclusion For BM-Case1 and BM-Case2 with a UE-side AI/ML model, the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) are considered at least from the following aspects: - information regarding model inference - Set A / Set B configuration - performance monitoring - data collection - assistance information

For a UE-sided model, to ensure consistency between training and inference, configuration information associated with the Set B should be indicated to UE so that the UE selects an appropriate AIML model for inference, i.e., an AIML model whose training data/inputs are a good match for the indicated Set B. The configuration signalling associated to a Set B should include information on beam IDs, beam periodicity, beam widths etc.

For gNB-sided model, we consider the spatial beam prediction for multiple cases of reported RSRP measurements selection (i.e., set B selection) when the reported RSRP measurements are subset of the total RSRP measurements (i.e., set B is a subset of set C). We consider a set C of RSRP measurements consisting of 16 Tx beams, following the pattern shown below (i.e., Pattern #1), which a subset of set A consisting of 64 Tx beams.

|  | Beam is member of set A and set C |
| --- | --- |
|  | Beam is member of set A |

Azimuth Angles

Elevation Angles

Pattern #1

|  | Beam is member of set A and set C |
| --- | --- |
|  | Beam is member of set A |

Pattern #2

Figure 4. Beam patterns of Set B for Uniformly selected 16 and 8 Tx beams

We consider multiple cases for selecting reported RSRP measurements for set B:

- Case 1 (Baseline): Size of set B = Size of set C = 8 following Pattern #2.

- Case 2-1: Size of set B = 8, where the members of set B are selected as the best 8 beams from set C.

- Case 2-2: Size of set B = 8, where the members of set B are selected randomly from set C.

- Case 3-1: Size of set B is variable, where the members of set B are selected as the beams of set C that are within 12.621 dB from best beam (so 8 beams on average are selected).

- Case 3-2: Size of set B is variable, where the members of set B are selected as the beams of set C that are within 12 dB from best beam. Also, a minimum of 4 beams must be selected.

- Case 4 (Upper Bound): Size of set B = Size of set C = 16 following Pattern #1.

Table 3: Spatial Prediction Accuracy of AIML-based Classification (Set B Selection)

| Case | Size of Set C | Size of Set B | Set B Selection | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 (Baseline) | 8 | 8 | All in set C | 55.36% | 86.04% | 93.33% |
| Case 2-1 | 16 | 8 | Best Beams in Set C | 70.19% | 94.14% | 97.50% |
| Case 2-2 | 16 | 8 | Random Beams from Set C | 51.60% | 80.21% | 89.07% |
| Case 3-1 | 16 | variable (Average Size of Set B = 7.6852 Beams) | Beams within 12.621 dB from best beam | 71.81% | 94.83% | 98.41% |
| Case 3-2 | 16 | max(variable,4) (Average Size of Set B = 7.9974 Beams) | Beams within 12 dB from best beam | 70.86% | 94.81% | 98.24% |
| Case 4 (Upper Bound) | 16 | 16 | All in set C | 76.67% | 96.29% | 98.86% |

We note that selecting set B as a subset of best measured beams (Case 2-1) outperforms measuring and reporting a uniform set of beams (Case 1). Moreover, the achieved performance with variable beam set selection (Case 3-1, Case 3-2) is slightly less than the upper bound (Case 4).

Observation 21: For a UE-sided model, configuration information associated to a Set B can help the UE in ensuring consistency between training and inference.

Observation 22: Set B selection as a subset of the best measured beams for input to the gNB-sided AIML model achieves a higher beam prediction accuracy than reporting a uniformly or randomly selected subset of the measured beams.

Proposal 21: Indicating configuration information associated with Set B to UE should be supported.

Proposal 22: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams).

#### NW-side additional conditions

In RAN1#116-bis [6], the following agreement was made on NW-side conditions for consistency across training and inference.

Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

Down selecting only Opt.1 limits UE to consider only NW-side additional conditions (e.g., Set B size, beam width, beam angles etc.) for consistency. Since channel conditions (e.g., path loss, UE speed) and other KPIs are crucial factors across training and inference as well as the NW-side additional conditions, flexibility to consider channel conditions and other KPIs should be provided. Otherwise, the difference in channel conditions between training and inference results in loss of generalization and corresponding performance loss despite of consistent configurations/functionality. On the other hand, solely relying on performance monitoring-based solution (Opt.2) creates unnecessary overhead for the UE since in that scenario, configuration related inconsistencies need to be also checked through performance monitoring KPIs which is inefficient. Therefore, supporting a combination of both Opt.1 and Opt.2 is beneficial.

Proposal 23: Support both Opt.1 (based on an associated ID) and Opt.2 (performance monitoring based) for the consistency of NW-side additional conditions.

#### Lifecycle management for BM

The following aspects should be considered for AI/ML model LCM in BM:

- Procedure for identifying need of AI/ML model recovery.

  - Procedures for identifying need of AI/ML model recovery should be considered. For example, UE can monitor channel quality by measuring monitoring RSs and identify the need of AI/ML model recovery.

  - Configuration of evaluation methodology from gNB should be considered as different evaluation methodology can be beneficial for each scenario. For example, the gNB selection on how UE can identify current AI/ML model status can be considered.

- Procedure for UE request or gNB trigger on AI/ML model recovery.

  - When the need of AI/ML model recovery is identified, procedures to trigger the recovery procedure should be considered. For example, UE request on the recovery procedure or reporting the identified channel quality can be used.

- Procedure for AI/ML model recovery

  - In addition, procedures for recovery of AI/ML models should be considered. For example, triggering a procedure for identifying an adequate AI/ML model selection can be used.

Proposal 24: For AIML LCM in BM, consider procedures for identification of need of AI/ML model recovery, UE request/gNB trigger and AI/ML model recovery.

#### Considerations on Performance monitoring

In RAN1#117 [7], the following agreement was made on Type 1(NW-side) performance monitoring.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Option 1 (NW-side performance monitoring) causes additional reporting overhead for UE. To enable Option 1, the UE report is required to report additional measurement results of the resources configured for performance reporting. The benefit of Option 2 (UE-assisted performance monitoring) is two-fold. First, it reduces the performance monitoring report overhead for the UE, as the UE can just report UE calculated performance metrics based on the measurements of performance monitoring resources and predicted beam qualities/indexes. Secondly, the NW need not to compute performance metrics at gNB, thus saving gNB computations.

Proposal 25: Support Option 2 (UE-assisted performance monitoring).

In RAN1#118 [8], the following agreement was made on Option 2, UE-assisted performance monitoring:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN1#118-bis [10], the following agreements were made on Option 2, UE-assisted performance monitoring:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

For Type 1-Option 2 (UE-assisted performance monitoring), Option 2 (i.e., dedicated resource set(s) for monitoring and report configuration) adds more complexity and problems than it solves. Firstly, it introduces dependency between two reports (i.e., inference and monitoring reports). In a scenario when one of the two reports is not received by the gNB, it results in either an inference report received without corresponding performance metrics (i.e., when monitoring report is not receive by the gNB) or an irrelevant monitoring report (i.e., when inference report is not received by the gNB). Moreover, to calculate accurate performance metrics, the calculation should be based on the measurements and predictions of the same beam. This can be achieved by transmission of predicted Set A beams without the need of a dedicated monitoring resource set. Additionally, the UE also has measurements from other beams (i.e., Set B beams other than predicted beams) to compare against the measurement of a predicted beam. Therefore, in both cases, i.e., whether to calculate performance metric using measurements of predicted beams or beams other than predicted beams, a dedicated monitoring resource set is unnecessary.

One way to activate performance monitoring at a UE is to couple it with the transmission configuration of the monitoring resource(s) i.e., the UE activates performance monitoring when it receives RSs for performance monitoring. For example, based on the reception of RSs associated to Set A beams (i.e., when configured with physical transmission parameters). If the performance of an AI/ML model falls lower than an acceptable level, then there is no benefit in continually monitoring the performance of the model until model recovery procedure is completed. In this context, a timer based performance monitoring procedure is useful wherein the UE continually monitors the performance of AI/ML model at periodic time instances until the timer expires. The timer is reset based on gNB signalling or when determined model performance is within acceptable range.

One good candidate for UE reported assistance information in the context of performance monitoring is an indication on AIML model validity. For example, instead of reporting the calculated metrics, the UE sends an indication of AIML model validity based on a performance metric computed by the UE.

Proposal 26: For UE-assisted performance monitoring, Support:

Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

- FFS: the resource set(s) for monitoring

- UE measures the resource set(s) for monitoring.

- FFS how/when to report the monitoring results

Proposal 27: For Option 2 (UE-assisted performance monitoring), support the following options for activation of performance monitoring at UE:

- Based on the reception of RSs associated to Set A beams.

- Based on a periodic timer which starts/stops performance monitoring procedure based on gNB signaling or model performance.

Proposal 28: Support a UE based indication of AIML model validity.

### Location of AIML Inference

AI/ML inference/training at UE side can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead. 
On the other hand, AIML inference at gNB-side can lead to significant reporting overhead since inference generally requires detailed explicit information. Although gNB has much greater computational capacity than UE, gNB computation capacity can be insufficient in some scenarios. For example, for a pair of gNB and UE, the reporting overhead and computation requirement for inference is not a critical issue for the network, however, if we consider all available UEs in a cell, acquiring information for the inference from multiple UEs requires huge overheads. Moreover, gNB may not have enough computational capacity for performing inference operations for multiple UEs especially in UE dense environments. In such scenario, it is beneficial to dynamically enable UE-sided AIML inference for a subset of UEs e.g., based on UE capability or UE’s prediction performance. The network can switch back and forth between NW-side and UE-side inference based on its computational workload, capacity and/or UE’s prediction performance.

Observation 23: AI/ML inference at UE side can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Observation 24: AI/ML inference at NW side could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead. Moreover, although gNB has greater computational capacity than UE, the capacity can still prove to be insufficient in dense UE scenarios.

Proposal 29: Support a procedure to dynamically switch AIML inference location (e.g., based on NW workload and/or UE’s prediction performance).

# Summary

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management. From the discussions, we made the following observations and proposals:

Observation 1: To identify applicability of UE-side model, Option 1 requires excessive configuration overhead to configure candidate CSI report configurations.

Observation 2: If reduction of some IEs is applied to Option 1 to reduce configuration overhead, Option 1 cannot activate CSI report config without Step 5. Additional RRC reconfiguration is required.

Observation 3: Option 2 provides efficient applicability identification of UE side model without excessive configuration overhead of full CSI report configs.

Observation 4: RRC reconfiguration is Step 5 is essential for Option 2, but actual overhead should be still lower than Option 1 as only few CSI report configs are needed.

Observation 5: If similar applicability identification is supported in other functionalities, Option 2 can be reused by supporting a different sets of inference parameters which are designed for other functionalities.

Observation 6: Identification applicability of UE side model based on gNB configured candidate inference parameters is desired considering limited flexibility of gNB implementation.

Observation 7: Feasibility of Alt 1: one CSI-ResourceConfigId is configured for Set B is doubted as the gNB vendors are severely negative on providing the NW-sided additional conditions for constructing Set A.

Observation 8: Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B is beneficial as Alt 2 supports both scenarios (i.e., not transmitted Set A and transmitted Set A with larger periodicity) with minimum specification impacts.

Observation 9: Motivation of Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately is not clear as different channel/interference measurements, resource type or BWP ID are not needed.

Observation 10: Benefit of Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId is not clear as different CSI-ReportConfig needs to be supported considering different Set B even for an identical Set A.

Observation 11: When PDCCH/PDSCH are to be transmitted via a predicted best beam which is not measured, the UE cannot acquire QCL-related parameters, therefore a procedure to obtain QCL-related parameters for the unmeasured beam is needed.

Observation 12: Estimation of QCL-related parameters via neighbouring beams achieves Doppler shift estimation with error below 10 Hz for 90% of the time and RMS delay spread below 15 ns for 90% of the time.

Observation 13: Enhanced beam indication mechanism is needed to enable future beam indication based on prediction of AI/ML model in BM-Case 2.

Observation 14: The UE’s frequent reporting of AIML input measurements for inference at gNB greatly increases UE reporting overhead.

Observation 15: Beam prediction accuracy with both fixed overhead reduction (e.g., reporting odd/even beams alternatively or best 50% of beams) and variable overhead reduction (i.e., reporting beams within X dB of the best beam) is as good as baseline case (i.e., reporting all beams in Set B).

Observation 16: For BM-Case 1 with a UE-side AI/ML model, the current beam reporting with CRIs/SSBRIs and corresponding L1-RSRP values is enough to indicate the best beam(s) from AI/ML model inference.

Observation 17: For BM-Case 2 with a gNB-side AI/ML model, information about time stamp on measurement instances for reported beam related information needs to be reported for time domain prediction at gNB.

Observation 18: For BM-Case 2 with a UE side AI/ML model, reporting information of multiple future time instances in one report reduces latency as gNB does not have to wait for UE reports in queue.

Observation 19: No need to restrict purpose of UE beam reporting, especially for network sided model, as how to utilize reported information is up to gNB implementation.

Observation 20: For network sided model, enhancement of UE reporting can be considered to allow both training and inference via UE beam reporting.

Observation 21: For a UE-sided model, configuration information associated to a Set B can help the UE in ensuring consistency between training and inference.

Observation 22: Set B selection as a subset of the best measured beams for input to the gNB-sided AIML model achieves a higher beam prediction accuracy than reporting a uniformly or randomly selected subset of the measured beams.

Observation 23: AI/ML inference at UE side can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Observation 24: AI/ML inference at NW side could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead. Moreover, although gNB has greater computational capacity than UE, the capacity can still prove to be insufficient in dense UE scenarios.

Proposal 1: The following responses to the questions in RAN2 LS are adopted.

On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? - In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI. On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? - RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list. - Q3: Is NW-side additional condition functionality specific? - RAN 1 hasn’t made any conclusion on whether NW-side additional condition functionality specific or not yet. - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - Inference related parameters can be a basis to determine the applicability. For example, NW can provide candidate inference parameters (e.g., as a separate I.E. in Option 2 or a CSI report config in Option 1), so that UE can identify its applicable inference parameters among the candidate inference parameters. In another example, UE can indicate applicable inference parameters in Option 3 and so that NW can activate a CSI report configuration based on the reported applicable inference parameters. - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - It depends on the selected option. For example, inference parameters can be included in Step 3 for Option 1 and Option 2 while Option 3 does not need additional inference parameters as UE provides applicable inference configuration in Step 4. - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - Inference parameters represent applied parameters during training of AI/ML model based on NW-side additional condition. - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - According to RAN1 agreement, the inference parameters can be at least including Set A related information, Set B related information, Report content related information and Time instances related information for measurements and prediction for BM-Case 2. - Q5: What is the content of applicable functionality reporting in Step 4? - The content of applicable functionality reporting also depends on the selected option. For Option 1, UE indicates applicable CSI report config IDs while applicable inference parameter IDs are indicated in Option 2. In Option 3, UE needs to indicate applicable sets of inference configurations. - Q6: What is the content of inference configuration in Step 5? - The content of inference configuration also depends on the selected option. For Option 2 and Option 3, a CSI report configuration for inference should be configured based on the UE applicability reporting in Step 4 while Option 1 may not need Step 5. On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? - It does not activate the functionality immediately. - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? - For periodic CSI reporting, the configured CSI report configuration in Step 5 can be activated immediately upon receiving Step 5. For semi-persistent/aperiodic CSI reporting, additional activation/trigger is required. - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? - Multiple/all applicable functionalities can be activated within UE capability. - Q10: Is L1/L2 signaling for functionality activation/deactivation needed? - L1/L2 signaling for functionality activation/deactivation can be additionally considered for better flexibility.

Proposal 2: Clarify the definition and the detailed signaling of data collection related configuration before confirming the working assumption on the support of associated IDs in CSI framework.

Proposal 3: Data collection related configuration supports sub-data collection configurations, which can be dynamically activated, considering different scenarios/configurations associated with one associated ID.

Proposal 4: Support Option 2 for applicability for inference for UE-side model.

Proposal 5: Support Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B.

Proposal 6: Deprioritize configuring RS resource set for only Set B.

Proposal 7: Support Set A for both no transmission and transmission with longer periodicity.

Proposal 8: Support CSI resource configs with/without physical RS transmission parameters (e.g., resourceMapping, CSI-ResourcePeriodicityAndOffset and etc.).

Proposal 9: Support configuration of RS resources to estimate QCL-parameters for a RS resource without transmission in Set A.

Proposal 10: Support a beam indication mechanism with a beam pattern and corresponding TCI states required for the indicated beam pattern.

Proposal 11: Support a shared CPU counter between legacy and AIML-based CSI reporting.

Proposal 12: Support an overhead reduction mechanism for reporting data collection for inference (e.g., based on channel conditions).

Proposal 13: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

Proposal 14: Support a pattern-based reporting mechanism for data collection for inference wherein the UE reports a pattern ID(s) and corresponding beam measurements of a subset of beams in Set B.

Proposal 15: Information about the time stamp for measurement instances should be supported.

Proposal 16: Reporting prediction results of multiple future time instances in one report should be supported. Beam inference reporting periodicity should be aligned with CSI-Reporting periodicity as a baseline.

Proposal 17: Support “Option 2: Based on the CSI reference resource corresponding to the report” for determination of reference time.

Proposal 18: Support flexible configuration of multiple future time instance reporting (e.g., based on channel conditions, gNB configuration).

Proposal 19: Support both

‘Option A: Predicted RSRP’

‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.’

Additionally, support a UE based selection procedure to report according to Option A or Option B.

Proposal 20: For network sided model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances.

- No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used.

- Information on measured past instances (e.g., time stamp) is supported.

Proposal 21: Indicating configuration information associated with Set B to UE should be supported.

Proposal 22: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams).

Proposal 23: Support both Opt.1 (based on an associated ID) and Opt.2 (performance monitoring based) for the consistency of NW-side additional conditions.

Proposal 24: For AIML LCM in BM, support procedures for identification of need of AI/ML model recovery, UE request/gNB trigger and AI/ML model recovery.

Proposal 25: Support Option 2 (UE-assisted performance monitoring).

Proposal 26: For UE-assisted performance monitoring, Support Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

- FFS: the resource set(s) for monitoring

- UE measures the resource set(s) for monitoring.

- FFS how/when to report the monitoring results

Proposal 27: For Option 2 (UE-assisted performance monitoring), support the following options for activation of performance monitoring at UE:

- Based on the reception of RSs associated to Set A beams.

- Based on a periodic timer which starts/stops performance monitoring procedure based on gNB signaling or model performance.

Proposal 28: Support a UE based indication of AIML model validity.

Proposal 29: Support a procedure to dynamically switch AIML inference location (e.g., based on NW workload and/or UE’s prediction performance).

# References

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description,

- RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Study Item Description,

- 3GPP TR38.843 V18.0.0 (2023-12), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”,

- 3GPP TS38.214 V18.1.0 (2023-12), “Physical layer procedures for data (Release 18)”,

- “Final Report of 3GPP TSG RAN WG1 #116”, 3GPP RAN1 Meeting #116, February 26th – March 1st, 2024,

- “Final Report of 3GPP TSG RAN WG1 #116-bis”, 3GPP RAN1 Meeting #116-bis, April 15th – April 19th, 2024,

- “Final Report of 3GPP TSG RAN WG1 #117-bis”, 3GPP RAN1 Meeting #117-bis, May 20th – May 24th, 2024,

- “Final Report of 3GPP TSG RAN WG1 #118”, 3GPP RAN1 Meeting #118, August 19th – August 23rd, 2024,

- R1-2407604, “LS on applicable functionality reporting for beam management UE-sided model”,

- “Final Report of 3GPP TSG RAN WG1 #118-bis”, 3GPP RAN1 Meeting #118-bis, October 14th – October 18th, 2024.