3GPP TSG RAN WG1 #119		       	R1-2409461

Orlando, USA, November 18th – 22nd 2024

Agenda Item:		9.2.3

Source:				InterDigital, Inc.

Title:						Further Details on Rel-19 CB-based UL for 3TX UE

Document for:		Discussion and Decision

# Introduction

For Rel-19, a WID was proposed to further enhance the MIMO feature [1], where the fourth objective targets enhancements to support 3TX UL transmission. The initial WID scope of 3TX was completed in RAN1#117.

After the RAN#105 plenary meeting, the WID was revised [2] and expanded to include additional enhancements to support 3TX with ‘antennaSwitching’ and ‘nonCodebook’ usage:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Last meeting [3], non-codebook-based transmissions and 3T6R antenna switching were addressed. In addition, RAN4 sent an LS [4] regarding 3TX RF issues. In this contribution, we discuss and share our views on the LS from RAN4.

# Background and Motivation

In NR, there exists an imbalance between the performance of DL and UL transmissions. In the last couple of releases (Rel-17 and Rel-18), 3GPP worked on introducing additional enhancements in the UL to close the performance gap with the DL. To enable higher throughputs, the number of supported UL antennas has been increased to support additional layers for increased spatial multiplexing. Rel-18 introduced specification support for 8TX which included new TPMI codebooks with 8 antenna ports for different UE coherence and Full Power Transmission (FPTX) capabilities, and support for more than 4 layers with 2 CWs. However, the Rel-18 objective was mainly targeting more advanced devices with larger form factor such as CPE/FWA/vehicle/industrial devices which can accommodate the additional hardware for 8TX (e.g., antennas, power amplifiers). For UEs with smaller form factor, 2TX and 4TX implementations represent more practical antenna configurations to support up to 2 and 4 layer transmission, respectively.

In codebook-based UL transmission, the UE reports its capability which includes the number of UL TX antennas, the coherency between antennas (e.g., coherent, partially coherent, or fully coherent), the FPTX assumption, and the maximum number of layers supported. The UE is configured with an SRS-ResourceSet with usage parameter set to ‘codebook’. Optionally, a second SRS-ResourceSet is configured if multi-TRP or multi-panel cases are considered. The SRS resource set is configured with up to NSRS=2 SRS resources, and all SRS resources in the set are configured with the same number of SRS ports (if FPTX is not configured). The number of ports configured are equal to the number of supported UL TX antennas, and the UE maps the transmission one-to-one from its UL TX antennas to an SRS port. The SRI indication is used in a scheduling grant to indicate the UL beam where one beam is associated per configured SRS resource in the SRS resource set. Table 1 from 38.212 [5] shows that, in the codebook-based case, the SRI indication is a single bit which is used to switch between two different SRS resources in the SRS resource set.

| Bit field mapped to index | SRI(s), |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

Table 1 - SRI indication in codebook-based PUSCH transmission without FPTX

In addition to the SRI, the scheduling grant for codebook-based also indicates the precoder that UE shall use to map the layers to the UL TX antennas. The precoders are defined in separate codebooks for each combination of number of transmit antennas and number of layers. A precoding indication bit field in the grant maps to a set of TPMIs associated to the number of antenna ports in the indicated SRI. The TPMIs represent indices to precoders that are specified in codebooks where one codebook is given for each combination of TX antennas and number of layers. The TPMIs correspond to a subset of precoders from the codebooks, where the subset is defined based on the number of layers, antenna coherence (‘fullyAndPartialAndNonCoherent’, ‘partialAndNonCoherent’, ‘nonCoherent’), and FPTX capabilities (‘ul-FullPowerTransmission’). Table 2 illustrates the precoding information for 4TX without FPTX (Table 7.3.1.1.2-2 in 38.212 [5]). The grant also includes a field for the antenna port indication which indicates the mapping of DMRS resources to PUSCH layers. The UE then sends the PUSCH precoded according to the determined TPMI and DMRS.

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |

Table 2 - Precoder indication mapping to TPMIs for nonCoherent 4TX

Based on the feedback from UE vendors, current devices are mostly limited to 2TX because of the device size restrictions. Even though Rel-18 supports 4TX and 8TX UL, these antenna configurations do not represent realistic use cases for mobile devices; thus, the throughput gain from 4TX and 8TX is rarely realized in practice. For this WID, it has been agreed that 3TX represents a viable value proposition considering current technology and consumer demands.

In terms of antenna coherency, for 3TX, there are different cases that could be implemented. For example, with a co-polarized configuration, all 3TX antennas have the same polarization. With dual-polarized antenna setup, the UE could be considered partially or fully coherent, where two of the antennas have the same polarization and a third antenna is on the other polarization. Figure 1 illustrates the two possible configurations, where each polarization is assigned a different color (e.g., v-pol and h-pol are red and blue, respectively). To simplify the specification effort, Rel-19 WID is restricted to the nonCoherent case where the UE can transmit on only one antenna at a time.

Figure 1 - Co- and cross-polarized antennas

In Rel-19, the objective is to specify 3TX for the restricted case of codebook-based transmission without FPTX, only for nonCoherent case, and without introducing any enhancements to the SRS resources.

# RAN4 LS

RAN4 has sent an LS to RAN1 [4] on RF issues with 3TX. From the three questions in the LS, question 2 and 3 are related to 3TX UE, where we discuss them in the following.

RAN4 noticed that UL full power transmission mode 0 and legacy Rel-15 power scaling is supported in the WID for 3Tx. RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks as below for example but not with 1Layer 3-port codebooks. 13100100, 13100001, 13 001001. Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

The scope of Rel-19 enhancements for 3TX clarifies that only non-coherent precoders are supported, and UL full power transmission mode 1 and 2 are not supported. Therefore, full power transmission mode 0 can be assumed to be supported by configuring ul-FullPowerTransmission in PUSCH-Config with fullpower. In fullpower transmission mode, the UE behavior is to scale by s=1 the linear value of the PUSCH transmit. The UE then splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power as stated in [6]:

## 7.1 Physical uplink shared channel For a PUSCH transmission on active UL BWP b, as described in clause 12, of carrier f of serving cell c, a UE first calculates a linear value PPUSCH,b,f,c(i,j,qd,l) of the transmit power PPUSCH,b,f,c(i,j,qd,l), with parameters as defined in clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook', - if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where: - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource - if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2, - s=1 for full power TPMIs reported by the UE [18, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', - s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and - if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1 - else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource. The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.

Based on the context of the questions, it would better to discuss Question 3 first, and then go back to Question 2. For Question 3, RAN4 is asking whether a UE may support a full power transmission mode 0 with 2 layers but not with 1 layer. In full power transmission mode 0, it is assumed that each PA has the full power capability. This ensures that a UE with partial or non-coherent precoders can operate according to its power class. Therefore, if the UE is configured with fullpower, it should support both the 1- and 2-layer transmissions with full power.

Observation 1: In full power transmission mode 0, it is assumed that each PA has the full power capability. This ensures that a UE with partial or non-coherent precoders can operate according to its power class.

Proposal 1: For Question 3, a 3TX UE supporting UL full power transmission mode 0 should support full power transmission for both 1- and 2-layer transmissions.

For 2-layer transmission, a UE capable of full power mode 0, shall use two transmit chains where each is equipped with a full-rated power amplifier. Therefore, in such case, a UE can potentially take advantage of its full-rated power amplifiers. For question 2, RAN4 is asking whether a 3TX UE supporting UL full power transmission mode 0 should consider the same maximum power for both 1- and 2-layer transmissions.  Regarding Question 2, despites the capability of a 3TX UE supporting UL full power transmission mode 0 to deliver more power when transmitting 2 layers than when 1 layer, a UE should not transmit more than the power level permitted by its power class.

Observation 2: For 2-layer transmission, a UE capable of full power mode 0, shall use two transmit chains where each is equipped with a full-rated power amplifier.

Proposal 2: Regarding Question 2, despites the capability of a 3TX UE supporting UL full power transmission mode 0 to deliver more power when transmitting 2 layers than when 1 layer, a UE should not transmit more than the power level permitted by its power class.

# Conclusions

In this contribution, we discussed our views with respect to the RAN4 LS. We make the following observations and proposals:

Observation 1: In full power transmission mode 0, it is assumed that each PA has the full power capability. This ensures that a UE with partial or non-coherent precoders can operate according to its power class.

Proposal 1: For Question 3, a 3TX UE supporting UL full power transmission mode 0 should support full power transmission for both 1- and 2-layer transmissions.

Observation 2: For 2-layer transmission, a UE capable of full power mode 0, shall use two transmit chains where each is equipped with a full-rated power amplifier.

Proposal 2: Regarding Question 2, despites the capability of a 3TX UE supporting UL full power transmission mode 0 to deliver more power when transmitting 2 layers than when 1 layer, a UE should not transmit more than the power level permitted by its power class.

# References

- RP-234007, “New WID: NR MIMO Phase 5”, 3GPP TSG RAN Meeting #102, December 2023

- RP-242394, “WID revision: NR MIMO Phase 5”, 3GPP TSG RAN Meeting #105 September 2024

- Chairman’s notes, 3GPP TSG RAN Meeting #118-bis, October 2024

- R4-2417117, “LS on UE RF issues for Rel-19 MIMO enhancement”, 3GPP TSG RAN4 Meeting #112-bis,

- 38.212, “NR: Multiplexing and channel coding”, v18.2.0, March 2024

- 38.213, “NR: Physical layer procedures for control”, v18.4.0, September 2024